| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0136 |
| --- | --- |

| Title: | TE3-4.4.1: Inter prediction based on difference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents inter coding in difference domain (D-domain) at enhancement layer (EL). In the proposed technique, D-domain is basically obtained by subtracting base layer (BL) samples from EL samples. Inter prediction is performed in D-domain. And after obtaining reconstruction samples in D-domain, they are added to collocated samples at BL for the final reconstruction samples. For each CU in inter prediction mode, a flag is signalled to indicate whether the proposed technique is employed or not. The simulation results reportedly show that it achieves 1.8%, 2.4%, 1.5%, 2.0%, 2.8%, and 1.5% BD rate savings on average for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x, LD-P-SNR, respectively, compared with SMuC-0.1.1 anchors. Encoding times are increased by 42.9%-65.4%, and decoding times are increased by 0.5%-5.8%.

# Introduction

In [1], video coding based on difference picture has been introduced. In this contribution, inter coding in D-domain at EL is presented.

# Proposed technique

In the proposed technique, inter coding in D-domain at EL is performed. To obtain the reference picture in the D-domain, the reconstructed BL of the reference picture is up-sampled and then subtracted from the reconstructed EL. When the resolutions of EL and BL are the same, the up-sampling process is omitted. The process of difference reference picture acquisition is depicted in Figure 1.

Figure 1. Acquisition of a difference reference picture

We obtain the motion compensation prediction block in D-domain from difference reference picture. And then, we get reconstruction block in D-domain by adding residuals. And we obtain the final reconstruction block by adding collocated block from BL. Figure 2 shows the reconstruction process of a block to which the proposed technique is applied.

Figure 2. Reconstruction process of proposed technique

In the proposed method, prediction and reconstruction values in D-domain are clipped from [-255,255] into [-128, 127]. By doing it, we can keep the range size the same with the original domain.

# Results

The proposed method has been implemented on SMuC 0.1.1 and simulated under the common test conditions. Error: Reference source not found summarizes the experimental results. The simulation results reportedly show that it achieves 1.8%, 2.4%, 1.5%, 2.0%, 2.8%, and 1.5% BD rate savings on average for RA-2x, RA-1.5x, RA-SNR, LD-P-2x, LD-P-1.5x, LD-P-SNR, respectively, compared with SMuC-0.1.1 anchors. Encoding times are increased by 42.9%-65.4%, and decoding times are increased by 0.5%-5.8%.

Table 1. Experimental results on RA and LP configuration

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.6% | -4.2% | -4.2% |  |  |  | -1.1% | -3.7% | -4.0% |
| Class B | -1.9% | -2.9% | -2.9% | -2.4% | -4.3% | -4.5% | -1.6% | -3.4% | -3.6% |
| Overall (EL+BL) | -1.8% | -3.3% | -3.3% | -2.4% | -4.3% | -4.5% | -1.5% | -3.5% | -3.7% |
| Overall (EL) | -3.4% | -6.3% | -6.4% | -6.2% | -11.1% | -11.4% | -2.9% | -7.1% | -7.6% |
| Enc Time[%] | 165.4% |  |  | 154.1% |  |  | 144.6% |  |  |
| Dec Time[%] | 103.8% |  |  | 100.8% |  |  | 104.6% |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -3.7% | -3.8% |  |  |  | -1.2% | -3.3% | -3.4% |
| Class B | -2.2% | -2.6% | -2.6% | -2.8% | -4.2% | -4.3% | -1.7% | -2.8% | -3.0% |
| Overall (EL+BL) | -2.0% | -2.9% | -3.0% | -2.8% | -4.2% | -4.3% | -1.5% | -3.0% | -3.1% |
| Overall (EL) | -3.7% | -5.6% | -5.7% | -7.0% | -10.2% | -10.9% | -2.9% | -5.6% | -5.9% |
| Enc Time[%] | 161.3% |  |  | 149.0% |  |  | 142.9% |  |  |
| Dec Time[%] | 105.8% |  |  | 100.5% |  |  | 105.5% |  |  |

# References

[1] C. Kim, J. Park, J. Kim, Hendry, S. Park, Y. Jeon, J. Lim, N. Park, B. Jeon, S. Liu, X. Zhang, M. Guo, Z. Chen, T. Chuang, S. Hsiang, C. Chen, C. Tsai, C. Fu, C. Hsu, Y. Huang, S. Lei, “Description of scalable video coding technology proposal by LG Electronics and MediaTek (differential coding mode on)”, JCTVC-K0033, Oct. 2012

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).