| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0138 |
| --- | --- |

| Title: | On VPS extension design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Rusert Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 76 7602660 thomas.rusert@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents proposed modifications to the design of the VPS extension relative to the design in JCTVC-K1007. Two items related to claimed limitations in the VPS extension design in JCTVC-K1007 are addressed.

The first item is related to profile/tier/level signaling. It is proposed to change the number of iterations of the profile/tier/level loop in the VPS extension from vps_max_layers_minus1 to vps_num_op_sets_minus1.

The second item is related to layer dependency signaling. An alternative approach for layer dependency signaling is proposed which reportedly leads to savings of 11/30/56/60 bits for 2/4/8/16 layer configurations with 1 reference layer per layer, and savings of 11/42/92/144 bits for 2/4/8/16 layer configurations with 2 reference layers per layer. It is asserted that the worst case number of bits for dependency signaling is significantly lower than with the approach in JCTVC-K1007. It is also asserted that in cases when many layers are present in the bitstream, the proposed approach can be less bit efficient than the approach in JCTVC-K1007, however these cases are reportedly practically less relevant. It is claimed that the proposed approach of dependency signaling is conceptually similar to the “scalability_mask” approach of signaling scalability dimensions as already part of the VPS extension design in JCTVC-K1007, as well as the “operation_point_set()” signaling in JCTVC-K1003_v13.

# Introduction

Document JCTVC-K1007 includes NAL unit header and parameter set designs for scalable and 3D extensions to HEVC as agreed at the 11th JCT-VC meeting. In this contribution, modifications to the design of the VPS extension relative to design in JCTVC-K1007 are proposed. Specifically, two claimed limitations of the design in JCTVC-K1007 are addressed:

- In the VPS extension syntax in JCTVC-K1007, there is a loop that to the author’s understanding is intended to specify a set of profile/tier/level for operation points that can be extracted from the bitstream. The loop has vps_max_layers_minus1 iterations. It is suggested that the loop has vps_num_op_sets_minus1 iterations, where vps_num_op_sets_minus1 + 1 specifies the number of operation point sets specified in the VPS according to JCTVC-K1003_v13. The suggested change is detailed in Section 3 below.

- The layer dependency signaling in the VPS extension design in JCTVC-K1007 is asserted to be bit inefficient for typical layer configurations. A reportedly more efficient design is proposed in Section 4 below.

# Current VPS and VPS extension syntaxes

For convenience, this section summarizes the VPS syntax according to JCTVC-K1003_v13 and the VPS extension syntax according to JCTVC-K1007.

## Video parameter set RBSP syntax from JCTVC-K1003_v13

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bit_rate_pic_rate_info( 0, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| vps_num_op_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_op_sets_minus1; i++ ) |  |
| operation_point_set( i ) |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Video parameter set extension RBSP syntax from JCTVC-K1007

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

# Profile/tier/level signaling

In the VPS extension syntax in JCTVC-K1007, there is a loop that to the author’s understanding is intended to specify a set of profile/tier/level for operation points that can be extracted from the bitstream. The loop has vps_max_layers_minus1 iterations, where vps_max_layers_minus1 + 1 specifies the number of layers in the bitstream. However the number of layers is not necessarily the same as the number of operation points. It is thus suggested that the loop has vps_num_op_sets_minus1 iterations, where vps_num_op_sets_minus1 + 1 specifies the number of operation point sets specified in the VPS according to JCTVC-K1003_v13. The suggested change is detailed as follows.

## Proposed syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1vps_num_op_sets_minus1; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| … |  |
| } |  |

# Layer dependency signaling

For layer dependency signaling in the VPS extension design in JCTVC-K1007, a 6-bit fixed-length code is used to signal the number of direct reference layers for each layer above the base layer, and another 6-bit fixed length code is used to signal a value of layer_id for identifying each reference layer. That signaling is claimed to be inefficient, in particular in cases with relatively few layers such as 2, 4, 8, or 16, which is asserted to be more typical than cases with many layers such as 64.

A reportedly more compact signaling is proposed below, followed by a comparison of number of bits needed for dependency coding with the current and the proposed approach, as well as a discussion of the performance of the proposed approach.

## Proposed approach

It is proposed to (1) omit explicitly signaling the number of direct reference layers and, (2) instead of signaling each direct reference layer by a value of layer_id, to signal a flag for each potential direct reference layer that indicates whether that layer is a direct reference layer or not. Furthermore, it is assumed that the layers are arranged in a way that a layer with index “i” (the index variable used to loop over the layers) does only have reference layers with index j<I (no reference layers with index j>=i). This is claimed to allow for a more compact representation without adding limitations in the flexibility of the dependency signaling.

### Syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |

### Semantics

direct_dependency_flag[ i ][ j ] equal to 0 indicates that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 indicates that the layer with index j is a direct reference layer for the layer with index i. If not present, direct_dependency_flag[ i ][ j ] is inferred to be equal to 0.

## Comparison of bits needed for dependency coding

The number of bits required for layer dependency coding has been simulated for four different dependency configurations as follows.

- “0-ref”: each layer has 0 reference layers (essentially simulcast of the layers)

- “1-ref”: each layer with index i has min(i, 1) reference layers

- “2-ref”: each layer with index i has min(i, 2) reference layers

- “all-ref”: each layer with index i has i reference layers

Results are given for 1..64 layers and both the approach in JCTVC-K1007 and the proposed approach. Note that for the proposed approach, the number of bits used for dependency coding is independent of the dependency configuration.

The red marked table cells indicate cases where the proposed dependency signaling is more bit efficient than the dependency signaling in JCTVC-K1007.

| number of layers | number of bits for dependency signaling |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | K1007 0-ref | K1007 1-ref | K1007 2-ref | K1007 all-ref | proposed |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 6 | 12 | 12 | 12 | 1 |
| 3 | 12 | 24 | 30 | 30 | 3 |
| 4 | 18 | 36 | 48 | 54 | 6 |
| 5 | 24 | 48 | 66 | 84 | 10 |
| 6 | 30 | 60 | 84 | 120 | 15 |
| 7 | 36 | 72 | 102 | 162 | 21 |
| 8 | 42 | 84 | 120 | 210 | 28 |
| 9 | 48 | 96 | 138 | 264 | 36 |
| 10 | 54 | 108 | 156 | 324 | 45 |
| 11 | 60 | 120 | 174 | 390 | 55 |
| 12 | 66 | 132 | 192 | 462 | 66 |
| 13 | 72 | 144 | 210 | 540 | 78 |
| 14 | 78 | 156 | 228 | 624 | 91 |
| 15 | 84 | 168 | 246 | 714 | 105 |
| 16 | 90 | 180 | 264 | 810 | 120 |
| 17 | 96 | 192 | 282 | 912 | 136 |
| 18 | 102 | 204 | 300 | 1020 | 153 |
| 19 | 108 | 216 | 318 | 1134 | 171 |
| 20 | 114 | 228 | 336 | 1254 | 190 |
| 21 | 120 | 240 | 354 | 1380 | 210 |
| 22 | 126 | 252 | 372 | 1512 | 231 |
| 23 | 132 | 264 | 390 | 1650 | 253 |
| 24 | 138 | 276 | 408 | 1794 | 276 |
| 25 | 144 | 288 | 426 | 1944 | 300 |
| 26 | 150 | 300 | 444 | 2100 | 325 |
| 27 | 156 | 312 | 462 | 2262 | 351 |
| 28 | 162 | 324 | 480 | 2430 | 378 |
| 29 | 168 | 336 | 498 | 2604 | 406 |
| 30 | 174 | 348 | 516 | 2784 | 435 |
| 31 | 180 | 360 | 534 | 2970 | 465 |
| 32 | 186 | 372 | 552 | 3162 | 496 |
| 33 | 192 | 384 | 570 | 3360 | 528 |
| 34 | 198 | 396 | 588 | 3564 | 561 |
| 35 | 204 | 408 | 606 | 3774 | 595 |
| 36 | 210 | 420 | 624 | 3990 | 630 |
| 37 | 216 | 432 | 642 | 4212 | 666 |
| 38 | 222 | 444 | 660 | 4440 | 703 |
| 39 | 228 | 456 | 678 | 4674 | 741 |
| 40 | 234 | 468 | 696 | 4914 | 780 |
| 41 | 240 | 480 | 714 | 5160 | 820 |
| 42 | 246 | 492 | 732 | 5412 | 861 |
| 43 | 252 | 504 | 750 | 5670 | 903 |
| 44 | 258 | 516 | 768 | 5934 | 946 |
| 45 | 264 | 528 | 786 | 6204 | 990 |
| 46 | 270 | 540 | 804 | 6480 | 1035 |
| 47 | 276 | 552 | 822 | 6762 | 1081 |
| 48 | 282 | 564 | 840 | 7050 | 1128 |
| 49 | 288 | 576 | 858 | 7344 | 1176 |
| 50 | 294 | 588 | 876 | 7644 | 1225 |
| 51 | 300 | 600 | 894 | 7950 | 1275 |
| 52 | 306 | 612 | 912 | 8262 | 1326 |
| 53 | 312 | 624 | 930 | 8580 | 1378 |
| 54 | 318 | 636 | 948 | 8904 | 1431 |
| 55 | 324 | 648 | 966 | 9234 | 1485 |
| 56 | 330 | 660 | 984 | 9570 | 1540 |
| 57 | 336 | 672 | 1002 | 9912 | 1596 |
| 58 | 342 | 684 | 1020 | 10260 | 1653 |
| 59 | 348 | 696 | 1038 | 10614 | 1711 |
| 60 | 354 | 708 | 1056 | 10974 | 1770 |
| 61 | 360 | 720 | 1074 | 11340 | 1830 |
| 62 | 366 | 732 | 1092 | 11712 | 1891 |
| 63 | 372 | 744 | 1110 | 12090 | 1953 |
| 64 | 378 | 756 | 1128 | 12474 | 2016 |

## Discussion of performance of proposed approach

As depicted in the table above, in cases with a moderate number of layers (e.g. up to 23 layers when each layer has one reference layer), the proposed dependency signaling method is more bit efficient than the method in JCTVC-K1007. For instance, for a configuration with 1 reference layer per layer, the number of bits saved in case of a bitstream with 2/4/8/16 layers is 11/30/56/60. For a configuration with 2 reference layers, the number of bits saved in case of a bitstream with 2/4/8/16 layers is 11/42/92/144.

As can be further seen from the table, the worst case number of bits, in the “all-ref” configuration, is significantly lower than the worst case number of bits for the approach in JCTVC-K1007. If many layers are present in the bitstream, the proposed approach can be less bit efficient than the current approach. However, configurations with many layers are believed to be practically less relevant. Furthermore, with the proposed approach the number of syntax elements used for dependency signaling is not dependent on the actual layer configuration, which simplifies the parsing operation compared to the approach in JCTVC-K1007.

It is finally noted that the proposed approach of signaling dependencies using flags is conceptually similar to the “scalability_mask” approach of signaling scalability dimensions as already part of the VPS extension design in JCTVC-K1007, as well as the “operation_point_set()” signaling in JCTVC-K1003_v13. The author believes that it thus harmonizes the VPS extension design.

# Conclusion

It is recommended to adopt the proposed changes for profile/tier/level signaling and layer dependency signaling into the VPS extension design.

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).