| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 |  |  | Document: JCTVC-L0139 |
| --- | --- | --- | --- |
| Title: | Non-TE 5: Motion assignment for Intra/Intra BL at enhancement layer |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-8893-4332 chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

# Abstract

In this contribution, motion assignment for Intra/Intra BL is proposed. If a neighbouring PU at the enhancement layer is intra or intra BL coded, corresponding PU at the base layer, which might be inter coded, should be considered as a spatial merge candidate. Motion of corresponding PU at base layer is assigned to Intra/Intra BL PU at enhancement layer. Simulation results reportedly show 0.0%, 0.2%,0.0%,0.0%,0.1%,0.0%, BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x and LDP-SNR, respectively, compared with SMuC-0.1.1(EL only, SVC_MVP On, using uncompressed BL MV) anchors.

# Introduction

The motion information of a neighbouring PU is used as a spatial merge candidate. When the neighbouring PU is intra or intra BL coded, it cannot become a spatial merging candidate as there is no motion information. However, an intra or intra BL coded neighboring PU at the enhancement layer might have a corresponding inter coded PU at the base layer.

# Proposed method

This proposal suggests that when a neighbouring PU at the enhancement layer is intra or intra BL coded, corresponding PU at the base layer, which might be inter coded, should be considered as a spatial merging candidate. The possible number of candidates at the base layer obtained in this way is limited to 2. For an intra/intra BL coded neighbouring PU, the motion information of the corresponding PU at the lower layer is used.

In Table 1, the left column shows the merge candidate list in HSVC 0.1.1, and the right column shows the proposed candidate list for EL

Table 1. Proposed merge candidate list

| SMuC-0.1.1 | Proposed method |
| --- | --- |
| - Collocated PU at base layer - Left PU - Above PU - Above right PU - If(N<4) left bottom PU - If(N<4) above left PU - Col. - If(N<Max.) Combined bi-pred. candidates - If(N < Max.) Zero MV candidates | - Collocated PU at base layer - Left PU, if(!Left PU) Collocated PU at base layer - Above PU, if(!Above PU) Collocated PU at base layer - Above right PU, if(!Above right PU&&Nb<2) Collocated PU at base layer - If(N<4), left bottom PU, if(!left bottom PU &&Nb<2) Collocated PU at base layer - If(N<4), above left PU, if(!above left PU, &&Nb<2) Collocated PU at base layer - Col. - if(N < Max.), Combined bi-pred. candidates - if(N < Max.), Zero MV candidates |

# Experimental results

In this contribution, uncompressed motion data of the BL is used for the EL. After decoding all the pictures of all layers in an access unit, motion data compression is performed all together. The collocated base layer candidates are placed first in the merging candidate list. Table 2 shows the results of proposed method compared with SMuC-0.1.1anchors. Simulation results reportedly show 4.4%, 6.4%, 5.0%,2.3%,2.9%,3.6%,BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x and LDP-SNR, respectively.

Table 3 shows the results of proposed method compared with SMuC-0.1.1(EL, SVC_MVP On, AU compression) anchors. Simulation results reportedly show 0.0%, 0.2%,0.0%,0.0%,0.1%,0.0%, BD-rate savings on average for RA-2x, RA-1.5x, RA-SNR, LDP-2x, LDP-1.5x and LDP-SNR, respectively.

Table 2. Results of the proposed method (anchor: SMuC-0.1.1)

Table 3. Results of the proposed method (anchor: SMuC-0.1.1(SVC_MVP on, using uncompressed BL MV))

# Patent rights declaration(s)

(LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).