| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0140 |
| --- | --- |

| Title: | Non-TE3 : Modifications on inter-layer texture, DC, planar predictions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution presents three modifications on the current SHVC model. Firstly, we add boundary process for a block coded in inter-layer texture prediction (ILTP) mode. Secondly, intra DC prediction is replaced with ILTP plus boundary process which is stronger than the first one. Thirdly, intra planar prediction is replaced with ILTP plus modified planar prediction in difference domain. For all intra (AI) 2x and 1.5x, the first modification reportedly shows 0.3% and 0.0% BD rate savings on average, respectively, compared with SMuC-0.1.1 anchors. The second modification also shows the 0.3% and 0.0% BD rate savings on average. The third modification reportedly shows 0.4% and 0.1% BD rate savings on average. Encoding time for each modification doesn’t roughly change, and decoding time is increased by 1-3%. When all the three modifications are applied together, it reportedly shows 0.7% and 0.2% BD rate savings on average. For this case, encoding time doesn’t roughly change, and decoding time is increased by 3-4%.

# Introduction

In this contribution, we propose three modifications on the current SHVC model. From section 2.1 to section 2.3, they are described in detail.

# Proposed technique

## Modified ILTP

We propose to apply boundary process to ILTP block. Boundary process is performed using differences between reference samples of enhancement layer (EL) and base layer (BL). Figure 1 shows the flow chart of the proposed technique.

Figure 1. Flow chart of the proposed technique in section 2.1 and 2.2

p(x,y)=b(0,0)+[d(−1,0)+d(0,−1)+offset>>3],   if x=0&&y=0b(0,y)+[d(−1,y)+offset>>3],   else if x=0b(x,0)+[d(x,−1)+offset>>3],   else if y=0                   (1)

Equation (1) represents the boundary process. In the equation, p represents the final prediction values, b represents the values of ILTP block, and d represents the difference reference sample values. And offset values are set equal to 4. We apply the boundary process only to luma component.

## Modified intra DC prediction

We propose to replace intra DC prediction block with the prediction block from section 2.1 with slightly modified boundary process as below.

p(x,y)=b(0,0)+[d(−1,0)+d(0,−1)+offset>>1],   if x=0&&y=0b(0,y)+[d(−1,y)+offset>>1],   else if x=0b(x,0)+[d(x,−1)+offset>>1],   else if y=0                   (2)

In equation (2), offset values are set equal to 1. According to setting shift bits equal to 1, the boundary process becomes stronger than that of section 2.1.

## Modified planar prediction

We propose to replace intra planar prediction block with ILTP block plus modified planar prediction with difference reference samples. Figure 2 shows the flow chart of the proposed technique.

Figure 2. Flow chart of the proposed technique in section 2.3

In figure 3, the modified planar prediction with difference reference samples is depicted. Unlike the previous planar prediction, right-top and bottom-left reference samples are not employed. Instead, sample values at the bottom line are set equal to zero during vertical interpolation, and sample values at the rightmost line are set equal to zero during horizontal interpolation.

Figure 3. Modified planar prediction with difference reference samples

p(x,y)=b(x,y)+dp(x,y)                                                          (3)

dp(x,y)=[(N−1−x)∗d(−1,y)+(N−1−y)∗d(x,−1)+N]>>(log2N+1)                (4)

Equations (3) and (4) show how the prediction block is calculated in the proposed method. In equations, p represents the final prediction values, b represents the values of ILTP block, d represents the difference reference sample values, dp represents the modified prediction sample values with difference reference samples, and Nrepresents the current block size.

# Results

The proposed method is implemented on SMuC 0.1.1 and simulated under the common test conditions. In section 3.1, we provide the performance for each proposed technique, and in section 3.2, we provide the performance of all possible combinations among the proposed techniques.

## Performance for each proposed technique

Table 1 summarizes the results for modified ILTP, table 2 summarizes the results for modified DC prediction, and table 3 summarizes the results for modified planar prediction. For AI-2x and AI-1.5x, modified ILTP and DC prediction reportedly shows 0.3% and 0.0% BD rate savings on average, respectively, compared with SMuC 0.1.1. Modified planar prediction reportedly shows 0.4% and 0.1% BD rate savings on average.

Table 1. Experimental results on modified ILTP

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.1% | -0.2% |  |  |  |
| Class B | -0.2% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | -0.3% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | -0.5% | -0.2% | -0.2% | -0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101.3% |  |  | 101.5% |  |  |
| Dec Time[%] | 101.3% |  |  | 102.2% |  |  |

Table 2. Experimental results on modified DC prediction

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -2.0% | -2.5% |  |  |  |
| Class B | -0.3% | -1.0% | -1.1% | 0.0% | -0.8% | -0.9% |
| Overall (EL+BL) | -0.3% | -1.3% | -1.5% | 0.0% | -0.8% | -0.9% |
| Overall (EL) | -0.4% | -2.5% | -2.9% | 0.0% | -2.1% | -2.2% |
| Enc Time[%] | 97.9% |  |  | 97.3% |  |  |
| Dec Time[%] | 101.7% |  |  | 101.4% |  |  |

Table 3. Experimental results on modified planar prediction

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.8% | -1.0% |  |  |  |
| Class B | -0.4% | -0.5% | -0.5% | -0.1% | -0.5% | -0.6% |
| Overall (EL+BL) | -0.4% | -0.6% | -0.6% | -0.1% | -0.5% | -0.6% |
| Overall (EL) | -0.6% | -1.1% | -1.2% | -0.4% | -1.3% | -1.4% |
| Enc Time[%] | 98.7% |  |  | 97.8% |  |  |
| Dec Time[%] | 103.4% |  |  | 102.5% |  |  |

## Performance of all possible combinations among proposed techniques

In this section, we provide the performance of all possible combinations among the proposed techniques. Table 4 summarizes the results for combination of modified ILTP and DC prediction, table 5 summarizes the results for combination of modified ILTP and planar prediction, table 6 summarizes the results for combination of modified DC and planar prediction, and table 7 summarizes the results for combination of modified ILTP, DC and planar prediction. Table 7 shows that combination of all three modifications reportedly achieves 0.7% and 0.2% BD rate savings on average.

Table 4. Experimental results on modified ILTP and DC prediction

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -1.8% | -2.2% |  |  |  |
| Class B | -0.4% | -1.0% | -1.1% | 0.0% | -0.8% | -0.9% |
| Overall (EL+BL) | -0.4% | -1.3% | -1.5% | 0.0% | -0.8% | -0.9% |
| Overall (EL) | -0.6% | -2.3% | -2.7% | -0.1% | -1.7% | -1.9% |
| Enc Time[%] | 99.2% |  |  | 98.8% |  |  |
| Dec Time[%] | 103.2% |  |  | 102.8% |  |  |

Table 5. Experimental results on modified ILTP and planar prediction

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.9% | -1.1% |  |  |  |
| Class B | -0.6% | -0.6% | -0.7% | -0.2% | -0.5% | -0.6% |
| Overall (EL+BL) | -0.6% | -0.7% | -0.8% | -0.2% | -0.5% | -0.6% |
| Overall (EL) | -1.0% | -1.2% | -1.5% | -0.5% | -1.2% | -1.3% |
| Enc Time[%] | 99.0% |  |  | 98.2% |  |  |
| Dec Time[%] | 104.1% |  |  | 103.4% |  |  |

Table 6. Experimental results on modified DC and planar prediction

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.7% | -2.1% |  |  |  |
| Class B | -0.6% | -0.7% | -0.8% | -0.2% | -0.6% | -0.7% |
| Overall (EL+BL) | -0.6% | -1.0% | -1.2% | -0.2% | -0.6% | -0.7% |
| Overall (EL) | -0.9% | -2.0% | -2.3% | -0.5% | -1.5% | -1.7% |
| Enc Time[%] | 97.4% |  |  | 96.5% |  |  |
| Dec Time[%] | 103.4% |  |  | 102.5% |  |  |

Table 7. Experimental results on modified ILTP, DC and planar predictions

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -1.7% | -2.1% |  |  |  |
| Class B | -0.7% | -0.8% | -0.9% | -0.2% | -0.6% | -0.7% |
| Overall (EL+BL) | -0.7% | -1.1% | -1.3% | -0.2% | -0.6% | -0.7% |
| Overall (EL) | -1.1% | -2.0% | -2.4% | -0.5% | -1.4% | -1.6% |
| Enc Time[%] | 97.2% |  |  | 96.5% |  |  |
| Dec Time[%] | 103.9% |  |  | 103.1% |  |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).