| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0141 |
| --- | --- |

| Title: | TE3 : crosscheck report of generalized residual prediction (TE3-4.6.2.1) from Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joonyoung Park Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of the proposal JCTVC-L0078, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction” [1] submitted by Qualcomm. The proponents provide their source codes with the results. The crosschecker checked source codes and provided results to the proponents. It succeeded and produced as same results as those the proponent provided. The spreadsheets containing the detailed data are accompanied.

# Introduction

In [1], generalized residual prediction is provided. In addition, sigaling of GRP weighting mode, 4-tap up-sampling filter for GRP, fast GRP mode selection is proposed. the proponent had provided source codes for five test cases (Test 1-5).

Test 1 : GRP with two weighting modes (0, 0.5) is tested.

Test 2 : GRP with three weighting modes (0, 0.5, 1) and fast GRP mode selection is tested.

Test 3 : GRP with three weighting modes (0, 0.5, 1), fast GRP mode selection, 4 tap up-sampling filter is tested.

Test 4 : GRP with two weighting modes (0, 0.5) and additional ME is tested.

Test 5 : GRP with three weighting modes (0, 0.5, 1), fast GRP mode selection, and additional ME is tested.

Among five cases, we cross-verified Test 1, 3 and 4. In JCTVC-L0063, Test 2 and 5 are cross-verified by ETRI.

# Results

## Simulation environment

The encoding and decoding environment for cross-checking the method is as following:

OS: Red Hat Enterprise Linux Server release 5.3 (Tikanga) 2.6.18-128.el5

CPU: Intel Xeon Eight Core X5570

Memory : 28/96GB DDR3 UDIMM Memory

## Simulation conditions and experimental results

The simulation follows the test conditions provided by TE3 group [2] for RA and LD-P cases. There was no mismatch between reconstructed sequences at the encoder side and decoded sequences at the decoder. The results are same as the proponent provided. The spreadsheet contains the detailed data of the simulation is accompanied.

Table 1. Results for test 1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -3.5% | -3.5% |  |  |  | -1.2% | -3.3% | -3.5% |
| Class B | -1.5% | -2.7% | -2.8% | -2.3% | -3.7% | -4.0% | -1.9% | -3.8% | -4.1% |
| Overall (EL+BL) | -1.5% | -2.9% | -3.0% | -2.3% | -3.7% | -4.0% | -1.7% | -3.6% | -3.9% |
| Overall (EL) | -2.5% | -5.2% | -5.4% | -5.2% | -8.4% | -8.9% | -3.1% | -6.6% | -7.1% |
| Enc Time[%] | 120.0% |  |  | 115.4% |  |  | 114.1% |  |  |
| Dec Time[%] | 108.3% |  |  | 109.7% |  |  | 111.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.3% | -3.1% | -2.5% |  |  |  | -2.5% | -2.9% | -2.4% |
| Class B | -2.4% | -1.8% | -1.3% | -3.4% | -2.6% | -1.5% | -3.5% | -3.2% | -2.7% |
| Overall (EL+BL) | -2.4% | -2.2% | -1.7% | -3.4% | -2.6% | -1.5% | -3.2% | -3.1% | -2.6% |
| Overall (EL) | -3.9% | -3.7% | -3.0% | -7.2% | -5.2% | -3.1% | -5.3% | -5.0% | -4.3% |
| Enc Time[%] | 122.1% |  |  | 116.7% |  |  | 116.3% |  |  |
| Dec Time[%] | 109.2% |  |  | 107.4% |  |  | 110.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2. Results for test 3

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.0% | -4.0% | -4.1% |  |  |  | -3.3% | -4.9% | -5.5% |
| Class B | -2.4% | -3.1% | -3.4% | -3.0% | -4.1% | -4.6% | -2.6% | -4.1% | -4.6% |
| Overall (EL+BL) | -2.3% | -3.4% | -3.6% | -3.0% | -4.1% | -4.6% | -2.8% | -4.3% | -4.8% |
| Overall (EL) | -4.0% | -6.3% | -6.7% | -7.0% | -9.6% | -10.8% | -5.3% | -8.2% | -9.1% |
| Enc Time[%] | 119.0% |  |  | 111.1% |  |  | 113.5% |  |  |
| Dec Time[%] | 125.7% |  |  | 127.3% |  |  | 139.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.1% | -4.2% | -3.8% |  |  |  | -5.5% | -4.4% | -4.1% |
| Class B | -3.5% | -2.7% | -2.3% | -4.5% | -3.6% | -3.0% | -4.5% | -3.8% | -3.6% |
| Overall (EL+BL) | -3.4% | -3.1% | -2.7% | -4.5% | -3.6% | -3.0% | -4.8% | -4.0% | -3.7% |
| Overall (EL) | -5.8% | -5.5% | -5.1% | -10.2% | -7.7% | -7.2% | -8.4% | -6.7% | -6.5% |
| Enc Time[%] | 124.4% |  |  | 114.0% |  |  | 118.4% |  |  |
| Dec Time[%] | 129.2% |  |  | 127.5% |  |  | 142.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3. Results for test 4

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.9% | -4.4% | -4.3% |  |  |  | -1.9% | -4.3% | -4.4% |
| Class B | -2.0% | -3.3% | -3.3% | -2.7% | -4.1% | -4.4% | -2.6% | -4.6% | -4.9% |
| Overall (EL+BL) | -2.0% | -3.6% | -3.6% | -2.7% | -4.1% | -4.4% | -2.4% | -4.5% | -4.8% |
| Overall (EL) | -3.4% | -6.3% | -6.4% | -6.1% | -9.0% | -9.7% | -4.3% | -8.0% | -8.5% |
| Enc Time[%] | 169.2% |  |  | 158.6% |  |  | 147.1% |  |  |
| Dec Time[%] | 107.9% |  |  | 109.7% |  |  | 113.7% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3.1% | -4.3% | -3.7% |  |  |  | -3.5% | -4.0% | -3.3% |
| Class B | -3.0% | -2.4% | -1.8% | -4.1% | -3.0% | -2.0% | -4.5% | -4.0% | -3.5% |
| Overall (EL+BL) | -3.0% | -2.9% | -2.3% | -4.1% | -3.0% | -2.0% | -4.2% | -4.0% | -3.5% |
| Overall (EL) | -4.9% | -4.9% | -4.1% | -8.6% | -6.1% | -4.3% | -6.8% | -6.3% | -5.6% |
| Enc Time[%] | 167.1% |  |  | 156.2% |  |  | 146.8% |  |  |
| Dec Time[%] | 110.2% |  |  | 108.8% |  |  | 112.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# References

[1] X. Li, J. Chen, K. Rapaka, M. Karczewicz, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0078

[2] X. Li, E. Francois, P. Lai, D. Kwon, A. Saxena, “Description of Tool Experiment B3: Combined Prediction in SHVC”, JCTVC-K1103