| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 16–23 Jan. 2013 | Document: JCTVC-L0148 |
| --- | --- |

| Title: | CE1: Test 3 - Square transforms for Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents an approach to transforms for the extended chroma formats that avoids introduction of rectangular transforms for the chroma channel in the 4:2:2 case.  Instead of using rectangular transform, the block for each chroma channel is split into two square blocks, each of which uses a square transform.

# Problem statement

At the 11th JCT-VC meeting in Shanghai, a contribution presented HM software modified to introduce square transforms in specific cases for the 4:2:2 and 4:4:4 chroma formats [1].  These cases were where the chroma block had a side dimension (width or height) of 32, in these cases the chroma block was coded using multiple 16x16 transforms, thus reusing existing 16x16 transform logic already present for chroma processing.

Interest was expressed in testing an additional case, whereby the chroma blocks are always divided into two square blocks for the 4:2:2 chroma format and thus this case was added to a core experiment description [2].

The available transforms for the HM-8.0-dev-ahg7 branch for extended chroma formats are listed in Table 1.

Table 1. Transform sizes for HM-8.0 extended chroma formats K0181 implementation.

| Luma | Chroma |  |  |
| --- | --- | --- | --- |
|  | 4:2:0 | 4:2:2 | 4:4:4 |
| 4x4 | 4x4 | 4x8 | 4x4 |
| 8x8 | 4x4 | 4x8 | 8x8 |
| 16x16 | 8x8 | 8x16 | 16x16 |
| 32x32 | 16x16 | 16x32 | 32x32 |

Table 1 will therefore require introducing 32-point transforms into the chroma processing path. Especially for the 4:4:4 chroma format, it is asserted that the required 32x32 transform occupies significant area for a hardware design.

# Description of the proposed change

Square transforms are introduced for the 4:2:2 chroma format case as shown in Table 2.

Table 2. Transform sizes for HM-8.0 extended chroma formats.

| Luma | Chroma |  |  |
| --- | --- | --- | --- |
|  | 4:2:0 | 4:2:2 | 4:4:4 |
| 4x4 | 4x4 | two 4x4 | 4x4 |
| 8x8 | 4x4 | two 4x4 | 8x8 |
| 16x16 | 8x8 | two 8x8 | 16x16 |
| 32x32 | 16x16 | two 16x16 | 32x32 |

When the 4:2:2 chroma format is in use, for the chroma block of a given chroma channel, one residual_coding() syntax table is required for each of the two chroma transforms. Thus, a separate coded block flag is also required for each of the two chroma transforms.  This is repeated for both chroma channels.

Note the following:

- Intra-prediction reconstruction is performed over the rectangular chroma block (i.e.: the upper transform residual is not used to contribute to reference samples for the lower block).

- Deblocking behaviour is not modified in this proposal

- For 4x8 chroma blocks, the upper and lower 4x4 blocks have the same value for each transform skip flag.

# Experiment Results

Results are presented in Table 3 and Table 4 below. Simulations were performed under the common conditions [3]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Note that the presented results are with transform skip disabled. Results with transform skip enabled are introduced in ‘r1’.

Context initialization values were not updated in this simulation.

Table 3. Results for CE1 Test 3 (Transform skip disabled).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.9% | 3.0% | 4.6% | 0.7% | 1.8% | 2.5% |
| Overall | 0.4% | 0.9% | 1.4% | 0.3% | 0.6% | 0.8% |
|  | 0.3% | 0.9% | 1.4% | 0.3% | 0.6% | 0.8% |
| Enc Time[%] | 89% |  |  | 89% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.2% | 0.3% | 0.2% | 0.5% | 0.3% | 0.6% |
| YCbCr 4:4:4 | -0.1% | -0.1% | -0.3% | 0.0% | 0.1% | 0.1% |
| YCbCr 4:2:2 | 1.0% | 2.4% | 3.3% | 1.1% | 1.6% | 2.4% |
| Overall | 0.4% | 0.8% | 1.0% | 0.5% | 0.6% | 1.0% |
|  | 0.4% | 0.8% | 1.0% | 0.5% | 0.3% | 0.9% |
| Enc Time[%] | 94% |  |  | 93% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.3% | 0.4% | 0.4% | 0.6% | 0.4% | 0.8% |
| YCbCr 4:4:4 | -0.2% | -0.2% | -0.5% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 1.0% | 0.6% | 1.0% | 1.0% | 1.0% | 1.4% |
| Overall | 0.4% | 0.3% | 0.3% | 0.5% | 0.5% | 0.7% |
|  | 0.4% | 0.1% | 0.3% | 0.5% | 0.0% | 0.7% |
| Enc Time[%] | 95% |  |  | 95% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Table 4. Results for CE1 Test 3 (Transform skip enabled).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.8% | 3.1% | 4.6% | 0.6% | 2.0% | 2.6% |
| Overall | 0.3% | 1.0% | 1.5% | 0.2% | 0.6% | 0.8% |
|  | 0.3% | 0.9% | 1.5% | 0.2% | 0.7% | 0.9% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.8% | 2.9% | 3.8% | 0.5% | 2.3% | 3.0% |
| Overall | 0.3% | 0.9% | 1.2% | 0.2% | 0.7% | 0.9% |
|  | 0.3% | 0.9% | 1.2% | 0.2% | 0.5% | 0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.8% | 1.0% | 1.3% | 0.5% | 1.6% | 1.6% |
| Overall | 0.2% | 0.3% | 0.4% | 0.1% | 0.5% | 0.5% |
|  | 0.2% | 0.3% | 0.4% | 0.1% | 0.2% | 0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Working Draft changes

See file included in archive.

# Conclusion

This contribution presents an approach to the chroma transforms for extended chroma formats that only uses square transforms for the 4:2:2 chroma format.

# References

- C. Rosewarne, M. Maeda, “AHG7: Transforms for extended chroma formats”, Joint Collaborative Team on Video Coding, JCTVC-K0171, October 2012.

- J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, Joint Collaborative Team on Video Coding, JCTVC-K1121, October 2012.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-K1006, Shanghai, October 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).