| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 16–23 Jan. 2013 | Document: JCTVC-L0149 |
| --- | --- |

| Title: | Non-CE1: Test 3 - Transform skip for chroma in 4:2:2 Range Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents an approach for handling the transform skip functionality in the 4:2:2 chroma format for the CE1 Test 3 (square transform) case.  Presently, a transform skip flag is encoded for each 4x4 square block within a 4x8 chroma region, however the encoder only tests cases where each transform skip flag has an identical value. Thus, when both blocks are present (i.e. CBF flag is one for each 4x4 block), a redundant transform skip flag is encoded for the second block.

# Problem statement

At the 11th JCT-VC meeting in Shanghai, a contribution presented HM software modified to support the 4:2:2 and 4:4:4 chroma formats [1].  The software supports chroma channels in the 4:2:2 chroma format using non-square transforms (4x8, 8x16 and 16x32 sizes are supported). A core experiment was established regarding the chroma transforms for 4:2:2 that included a case where only square transforms are used [2]. This contribution presents approaches to the transform skip functionality for this [3].

Presently, a transform skip flag is encoded for each 4x4 square block within a 4x8 chroma region, however the encoder only tests cases where each transform skip flag has an identical value. Thus, when both blocks are present (i.e. CBF flag is one for each 4x4 block), a redundant transform skip flag is encoded for the second block.

# Description of the proposed change

In the case where both 4x4 transforms in the 4x8 chroma region are present (i.e. coded block flag is set to one for each transform), transform skip is not signaled for the lower 4x4 transform, instead the flag that was signalled for the upper 4x4 transform is used, as shown in Figure 1. Transforms with explicitly signalled transform skip flags are indicated with a shaded box in the upper left corner. Transforms relying on the above transform skip flag value are indicated with a clear box in the upper left corner.

Figure 1. Transform skip for square transform case of 4:2:2.

# Experiment Results

Results are presented in Table 1 below. Simulations were performed under the common conditions [4]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Context initialization values were not updated in this simulation.

Table 1. Results for 4:2:2 chroma format in HM-9.0.1-ahg7-r3055 with square transforms and redundant transform skip flags removed.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.8% | 3.1% | 4.6% | 0.6% | 2.0% | 2.6% |
| Overall | 0.3% | 1.0% | 1.5% | 0.2% | 0.6% | 0.8% |
|  | 0.3% | 0.9% | 1.4% | 0.2% | 0.7% | 0.8% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.8% | 2.9% | 3.7% | 0.5% | 2.2% | 2.9% |
| Overall | 0.3% | 0.9% | 1.2% | 0.2% | 0.7% | 0.9% |
|  | 0.3% | 0.9% | 1.2% | 0.2% | 0.5% | 0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.8% | 1.0% | 1.4% | 0.5% | 1.5% | 1.5% |
| Overall | 0.2% | 0.3% | 0.4% | 0.1% | 0.5% | 0.5% |
|  | 0.2% | 0.3% | 0.4% | 0.1% | 0.2% | 0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Working Draft changes

See file included in archive.

# Conclusion

This contribution presents an approach to the transform skip flag for the 4:2:2 chroma format that removes signalling of a redundant transform skip flag in the bitstream when two 4x4 chroma blocks are present.

# References

- K. Sharman, N. Saunders, J. Gamei, “AHG7: Options present in Extended Chroma Format model”, Joint Collaborative Team on Video Coding, JCTVC-K0181, October 2012.

- J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, Joint Collaborative Team on Video Coding, JCTVC-K1121, October 2012.

- C. Lan, J. Xu, G. Sullivan, F. Wu, “Intra transform skipping”, Joint Collaborative Team on Video Coding, JCTVC-I0408, April 2012.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-K1006, Shanghai, July 2012.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).