| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 16–23 Jan. 2013 | Document: JCTVC-L0150 |
| --- | --- |

| Title: | Non-CE1: Test 3 - Square transform deblocking for 4:2:2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V. Kolesnikov 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 [chris.rosewarne,vladimir.kolesnikov]@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution introduces deblocking to the boundaries between the square transforms in chroma for CE1 Test 3 when the 4:2:2 chroma format is in use.

# Problem statement

At the 11th JCT-VC meeting in Shanghai, a core experiment for selecting the transform shape for the 4:2:2 chroma format was established [1].

When square transforms are used, additional transform boundaries are created which are not deblocked in the present design.  Transform boundaries without deblocking could result in reduced subjective performance. Transform boundaries without deblocking is also inconsistent with the design of 4:2:0, where deblocking is always applied on transform boundaries.

# Description of the proposed change

For the 4:2:2 chroma format, deblocking is introduced for the boundaries between the square transforms used in chroma (see Figure 1). The light blue lines are already deblocked and the dark blue (dashed) lines are the additional chroma boundaries that are deblocked as a result of this proposal.

The conditions under which chroma deblocking occur are not altered with this proposal (i.e. intra only boundaries).

Figure 1. Deblocking of square transforms in CE1 Test 3.

# Experiment Results

Results are presented in Table 1d Table 2 and Table 3 below. Simulations were performed under the common conditions [2]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Also presented here are the results of implementing this proposal on top of the joint proposal JCTVC-L0351 method 2 by Qualcomm and the BBC [3].

Context initialization values were not updated in this simulation.

Table 1. Results for deblocking in CE1 Test 3 compared with AHG7 Anchor (CE1 Test 1).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:2:2 | 0.8% | 2.9% | 4.3% | 0.6% | 2.0% | 2.5% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 93% |  |  | 95% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:2:2 | 1.1% | 2.7% | 3.6% | 0.8% | 2.2% | 2.8% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 85% |  |  | 85% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:2:2 | 1.3% | 1.0% | 1.2% | 0.8% | 1.6% | 1.6% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 84% |  |  | 84% |  |  |

Table 2. Results for deblocking in CE1 Test 3 compared with Canon’s CE1 Test 3 result (TS enabled).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:4:4 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| YCbCr 4:2:2 | 0.00% | -0.21% | -0.29% | 0.00% | -0.03% | -0.17% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 93% |  |  | 95% |  |  |

Table 3. Results for L0150 implemented with L0351 method 2 (and L0351 method 2 used as the anchor).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.00% | -0.18% | -0.25% | 0.00% | -0.02% | -0.15% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.00% | -0.19% | -0.19% | 0.00% | -0.04% | -0.11% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.01% | -0.11% | -0.10% | 0.01% | -0.03% | -0.07% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

- Working Draft changes

See file included in archive.

# Conclusion

This contribution presents an approach for deblocking of the boundaries between square transforms resulting from splitting the rectangular chroma regions in the 4:2:2 chroma format into square regions and using a square transform for each region.

# References

- J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, Joint Collaborative Team on Video Coding, JCTVC-K1121, October 2012.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-K1006, Shanghai, October, 2012.

- J. Sole, R. Joshi, M. Karczewicz, A. Gabriellini, M. Mrak, “Non-CE1: Square transform blocks for 4:2:2”, JCTVC-L0351, 12th JCT-VC Meeting, Geneva, CH, Jan 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).