#### Changes relative to K0383_CE1_Test3

#### 7.3.8.6	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPcmUnits = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : 0 [Ed. (TK) Does this create an infinite loop when pbOffset = 0?] |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| if ( ChromaArrayType != 0 ) { |  |
| if ( ChromaArrayType = = 3 ) { |  |
| for( j = 0; j <= pbOffset; j = j + pbOffset ) |  |
| for( i = 0; i <= pbOffset; i = i + pbOffset ) { |  |
| intra_chroma_pred_mode [ x0 + i ][ y0+ j ] | ae(v) |
| } |  |
| } else |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.3.8.8  Transform tree syntax

| transform_tree( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx, chroma32x32 ) { | Descriptor |
| --- | --- |
| if( log2TrafoSize <= ( Log2MaxTrafoSize + 1 ) && log2TrafoSize > Log2MinTrafoSize && !chroma32x32 && trafoDepth < MaxTrafoDepth && !(IntraSplitFlag && trafoDepth = = 0) ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize > 2 && ChromaArrayType!=0 && (!chroma32x32 ¦¦ blkIdx == 0 ) ) { |  |
| for( tIdx = 0; tIdx < TrafoCrCbCnt; tIdx++ ) |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 + tIdx << log2TrafoSize ][ trafoDepth ] | ae(v) |
| for( tIdx = 0; tIdx < TrafoCrCbCnt; tIdx++ ) |  |
| if( ( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 + tIdx << log2TrafoSize ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ¦¦ log2TrafoSize == Log2MaxTrafoSize + 1 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| chroma32x32 = log2TrafoSize == Log2MaxTrafoSize + 1 && !split_transform_flag[ x0 ][ y0 ][ trafoDepth ] |  |
| transform_tree( x0, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0, chroma32x32 ) |  |
| transform_tree( x1, y0, x0, y0, log2TrafoSize − 1 trafoDepth + 1, 1, chroma32x32 ) |  |
| transform_tree( x0, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 2, chroma32x32 ) |  |
| transform_tree( x1, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 3, chroma32x32 ) |  |
| } else { |  |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ¦¦ (ChromaArrayType == 2 && (cbf_cb[ x0 ][ y0 + 1 << log2TrafoSize ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 + 1 << log2TrafoSize ][ trafoDepth ] ) ) ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| transform_unit (x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx, chroma32x32) |  |
| } |  |
| } |  |

#### 7.3.8.10	Transform unit syntax

| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx, chroma32x32 ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign | ae(v) |
| } |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSize, 0 ) |  |
| if ( ChromaArrayType == 3 ) { |  |
| if( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSize, 1 ) |  |
| if( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSize, 2 ) |  |
| } else if( log2TrafoSize > 2 && !chroma32x32 ) { |  |
| for ( tIdx = 0; tIdx < TrafoCrCbCnt; tIdx++ ) |  |
| if( cbf_cb[ x0 ][ y0 + tIdx << log2TrafoSize ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + tIdx << log2TrafoSize, log2TrafoSize - (1 - chroma32x32), 1 ) |  |
| for ( tIdx = 0; tIdx < TrafoCrCbCnt; tIdx++ ) |  |
| if( cbf_cr[ x0 ][ y0 + tIdx << log2TrafoSize ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + tIdx << log2TrafoSize, log2TrafoSize - (1 - chroma32x32), 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| for ( tIdx = 0; tIdx < TrafoCrCbCnt; tIdx++ ) |  |
| if( cbf_cb[ xBase ][ yBase + tIdx << log2TrafoSize ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase + tIdx << log2TrafoSize, log2TrafoSize, 1 ) |  |
| for ( tIdx = 0; tIdx < TrafoCrCbCnt; tIdx++ ) |  |
| if( cbf_cr[ xBase ][ yBase + tIdx << log2TrafoSize ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase + tIdx << log2TrafoSize, log2TrafoSize, 2 ) |  |
| } |  |
| } |  |
| } |  |

#### Transform tree semantics

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into four blocks with half horizontal and half vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

The variable interSplitFlag is derived as follows.

- If max_transform_hierarchy_depth_inter is equal to 0 and PredMode[ x0 ][ y0 ] is equal to MODE_INTER and PartMode is not equal to PART_2Nx2N and trafoDepth is equal to 0, interSplitFlag is set to 1.

- Otherwise, interSplitFlag is set to 0.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

- log2TrafoSize is greater than Log2MaxTrafoSize + 1

- IntraSplitFlag is equal to 1 and trafoDepth is equal to 0

- interSplitFlag is equal to 1

Otherwise, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

The variable TrafoCrCbCnt is derived as follows:

- If ChromaArrayType is equal to 2, TrafoCrCbCnt is derived as follows:

- If split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is equal to 0, TrafoCrCbCnt is equal to 2.

- Otherwise, split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is equal to 1, TrafoCrCbCnt is equal to 1.

- Otherwise, TrafoCrCbCnt is equal to 1.