| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0152 |
| --- | --- |

| Title: | On random access at CRA access units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong Jin Cho Byeongdoo Choi Youngo Park Chanyul Kim | Email: | yongjin9.cho@samsung.com b.d.choi@samsung.com youngo.park@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document introduces methods to avoid decoder misbehaviour at CRA access units. Decoder misbehaviours at CRA access units are caused by 1) parameter set mismatch and 2) failure to random access signalling and/or detection. In this document, informative notes to avoid decoder misbehaviour at CRA access units are proposed, and also, a condition to set the value of HandleCraAsBlaFlag is proposed for clarification.

# Introduction

When parameter set RBSPs are conveyed in the bitstream, the trick mode play may cause decoder misbehaviour due to the failure to detect the current play mode – i.e., normal or trick play, and/or delivery failure of matching parameter sets. Parameter sets are assumed to be available at the activation time in the latest HEVC draft. However, because parameter sets are activated based on their identifier values whose scope is confined in a coded video sequence, identifier values are not able to distinguish parameter sets corresponding to different coded video sequences. Then, decoder misbehaviour may be caused by the use of mismatching parameter sets with random access. In this proposal, we introduce possible causes of the decoder misbehaviour and propose solutions to them.

# Problem description

(a) Random access to a CRA access unit, identical parameter set ids, normal play

(b) Random access to a CRA access unit, identical parameter set ids, trick play

(c) Random access to a CRA access unit, different parameter set ids, normal play

(d) Random access to a CRA access unit, different parameter set ids, trick play

Figure 1. Random access scenarios at CRA access units

Figure 1 demonstrates two random access scenarios, where the parameter set ids referred by a CRA access unit are identical (Figure 1(a) and 1(b)), and different (Figure 1(c) and 1(d)). In the following scenarios, two parameter sets – active parameter sets right before the parsing of the CRA access unit slice segment header, and the parameter sets to be referred by the CRA access unit.

Scenario 1: Parameter sets identifier values are identical

When random access occurs as demonstrated in Figure 1(b), following scenarios are possible.

- Parameter sets are identical

  - Decoder misbehaviour due to the failure to detect the random access. That is, the CRA access unit has to be changed to a BLA access unit.

- Only parameter identifier values are identical

  - Decoder misbehaviour due to parameter mismatch

Scenario 2: Parameter sets are different.

When random access occurs as illustrated in Figure 1(d), the parameter sets for the CRA access unit have to be delivered by an external means or they are lost. Even though the availability of the parameter sets may be assumed, if the current access unit is a CRA access unit, it may still cause decoder misbehaviour due to the random access failure.

To summarize, a decoder may misbehave at a CRA access unit because of 1) failure to detect the random access, and 2) parameter mismatch.

# Proposed solutions

Proposal 1: Parameter set mismatch

An informative note can be added to recommend the attachment of parameter sets. If a CRA access unit always comes with corresponding parameter sets, a decoder will not misbehave because of the parameter mismatch. Another possibility is to require external means to put corresponding parameter sets in a CRA access unit whenever random access occurs to a CRA access unit. Then, the decoder misbehaviour due to the parameter set mismatch can be avoided.

Proposal 2: Detection of random access

When the current picture is a CRA picture, and the identifier values of the currently active parameter sets, and the parameter sets to be referred by the CRA picture are different, it can be inferred that random access has occurred at the current CRA picture. Then, HandleCraAsBlaFlag can be set to 1 to avoid decoder misbehaviour due to the random access detection failure.

# Proposed text changes in the specification

The text changes corresponding to the proposals are highlighted in yellow texts.

- Proposal 1: Parameter mismatch avoidance

- 3.102 random access point (RAP) access unit: An access unit in which the coded picture is a RAP picture.

- NOTE 12 – RAP access units may always contain parameter sets corresponding to the coded RAP picture to avoid possible parameter set mismatch.

- 7.4.1.4.1 Order of video, sequence, and picture parameter set RBSPs and their activation

- …

- Any video parameter set NAL unit containing the value of vps_video_parameter_set_id for the active video parameter set RBSP for a coded video sequence shall have the same content as that of the active video parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit, the first sequence parameter set NAL unit, and the first SEI NAL unit containing an active parameter sets SEI message (when present) of another coded video sequence.

- NOTE 1 – If video parameter set RBSP, sequence parameter set RBSP, or picture parameter set RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the video parameter set RBSP, sequence parameter set RBSP, or picture parameter set RBSP, respectively. Otherwise (video parameter set RBSP, sequence parameter set RBSP, or picture parameter set RBSP are conveyed by other means not specified in this Specification), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

NOTE 4 – Because the scope of parameter set identifiers is confined within a coded video sequence, it is possible for two parameter sets to have different contents but the identifier value, which may cause incorrect decoding of RAP access units in case of (but, not limited to) bitstream splicing and random access due to incorrect parameter information. When two parameter sets have a common identifier value with different contents, the parameter sets must be managed appropriately by external means not specified in this Specification. For example, if video parameter set RBSP, sequence parameter set RBSP, or picture parameter set RBSP are conveyed within a bitstream, when random access or bitstream splicing occurs, external means not specified in thie Specification may attach corresponding parameter sets to corresponding RAP access unit to avoid such decoder failure.

- Proposal 2: Random access detection at CRA access units

8.1 General decoding process

When the current picture is a CRA picture, the following applies:

–	If some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value, HandleCraAsBlaFlag is set to the value provided by the external means.

–	Otherwise, if pps_seq_parameter_set_id is different from that of the active sequence parameter set, HandleCraAsBlaFlag is set to 1.

–	Otherwise, the value of HandleCraAsBlaFlag is set to 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).