| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0154 |
| --- | --- |

| Title: | Single-loop SNR scalability using Binary Residual Refinement Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Christian Feldmann Fabian Jäger Juliana Hsu Mathias Wien Institut für Nachrichtentechnik RWTH Aachen University | Email: Email: Email: Email: | feldmann@ient.rwth-aachen.de jaeger@ient.rwth-aachen.de hsu@ient.rwth-aachen.de wien@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

Following the Call for Proposals on Scalable High Efficient Video Coding (SHVC) all responses proposed a multi-loop decoder design. The reconstructed base layer can be used for prediction in the enhancement layer. This approach implies that full layer decoding and reconstruction has to be performed in all layers. While in the case of spatial scalability, the complexity increase for multi-loop decoding may be limited, multi-loop SNR scalable decoding is expected to induce a significant increase of the overall decoder complexity compared to single layer decoding.

This contribution proposes the consideration of a single-loop SNR scalability concept for the scalable extension of HEVC. An approach for this concept is presented and compared to the SMuC performance. The proponents suggest considering single-layer SNR scalable coding in the SHVC design.

The presented approach re-uses the SVC key picture concept and applies inter-layer prediction mechanisms which include an inherited coding tree and inter-layer prediction for inter and intra prediction tools. For residual coding, a binary residual refinement of the transform coefficients is proposed which is asserted to allow for re-writing of the multi-layer residual signal to a single layer residual.

It should be noted that the current implementation of the proposed scheme does not yet include the implementation of Sign Data Hiding. The encoder does not yet include RDOQ, and no multi-layer encoder decisions are implemented. Consequently, Sign Data Hiding and RDOQ are not used in the reported simulation results. The presented approach reportedly yields BD-rate savings in both layers of -18.55% with the random access configuration compared to -22.13% for the SMuC reference. In the all intra configuration the approach reportedly yields BD-rate savings of -30.42% compared to -31.12% for the SMuC reference. Based on the observed results, the proponents suggest to set up an investigation of single-loop SNR scalable coding in a Core Experiment.

# Introduction

In the responses to the call for proposals as well as in the SMuC, a dual loop approach has been chosen for both, spatial and quality scalability. [1] In SNR scalable video coding a dual loop approach for quality scalability might not considered to be practical because it basically scales the complexity at the decoder side linearly by the number of included SNR layers. Due to limited compression efficiency improvements which can be observed with the SMuC SNR scalable approach, the trade-off between the achievable rate-distortion gain and the required computational complexity may be considered as a major drawback of the design. It is therefore suggested to consider single-layer SNR scalable coding in the SHVC design.

This contribution proposes the consideration of a single-loop SNR scalable coding scheme for the scalable extension of HEVC. Such an approach was pursued in SVC as well. In this proposal, a possible single loop coding approach is presented which performs residual refinement in the transform domain. The scheme employs a binary refinement step in successive SNR layers and allows for re-writing of the multi-layer residual signal to a single layer residual.

The proposed codec is currently implemented based on the HEVC reference Software HM-6.1. [2] This implementation however is not yet optimized. Specifically, no optimization between the layers is performed yet. The current does not use rate distortion optimal quantization (RDOQ) or sign data hiding. Although these techniques are compatible with the proposed methods they are not yet implemented in the software. For the comparison to the SMuC scheme, these tools are consequently switched off in all cases.

# Proposed Coding Scheme

In the following chapter the coding scheme and the used tools are explained.

## Coding order

The video sequence is split into groups of pictures (GOPs) as is done in the HEVC reference software. The GOP structure is identical in base and enhancement layer. For every frame the base layer picture is encoded first, followed by the corresponding enhancement layer picture. The base layer is encoded using the conventional HEVC coding tools implemented in HM6.1 with constrained intra prediction activated. When encoding the base layer no optimization is currently done regarding the enhancement layer which is encoded in a second step.

## Prediction structure

Figure 1 - Key picture concept prediction structure for a GOP size of 4 with coding order. Key pictures are marked in grey.

The proposed scheme uses the key picture concept as is was used in the scalable extension of H.264/AVC. Error: Reference source not found shows the exemplary prediction structure of the used key picture concept for a GOP size of 4. In the presented simulation results, a GOP size of 8 was used. Within the GOP, the base layer pictures use the enhancement layer pictures as reference for inter prediction, if available. Every GOP is delimited by key pictures (POC 0, 8, 16…), which state an exception to this rule. For every key picture the base layer picture is used as a reference for the base layer rather than the enhancement layer picture. Thereby, drift in case of the absence of the enhancement layer is limited to the pictures within each GOP. Error: Reference source not found shows the reference picture lists configuration that is used in both base and enhancement layer.

Although the base layer prediction is performed using the enhancement layer reconstructed pictures, for the motion prediction as well as for merge modes only the base layer motion information is utilized in the base layer. The enhancement layer can make full use of the enhancement layer motion info as well as the base layer motion info.

| POC | Reference Pictures |
| --- | --- |
| 8 | -8, -16 |
| 4 | -4, 4, -12 |
| 2 | -2, 2, 6, -10 |
| 1 | -1, 1, 3, 7 |
| 3 | -1, -3, 1, 5 |
| 6 | -2, -4, -6, 2 |
| 5 | -1, -5, 1, 3 |
| 7 | -1, -3, -7, 1 |

Table 1 - Used reference picture configuration.

## Inherit CU tree

In the current implementation, the enhancement layer inherits the CU tree as well as the prediction structure and the TU tree from the base layer. Hence, signaling of a CU tree, TU tree, and prediction partitioning in the enhancement layer is not necessary and therefore omitted. Altering the trees or the prediction partitioning in the enhancement layer is not enabled in the current implementation.

## Prediction modes

For every CU in the enhancement layer the following prediction modes are tested:

### EL Intra prediction

In Intra mode the CU is coded using the conventional HEVC Intra prediction as it is implemented in HM-6.1. Also the residual signal is encoded conventionally. This mode uses the enhancement layer reconstructed pixels for prediction.

### EL Inter prediction

In Inter mode the CU is coded using the conventional HEVC Inter prediction as it is implemented in HM-6.1. The possible prediction modes 2Nx2N, NxN, Nx2N and 2NxN are tested. The AMP partitionings have not been enabled. The residual signal is coded conventionally as in single layer HEVC. This mode uses the reconstructed enhancement layer pictures for prediction (see Error: Reference source not found).

Some modifications to the AMVP process are implemented to better utilize the base layer motion info:

- The first candidate that is added to the list is the motion vector of the corresponding CU in the base layer

- The remaining candidates are added in the conventional AMVP order

- For all spatial and temporal candidates: Use base layer motion vector if enhancement layer motion vector is not available

Also the merge candidate list creation is modified:

- For all spatial candidates: If a merge candidate is not available it is replaced by the corresponding base layer merge candidate if available

- For all temporal candidates: Use available base layer merge candidate if enhancement layer merge candidate is not available

### Base layer mode

In base layer mode the prediction signal as well as the residual signal from the base layer is inherited in the enhancement layer. The enhancement layer residual can be coded in several different ways:

- If the base layer residual has no significant coefficients, a new residual signal can be coded in the enhancement layer which is added to the base layer prediction. The new residual signal is coded using conventional HEVC residual coding.

- The enhancement layer residual can be skipped. In this case the enhancement layer reconstruction is inherited from the base layer reconstruction.

- The residual from the base layer is refined using the new binary residual refinement coding as explained in 2.5. The prediction from the base layer and the refined residual then yield the enhancement layer reconstruction.

## Residual Coding

In this new mode the prediction signal from the base layer is used and the residual signal from the base layer is refined before adding it to the prediction. The transform tree is also inherited from the base layer so that the same transforms are used in both layers.

As in the base layer, the position of the last significant coefficient is coded first. However in the enhancement layer we can utilize the position of the last significant coefficient in the base layer. Since coefficients that are significant in the base layer are also significant in the enhancement layer, the position cannot be closer to the (0,0) position in coefficient scanning order than the base layer position. This property can be exploited by only encoding a position difference from the position of the last significant coefficient in the base layer.

In the subsequent significance scan,, all coefficients that were significant in the base layer are skipped. Also the sign information only needs to be coded for the coefficients that get significant during this scan. For already significant coefficients, a refinement flag is coded which indicates how the quantized coefficient shall be refined.

In Error: Reference source not found the quantization steps for the QP values 30 and 36 are shown. For every coefficient that is significant in the base layer a binary refinement decision is encoded that maps the base layer level to one of two possible levels in the enhancement layer. Error: Reference source not found illustrates a possible mapping from base layer levels to enhancement layer levels. Similar mappings can be established for other delta QP step sizes. A possible mapping from QP value 36 to QP value 32 is shown in Figure 4.

The mapping for each QP delta can be pre-stored in a table or adjusted, e.g. in the SPS. Currently, the scheme has been implemented up to a QP delta of 6. If the QP difference between the two layers is greater than 6, a binary decision is not sufficient anymore. In this situation multiple binary decisions have to be combined or a one to many mapping has to be used. This approach has not yet been applied in the current version. Therefore, the case of a QP delta equal to 8 is omitted for now.

Figure 2 - Quantization steps for QP=36 and QP=30.

Figure 3 - Example ∆QP=6 binary mapping from QP=36 to QP=30.

Figure 4 – Example ∆QP=4 binary mapping from QP=36 to QP=30.

# Coding

In the following section the changes that were made to the coding process are explained as well as the coding of the new modes.

## Prediction mode

Since the CU tree is inherited from the base layer, at first the prediction mode of every CU is coded. The possible modes are: Inter, Intra and residual refinement coding.

A first bit codes if residual refinement coding is used or not. If this bit is set (no residual refinement coding is used) a second bit codes if intra (bit set) or inter coding (bit not set) is used. The second bit is omitted if the current frame is an I-frame in which case only Intra prediction and residual refinement coding is allowed. Currently each bit is encoded in one CABAC context that is initially set to being equiprobable (initValue 154).

## Residual refinement

If residual refinement coding is used, the prediction signal from the base layer is inherited and the following modifications to the residual coding are employed.

### Transform tree

In residual refinement coding the transform tree from the base layer is inherited and no split_transform_flag is coded. The no_residual_data_flag, cbf_cb, cbf_cr and cbf_luma flags are HEVC coded.

### Last significant coefficient

Since the position of the last significant coefficient in the base layer is known and significant coefficients in the base layer are also significant in the enhancement layer, only a position difference is encoded in the enhancement layer.

The difference values are encoded similar to the last significant position coding in the conventional HEVC implementation in HM-6.1 using an entropy coded prefix part and a suffix part coded in bypass mode.

### Significance Scan

The significance scan is performed as in the conventional HEVC implementation in HM-6.1. The only modification is that all coefficients that are significant in the base layer are skipped in this scan.

### Sign data

The sign data of the coefficients that get significant in the enhancement layer scan is conventionally coded in bypass mode.

### Coefficient refinement

In the final step all the coefficients that are significant in the base layer are refined. A binary mapping decision is coded that maps each coefficient to one of two possible coefficients in the enhancement layer. The sign information for the coefficient is inherited from the base layer. Currently the binary decision is encoded in one CABAC context that is initialized as being equiprobable (initValue 154).

# Simulation results

## Simulation conditions

As in the CfP, two layers are coded. RDOQ and sign data hiding have not yet been implemented in the current test software and will be implemented as a next step. In order to make a fair comparison, RDOQ and sign data hiding have also been disabled in the simulcast coder as well as in the SMUC coder in the presented simulations.

The simulation conditions are set as defined in the call for proposals on scalable video coding extensions of HEVC [3]. Here, all class B sequences (Kimono, ParkScene, Cactus, BasketballDrive and BQTerrace) were tested in random access and all intra configurations.

As stated above, a QP difference of 8 is not tested in these simulations. The tested QP values for the base layer are QP_base = 26, 30, 34, 38 and the QP values for the enhancement layer are QP_base-2, QP_base-4 and QP_base-6.

## Random access results

Table 2 lists the summary report of the BD rate savings for the SMuC and the proposed scheme. The averaging follows the method used in the CfP Excel sheet. The full data are available in the accompanying data sheet [JCTVC-L154-Results_RA.xlsm].

|  | SMuC reference |  |  | Proposal |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -24.92% | -22.59% | -19.31% | -22.40% | -24.74% | -26.00% |
| ParkScene | -23.12% | -21.13% | -20.33% | -20.53% | -23.53% | -24.56% |
| Cactus | -21.70% | -19.31% | -16.22% | -17.66% | -21.08% | -18.60% |
| BaskteballDrive | -23.79% | -15.81% | -17.82% | -18.60% | -19.53% | -18.54% |
| BQTerrace | -17.11% | 1.22% | 5.10% | -13.57% | -18.65% | -14.25% |
| Average | -22.13% | -15.53% | -13.72% | -18.55% | -21.51% | -20.39% |

Table 2 - Average BD-Rate savings per sequence of the SMuC reference and the proposed coded.

## All intra results

Table 2 lists the summary report of the BD rate savings for the SMuC and the proposed scheme. The averaging follows the method used in the CfP Excel sheet. The full data are available in the accompanying data sheet [JCTVC-L154-Results_AI.xlsm].

|  | SMuC reference |  |  | Proposal |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -33.78% | -33.45% | -34.23% | -31.21% | -29.34% | -30.91% |
| ParkScene | -31.63% | -34.55% | -35.18% | -30.85% | -31.74% | -33.27% |
| Cactus | -30.78% | -33.46% | -33.01% | -30.92% | -31.58% | -27.79% |
| BaskteballDrive | -28.93% | -31.66% | -30.50% | -28.64% | -28.43% | -27.33% |
| BQTerrace | -30.51% | -33.16% | -35.08% | -30.50% | -34.34% | -32.00% |
| Average | -31.12% | -33.26% | -33.60% | -30.42% | -31.08% | -30.26% |

Table 3 - Average BD-Rate savings per sequence of the SmuC reference and the proposed coded.

## Performance of RDOQ, Sing Bit Hiding and Constrained Intra Prediction

As stated above, the simulation conditions do not fully include the tools enabled in the default configurations. In order to provide an indication on the overall performance impact, single layer simulations have been conducted with each of the tools separately switched on/off. Table 4 shows the average BD-rate loss for individually deactivating the tools RDOQ, sign bit hiding and activating constrained intra prediction. For the measurements HM-6.1 was used with random access configuration. The QP values 26, 30, 34, 38 and the sequences Kimono, ParkScene, Cactus, BasketballDrive and BQTerrace were simulated.

|  | Y | U | V |
| --- | --- | --- | --- |
| No RDOQ | -4.84% | -4.29% | -4.68% |
| No Sign Bit Hiding | -0.57% | -0.59% | -0.94% |
| Constrained Intra | -1.94% | -2.33% | -2.50% |

Table 4 - Average BD-rate loss for deactivating RDOQ, sign bit hiding and using constrained intra prediction.

# Discussion and Conclusions

The proposed single layer coding comes at some RD-cost compared to SMuC. However the proposed scheme is believed to yield a significant reduction in complexity compared to the dual loop approach of SMuC. In our scheme, a single coding loop is required for prediction. For the refined blocks, a second iteration of the regular coefficient coding passes is applied for not-yet significant coefficients. The refinement of previously significant coefficients is realized in a single coding pass with a refinement flag per significant base layer coefficient.

The current implementation is limited to two layers and does not support RDOQ as well as sign bit hiding. However these limitations do not result from the described method but rather from the current software implementation. The current implementation further does not include specific modifications to the encoder mode decision. Specifically, no joint optimization of base and enhancement layer prediction modes has been applied.

The presented results seem to suggest that single-loop SNR scalable decoding can be achieved at a reasonable rate-distortion trade-off compared to the dual-loop approach investigated so far. We propose to continue this investigation and to set up a tool or core experiment for a single layer coding approach for SNR scalability.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Acknowledgement

This work was co-funded by the German federal state North Rhine Westphalia (NRW) and the European Union (European Regional Development Fund: Investing In Your Future).

# References

- JCTVC-K1008, “Report of Results of the Joint Call for Proposals on Scalable High Efficiency Video Coding (SHVC),” Shanghai, CN, 10–19 Oct. 2012.

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, and T. Wiegand, High efficiency video coding (HEVC) text specification draft 6, ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) document JCTVC-H1003, February 2012.

- ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC), ITU-T SG16/Q6,Document  VCEG-AS90, Jul. 2012