| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0155 |
| --- | --- |

| Title: | AHG9: On column_width_minus1 syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to change column_width_minus1 syntax into column_width_minus4 to align the profile restriction which defines the minimum size of ColumnWidthInLumaSamples as 256 pixel. The proposal prevents an illegal bitstream regarding a tile width when coding tree block size is 64. Besides, it saves redundant bit to code the syntax.

# Problem Statement

column_width_minus1 is a picture parameter set syntax which specifies the width of tile column in units of coding tree blocks(CTB). Since “_minus1” is appended in the name, the minimum value of column_width is 1. Thus, the minimum width of a tile can be 16 since the minimum CTB size is 16.

On the other hand, there is a profile restriction in Annex A. According to the latest draft, all version 1 profiles (Main, Main10 and Still picture) shall satisfy the following restriction.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

It is noted that the yellow highlighted part comes from decoder complexity point of view. It is believed that the restriction will remain even in a future profile including extensions (fidelity range, 3D, scalability and so on).

Thus, in a conformance bitstream, column_width_minus1 shall be greater than or equal to 3, 7, or 15 for CTB size 64, 32 and 16, respectively.

This is a bit redundant and can cause an illegal bitstream for careless encoders.

# Proposal

It is proposed to change column_width_minus1 syntax into column_width_minus4 to align the profile restriction.

In a conformance bitstream, column_width_minus4 shall be greater than or equal to 0, 4, or 12 for CTB size 64, 32 and 16, respectively.

When coding tree block size is 64, it prevents an illegal bitstream in syntax level. Since the syntax is unsigned integer coded (ue(v)), the value of column_width_minus4 is greather than or equal to 0. Thus, the value of column_width is greather than or equal to 4. When CTB size is 64, it means a tile width is greater than or equal to 256 pixel. So, it always satisfies the profile restriction.

Besides, it saves some redundant bit to code the syntax. For example, when a tile width is 1024 and CTB size is 64, the syntax value of column_width_minus1 and column_width_minus4 is 15 and 12, respectively. Since it is coded by ue(v), the coded bit can be reduced from 9 to 7 bit by proposal. It is noted that the syntax is coded for all columns in a picture, and for every PPS if necessary.

As a side effect, proposed “_minus4” syntax prevents a smaller tile width than 256 pixel. However, proponents believe the functionality is not necessary even in the future profiles.

# Draft

The proposed text on top of JCTVC-K1003_v13 is attached in the document.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).