| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12nd Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0157 |
| --- | --- |

| Title: | Editor’s proposed draft HEVC conformance spec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
|  | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Tel: Email: | +1 425 703 5308 garysull@microsoft.com |
|  | Wade Wan 5300 California Ave Irvine, CA 92617 USA | Tel: Email: | wwan@broadcom.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document is a proposed draft 2 of conformance test of HEVC.

- Conformance specification for HEVC

# Scope

This Recommendation | International Standard specifies a set of tests and procedures designed to indicate whether encoders or decoders meet the normative requirements specified in ITU-T H.HEVC | ISO/IEC 23008-2.

# Normative references

## General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None.

## Paired Recommendations | International Standards equivalent in technical content

–	ITU-T H.HEVC (in force), High efficiency coding.

ISO/IEC 23008-2: in force, Information technology – High efficiency video coding and media delivery in heterogeneous environment – Part 2: High Efficiency Video Coding.

–	ITU-T H.HEVC.2 (in force), Reference software for H.HEVC advanced video coding.

ISO/IEC 14496-5: in force, Information technology – Coding of audio-visual objects – Part 5: Reference software.

## Additional references

–	None.

# Definitions

For the purposes of this Recommendation | International Standard, the terms, definitions, abbreviations and symbols specified in ITU-T H.HEVC | ISO/IEC 23008-2 (particularly in clauses 3) apply. The following terms are further clarified for purposes herein as follows.

3.1	bitstream: An ITU-T H.HEVC | ISO/IEC 23008-2 video bitstream. A bitstream may contain I, P and B slices.

3.2	decoder: An ITU-T H.HEVC | ISO/IEC 23008-2 video decoder, i.e., an embodiment of the decoding process specified by ITU-T H.HEVC | ISO/IEC 23008-2. The decoder does not include the display process, which is outside the scope of this Recommendation | International Standard.

3.3	reference software decoder: The software decoder contained in ITU-T H.HEVC.2 | ISO/IEC 14496-5.

# Abbreviations and acronyms

For the purposes of this Recommendation | International Standard, relevant abbreviations and acronyms are specified in clause 4 of ITU-T H.HEVC | ISO/IEC 23008-2.

# Conventions

For the purposes of this Recommendation | International Standard, relevant conventions are specified in clause 5 in ITU-T H.HEVC | ISO/IEC 23008-2.

# Conformance for ITU-T H.HEVC | ISO/IEC 23008-2

## Introduction

The following clauses specify the normative tests for verifying conformance of video bitstreams as well as decoders. Those normative tests make use of test data (bitstream test suites) provided as an electronic annex to this Recommendation | International Standard and the reference software decoder with source code included in electronic format.

## Bitstream conformance

The bitstream conformance of ITU-T H.HEVC | ISO/IEC 23008-2 is specified by clause C.4 of ITU-T H.HEVC | ISO/IEC 23008-2.

## Decoder conformance

The decoder conformance of ITU-T H.HEVC | ISO/IEC 23008-2 is specified by clause C.5 of ITU-T H.HEVC | ISO/IEC 23008-2.

## Procedure to test bitstreams

A bitstream that claims conformance with ITU-T H.HEVC | ISO/IEC 23008-2 shall pass the following normative test:

The bitstream shall be decoded by processing it with the reference software decoder. When processed by the reference software decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstreams that lack conformance to ITU-T H.HEVC | ISO/IEC 23008-2.

Successfully passing the reference software decoder test provides only a strong presumption that the bitstream under test is conforming to the video layer, i.e., that it does indeed meet all the requirements for the video layer (except Annexes C, D and E) specified in ITU-T H.HEVC | ISO/IEC 23008-2 that are tested by the reference software decoder.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in ITU-T H.HEVC | ISO/IEC 23008-2 including the hypothetical reference decoder (HRD) conformance (based on Annexes C, D and E). These complementary tests may be performed using other video bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

ITU-T H.HEVC | ISO/IEC 23008-2 contains several informative recommendations that are not an integral part of that Recommendation | International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntax elements and other values derived from those syntactic elements and used by the decoding process specified in ITU-T H.HEVC | ISO/IEC 23008-2.

A verifier may not necessarily perform all stages of the decoding process specified in ITU-T H.HEVC | ISO/IEC 23008-2 in order to verify bitstream correctness. Many tests can be performed on syntax elements in a state prior to their use in some processing stages.

## Procedure to test decoder conformance

### Conformance bitstreams

A bitstream has values of general_profile_idc, general_level_idc corresponding to a set of specified constraints on a bitstream for which a decoder conforming to a specified profile and level is required in Annex A of ITU-T H.HEVC | ISO/IEC 23008-2 to properly perform the decoding process.

### Contents of the bitstream file

The conformance bitstreams are included in this Recommendation | International Standard as an electronic attachment. The following information is included in a single zipped file for each such bitstream.

–	bitstream;

–	decoded pictures or hashes of decoded pictures (may not be present);

–	short description of the bitstream;

–	trace file (the bitstream in ASCII format).

In cases where the decoded pictures or hashes of decoded pictures are not available, the reference software decoder shall be used to generate the necessary reference decoded pictures from the bitstream.

### Requirements on output of the decoding process and timing

Two classes of decoder conformance are specified:

–	output order conformance; and

–	output timing conformance.

The output of the decoding process is specified in clauses 8 and Annex C of ITU-T H.HEVC | ISO/IEC 23008-2.

For output order conformance, it is a requirement that all of the decoded pictures specified for output in Annex C of ITU-T H.HEVC | ISO/IEC 23008-2 shall be output by a conforming decoder in the specified order and that the values of the decoded samples in all of the pictures that are output shall be (exactly equal to) the values specified in clause 8 of ITU-T H.HEVC | ISO/IEC 23008-2.

For output timing conformance, it is a requirement that a conforming decoder shall also output the decoded samples at the rates and times specified in Annex C of ITU-T H.HEVC | ISO/IEC 23008-2.

The display process, which ordinarily follows the output of the decoding process, is outside the scope of this Recommendation | International Standard.

### Recommendations (informative)

This clause does not form an integral part of this Recommendation | International Standard.

In addition to the requirements, it is desirable that conforming decoders implement various informative recommendations specified in ITU-T H.HEVC | ISO/IEC 23008-2 that are not an integral part of that Recommendation | International Standard. This clause lists some of these recommendations.

It is recommended that a conforming decoder be able to resume the decoding process as soon as possible after the loss or corruption of part of a bitstream. In most cases it is possible to resume decoding at the next start code or slice header. It is recommended that a conforming decoder be able to perform concealment for the macroblocks or video packets for which all the coded data has not been received.

### Static tests for output order conformance

Static tests of a video decoder require testing of the decoded samples. This clause will explain how this test can be accomplished when the decoded samples at the output of the decoding process are available. It may not be possible to perform this type of test with a production decoder (due to the lack of an appropriate accessible interface in the design at which to perform the test). In that case this test should be performed by the manufacturer during the design and development phase. Static tests are used for testing the decoding process. The test will check that the values of the samples decoded by the decoder under test shall be identical to the values of the samples decoded by the reference decoder. When a hash of the values of the samples of the decoded pictures is attached to the bitstream file, a corresponding hash operation performed on the values of the samples of the decoded pictures produced by the decoder under test shall produce the same results.

### Dynamic tests for output timing conformance

Dynamic tests are applied to check that all the decoded samples are output and that the timing of the output of the decoder's decoded samples conforms to the specification of clauses 8; and Annex C of ITU-T H.HEVC | ISO/IEC 23008-2, and to verify that the HRD models (as specified by the CPB and DPB specification in Annex C of ITU-T H.HEVC | ISO/IEC 23008-2) are not violated when the bits are delivered at the proper rate.

The dynamic test is often easier to perform on a complete decoder system, which may include a systems decoder, a video decoder and a display process. It may be possible to record the output of the display process and to check that display order and timing of fields or frames are correct at the output of the display process. However, since the display process is not within the normative scope of ITU-T H.HEVC | ISO/IEC 23008-2, there may be cases where the output of the display process differs in timing or value even though the video decoder is conforming. In this case, the output of the video decoder itself (before the display process) would need to be captured in order to perform the dynamic tests on the video decoder. In particular the field or frame order and timing shall be correct.

If buffering period SEI and picture timing SEI are included in the test bitstream, HRD conformance shall be verified using the values of initial_cpb_removal_delay, initial_cpb_removal_delay_offset, cpb_removal_delay and dpb_removal_delay that are included in the bitstream.

If buffering period SEI and picture timing SEI are not included in the bitstream, the following inferences shall be made to generate the missing parameters:

–	fixed_pic_rate_flag shall be inferred to be 1.

–	low_delay_hrd_flag shall be inferred to be 0.

–	cbr_flag shall be inferred to be 0.

–	The frame rate of the bitstream shall be inferred to be the frame rate value specified in the corresponding table of clause 6.7, where the bitstream is listed. If this is missing, then a frame rate of either 25 or 30000 ÷ 1001 can be inferred.

–	time_scale shall be set to 90,000 and the value of num_units_in_tick shall be computed based on field rate (twice the frame rate).

–	The bit rate of the bitstream shall be inferred to be the maximum value for the level specified in Table A-1 in ITU-T H.HEVC | ISO/IEC 23008-2.

–	CPB and DPB sizes shall be inferred to be the maximum value for the level specified in Table A-1 in ITU-T H.HEVC | ISO/IEC 23008-2.

With the above inferences, the HRD shall be operated as follows.

–	The CPB is filled starting at time t = 0, until it is full, before removal of the first access unit. This means that the initial_cpb_removal_delay shall be inferred to be equal to the total CPB buffer size divided by the bit rate divided by 90000 (rounded downwards) and initial_cpb_removal_delay_offset shall be inferred to be equal to zero.

–	The first access unit is removed at time t = initial_cpb_removal_delay ÷ 90000 and subsequent access units are removed at intervals based on the frame distance, i.e., 2 * (90000 ÷ num_units_in_tick) or the field distance, i.e., (90000 / num_units_in_tick), depending on whether the access unit is coded as a frame picture or field picture.

–	Using these inferences, the CPB will not overflow or underflow and the DPB will not overflow.

### Decoder conformance test of a particular profile-and-level

In order for a decoder of a particular profile-and-level to claim output order conformance to ITU-T H.HEVC | ISO/IEC 23008-2 as specified by this Recommendation | International Standard, the decoder shall successfully pass the static test specified in clause 6.5.5 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level.

In order for a decoder of a particular profile and level to claim output timing conformance to ITU-T H.HEVC | ISO/IEC 23008-2 as specified by this Recommendation | International Standard, the decoder shall successfully pass both the static test specified in clause 6.5.5 and the dynamic test specified in clause 6.5.6 with all the bitstreams of the normative test suite specified for testing decoders of this particular profile-and-level. Tables 1 through 5 specify the normative test suites for each profile-and-level combination. The test suite for a particular profile-and level combination is the list of bitstreams that are marked with an 'X' in the column corresponding to that profile-and-level combination.

'X' indicates that the bitstream is designed to test both the dynamic and static conformance of the decoder.

The bitstream column specifies the bitstream used for each test.

A decoder that conforms to the Main profile at a specific level shall be capable of decoding the specified bitstreams in Tables 1.

## Specification of the test bitstreams

Some characteristics of each bitstream listed in Tables 1 are specified in this clause. In Tables 1, the value "29.97" shall be interpreted as an approximation of an exact value of 30000 ÷ 1001 and the value "59.94" shall be interpreted as an approximation of an exact value of 60000 ÷ 1001.

### Test bitstreams – Block structure

#### Test bitstreams #STRUCT_A

Specification:  All slices are coded as I, P or B slices. Each picture contains one slice. Various CTU and maximum CU size are used.

- Functional stage:  Test the reconstruction process of slices.

Purpose:  Check that decoder can properly decode I, P and B slices with various CTU and maximum CU size.

#### Test bitstreams #STRUCT_B

Specification:  All slices are coded as I, P or B slices. Each picture contains one slice. Various CTU and minimum CU size are used.

- Functional stage:  Test the reconstruction process of slices.

Purpose:  Check that decoder can properly decode I, P and B slices with various CTU and minimum CU size.

### Test bitstreams – Intra coding

#### Test bitstreams #IPRED_A, IPRED_B, IPRED_C

Specification:  All slices are coded as I slices. Each picture contains one slice. All intra prediction modes (35 modes for each of luma 32x32, luma 16x16, luma 8x8, luma 4x4, chroma 16x16, chroma 8x8 and chroma 4x4 for a total 245 modes) are used.

- Functional stage:  Test the reconstruction process of I slice.

Purpose:  Check that decoder can properly decode I slices with all intra prediction modes.

#### Test bitstreams #CIP_A

Specification:  The bitstream contains one I slice and one B slice, using one slice per picture. Both SAO and DBF are disabled.

- Functional stage:  Test the reference sample substitution process for intra sample prediction.

Purpose:  Check that decoder can correctly decode slices of coded pictures containing intra PUs with unavailable samples for intra prediction.

#### Test bitstreams #CIP_B

Specification:  The bitstream contains an I-picture and 4 P-pictures. Each picture contains only one slice. constrained_intra_pred_flag is equal to 1.

- Functional stage:  Test the reference sample substitution process for intra sample prediction.

Purpose:  Check that decoder can correctly decode slices of coded pictures containing intra PUs with unavailable samples for intra prediction.

### Test bitstreams – Inter frame coding

#### Test bitstreams #MERGE_A

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 4.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_B

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 3.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_C

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 2.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_D

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 1.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_E

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 0.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the maximum number of merging canidates permitted by the standard (i.e. 1, 2, 3, 4, 5)

#### Test bitstreams #MERGE_F

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. sps_temporal_mvp_enable_flag  is equal to 0 and five_minus_max_num_merge_cand is equal to 0.

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check whether Merge operations perform correctly when temporal merging candidate is not included in merge candidate set.

#### Test bitstreams #MERGE_G

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. five_minus_max_num_merge_cand is set equal to 0

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check if decoder properly handles merge index ranging from 0 to 4.

      - Test bitstreams #PMERGE_A

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 0.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_B

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 1.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_C

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 2.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_D

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 3.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #PMERGE_E

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. log2_parallel_merge_level_minus2 is set equal to 4.

Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check the parallel merge level permitted by the standard (i.e. 2, 3, 4, 5, 6 for CTB size 64x64)

#### Test bitstreams #AMVP_A

Specification:  All slices are coded as I or P slices. Each picture contains only one slice.

num_ref_idx_l0_default_active_minus1 is  equal to 0, num_ref_idx_l1_default_active_minus1 is  equal to 0 and num_ref_idx_active_override_flag is equal to 0.

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check whether motion vector prediction candidate generation and signalling perform correctly when MV scaling is not needed for spatial MVP candidate generation (All inter-coded PUs within the same slice have the same inter_pred_idc and ref_idx_l0).

#### Test bitstreams #AMVP_B

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. Multiple reference pictures are used. For GPB slices,  num_ref_idx_l0_default_active_minus1 is  equal to 3 and num_ref_idx_active_override_flag is equal to 0. For non-GPB B slices, num_ref_idx_l0_default_active_minus1 is  equal to 1, num_ref_idx_l1_default_active_minus1 is  equal to 1 and num_ref_idx_active_override_flag is equal to 0

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check whether motion vector prediction candidate generation and signalling perform correctly when MV scaling is not needed for spatial MVP candidate generation.

#### Test bitstreams #AMVP_C

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. The purpose of the stream is to exercise AMVP for low delay condition.

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check whether motion vector prediction candidate generation and signalling perform correctly when MV scaling is not needed for spatial MVP candidate generation.

#### Test bitstreams #TMVP_A

Specification:  Each picture contains only one slice. slice_temporal_mvp_enable_flag is set to 0 for picture 0-8 and 1 for picture 9-16.

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check if slice_temporal_mvp_enable_flag is parsed and temporal motion vector predictor is generated correctly.

#### Test bitstreams #MVDL1ZERO_A

Specification:  The bitstream contains multiple B slices per picture. Mvd_l0_zero_flag is set equal to 1.

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  The purpose of the stream is to exercise the parsing of list 1 motion vecotor difference for bi-prediction according to mvd_l1_zero_flag in slice header (randamized on/off switching of mvd_l1_zero_flag for multiple B slices).

#### Test bitstreams #MVCLIP_A

Specification:  Each picture contains only one slice. MVP and Merge candidate motion vectors are clipped to 16-bit values. Clipped MVP and merge candidates are selected as MVP and Merge motion vectors.

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check if decoder properly handles clipping of MVP and Merge candidate motion vectors to 16-bit values

#### Test bitstreams #MVEDGE_A

Specification:  Each picture contains only one slice. The bitstream includes motion vectors pointing to the padded edge regions in a picture.

- Functional stage:  Test the reconstruction process of motion vector prediction.

Purpose:  Check if decoder properly handles motion vectors pointing to the padded edge regions in a picture.

#### Test bitstreams #WP_A

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. weighted_pred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage:  Weighted sample prediction process for P slices with plural reference indices.

Purpose:  Check that the decoder handles weighted sample prediction for P slices with plural reference indices.

#### Test bitstreams #WP_B

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. weighted_pred_flag is equal to 1 and weighted_bipred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage:  Weighted sample prediction process for P and B slices with plural reference indices.

Purpose:  Check that the decoder handles weighted sample prediction for P and B slices with plural reference indices.

    - Test bitstreams – Transform and quantization

#### Test bitstreams #RQT_A

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 0.

- Functional stage:  Test the reconstruction process of slices with RQT.

Purpose:  Check if decoder properly decodes slices with RQT with intra and inter depth 0.

#### Test bitstreams #RQT_B

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 1.

- Functional stage:  Test the reconstruction process of slices with RQT.

Purpose:  Check if decoder properly decodes slices with RQT with intra and inter depth 1.

#### Test bitstreams #RQT_C

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 2.

- Functional stage:  Test the reconstruction process of slices with RQT.

Purpose:  Check if decoder properly decodes slices with RQT with intra and inter depth 2.

#### Test bitstreams #RQT_D

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 3.

- Functional stage:  Test the reconstruction process of slices with RQT.

Purpose:  Check if decoder properly decodes slices with RQT with intra and inter depth 3.

#### Test bitstreams #RQT_E

Specification:  All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter and max_transform_hierarchy_depth_intra are both set equal to 4.

- Functional stage:  Test the reconstruction process of slices with RQT.

Purpose:  Check if decoder properly decodes slices with RQT with intra and inter depth 4.

#### Test bitstreams #RQT_F

Specification:   All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter is set equal to 2 and max_transform_hierarchy_depth_intra is set equal to 0.

- Functional stage:  Test the reconstruction process of slices with RQT.

Purpose:   Check if decoder properly decodes slices with RQT with different intra and inter depths.

#### Test bitstreams #RQT_G

Specification:   All slices are coded as I, or B slices. Each picture contains only one slice. max_transform_hierarchy_depth_inter is set equal to 0 and max_transform_hierarchy_depth_intra is set equal to 2.

- Functional stage:  Test the reconstruction process of slices with RQT.

Purpose:   Check if decoder properly decodes slices with RQT with different intra and inter depths.

      - Test bitstreams #DELTAQP_A

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. Max CU size is equal to 64 and Min CU size is equal to 8. Diff_cu_qp_delta_depth is set randomly from 0 to 3. cu_qp_delta is set randomly from -26 to 25.

- Functional stage:  Test the reconstruction process of slices with delta QP.

Purpose:  Check if decoder properly decodes slices with delta QP.

#### Test bitstreams #DELTAQP_B

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. Max CU size is equal to 64 and Min CU size is equal to 8. cu_qp_delta is set randomly from -26 to 25. slice_cb_qp_offset and slice_cr_qp_offset are set randomly from -4 to 4.

- Functional stage:  Test the reconstruction process of slices with delta QP.

Purpose:  Check if decoder properly handles various combination of chroma QP offset.

#### Test bitstreams #DELTAQP_C

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. Max CU size is equal to 64 and Min CU size is equal to 8. Diff_cu_qp_delta_depth is set randomly from 0 to 3. cu_qp_delta is set randomly from -26 to 25. In some TU, Cbf is set equal to 0.

- Functional stage:  Test the reconstruction process of slices with delta QP.

Purpose:  Check if decoder properly decodes slices with delta QP.

      - Test bitstreams #SLIST_A

Specification:  All slices are coded as I or B slices. Each picture contains one slice. One sequence parameter set and more than one picture parameter sets are included. The sequence parameter set includes scaling list data. One of the picture parameter set doesn’t include scaling list data. In other picture parameter sets, different scaling lists data is included. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 1.

- Functional stage:  Test the reconstruction process of scaling list. Tests switching of scaling list in sequence parameter set and picture parameter set.

Purpose:  Check that decoder can properly decode slices of coded frames with scaling list. Check that decoder can properly reconstruct scaling list with all coding modes of scaling list. Check that scaling list in sequence parameter set is applied when no scaling list is included in picture parameter set. Check that appropriate scaling list in picture parameter set is applied when scaling list data is included in picture parameter set.

#### Test bitstreams #SLIST_B

Specification:  All slices are coded as I or B slices. Each picture contains one slices. One sequence parameter set and more than one sequence parameter sets are included. One of the picture parameter set includes scaling list data. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 0.

- Functional stage:  Test the functionality of scaling_list_enabled_flag. Test reconstruction process of scaling list.

Purpose:  Check that decoder can properly decode slices of coded frames with scaling_list_enabled_flag equal to 0, even if scaling lists are included in picture parameter set.

#### Test bitstreams #SLIST_C

Specification:  All slices are coded as I or B slices. Each picture contains one slice. One sequence parameter set and more than one picture parameter sets are included. The sequence parameter set doesn’t include scaling list data. One of the picture parameter set doesn’t include scaling list data. In other picture parameter sets, different scaling lists data is included. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 1.

- Functional stage:  Test the reconstruction process of scaling list. Tests switching of default scaling list and scaling list in picture parameter set.

Purpose:  Check that decoder can properly decode slices of coded frames with scaling list. Check that decoder can properly reconstruct scaling list with all coding modes of scaling list. Check that default scaling list is applied when no scaling list data is present. Check that decoder properly handle switching of default scaling list and scaling list data in picture parameter set.

#### Test bitstreams #SLIST_D

Specification:  All slices are coded as I or B slices. Each picture contains one slice. More than one sequence parameter set and more than one picture parameter sets are included. One of the sequence parameter set doesn’t includes scaling list data. One of the picture parameter set doesn’t include scaling list data. In other sequence parameter sets and picture parameter sets, different scaling lists data is included. In each picture, picture parameter set is override. scaling_list_enabled_flag is set equal to 0 or 1.

- Functional stage:  Test the reconstruction process of scaling list. Tests switching of scaling list off, default scaling list and scaling list in parameter sets.

Purpose:  Check that decoder can properly decode slices of coded frames with scaling list. Check that decoder can properly reconstruct scaling list with all coding modes of scaling list. Check that appropriate scaling list in sequence parameter set and picture parameter set is applied with multiple sequence parameter sets and picture parameter sets.

### Test bitstreams – Deblocking filter

#### Test bitstreams #DBLK_A

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. More than one picture parameter sets are included. QP is set randomly from 22 to 51. pps_beta_offset_div2 is randomly set in each picture from -6 to 6. slice_beta_offset_div2 and slice_tc_offset_div2 are randomly set in each slice from -6 to 6.

- Functional stage:  Test the deblocking filter process.

Purpose:  Check that decoder can properly decode slices with various combinations of deblocking filter control parameters.

#### Test bitstreams #DBLK_B

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. More than one picture parameter sets are included. pps_cb_qp_offset and pps_cr_qp_offset are randomly set from -12 to 12. slice_cb_qp_offset and slice_cr_qp_offset are randomly set from -4 to 4.

- Functional stage:  Test the deblocking filter process.

Purpose:  Check that decoder can properly handles deblocking filter with various combinations of QP.

#### Test bitstreams #DBLK_C

Specification:  All slices are coded as I, P or B slices. Each picture contains more than one slice. pps_disable_deblocking_filter_flag is set equal to 0. slice_disable_deblocking_filter_flag is randomly set equal to 0 or 1.

- Functional stage:  Test the deblocking filter process.

Purpose:  Check that decoder can properly handles deblocking filter with random deblocking filter on/off in each slice.

    - Test bitstreams – Sample adaptive offset

#### Test bitstreams #SAO_A

Specification:  All slices are coded as I or B slices.

[To be fill]

Functional stage:  Test the reconstruction process of sample adaptive offset.

Purpose:  Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with random SAO merge left/up flag values.

#### Test bitstreams #SAO_B

Specification:  All slices are coded as I or B slices.

[To be fill]

Functional stage:  Test the reconstruction process of sample adaptive offset.

Purpose:  Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with tiles, and randomely enabled SAO for luma and/or SAO for chroma per slice.

#### Test bitstreams #SAO_C

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. All SAO offset values in this bitstream have maximum allowed magnitude 7 and random sign.

Functional stage:  Test the reconstruction process of sample adaptive offset.

Purpose:  Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with extreme SAO offset values.

#### Test bitstreams #SAO_D

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. SAO offset values in this bitstream have random values in range -7..7.

Functional stage:  Test the reconstruction process of sample adaptive offset.

Purpose:  Check that decoder can properly decode with sample adaptive offset. Check that decoder can properly decode with random SAO offset values.

#### Test bitstreams #SAO_E

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. A set of SAO parameters is associated to each CTB for all frames. This means that no SAO merge flag (up or left) is used. Only the Band Offset (BO) SAO type is used and the 4 BO offsets are forced to be -7 or 7 in a random way. The Coded Tree Block size is set to 16x16.

Functional stage:  Tests loading of maximum SAO information at CTB level and frame.

Purpose:  Check that decoder can properly decode slices of coded frames with full SAO information.

#### Test bitstreams #SAO_F

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. A set of SAO parameters is associated to each CTB for all frames. This means that no SAO merge flag (up or left) is used. Only the Band Offset (BO) SAO type is used and the 4 BO offsets are forced to be -7 or 7 in a random way. The Coded Tree Block size is set to 32x32.

Functional stage:  Tests loading of maximum SAO information at CTB level and frame.

Purpose:  Check that decoder can properly decode slices of coded frames with full SAO information.

#### Test bitstreams #SAO_G

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. A set of SAO parameters is associated to each CTB for all frames. This means that no SAO merge flag (up or left) is used. Only the Band Offset (BO) SAO type is used and the 4 BO offsets are forced to be -7 or 7 in a random way. The Coded Tree Block size is set to 64x64.

Functional stage:  Tests loading of maximum SAO information at CTB level and frame.

Purpose:  Check that decoder can properly decode slices of coded frames with full SAO information.

    - Test bitstreams – Entropy coding

#### Test bitstreams #MAXBINS_A

Specification:  All slices are coded as I slices. Each picture contains only one slice. The number of bins per LCU is reached within 95% of the maximum number which is 4096 bits per 16x16 LCU. pcm_enabled_flag is set equal to 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that decoder can properly decode slices with the maximum number of bins per LCU.

#### Test bitstreams #MAXBINS_B

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. The number of bins per LCU is reached within 95% of the maximum number which is 4096 bits per 16x16 LCU. pcm_enabled_flag is set equal to 1. Low delay configuration.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that decoder can properly decode slices with the maximum number of bins per LCU.

#### Test bitstreams #MAXBINS_C

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. The number of bins per LCU is reached within 95% of the maximum number which is 4096 bits per 16x16 LCU. pcm_enabled_flag is set equal to 1. Random access configuration.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that decoder can properly decode slices with the maximum number of bins per LCU.

#### Test bitstreams #CAINIT_A

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is equal to 0 in picture parameter set.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check whether setting cabac_init_present_flag to 0 disables transmission of cabac_init_flag in slice header referring to the picture parameter set.

#### Test bitstreams #CAINIT_B

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is  equal to 1 in picture parameter set. cabac_init_flag is signaled for B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that the CABAC initialization type is correctly determined based on cabac_init_flag as follows:

if( slice_type = = I )

initType =  0

if( slice_type = = B )

initType = cabac_init_flag ? 1 : 2

#### Test bitstreams #CAINIT_C

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is  equal to 1 in picture parameter set. cabac_init_flag is signaled for P slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check

that the CABAC initialization type is correctly determined based on cabac_init_flag as follows:

if( slice_type = = I )

initType =  0

if( slice_type = = P )

initType = cabac_init_flag ? 2 : 1

#### Test bitstreams #CAINIT_D

Specification:  All slices are coded as I or B slices which are uni-directionally predicted. Each picture contains only one slice. There is one picture parameter set. cabac_init_present_flag is  equal to 1 in picture parameter set. cabac_init_flag is signaled for B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that the CABAC initialization type is correctly determined based on cabac_init_flag as follows:

if( slice_type = = I )

initType =  0

if( slice_type = = B )

initType = cabac_init_flag ? 1 : 2

#### Test bitstreams #CAINIT_E

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. Each slice contains 4 tiles (2 columns of tiles and 2 rows of tiles with uniform spacing). There is one picture parameter set. cabac_init_present_flag is  equal to 1 in picture parameter set. cabac_init_flag is signaled for P slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that the CABAC initialization type is correctly determined based on cabac_init_flag in presence of four tiles as follows:

if( slice_type = = I )

initType =  0

if( slice_type = = P )

initType = cabac_init_flag ? 2 : 1

#### Test bitstreams #CAINIT_F

Specification:  All slices are coded as I or uni-directionally predicted B slices. Each picture contains only one slice. Each slice contains 4 tiles (2 columns of tiles and 2 rows of tiles with uniform spacing). There is one picture parameter set. cabac_init_present_flag is  equal to 1 in picture parameter set. cabac_init_flag is signaled for uni-directionally predicted B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that the CABAC initialization type is correctly determined based on cabac_init_flag in presence of four tiles as follows:

if( slice_type = = I )

initType =  0

if( slice_type = = B )

initType = cabac_init_flag ? 1 : 2

#### Test bitstreams #CAINIT_G

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. Each slice contains multple dependent slices. Each dependent slice contains 3 CTBs or less. There is one picture parameter set. cabac_init_present_flag is  equal to 1 in picture parameter set. cabac_init_flag is signaled for P slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that the CABAC initialization type is correctly determined based on cabac_init_flag in presence of dependent slices as follows:

if( slice_type = = I )

initType =  0

if( slice_type = = P )

initType = cabac_init_flag ? 2 : 1

#### Test bitstreams #CAINIT_H

Specification:  All slices are coded as I or uni-directionally predicted B slices. Each picture contains only one slice. Each slice contains multple dependent slices. Each dependent slice contains 3 CTBs or less. There is one picture parameter set. cabac_init_present_flag is  equal to 1 in picture parameter set. cabac_init_flag is signaled for uni-directionally predicted B slices in the slice header referring the picture parameter set. cabac_init_flag can take on values 0 or 1.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Check that the CABAC initialization type is correctly determined based on cabac_init_flag in presence of dependent slices as follows:

if( slice_type = = I )

initType =  0

if( slice_type = = B )

initType = cabac_init_flag ? 1 : 2

#### Test bitstreams #SDH_A

Specification:

A. sign_data_hiding_flag is on:

At least 100 CGs have lastSigScanPos-firstSigScanPos = 3.

At least 2000 CGs have lastSigScanPos-firstSigScanPos < 3.

At least 1000 CGs have lastSigScanPos-firstSigScanPos > 3 and infer the sign bit to be positive.

At least 1000 CGS have lastSigScanPos-firstSigScanPos > 3 and infer the sign bit to be negative.

o	Testing position of CG with SDH:

Sign inference happens for all possible combinations of (TU size, CG position in TU) at least once in the frame.

o	Testing significance positions:

All possible combinations for (lastSigScanPos, firstSigScanPos ) appear at least once in the frame.

o	Testing the correctness of the inference of the sign (odd/even) and testing if the calculation of the sum creates

overflows (e.g. implementations with 8, 16 or 32 bits adder):

*	4x4 TU

1.	lastSigScanPos-firstSigScanPos = 3, 4 combinations of (first sign bit, sum parity): (negative, even), (positive, odd).

2.	lastSigScanPos-firstSigScanPos < 3, 4 combinations of (first sign bit, sum parity) : (negative, even), (positive, odd).

3.	lastSigScanPos-firstSigScanPos > 3, first sign bit even, sum of signed values include at least

(0, ｱ2, ｱ4, ｱ6, ｱ8, ｱ10, ｱ12, ｱ14, ｱ16, ｱ32, ｱ64, ｱ128, ｱ256, ｱ512, ｱ1024, ｱ2048, ｱ4096, ｱ8192, ｱ16384)

4.	lastSigScanPos-firstSigScanPos  > 3, first sign bit odd, sum of signed values include at least

(ｱ1, ｱ3, ｱ5, ｱ7, ｱ9, ｱ11, ｱ13, ｱ15, ｱ17, ｱ33, ｱ129, ｱ257, ｱ513, ｱ1025, ｱ2049, ｱ4097, ｱ8193, ｱ16385)

5.	One CG with all coefficients to maximum absolute value and sign inference.

6.	One CG with all coefficients to minimum absolute value and sign inference.

*	8x8 TU

1.	Repeat the test cases for 4x4 TU for all 4 CGs.

*	16x16 TU

1.	Repeat the test cases for 4x4 TU for all 16 CGs.

*	32x32 TU

1.	Repeat the test cases for 4x4 TU for all 64 CGs.

B. sign_data_hiding_flag is on:

At least 500 CGs have lastSigScanPos-firstSigScanPos > 3  and the CU they belong to has cu_ transquant_bypass _flag=1.

At least 500 CGs have lastSigScanPos-firstSigScanPos > 3  and the CU they belong to has cu_ transquant_bypass _flag=0.

Functional stage:  Test the reconstruction process of entropy coding.

Purpose:  Test SDH sign inference, and test SDH in conjunction with cu_ transquant_bypass _flag.

    - Test bitstreams – Temporal scalability

### Test bitstreams – Parallel processing tools

#### Test bitstreams #TILES_A

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. num_tiles_columns_minus1 and num_tiles_rows_minus1 are set equal to 4, which is the maximum value for level 4.1. uniform_spacing_flag is set equal to 0. The values of column_width_minus1[i] and row_height_minus1[i] are set randomly for each picture. loop_filter_across_tiles_enabled_flag is set randomly for each picture.

Functional stage:  Test dependency breaks at tile boundaries.

Purpose:  Test random non-uniform tile spacing with maximum number of tiles and disabling/enabling of deblocking filter across tile boundaries.

      - Test bitstreams #WPP_A

Specification:  entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,   QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_B

Specification:  entropy_coding_sync_enabled_flag is set to 1. A CTU size of 32x32 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,   QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_C

Specification:  entropy_coding_sync_enabled_flag is set to 1. A CTU size of 16x16 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,   QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_D

Specification:  entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 1 CTU wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised when a picture is 1 CTU wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 1 CTU wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_E

Specification:  entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 2 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised when a picture is 2 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 2 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_F

Specification:  entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 3 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised when a picture is 3 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 3 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #ENTP_A

Specification:  All slices are coded as I or Bslices. Each picture contains only one slice. num_tiles_columns_minus1 and num_tiles_rows_minus1 are set equal to 2. uniform_spacing_idc is set equal to 1.

- Functional stage:  Test entry point signaling.

Purpose:  Check that decoder can correctly handles entry point signalling in slice header when tile is used.

#### Test bitstreams #ENTP_B

Specification:  All slices are coded as I or Bslices. Each picture contains only one slice. WaveFrontSynchro is set equal to 1.

- Functional stage:  Test entry point signaling.

Purpose:  Check that decoder can correctly handles entry point signalling in slice header when WPP is used.

#### Test bitstreams #ENTP_C

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. WaveFrontSynchro is set equal to 1. At the end of some random substreams, redundant bytes with emulation prevention bytes (0x00000300) are added.

- Functional stage:  Test entry point signaling.

Purpose:  Test entry point signalling in slice header when WPP is used and when emulation prevention bytes occur in substream(s).

    - Test bitstreams – Other coding tools

#### Test bitstreams #IPCM_A

Specification:  All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 7. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 2 and 0, respectively.

- Functional stage:  Test parsing of pcm_flags in Coding unit syntax.

Purpose:  Check that decoder can correctly decode slices of coded pictures containing pcm_flags.

#### Test bitstreams #IPCM_B

Specification:  All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 5. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 1 and 0, respectively.

- Functional stage:  Test parsing of pcm_flags in Coding unit syntax. Test parsing of pcm_sample_luma and pcm_sample_chroma data in PCM sample syntax.

Purpose:  Check that decoder can correctly decode slices of coded pictures containing pcm_flags, and pcm_sample_luma and pcm_sample_chroma data.

#### Test bitstreams #IPCM_C

Specification:  All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 7. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 1 and 1, respectively.

- Functional stage:  Test parsing of pcm_flags in Coding unit syntax. Test parsing of pcm_sample_luma and pcm_sample_chroma data in PCM sample syntax. Test skipping of loop filtering on samples associated with pcm_flag equal to 1.

Purpose:  Check that decoder can correctly decode slices of coded pictures containing pcm_flags, and pcm_sample_luma and pcm_sample_chroma data. Check that decoder can skip loop filtering on samples associated with pcm_flag equal to 1.

#### Test bitstreams #IPCM_D

Specification:  All slices are coded as I slices. Each picture contains only one slice. pcm_enabled_flag is equal to 1. Both pcm_sample_bit_depth_luma_minus1 and pcm_sample_bit_depth_chroma_minus1 are equal to 7. log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_coding_block_size, and pcm_loop_filter_disable_flag are equal to 0, 1 and 0, respectively. transquant_bypass_enable_flag is equal to 1.

- Functional stage:  Test parsing of pcm_flags in Coding unit syntax. Test parsing of pcm_sample_luma and pcm_sample_chroma data in PCM sample syntax. Test skipping of loop filtering on samples associated with both cu_transquant_bypass_flag and pcm_flag equal to 1.

Purpose:  Check that decoder can correctly decode slices of coded pictures containing pcm_flags, and pcm_sample_luma and pcm_sample_chroma data. Check that decoder can skip loop filtering on samples associated with both cu_transquant_bypass_flag and pcm_flag equal to 1.

#### Test bitstreams #TS_A

Specification:  Each picture contains only one slice. transform_skip_enabled_flag is set to 1 for picture 0-8 and 0 for picture 9-16.

- Functional stage:  Test reconstruction process of slices with transform_skip_enabled_flag is equal to 1.

Purpose:  Check if transform_skip_enabled_flag is parsed and transform skipping functions correctly.

pecification:

#### Test bitstreams #AMP_A

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. All AMP modes (2NxnU, 2NxND, nLx2N, nRx2N) are included.

- Functional stage:  Test reconstruction process of slices with amp_enabled_flag equal to 1.

Purpose:  Check if decoder properly decodes slices with all AMP modes.

#### Test bitstreams #AMP_B

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. AMP is only utilized for PUs of which size is larger than minimum CU.

- Functional stage:  Test reconstruction process of slices with amp_enabled_flag equal to 1.

Purpose:  Check if decoder properly decodes slices with a constraint of AMP.

### Test bitstreams – High level syntax

#### Test bitstreams #RPS_A

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. The slice header includes the inter-RPS prediction method for sending the RPS for short-term pictures. The last 3 frames of this 44 frames sequence contain Slice header RPS using the inter-RPS in addition to the RPS sent in the PPS.

- Functional stage:  Test reconstruction process of slices with inter-RPS prediction.

Purpose:  Check if decoder properly decodes slices with inter-RPS prediction method for sending the RPS for short-term pictures in the slice header.

#### Test bitstreams #RPS_B

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. The bitstream includes random RPS signaling in slice header along with random picture coding order within a GOP.

- Functional stage:  Test reconstruction process of slices without inter-RPS prediction.

Purpose:  Check if decoder properly decodes slices with the inter-RPS prediction method for sending the RPS for short-term pictures in the slice header.

#### Test bitstreams #RPLM_A

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. The bitstream includes random reference picture list modification with varying list sizes.

- Functional stage:  Test reconstruction process of slices with reference list modification.

Purpose:  Check if decoder properly decodes slices with reference list modification.

#### Test bitstreams #RPLM_B

Specification:  All slices are coded as I or B slices. Each picture contains more than one slice. The bitstream includes random reference picture list modification with varying list sizes.

- Functional stage:  Test reconstruction process of slices with reference list modification.

Purpose:  Check if decoder properly decodes slices with reference list modification.

#### Test bitstreams #DSLICE_A

Specification:  To be added

- Functional stage:  Test reconstruction process of independent and dependent slice segments.

Purpose:  Check if decoder properly decodes independent and dependent slice segments.

#### Test bitstreams #DSLICE_B

Specification:  To be added

- Functional stage:  Test reconstruction process of dependent slice segments.

Purpose:  Check if decoder properly decodes dependent slice segments in combination with substreams (WPP).

#### Test bitstreams #DSLICE_C

Specification:  To be added

- Functional stage:  Test reconstruction process of dependent slice segments.

Purpose:  Check if decoder properly decodes dependent slice segments in combination with tiles.

#### Test bitstreams #RAP_A

Specification:  All slices are coded as I or B slices. Each picture contains only one slice. The first picture in the bitstream is CRA and is followed by 7 RASL pictures that are not decodable. There are two subsequent CRA pictures with RASL pictures, following the first CRA picture in this bitstream. These subsequent RASL pictures should be decodable since the associated CRA is not the first CRA picture in the bitstream.

- Functional stage:  Test reconstruction process starting with a CRA picture followed by RASL pictures that cannot be decoded of slices with the inter RPS prediction.

Purpose:  Check if decoder properly decodes a conforming bitstream where the CRA is the first picture in the bitstream and is followed by RASL pictures that are not decodable.

#### Test bitstreams #HRD_A

Specification:

Each AU contains 4 DUs.

sub_pic_cpb_params_present_flag = 1

du_common_cpb_removal_delay_flag = 0

- Functional stage:  Test the DU-based HRD.

Purpose:  Check that decoder can properly decode with DU-based CPB removal time signaling.

### Test bitstreams – 10 bit

#### Test bitstreams #WP_A_MAIN10

Specification:  All slices are coded as I or P slices. Each picture contains only one slice. weighted_pred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage:  Weighted sample prediction process for P slices with plural reference indices.

Purpose:  Check that the decoder handles weighted sample prediction for P slices with plural reference indices.

#### Test bitstreams #WP_B_MAIN10

Specification:  All slices are coded as I, P or B slices. Each picture contains only one slice. weighted_pred_flag is equal to 1 and weighted_bipred_flag is equal to 1. Plural reference indices are assigned to each reference picture.

- Functional stage:  Weighted sample prediction process for P and B slices with plural reference indices.

Purpose:  Check that the decoder handles weighted sample prediction for P and B slices with plural reference indices.

#### Test bitstreams #WPP_A_MAIN10

Specification:  bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,   QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_B_MAIN10

Specification:  bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 32x32 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,   QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_C_MAIN10

Specification:  bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 16x16 is used to encode the sequence. The sequence contains six GOPs, which are all 8 pictures long. The first three GOPs have pictures with the following characteristics:

1.	One slice in the frame, constant QP

2.	One slice in the frame, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

3.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long, constant QP

4.	Maximum number of independent slice segments in the frame, at least one slice segment is 1 CTU long, at least one slice segment is 2 CTUs long,   QP of each TU is set to a value such that abs(QP - QPSlice)>2.

5.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, constant QP

6.	Maximum number of dependent slice segments in the frame, at least one dependent slice segment is 1 CTU long, at least one dependent slice segment is 2 CTUs long, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

7.	Random combination of independent/dependent slice segments, constant QP

8.	Random combination of independent/dependent slice segments, QP of each TU is set to a value such that abs(QP - QPSlice)>2.

The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. The final three GOPs feature a mixture of single slice pictures, and pictures coded using multiple slices and multiple slice segments. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation with and without different types of slice. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_D_MAIN10

Specification:  bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 1 CTU wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised when a picture is 1 CTU wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 1 CTU wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_E_MAIN10

Specification:  bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 2 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised when a picture is 2 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 2 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

#### Test bitstreams #WPP_F_MAIN10

Specification:  bit_depth_luma_minus8 is set equal to 2 and bit_depth_chroma_minus8 is set equal to 2. entropy_coding_sync_enabled_flag is set to 1. A CTU size of 64x64 is used to encode the sequence. The picture is 3 CTUs wide. The sequence contains six GOPs, which are all 8 pictures long. The GOPs are encoded with varying numbers of slices and slice segments. Even frames have fixed QP, while the QP in odd frames is set so that abs(QP - QPSlice)>2. The first GOP is coded using all Intra CUs, the second GOP is coded with skip disabled. Random amounts of slice extension bytes are encoded in each slice header.

Functional stage:  Tests that entropy coding is correctly synchronised when a picture is 3 CTUs wide. Tests that the QP predictor is reset to QPslice at the beginning of every CTU row. May be used to test handling of entry points by a parallel decoder.

Purpose:  The bitstream checks that a decoder can correctly perform entropy coding synchronisation when a picture is 3 CTUs wide. It checks that a decoder can correctly derive QP predictors when entropy_coding_sync_enabled_flag is set to 1. It can also be used to check that a decoder can correctly handle entry points when slice header extensions are used.

## Normative test suites for ITU-T H.HEVC | ISO/IEC 23008-2

Legend:

- X – Bitstream is for static and dynamic test

| Categories | Sub category |  | Bitstream | Donated by | File name | Main | Main 10 | Main still picture | Level | Frame rate (Frames/sec) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block structure | Block structure and partitioning |  | STRUCT_A | Samsung | STRUCT_A_Samsung_2 | X | X |  |  |  |
|  |  |  | STRUCT_B | Samsung | STRUCT_B_Samsung_1 | X | X |  |  |  |
| Intra coding | Intra prediction |  | IPRED_A | Docomo | IPRED_A_docomo_1 | X | X |  | 3.0 and higher | 30 |
|  |  |  | IPRED_B | Nokia | IPRED_B_Nokia_1 | X | X | X | 4.0 and higher | N/A |
|  |  |  | IPRED_C | Mitsubishi | IPRED_C_Mitsubishi_1 | X | X |  |  |  |
|  | Constraint intra prediction |  | CIP_A | Panasonic | CIP_A_Panasonic_1 | X | X |  |  |  |
|  |  |  | CIP_B | NEC | CIP_B_NEC_1 | X | X |  | 2.0 and higher | 30 |
| Inter frame coding | Merge |  | MERGE_A | TI | MERGE_A_TI_1 | X | X |  |  |  |
|  |  |  | MERGE_B | TI | MERGE_B_TI_1 | X | X |  |  |  |
|  |  |  | MERGE_C | TI | MERGE_C_TI_1 | X | X |  |  |  |
|  |  |  | MERGE_D | TI | MERGE_D_TI_1 | X | X |  |  |  |
|  |  |  | MERGE_E | TI | MERGE_E_TI_1 | X | X |  |  |  |
|  |  |  | MERGE_F | Media Tek | MERGE_F_MTK_1 | X | X |  |  |  |
|  |  |  | MERGE_G | HHI | MERGE_G_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  | Parallel merge |  | PMERGE_A | TI | PMERGE_A_TI_1 | X | X |  |  |  |
|  |  |  | PMERGE_B | TI | PMERGE_B_TI_1 | X | X |  |  |  |
|  |  |  | PMERGE_C | TI | PMERGE_C_TI_1 | X | X |  |  |  |
|  |  |  | PMERGE_D | TI | PMERGE_D_TI_1 | X | X |  |  |  |
|  |  |  | PMERGE_E | TI | PMERGE_E_TI_1 | X | X |  |  |  |
|  | AMVP |  | AMVP_A | Media Tek | AMVP_A_MTK_1 | X | X |  |  |  |
|  |  |  | AMVP_B | Media Tek | AMVP_B_MTK_1 | X | X |  |  |  |
|  |  |  | AMVP_C | Samsung | AMVP_C_Samsung_1 | X | X |  |  |  |
|  | Temporal MVP |  | TMVP_A | Microsoft | TMVP_A_MS_1 | X | X |  |  |  |
|  | mvd_l1_zero |  | MVDL1ZERO_A | Docomo | MVDL1ZERO_A _Docomo_1 | X | X |  |  |  |
|  | MVP clipping |  | MVCLIP_A | Qualcomm | MVCLIP_A_qualcomm_1 | X | X |  |  |  |
|  | MV pointing to picture edge |  | MVEDGE_A | Qualcomm | MVEDGE_A_qualcomm_1 | X | X |  |  |  |
|  | Weighted prediction |  | WP_A | Toshiba | WP_A_Toshiba_2 | X | X |  |  |  |
|  |  |  | WP_B | Toshiba | WP_B_Toshiba_1 | X | X |  |  |  |
| Transform and quantization | RQT |  | RQT_A | HHI | RQT_A_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | RQT_B | HHI | RQT_B_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | RQT_C | HHI | RQT_C_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | RQT_D | HHI | RQT_D_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | RQT_E | HHI | RQT_E_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | RQT_F | HHI | RQT_F_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | RQT_G | HHI | RQT_G_HHI_1 | X | X |  | 3.1 and higher | 60 |
|  | Delta QP |  | DELTAQP_A | Broadcom | DELTAQP_A_BRCM_2 | X | X |  |  |  |
|  |  |  | DELTAQP_B | Sony | DELTAQP_B_Sony_1 | X | X |  | 3.0 and higher | 30 |
|  |  |  | DELTAQP_C | Sony | DELTAQP_C_Sony_1 | X | X |  | 3.0 and higher | 30 |
|  | Scaling list |  | SLIST_A | Sony | SLIST_A_Sony_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | SLIST_B | Sony | SLIST_B_Sony_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | SLIST_C | Sony | SLIST_C_Sony_1 | X | X |  | 3.1 and higher | 60 |
|  |  |  | SLIST_D | Sony | SLIST_D_Sony_2 | X | X |  | 3.1 and higher | 60 |
| In-loop filter | Deblocking filter |  | DBLK_A | Sony | DBLK_A_Sony_1 | X | X |  | 3.0 and higher | 30 |
|  |  |  | DBLK_B | Sony | DBLK_B_Sony_1 | X | X |  | 3.0 and higher | 30 |
|  |  |  | DBLK_C | Sony | DBLK_C_Sony_1 | X | X |  | 3.0 and higher | 30 |
|  | Sample adaptive offset |  | SAO_A | Media Tek | SAO_A_MediaTek_1 | X | X |  |  |  |
|  |  |  | SAO_B | Media Tek | SAO_B_MediaTek_1 | X | X |  |  |  |
|  |  |  | SAO_C | Samsung | SAO_C_Samsung_1 | X | X |  |  |  |
|  |  |  | SAO_D | Samsung | SAO_D_Samsung_1 | X | X |  |  |  |
|  |  |  | SAO_E | Canon | SAO_E_Canon_2 | X | X |  |  |  |
|  |  |  | SAO_F | Canon | SAO_F_Canon_1 | X | X |  |  |  |
|  |  |  | SAO_G | Canon | SAO_G_Canon_1 | X | X |  |  |  |
| Entropy coding | Maximum bins |  | MAXBINS_A | TI | MAXBINS_A_TI_2 | X | X |  |  |  |
|  |  |  | MAXBINS_B | TI | MAXBINS_B_TI_2 | X | X |  |  |  |
|  |  |  | MAXBINS_C | TI | MAXBINS_C_TI_2 | X | X |  |  |  |
|  | CABAC initialization |  | CAINIT_A | Sharp | CAINIT_A_Sharp_1 | X | Ｘ |  |  |  |
|  |  |  | CAINIT_B | Sharp | CAINIT_B_Sharp_1 | X | Ｘ |  |  |  |
|  |  |  | CAINIT_C | Sharp | CAINIT_C_Sharp_1 | X | Ｘ |  |  |  |
|  |  |  | CAINIT_D | Sharp | CAINIT_D_Sharp_1 | X | Ｘ |  |  |  |
|  |  |  | CAINIT_E | Sharp | CAINIT_E_Sharp_1 | X | Ｘ |  |  |  |
|  |  |  | CAINIT_F | Sharp | CAINIT_F_Sharp_1 | X | Ｘ |  |  |  |
|  |  |  | CAINIT_G | Sharp | CAINIT_G_Sharp_1 | X | Ｘ |  |  |  |
|  |  |  | CAINIT_H | Sharp | CAINIT_H_Sharp_1 | X | Ｘ |  |  |  |
|  | Sign data hiding |  | SDH_A | Orange | SDH_A_Orange_1 | X | X |  |  |  |
| Temporal scalability |  |  |  |  |  |  |  |  |  |  |
| Parallel processing tools | Tiles |  | TILES_A | Cisco | TILES_A_Cisco_1 | X | X |  | 4.1 and higher | 60 |
|  | Wavefront parallel processing |  | WPP_A | Ericsson | WPP_A_Ericsson_1 | X | X |  |  |  |
|  |  |  | WPP_B | Ericsson | WPP_B_Ericsson_1 | X | X |  |  |  |
|  |  |  | WPP_C | Ericsson | WPP_C_Ericsson_1 | X | X |  |  |  |
|  |  |  | WPP_D | Ericsson | WPP_D_Ericsson_1 | X | X |  |  |  |
|  |  |  | WPP_E | Ericsson | WPP_E_Ericsson_1 | X | X |  |  |  |
|  |  |  | WPP_F | Ericsson | WPP_F_Ericsson_1 | X | X |  |  |  |
|  | Entry point |  | ENTP_A | LG | ENTP_A_LG_1 | X | X |  | 3.0 and higher | 60 |
|  |  |  | ENTP_B | LG | ENTP_A_LG_1 | X | X |  | 3.0 and higher | 60 |
|  |  |  | ENTP_C | LG | ENTP_C_LG_2 | X | X |  | 3.0 and higher | 60 |
| Other coding tools | IPCM |  | IPCM_A | NEC | IPCM_A_NEC_1 | X | X |  | 2.0 and higher | 30 |
|  |  |  | IPCM_B | NEC | IPCM_B_NEC_1 | X | X |  | 2.0 and higher | 30 |
|  |  |  | IPCM_C | NEC | IPCM_C_NEC_1 | X | X |  | 2.0 and higher | 30 |
|  |  |  | IPCM_D | NEC | IPCM_D_NEC_1 | X | X |  | 2.0 and higher | 30 |
|  | Transform skip |  | TS_A | Microsoft | TS_A_MS_1 | X | X |  |  |  |
|  | AMP |  | AMP_A | Samsung | AMP_A_Samsung_1 | X | X |  |  |  |
|  |  |  | AMP_B | Samsung | AMP_B_Samsung_1 | X | X |  |  |  |
| High level syntax | RPS |  | RPS_A | Docomo | RPS_A_Docomo_1 | X | X |  |  |  |
|  |  |  | RPS_B | Qualcomm | RPS_B_qualcomm_1 | X | X |  |  |  |
|  | Reference picture list modification |  | RPLM_A | Qualcomm | RPLM_A_qualcomm_1 | X | X |  |  |  |
|  |  |  | RPLM_B | Qualcomm | RPLM_B_qualcomm_1 | X | X |  |  |  |
|  | Dependent slice |  | DSLICE_A | HHI | DSLICE_A_HHI_1 | X | X |  |  |  |
|  |  |  | DSLICE_B | HHI | DSLICE_B_HHI_1 | X | X |  |  |  |
|  |  |  | DSLICE_C | HHI | DSLICE_C_HHI_1 | X | X |  |  |  |
|  | Random access |  | RAP_A | Docomo | RAP_A_Docomo_1 | X | X |  |  |  |
|  | HRD |  | HRD_A | Fujitsu | HRD_A_Fujitsu_1 | X | X |  |  |  |
| 10 bit |  |  |  |  |  |  |  |  |  |  |
|  | Weighted prediction |  | WP_A_ MAIN10 | Toshiba | WP_A_MAIN10_ Toshiba_1 |  | X |  |  |  |
|  |  |  | WP_B_ MAIN10 | Toshiba | WP_B_MAIN10_ Toshiba_2 |  | X |  |  |  |
|  | Wavefront parallel processing |  | WPP_A_ MAIN10 | Ericsson | WPP_A_MAIN10_ Ericsson_1 |  | X |  |  |  |
|  |  |  | WPP_B_ MAIN10 | Ericsson | WPP_B_MAIN10_ Ericsson_1 |  | X |  |  |  |
|  |  |  | WPP_C_ MAIN10 | Ericsson | WPP_C_MAIN10_ Ericsson_1 |  | X |  |  |  |
|  |  |  | WPP_D_ MAIN10 | Ericsson | WPP_D_MAIN10_ Ericsson_1 |  | X |  |  |  |
|  |  |  | WPP_E_ MAIN10 | Ericsson | WPP_E_MAIN10_ Ericsson_1 |  | X |  |  |  |
|  |  |  | WPP_F_ MAIN10 | Ericsson | WPP_F_MAIN10_ Ericsson_1 |  | X |  |  |  |

This Recommendation | International Standard includes an electronic attachment containing the conformance bitstreams identified within the text. The bitstreams can also be downloaded from the ITU-T Test Signal Database.