| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0159 |
| --- | --- |

| Title: | CE1: Cross-verification of CE1 Test 3 square transforms for 4:2:2 chroma format coding (JCTVC-L0148, L0333) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In CE1, the usage of square or non-square transforms for the chroma components in 4:2:2 chroma format is studied.

This contribution reports cross-verification results of CE1 Test 3 square transform for 4:2:2 chroma format coding.

The BD-rate results tested by the cross-checker perfectly match those provided by the proponents.

# Introduction

In CE1 [1], the usage of square or non-square transforms for the chroma components in 4:2:2 chroma format is studied.

This contribution reports cross-verification results of CE1 Test 3 square transform for 4:2:2 chroma format coding.

# Experiments

## Tests and software

Tests are to be done for the 4:2:2 case by enabling square or non-square transforms for the chroma components. Following configurations are to be tested [1]:

- Anchor: 
Non-square transforms: method presented in JCTVC-K0181
Revision 3055 of the common software (based upon HM-9.0.1) with range extension support [2] is used.

- CE1 Test 3: 
Square transforms: disallow non-square transforms for all PU/TU sizes in JCTVC-K0171
Revision 3055 of the common software (based upon HM-9.0.1) with range extension support is modified by Canon and Qualcomm.
The following two settings are tested.

  - JCTVC-L0148 provided by Cannon [3]: 
transform skip is enabled and disabled.

  - JCTVC-L0333 provided by Qualcomm [4]: 
RDOQ is enabled and disabled.

## Test conditions

The common test conditions for HEVC range extensions [2] are used.

## Experimental environment

The experimental environment is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

    - Simulation results of JCTVC-L0148

Table 2 and Table 3 present the simulation results for JCTVC-L0148.

Table 2 presents the simulation results of the 4:2:2 chroma format in the case that transform skip is enable on anchor test (non-square transforms) and transform skip is disabled on CE1 Test 3 (square transforms).

Table 2 Simulation results for CE1 Test 3 (JCTVC-L0148 Cannon) : transform skip is disabled

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.9% | 3.0% | 4.6% | 0.7% | 1.8% | 2.5% |
| Enc Time[%] | 87.8% |  |  | 86.8% |  |  |
| Dec Time[%] | 97.5% |  |  | 98.0% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 1.0% | 2.4% | 3.3% | 1.1% | 1.6% | 2.4% |
| Enc Time[%] | 94.2% |  |  | 92.5% |  |  |
| Dec Time[%] | 99.3% |  |  | 99.0% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 1.0% | 0.6% | 1.0% | 1.0% | 1.0% | 1.4% |
| Enc Time[%] | 95.0% |  |  | 93.3% |  |  |
| Dec Time[%] | 100.1% |  |  | 99.9% |  |  |

Table 3 presents the simulation results of the 4:2:2 chroma format in the case that transform skip is enable on anchor test (non-square transforms) and CE1 Test 3 (square transforms).

Table 3 Simulation results for CE1 Test 3 (JCTVC-L0148 Cannon) : transform skip is enbled

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.8% | 3.1% | 4.6% | 0.6% | 2.0% | 2.6% |
| Enc Time[%] | 103.8% |  |  | 103.5% |  |  |
| Dec Time[%] | 99.6% |  |  | 100.1% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.8% | 2.9% | 3.8% | 0.5% | 2.3% | 3.0% |
| Enc Time[%] | 101.2% |  |  | 101.1% |  |  |
| Dec Time[%] | 102.5% |  |  | 102.3% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.8% | 1.0% | 1.3% | 0.5% | 1.6% | 1.6% |
| Enc Time[%] | 101.1% |  |  | 101.0% |  |  |
| Dec Time[%] | 101.5% |  |  | 101.5% |  |  |

### Simulation results of JCTVC-L0333

Table 4 and Table 5 present the simulation results for JCTVC-L0333.

Table 4 presents the simulation results of the 4:2:2 chroma format in the case that transform skip is enabled on both anchor test (non-square transforms) and CE1 Test 3 (square transforms).

Table 4 Simulation results for CE1 Test 3 (JCTVC-L0333 Qualcomm) : transform skip is enabled

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.7% | 3.6% | 5.0% | 0.6% | 2.0% | 2.4% |
| Enc Time[%] | 101.6% |  |  | 101.6% |  |  |
| Dec Time[%] | 94.3% |  |  | 96.9% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.4% | 4.3% | 5.4% | 0.3% | 2.4% | 3.6% |
| Enc Time[%] | 100.2% |  |  | 100.0% |  |  |
| Dec Time[%] | 99.0% |  |  | 99.0% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.2% | 3.7% | 4.7% | 0.1% | 2.3% | 3.3% |
| Enc Time[%] | 100.0% |  |  | 99.9% |  |  |
| Dec Time[%] | 99.8% |  |  | 99.6% |  |  |

Table 5 presents the simulation results of the 4:2:2 chroma format in the case that transform skip is enabled and RDOQ is disabled on both anchor test (non-square transforms) and CE1 Test 3 (square transforms).

Table 5 Simulation results for CE1 Test 3 and RDOQ off (JCTVC-L0333 Qualcomm) : transform skip is enabled

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.5% | 3.3% | 5.4% | 0.3% | 1.8% | 3.1% |
| Enc Time[%] | 101.0% |  |  | 101.1% |  |  |
| Dec Time[%] | 94.9% |  |  | 97.5% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.4% | 4.6% | 5.7% | 0.3% | 2.5% | 4.1% |
| Enc Time[%] | 99.8% |  |  | 99.7% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.7% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| YCbCr 4:2:2 | 0.2% | 4.2% | 4.9% | 0.2% | 2.4% | 3.6% |
| Enc Time[%] | 99.8% |  |  | 99.7% |  |  |
| Dec Time[%] | 99.4% |  |  | 99.4% |  |  |

# Conclusions

The BD-rate results tested by the cross-checker perfectly match those provided by the proponents.

# References

[1] J. Sole, “HEVC Range Extensions Core Experiment 1: Square and non-square transforms for 4:2:2 chroma format coding”, JCTVC-K1121, Shanghai, October 2012

[2] D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-K1006, Shanghai, October 2012

[3] C. Rosewarne, M. Maeda, " CE1: Test 3 - Square transforms for Range Extensions ", JCTVC-L0148, Geneva, January 2013

[4] J. Sole, R. Joshi, M. Karczewics, " CE1: Test 3 - Square transform blocks for 4:2:2", JCTVC-L0333, Geneva, January 2013

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).