| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0163 |
| --- | --- |

| Title: | non-TEC5: Cross-verification of simplified TEC5 Test 5.1.5 Inter-layer intra prediction mode coding (ILIPM) (JCTVC-L0156) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

TEC5 (Tool Experiment C5) is Tool Experiment to study inter-layer syntax prediction using HEVC base layer. TE5.1 is Tool Experiment for inter-layer intra mode prediction.

In TE5.1, Test 5.1.5, inter-layer intra prediction mode coding (ILIPM) is proposed by Leibniz University Hannover. In addition, simplified methods of ILIPM are proposed by the proponent. This contribution reports the cross-verification results of the simplified ILIPM.

The BD-rate results tested by the cross-checker perfectly match those provided by the proponent.

# Introduction

TEC5 (Tool Experiment C5) is Tool Experiment to study inter-layer syntax prediction using HEVC base layer [1]. TE5.1 is Tool Experiment for inter-layer intra mode prediction.

In TE5.1, Test 5.1.5, inter-layer intra prediction mode coding (ILIPM) is proposed by Leibniz University Hannover [2]. In addition, simplified methods of ILIPM are proposed by the proponent [3]. This contribution reports the cross-verification results of the simplified ILIPM.

# Experiments

## Tests and software

Following configurations are to be tested [1]:

- Anchor: 
SMuC 0.1.1 (revision 7 of the common software with SHVC support) is used.

- Original ILIPM (TEC5 Test 5.1.5): 
Base layer based MPM coding and inter-layer intra prediction mode coding in JCTVC-K0238

  - Inter-layer intra prediction mode coding (ILIPM)
In this approach, a new mode, called inter-layer intra prediction mode (ILIPM), is added as an extra intra prediction mode for an intra coded CU in the enhancement layer. ILIPM is only evaluated if the current PU size of the enhancement layer is twice the co-located PU size of the base layer in terms of RD cost. In the IPILM mode, the current PU utilizes the intra prediction mode of the co-located PU in the base layer as its intra prediction mode. A flag, intra_bl_mode_flag, is introduced before prev_intra_luma_pred_flag in the coding unit syntax. Intra_bl_mode_flag is only coded in the coding unit syntax if the current PU size of the enhancement layer is twice the co-located PU size of the base layer. If intra_bl_mode_flag is true, prev_intra_luma_pred_flag and possible rem_intra_luma_pred_mode are not coded any more. If the current PU size of the enhancement layer is twice the co-located PU size of the base layer and IPILM is not chosen as the intra prediction mode for the current PU, intra_bl_mode_flag is set to false and MPM coding as in the single layer coding is performed.

  - with prediction unit (PU) size restriction and mode dependent coefficient scanning (MDCS)

- Simplified ILIPM with MDCS:

  - without PU size restriction and with MDCS

- Simplified ILIPM without MDCS:

  - without PU size restriction and MDCS

## Test conditions

The common test conditions for SHVC are used [1].

MD5 hash of each decoding picture is computed.

## Experimental environment

The experimental environment is presented in Table 1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 presents the simulation results of original ILIPM (TEC5 Test 5.1.5) with PU restriction and MDCS.

Table 2 Simulation results of Original ILIPM

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 103.0% |  |  | 102.8% |  |  |
| Dec Time[%] | 99.9% |  |  | 99.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3 presents the simulation results of simplified ILIPM without PU restriction and with MDCS.

Table 3 Simulation results of simplified ILIPM with MDCS

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.1% | 0.0% |  |  |  |
| Class B | -0.4% | -0.2% | -0.2% | -0.1% | 0.1% | 0.1% |
| Overall (EL+BL) | -0.3% | -0.2% | -0.2% | -0.1% | 0.1% | 0.1% |
| Overall (EL) | -0.6% | -0.3% | -0.2% | -0.4% | 0.2% | 0.2% |
| Enc Time[%] | 102.9% |  |  | 102.7% |  |  |
| Dec Time[%] | 100.8% |  |  | 100.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 4 presents the simulation results of ILIPM without PU size restriction and MDCS.

Table 4 Simulation results of simplified ILIPM without MDCS

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.1% |  |  |  |
| Class B | -0.2% | 0.0% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall (EL+BL) | -0.2% | 0.0% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall (EL) | -0.4% | 0.0% | 0.2% | -0.3% | 0.2% | 0.3% |
| Enc Time[%] | 101.8% |  |  | 101.7% |  |  |
| Dec Time[%] | 100.1% |  |  | 100.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusions

The BD-rate results tested by the cross-checker perfectly match those provided by the proponents.

# References

[1] “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, October 2012

[2] Z. Zhao et al. “TE5: Test Results of TE5.1.5”, JCTVC-L0153, Geneva, January 2013

[3] Z. Zhao et al., “Simplification of TE5.1.5 on inter-layer intra mode prediction”, JCTVC-L0156, Geneva, January 2013