| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0164 |
| --- | --- |

| Title: | TE5: Results of TE5 tools from Huawei |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Li Bin Li Houqiang Li Univ. of Science and Technology of China Haitao Yang Huawei Technologies | Tel: Email: | +86 755 28976972 haitao.yang@huawei.com |
| Source: | USTC, Huawei |  |  |

_____________________________

# Abstract

This contribution presents three TE5 tools from Huawei, they are 1). EL AMVP extension, 2) CU level Inter BL, and 3) PU level Inter BL. The three tools are implemented in SMUC-0.1.1. Considering the BL+EL bitrate, for the six inter configurations (RA 2x, RA 1.5x, RA SNR, LDP 2x, LDP 1.5x, LDP SNR), EL AMVP extension provides average BD-rate reduction of 1.5%, 1.5%, 1.6%, 1.0% , 0.8%, 1.1%, CU level Inter BL provides average BD-rate reduction of 2.3%, 2.1%, 2.4%, 1.4%, 1.1% and 1.8% , and PU level Inter BL provides average BD-rate reduction of 2.2%, 2.1%, 2.4%, 1.3%, 0.9% and 1.9%.

# Introduction

The three TE5 tools proposed by Huawei are 1). EL AMVP extension, 2) CU level Inter BL, and 3) PU level Inter BL. EL AMVP adds a base layer motion vector candidate into the candidate list, to improve motion vector coding in the enhancement layer. For the two Inter BL tools, both derive motion information from base layer, but the signalling is different. CU level Inter BL applies to a CU, and a flag is added to CU header for signalling. PU level Inter BL applies to a PU, and is signalled by a merge index. The three tools are described in detail in the following section.

# Algorithm description

Motion derivation is the common module shared by the three tools, and therefore it is introduced firstly in section 2.1. Then in section 2.2 and 2.3, EL AMVP extension and two Inter BL modes are explained in details respectively.

### 2.1 Motion derivation

In this proposal, SPU (Smallest Prediction Unit) is used as the basic unit to retrieve motion information from the base layer. Generally, the motion information of the collocated BL (Base Layer) SPU is used as the motion information of the EL (Enhancement Layer) SPU after proper scaling. To find the collocated BL SPU, the center position of the EL SPU is mapped into the base layer to find a corresponding position, and the base layer SPU containing the corresponding position is the collocated BL SPU. The center position is (3, 3) inside an SPU of size 8x8. For the 4x4 SPU, center position is (1, 1).

Not all the SPUs in a CU can get valid motion information from the base layer. For example, if the collocated base layer SPU is in an intra coded CU, the enhancement layer SPU cannot get valid motion information. In such a case, a recursive padding method is proposed to derive the motion information for these SPUs. Shown in Fig. 1 is a CU containing 16 SPUs. The index of SPU indicates the processing order in the padding process, which is the zigzag order. This CU is taken as an example to illustrate the recursive padding method.

Fig.1 An example of the processing unit (marked by blue square) 
and its four partitions (marked by green square)

Firstly, every four neighboring SPUs are grouped together as the current processing unit for the padding. For example, the SPU 0, 1, 2, and 3 are grouped together for processing, as shown in Fig. 1. In case one of the SPUs has no motion information, it will get the motion information from its neighboring SPUs within this group. It will check the three neighbors in the order of the horizontal neighbor, the vertical neighbor and the diagonal neighbor. If any of the three neighbors has motion information, the motion information is used as the motion information of the current SPU. For example, if SPU 3 has no motion information, it will firstly check SPU 2, and then SPU 1, and finally SPU 0, to see if any of them has motion information, and use the motion information as its motion information. For the current SPU with index, the index of the horizontal neighbor, the index of the vertical neighbor, and the index of the diagonal neighbor can be determined by (1), (2) and (3), respectively.

(1)

(2)

(1)

where  is the number of SPUs in the current processing unit divided by four. For example, N is equal to 1 if the current processing unit comprises 4 SPUs.

The same method can be used for padding the vacant motion information in other processing units. After the first round padding at the base level, for each processing unit, either all the four SPUs in it have motion information or all the four SPUs have no information.

Fig.2 Another example of the processing unit (marked by blue square) 
and its four partitions (marked by green square)

Then the second round padding will be started at a higher level. Specifically, every four processing units in the lower level form a new processing unit in the higher level. As shown in Fig. 2, the processing unit comprises four partitions, and each partition is a processing unit in the lower level. At this higher level, exactly the same method and the same equations can be used for motion information padding of each partition. For example, if the second partition (comprises SPU 4, 5, 6, and 7) has no motion information, it will check its neighbors in horizontal, vertical and diagonal direction to see if any of its neighbor has motion information, and use that motion information as its own motion information. Note that the motion information of the upper left SPU in a partition is considered as the representative motion information of the partition. The benefit is that equation (1), (2) and (3) can be used without modification to the calculation of the SPU index in a neighbor partition.

If the CU is composed of 16 SPUs as shown in Fig. 2, the padding process is terminated. In case the CU is bigger, the same padding process is performed recursively until the size of the processing unit is the same as the size of the CU.

After the recursive padding process described above, either all the SPUs in a CU have valid motion information, or they have no motion information. In the latter case, zero motion information is constructed and used for the CU. For P slices where forward prediction is performed, motion vector (0,0) and reference index 0 are set for forward reference picture list. For B slices where bi-directional prediction is performed, motion vectors (0,0) and reference index 0 are set for both forward and backward reference picture list.

After the recursive padding process and the zero padding described above, all SPUs in a CU have valid information.

### 2.2 EL AMVP extension

For EL AMVP extension, an additional AMVP candidate, i.e. the base layer AMVP candidate, is added in the first place in the AMVP candidate list. SPU size is set to 4x4 for this tool. The motion information of each SPU  inside the current PU is derived using the motion derivation algorithm described in section 2.1. Then the motion information of the center SPU is used to derive the base layer AMVP candidate. Specifically, the motion vector of the center SPU is scaled according to the POC difference, in the same way as the scaling operation on the temporal co-located motion vector for the derivation of the temporal AMVP candidate in the base layer. Motion compensation is performed for the current PU using the derived base layer AMVP candidate. The max number of the AMVP candidate is set to 2. The construction of the AMVP candidate list is shown in Table 1. The pruning process is not changed.

Table 1 EL AMVP candidate list construction

Proposed

- BL AMVP candidate - A - if(N < Max), B - pruning - Col - if(N < Max), Zero MV candidates

### 2.3 Inter BL

CU level inter BL and PU level Inter BL are described in section 2.2.1 and section 2.2.2, respectively.

- 2.3.1 CU level Inter BL

The proposed CU level Inter BL mode is applied to CUs in enhancement layer P/B slices. As shown in Table 2, a flag inter_bl_flag is put into the CU header, right after intra_bl_flag, to indicate whether this mode is used. The motion information of all SPU in the CU is derived using the motion derivation algorithm described in section 2.1. Then the motion compensation is performed separately for each SPU in the CU. SPU size is set to 8x8. The residual coding part is the same as the ordinary inter coded CUs in the base layer. Deblocking filter is also applied to SPU boundaries.

Table 2 Syntax change for the proposed CU level Inter BL

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( layerID > 0 ) { |  |
| intra_bl_flag | ae(v) |
| if( !intra_BL_flag && slice_type != I) |  |
| inter_bl_flag | ae(v) |
| } |  |
| if( !intra_BL_flag&& !inter_BL_flag ) { |  |
| if( slice_type != I ) |  |
| part_mode_flag | ae(v) |

- 2.3.2 PU level Inter BL

For PU level inter BL, an additional merge candidate, i.e. the base layer merge candidate, is added in the first place in the merge candidate list. The motion information of the base layer merge candidate is derived using the motion derivation algorithm described in section 2.1. SPU size is set to 8x8. Using the derived base layer merge candidate, the motion compensation is performed for each SPU if the current PU size is bigger than or equal to the SPU size. Otherwise, the motion compensation is performed for the current PU. Deblocking filter is also applied to SPU boundaries.

The max number of the merge candidate is set to 5. The construction of the merge candidate list is shown in Table 3. The pruning process is not changed.

Table 3 EL merge candidate list construction

Proposed

- BL merge candidate - A1 - B1 - B0 - if(N < Max-1), A0 - if(N < Max-1), B2 - Col - if(N < Max), Combined bi-pred. candidates - if(N < Max), Zero MV candidates

# Experiment Results

As specified in TE5 description [1], the proposed tools are integrated into SHVC reference software SMuC0.1.1. The RD performance of SMuC0.1.1 is used as the anchor for comparison. For accurate evaluation of decoding time, all the decoding tasks are run sequentially on the same machine, without YUV writing.

### 3.1 EL AMVP extension

The experimental results of EL AMVP extension are shown in Table 4.

Table 4 Experimental results of EL AMVP extension

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -2.8% | -2.8% |  |  |  | -1.6% | -3.1% | -3.3% |
| Class B | -1.4% | -2.0% | -2.2% | -1.5% | -1.8% | -2.1% | -1.5% | -2.2% | -2.5% |
| Overall (EL+BL) | -1.5% | -2.3% | -2.4% | -1.5% | -1.8% | -2.1% | -1.6% | -2.5% | -2.7% |
| Overall (EL) | -2.7% | -4.0% | -4.2% | -3.8% | -4.3% | -5.0% | -3.0% | -4.4% | -4.9% |
| Enc Time[%] | 98.1% |  |  | 96.9% |  |  | 98.7% |  |  |
| Dec Time[%] | 102.0% |  |  | 101.7% |  |  | 102.3% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -2.0% | -2.0% |  |  |  | -1.3% | -2.1% | -2.1% |
| Class B | -0.8% | -1.2% | -1.3% | -0.8% | -1.0% | -1.0% | -1.0% | -1.3% | -1.4% |
| Overall (EL+BL) | -1.0% | -1.4% | -1.5% | -0.8% | -1.0% | -1.0% | -1.1% | -1.5% | -1.6% |
| Overall (EL) | -1.6% | -2.4% | -2.6% | -1.7% | -2.1% | -2.1% | -1.8% | -2.4% | -2.6% |
| Enc Time[%] | 98.5% |  |  | 97.7% |  |  | 99.0% |  |  |
| Dec Time[%] | 102.2% |  |  | 102.2% |  |  | 102.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

### 3.2 CU level inter BL

The experimental results of CU level Inter BL are shown in Table 5.

Table 5 Experimental results of CU level Inter BL

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.8% | -5.4% | -5.2% |  |  |  | -2.7% | -6.3% | -6.6% |
| Class B | -2.2% | -3.6% | -3.7% | -2.1% | -3.4% | -3.6% | -2.3% | -3.9% | -4.1% |
| Overall (EL+BL) | -2.3% | -4.1% | -4.1% | -2.1% | -3.4% | -3.6% | -2.4% | -4.6% | -4.8% |
| Overall (EL) | -4.5% | -7.7% | -7.7% | -5.6% | -8.7% | -9.2% | -5.1% | -9.2% | -9.5% |
| Enc Time[%] | 100.5% |  |  | 99.9% |  |  | 105.6% |  |  |
| Dec Time[%] | 102.1% |  |  | 103.2% |  |  | 105.2% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.8% | -3.5% | -3.4% |  |  |  | -2.3% | -4.1% | -4.0% |
| Class B | -1.3% | -2.1% | -2.2% | -1.1% | -1.8% | -2.0% | -1.7% | -2.5% | -2.6% |
| Overall (EL+BL) | -1.4% | -2.5% | -2.5% | -1.1% | -1.8% | -2.0% | -1.8% | -3.0% | -3.0% |
| Overall (EL) | -2.5% | -4.5% | -4.5% | -2.5% | -4.2% | -4.4% | -3.4% | -5.4% | -5.4% |
| Enc Time[%] | 101.5% |  |  | 101.4% |  |  | 101.6% |  |  |
| Dec Time[%] | 101.9% |  |  | 102.2% |  |  | 104.5% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

### 3.3 PU level inter BL

The experimental results of PU level inter BL are shown in Table 6.

Table 6 experimental results of PU level inter BL

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.7% | -6.0% | -5.7% |  |  |  | -2.7% | -6.8% | -7.1% |
| Class B | -2.1% | -4.0% | -4.0% | -2.1% | -3.7% | -4.0% | -2.2% | -4.3% | -4.5% |
| Overall (EL+BL) | -2.2% | -4.5% | -4.5% | -2.1% | -3.7% | -4.0% | -2.4% | -5.1% | -5.3% |
| Overall (EL) | -4.4% | -8.4% | -8.5% | -5.7% | -9.7% | -10.3% | -5.1% | -10.1% | -10.5% |
| Enc Time[%] | 96.4% |  |  | 95.7% |  |  | 97.5% |  |  |
| Dec Time[%] | 106.2% |  |  | 108.0% |  |  | 109.3% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -3.8% | -3.6% |  |  |  | -2.3% | -4.3% | -4.4% |
| Class B | -1.1% | -2.4% | -2.5% | -0.9% | -2.0% | -2.2% | -1.7% | -2.8% | -2.9% |
| Overall (EL+BL) | -1.3% | -2.8% | -2.8% | -0.9% | -2.0% | -2.2% | -1.9% | -3.3% | -3.3% |
| Overall (EL) | -2.4% | -5.0% | -4.9% | -2.2% | -4.6% | -4.9% | -3.5% | -5.8% | -5.9% |
| Enc Time[%] | 96.4% |  |  | 95.9% |  |  | 98.1% |  |  |
| Dec Time[%] | 104.9% |  |  | 104.7% |  |  | 107.1% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

This contribution presents three TE5 tools for SHVC. The proposed algorithm is implemented in SMUC-0.1.1. Considering the overall bitrate, for the six inter configurations (RA 2x, RA 1.5x, RA SNR, LDP 2x, LDP 1.5x, LDP SNR), EL AMVP extension provides average BD-rate reduction of 1.5%, 1.5%, 1.6%, 1.0% , 0.8%, 1.1%, CU level Inter BL provides average BD-rate reduction of 2.3%, 2.1%, 2.4%, 1.4%, 1.1% and 1.8% , and PU level Inter BL provides average BD-rate reduction of 2.2%, 2.1%, 2.4%, 1.3%, 0.9% and 1.9%.

# References

[1] Vadim Seregin, Patrice Onno, Shan Liu, Tammy Lee, Chulkeun Kim, Haitao Yang, Haricharan Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, China, Oct 2012.

# Patent rights declaration(s)

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).