| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0165 |
| --- | --- |

| Title: | TE2: Results of test 3.1.3 on CU based IntraBL signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal/Information |  |  |
| Author(s) or Contact(s): | Jinwen Zan Xianghui Wei Haitao Yang Section F1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86-755-2897-9016 jinwen.zan@huawei.com weixianghui@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In scalable high efficiency video coding (SHVC) [1] tool experiment 2 (TE2)[2], inter-layer texture prediction signaling techniques as in the call for proposals (Cfp) are experimented. This document reports the performance of SHVC-TE2 Section 3.1.3 [2] proposed by Huawei Technologies, which signals the inter-layer texture prediction (ILTP) without skip mode in the enhancement layer. Simulation results show that the proposed ILTP signaling technique, under the SHVC common test conditions, provides BD-rate improvements for AI, RA, and LDP of -23.1%,    -16.6%, and -12.5% in the case of 2x; -32.8%, -26.3% and -22.1% in the case of 1.5x, respectively.

# Introduction

During the 11th JCT-VC meeting in Shanghai, China, techniques from SHVC Cfp responses were grouped into 8 tool experiments (TEs). TE2 focuses on ILTP signalling, including 9 CU based techniques, 3 reference picture based techniques, and 1 slice based technique. This contribution is one of the CU based techniques [3].

# Tools description

In this contribution, the ILTP signaling technique as described in Section 3.1.3 of [2] is implemented onto the SMuC-0.1.1 software. A new syntax element, inter_layer_texture_pred_flag, is defined to signal the ILTP mode in the enhancement layer, when the current CU is not skipped.

For the CU employing the ILTP, a new CU mode is defined, with the partition size 2Nx2N and merge flag false. The residue of the ILTP CUs is encoded by employing the no_residual_syntax_flag.

# Syntax table and semantics

| coding_unit( x0, y0, log2CbSize, layerID ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( layerID > 0 ) { |  |
| inter_layer_texture_pred_flag[x0][y0] | ae(v) |
| } |  |
| if( !inter_layer_texture_pred_flag ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && (!(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ¦ ¦ PredMode[ x0 ][ y0 ] = = MODE_INTRA_BL) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Results

The proposed ILTP signaling technique is implemented onto the SMuC-0.1.1 software (without the DCT bug fix). Simulations are run under the common test conditions [2] and results summarized into Table 1. It is noted that the anchor data used for comparison is HM-8.1 simulcast.

The decoding time is not reliable as it was recorded on a heterogeneous cluster.

Table 1 TE2-3.1.3 Simulation Results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.2% | -27.0% | -27.9% |  |  |  |
| Class B | -21.4% | -21.0% | -20.9% | -32.8% | -32.8% | -33.0% |
| Overall (EL+BL) | -23.1% | -22.8% | -22.9% | -32.8% | -32.8% | -33.0% |
| Overall (EL) | -35.2% | -34.9% | -35.2% | -58.4% | -58.0% | -58.5% |
| Enc Time[%] | 106.2% |  |  | 104.9% |  |  |
| Dec Time[%] | 100.6% |  |  | 100.1% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.5% | -9.0% |  |  |  |
| Class B | -15.8% | -8.8% | -7.5% | -26.3% | -19.8% | -18.4% |
| Overall (EL+BL) | -16.6% | -8.4% | -7.9% | -26.3% | -19.8% | -18.4% |
| Overall (EL) | -25.5% | -13.0% | -12.4% | -47.6% | -37.2% | -34.7% |
| Enc Time[%] | 105.5% |  |  | 107.9% |  |  |
| Dec Time[%] | 130.8% |  |  | 129.6% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -13.7% | -1.7% | -2.8% |  |  |  |
| Class B | -11.9% | -6.7% | -4.8% | -22.1% | -16.6% | -14.9% |
| Overall (EL+BL) | -12.4% | -5.2% | -4.2% | -22.1% | -16.6% | -14.9% |
| Overall (EL) | -19.8% | -8.2% | -6.8% | -41.1% | -31.8% | -28.6% |
| Enc Time[%] | 105.1% |  |  | 107.8% |  |  |
| Dec Time[%] | 138.4% |  |  | 134.9% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

A candidate ILTP signaling algorithm [2] as originally proposed in [3] was implemented onto the SMuC-0.1.1 software. The proposed technique employs only one new syntax element to signal the ILTP mode. The modification to the HEVC syntax is simple and clean.

Simulation results have demonstrated the effectiveness of the proposed algorithm, and it is recommended to adopt the proposed ILTP signaling technique into SHVC.

# References

[1] A. Segall, J. Boyce, J.-R. Ohm, G.J. Sullivan, “Report of results of the joint call for proposals on scalable high efficiency video coding (SHVC)”, JCTVC-K1008, 11th JCT-VC meeting, Shanghai, China, October, 2012.

[2] L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang, “Description of Tool Experiment A2: Inter-layer texture prediction signaling in SHVC”, JCTVC-K1102 , 11th JCT-VC meeting, Shanghai, China, October, 2012.

[3] H. Yang, J.Zan, X. Wei, W. Gao, H. Yu (Huawei), L. Li, J. Zhang, B. Li, H. Li (USTC), “Description of scalable video coding technology proposal by Huawei Technologies”, JCTVC-K0046, 11th JCT-VC meeting, Shanghai, China, October, 2012.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).