| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0166 |
| --- | --- |

| Title: | TE2: Results of test 3.1.4 on CU based IntraBL signalling with skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal/Information |  |  |
| Author(s) or Contact(s): | Jinwen Zan Xianghui Wei Haitao Yang Section F1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86-755-2897-9016 jinwen.zan@huawei.com weixianghui@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

In scalable high efficiency video coding (SHVC) [1] tool experiment 2 (TE2), inter-layer texture prediction signalling techniques as in the call for proposals (CfP) are experimented. This document reports the performance of SHVC-TE2 Section 3.1.4 [2] proposed by Huawei Technologies, which signals the inter-layer texture prediction (ILTP) with skip mode in the enhancement layer. Simulation results show that the proposed ILTP signalling technique, under the SHVC common test conditions, provides BD-rate improvements for AI, RA, and LDP of -23.1%, -16.6%, and -12.5% in the case of 2x; -32.8%, -26.3% and -22.1% in the case of 1.5x, respectively.

# Introduction

During the 11th JCT-VC meeting in Shanghai, China, techniques from SHVC CfP responses were grouped into 8 tool experiments (TEs). TE2 focuses on ILTP signalling, including 9 CU based techniques, 3 reference picture based techniques, and 1 slice based technique. This contribution is one of the CU based techniques [3].

# Tool description

The ILTP with skip mode is applied to CUs in both intra and inter slices.

## ILTP with skip for CUs in I slices

In HEVC, the syntax element skip_flag is applied to CUs in P/B slices, but not I slices. In this TE, the skip_flag is applied to I slices in the enhancement layer to indicate whether the proposed ILTP with skip is used or not.

When the skip_flag is equal to 1 for a CU in I slice of enhancement layer, no more syntax elements need to be parsed from the bitstream, and the ILTP is used as the reconstruction of the CU; otherwise, more syntax elements need to be parsed for the reconstruction.

## ILTP with skip for CUs in P/B slices

In HEVC, the skip_flag is used to indicate the usage of the skip mode in the case of P/B slices. In the proposed SHVC TE2, this flag is also used to signal the ILTP skip mode in the enhancement layer. In order to differentiate these cases, an addition flag, inter_layer_texture_skip_flag is added. Only when all the conditions listed below are true, this added flag is parsed.

- Enhancement layer

- P/B slice

- skip_flag is equal to 1

If the inter_layer_texture_skip_flag is equal to 1, the ILTP is used as the reconstruction of the current CU; otherwise, the HEVC skip mode is performed.

## ITLP signalling

A new syntax element, inter_layer_texture_pred_flag, is defined to signal the ILTP mode in the enhancement layer, when the current CU is not skipped.

If the inter_layer_texture_pred_flag is equal to 1, the ILTP is used as the prediction of the current CU; otherwise, HEVC intra and inter mode are performed.

It is noted that in case a CU is signaled with inter_layer_texture_pred_flag, the no_residual_syntax_flag should not be inferred as 1.

Details of the syntax and semantics are listed below in section 3.

# Syntax and semantics description

| coding_unit( x0, y0, log2CbSize, layerID ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ¦¦ (layerID > 0 && (slice_type == I) )) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| if( slice_type != I && (layerID > 0) ) |  |
| inter_layer_texture_skip_flag[x0][y0] |  |
| if( !inter_layer_texture_skip_flag[x0][y0] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( layerID > 0 ) |  |
| inter_layer_texture_pred_flag |  |
| if( !inter_layer_texture_pred_flag ) { |  |
| pred_mode_flag |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦¦ log2CbSize == Log2MinCbSizeY ) |  |
| part_mode |  |
| if( PredMode[ x0 ][ y0 ] == MODE_INTRA ) { | ae(v) |
| if( PartMode == PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | ae(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode == PART_NxN ) ? ( nCbS / 2 ) : nCbS | f(1) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) { |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] |  |
| } |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) { | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] |  |
| Else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] |  |
| } | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] |  |
| } | ae(v) |
| } else { |  |
| if( PartMode == PART_2Nx2N ) | ae(v) |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode == PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode == PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode == PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode == PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode == PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode == PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0] != MODE_INTRA && (!( PartMode == PART_2Nx2N && merge_flag[x0][y0]) ¦ ¦ inter_layer_texture_pred_flag) ) |  |
| no_residual_syntax_flag |  |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] == MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Results

The proposed TE2 technique is implemented on SMuC-0.1.1, and simulation output compared with that of HM-8.1 simulcast. The simulation configuration conforms to the common test conditions defined in [1].

The summary of this TE is reported in Table 1.

Table 1. TE2: 3-1-4 simulation results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.2% | -27.1% | -27.9% |  |  |  |
| Class B | -21.4% | -21.0% | -20.9% | -32.9% | -32.7% | -33.0% |
| Overall (EL+BL) | -23.1% | -22.8% | -22.9% | -32.9% | -32.7% | -33.0% |
| Overall (EL) | -35.2% | -34.9% | -35.2% | -58.5% | -57.8% | -58.3% |
| Enc Time[%] | 107.7% |  |  | 105.2% |  |  |
| Dec Time[%] | 102.5% |  |  | 98.1% |  |  |

- All intra

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.8% | -9.4% |  |  |  |
| Class B | -15.7% | -8.9% | -7.5% | -26.3% | -20.0% | -18.6% |
| Overall (EL+BL) | -16.5% | -8.6% | -8.1% | -26.3% | -20.0% | -18.6% |
| Overall (EL) | -25.5% | -13.3% | -12.7% | -47.7% | -37.5% | -35.2% |
| Enc Time[%] | 105.6% |  |  | 108.2% |  |  |
| Dec Time[%] | 129.3% |  |  | 127.0% |  |  |

- Random Access

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -13.7% | -2.0% | -3.1% |  |  |  |
| Class B | -12.0% | -6.6% | -4.9% | -22.3% | -16.7% | -14.9% |
| Overall (EL+BL) | -12.5% | -5.3% | -4.4% | -22.3% | -16.7% | -14.9% |
| Overall (EL) | -19.9% | -8.3% | -7.0% | -41.5% | -31.8% | -28.6% |
| Enc Time[%] | 105.1% |  |  | 108.1% |  |  |
| Dec Time[%] | 133.7% |  |  | 131.2% |  |  |

- Low Delay-P

# Conclusion

A signalling algorithm applied onto the SMuC-0.1.1 software has been implemented. The proposed technique includes both the signalling of the ILTP mode, and the signalling of ILTP with skip mode, to fully exploit the correlation between the base layer and enhancement layer in SHVC.

Simulation results demonstrated the effectiveness of the proposed algorithm, and it is recommended to adopt this signalling technique in SHVC.

# Reference

[1] JCTVC-K1102, Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC.

[2] JCTVC-K0046, Description of scalable video coding technology proposal by Huawei Technologies.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).