| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0170r1 |
| --- | --- |

| Title: | HEVC v1 scalability hook: long-term pictures with layer_id values |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela and Antti Hallapuro Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is proposed in this contribution to enable SHVC to use a long-term reference picture having nuh_layer_id equal to A as reference for inter prediction for a picture having nuh_layer_id greater than A. This functionality would, for example, enable storing a long-term reference picture at a low resolution and hence consume a relatively moderate amount of decoded picture buffer (DPB) memory rather than storing long-term reference pictures separately at each layer they are intended to be used as reference for inter prediction.

In order to realize the proposed functionality, the following changes are proposed for HEVC version 1:

- Association of nuh_reserved_zero_6bits (which is expected to be called nuh_layer_id in SHVC) with each long-term reference picture in the reference picture set (RPS).

- Syntax changes to indicate the nuh_reserved_zero_6bits for long-term reference pictures in RPS related syntax, the presence of which is controlled by a sequence parameter set flag.

In revision 1 of the contribution some additions within the proposed specification text were deleted, as they were not necessary for maintaining long-term pictures with their nuh_layer_id values (but originated from the more general specification text provided with JCTVC-L0188). In addition, revision 1 includes example results using the low-delay common conditions were added.

# Problem Statement

It is proposed in this contribution to enable SHVC to use a long-term reference picture having nuh_layer_id equal to A as reference for inter prediction for a picture having nuh_layer_id greater than A. This functionality would, for example, enable storing a long-term reference picture at a low resolution and hence consume a relatively moderate amount of decoded picture buffer (DPB) memory rather than storing long-term reference pictures separately at each layer they are intended to be used as reference for inter prediction. However, it is also desirable to enable storage of more than one long-term reference picture per access unit, for example for keeping long-term reference pictures for each view in MV-HEVC.

Long-term reference pictures may be used for example in error-prone transmission environments which provide feedback capability from the far-end decoder to the encoder indicating which pictures have been correctly or incorrectly received or decoded. The encoder may choose to use a long-term reference picture as reference for inter prediction in cases where the feedback of incorrect decoding has been received. In general, the encoder would need to keep two pictures marked as used for long-term reference, one which has been confirmed to have been decoded correctly by the far-end decoder and another later one for which the encoder is waiting for the confirmation from the far-end receiver. If e.g. two long-term reference pictures were maintained for each enhancement layer and the number of short-term reference pictures available for enhancement-layer coding was reduced accordingly, the compression penalty could be significant. For example, if the number of EL short-term reference pictures is reduced by 2 in the TE software and using the common RA test conditions, the penalty in EL dBR is at least 25%:

In another example, if the number of EL short-term reference pictures is reduced from two to one in the low-delay common conditions, the penalty in EL dBR is more than 10%.

A key idea of the RPS is that all pictures that may be used as reference for the current picture or any subsequent picture in decoding order are included in the RPS. Pictures that are not included in the RPS are marked as “unused for reference”.

In SHVC, the RPS may be considered to operate layer-wise for short-term reference pictures, i.e. all short-term reference pictures that are in the same layer as the current picture and may be used as reference for the current picture or any subsequent picture in decoding order in the same layer as the current picture are included in the RPS. In this contribution it is suggested that long-term reference pictures may be used across layers and that the same access unit (and hence the same POC value) may include more than one long-term reference picture in different layers. In order to keep long-term reference pictures from a different layer (than that of the current picture) marked as “used for long-term reference”, all the long-term reference pictures along with their layer_id values have to be explicitly listed in RPS – otherwise, they would be marked as “unused for reference”. Note that this applies also to RPS applied for the base layer, as the RPS of a base-layer picture has to include those long-term pictures (originating from any layer) that are kept marked as “used for long-term reference”. Consequently, this contribution proposes a change into HEVC version 1.

# Proposed Specification Text Changes

The changes indicated in red font color are proposed.

## Sequence parameter set

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| nonbase_layer_long_term_ref_pics_present_flag | u(1) |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| if( nonbase_layer_long_term_ref_pics_present_flag ) |  |
| lt_ref_reserved_zero_6bits_sps[ i ] | u(6) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |

nonbase_layer_long_term_ref_pics_present_flag specifies the presence of the syntax elements lt_ref_reserved_zero_6bits_sps and reserved_zero_6bits_lt.

lt_ref_reserved_zero_6bits_sps[ i ] specifies a nuh_reserved_zero_6bits value of the i-th candidate long-term reference picture specified in the sequence parameter set. If not present, the value of lt_ref_reserved_zero_6bits_sps[ i ] is inferred to be equal to 0.

## Slice segment header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| if( nonbase_layer_long_term_ref_pics_present_flag ) |  |
| reserved_zero_6bits_lt[ i ] | u(6) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| … |  |

reserved_zero_6bits_lt[ i ] specifies that the i-th candidate long-term reference picture to be included in the long-term reference picture set of the current picture has nuh_reserved_zero_6bits equal to reserved_zero_6bits_lt[ i ]. If not present, reserved_zero_6bits_lt[ i ] is inferred to be equal to 0.

The variable ReservedZero6BitsLt[ i ] is derived as follows:

–	If i is less than num_long_term_sps, ReservedZero6BitsLt[ i ] is set equal to lt_ref_reserved_zero_6bits_sps[ lt_idx_sps[ i ] ].

–	Otherwise, ReservedZero6BitsLt[ i ] is set equal to reserved_zero_6bits_lt[ i ].

## Reference picture set decoding

- 8.3.2	Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction for the slice as specified in subclause Error: Reference source not found. This process may result in one or more reference pictures in the DPB being marked as "unused for reference" or "used for long-term reference".

NOTE 1 – The reference picture set is an absolute description of the reference pictures used in the decoding process of the current and future coded pictures. The reference picture set signalling is explicit in the sense that all reference pictures included in the reference picture set are listed explicitly and there is no default reference picture set construction process in the decoder that depends on the status of the DPB. [Ed. (GJS): Search for line spacing exactly 9.95 pt and change to "single".]

A picture can be marked as "unused for reference", "used for short-term reference", or "used for long-term reference", but only one among these three. Assigning one of these markings to a picture implicitly removes another of these markings when applicable. When a picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both).

When the current picture is the first picture in the bitstream, the DPB is initialized to be an empty set of pictures.

When the current picture is an IDR picture or a BLA picture, all reference pictures currently in the DPB (if any) are marked as "unused for reference".

Short-term reference pictures are identified by their PicOrderCntVal values. Long-term reference pictures are identified either by their PicOrderCntVal values or their pic_order_cnt_lsb values. When nonbase_layer_long_term_ref_pics_present_flag is equal to 1, long-term reference pictures are additionally identified by their nuh_reserved_zero_6bits values.

Five lists of picture order count values are constructed to derive the reference picture set; PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll with NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of elements, respectively. Two lists of nuh_reserved_6bits values are additionally constructed to derive the reference picture set; LayerIdLtCurr and LayerIdLtFoll with NumPocLtCurr and NumPocLtFoll number of elements, respectively.

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll are all set to empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set to 0.

- Otherwise, the following applies for derivation of the five lists of picture order count values and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ StRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(8-1)
for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt += PicOrderCntVal − 	DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb − pic_order_cnt_lsb
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt
		LayerIdLtCurr[ j ] = ReservedZero6BitsLt[ i ]
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		LayerIdLtFoll[ k ] = ReservedZero6BitsLt[ i ]
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in subclause Error: Reference source not found.

NOTE 2 – A value of StRpsIdx in the range from 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a short-term reference picture set from the active sequence parameter set is being used, where StRpsIdx is the index of the short-term reference picture set to the list of short-term reference picture sets in the order in which they are signalled in the sequence parameter set. StRpsIdx equal to num_short_term_ref_pic_sets indicates that a short-term reference picture set explicitly signalled in the slice header is being used.

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to PocLtCurr[ j ] and LayerIdLtCurr[ i ] is equal to LayerIdLtCurr[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to PocLtFoll[ j ] and LayerIdLtFoll[ i ] is equal to LayerIdLtFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to PocLtCurr[ j ] and LayerIdLtFoll[ i ] is equal to LayerIdLtCurr[ j ].

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to LayerIdLtCurr[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to ( PocLtFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to LayerIdLtFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to LayerIdLtCurr[ j ].

The reference picture set consists of five lists of reference pictures; RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetStFoll, RefPicSetLtCurr and RefPicSetLtFoll. The variable NumPocTotalCurr is derived in Error: Reference source not found, reference picture list modification semantics.

It is a requirement of bitstream conformance that the following applies to the value of NumPocTotalCurr:

- If the current picture is a BLA or CRA picture, the value of NumPocTotalCurr shall be equal to 0.

- Otherwise, when the current picture contains a P or B slice, the value of NumPocTotalCurr shall not be equal to 0.

NOTE 3 – RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr contains all reference pictures that may be used in inter prediction of the current picture and that may be used in inter prediction of one or more of the pictures following the current picture in decoding order. RefPicSetStFoll and RefPicSetLtFoll consists of all reference pictures that are not used in inter prediction of the current picture but may be used in inter prediction of one or more of the pictures following the current picture in decoding order.

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ )
	if( !CurrDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB [Ed. (JB): Should be made more precise. (GJS): Seems roughly OK to me.]
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
																						(8-2)
for( i = 0; i < NumPocLtFoll; i++ )
	if( !FollDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else 
				RefPicSetLtFoll[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
					with PicOrderCntVal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
			else if( there is a short-term reference picture picY in the DPB
					with PicOrderCntVal equal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else
				RefPicSetLtFoll[ i ] = "no reference picture"

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference".

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrBefore[ i ]
			
		RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrAfter[ i ]
			
		RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(8-3)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStFoll[ i ]
			
		RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

NOTE 4 – There may be one or more reference pictures that are included in the reference picture set but not present in the decoded picture buffer. Entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr that is equal to "no reference picture".

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr for which one or more of the following are true.

  - The entry is equal to "no reference picture".

  - The entry is a picture that has nal_unit_type equal to TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12, or RSV_VCL_N14 and TemporalId equal to that of the current picture.

  - The entry is a picture that has TemporalId greater than that of the current picture.

- There shall no entry in RefPicSetLtCurr or RefPicSetLtFoll for which the difference between the picture order count value of the current picture and the picture order count value of the entry is equal to or greater than 224.

- There shall be no picture in RefPicSetLtCurr with LayerIdLtCurr[ i ] greater than nuh_reserved_zero_6bits of the current picture.

- When the current picture is a TSA picture, there shall be no picture included in the reference picture set with TemporalId greater than or equal to the TemporalId of the current picture.

- When the current picture is an STSA picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that preceded the STSA picture in decoding order.

- When the current picture is a CRA picture, there shall be no picture included in the reference picture set that precedes, in decoding order, any preceding RAP picture in decoding order (when present).

- When the current picture is a trailing picture, there shall be no picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that was generated by the decoding process for generating unavailable reference pictures as specified in subclause 2.4.

- When the current picture is a trailing picture, there shall be no picture in the reference picture set that precedes the associated RAP picture in output order or decoding order.

- When the current picture is a RADL picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is any of the following types of pictures.

  - A RASL picture.

  - A picture that was generated by the decoding process for generating unavailable reference pictures as specified in subclause 2.4.

  - A picture that precedes the associated RAP picture in decoding order.

- When the sps_temporal_id_nesting_flag is equal to 1, the following applies. Let tIdA be the value of TemporalId of the current picture picA. Any picture picB with TemporalId equal to tIdB that is less than or equal to tIdA shall not be included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of picA when there exists a picture picC with TemporalId equal to tIdC that is less than tIdB, which follows the picture picB in decoding order and precedes the picture picA in decoding order.

NOTE 5 – A picture cannot be included in more than one of the five reference picture set lists.

## Generation of unavailable reference pictures

- 8.3.3	Decoding process for generating unavailable reference pictures

- 8.3.3.1	General decoding process for generating unavailable reference pictures

[Ed. (GJS): Scan the table of contents to look for any cases where headings do not appear there, as this is almost certainly due to having some style other than the ordinary heading style used on the headings of the missing sections.]

This process is invoked once per coded picture when the current picture is a BLA picture or is a CRA picture that is the first picture in the bitstream.

NOTE – This process is primarily specified only for the specification of syntax constraints for RASL pictures. The entire specification of the decoding process for RASL pictures associated with a CRA picture at the beginning of the bitstream or for RASL pictures associated with a BLA picture is only included herein for purposes of specifying constraints on the allowed syntax content of such RASL pictures. During the decoding process, any RASL pictures associated with a CRA picture at the beginning of the bitstream or any RASL pictures associated with a BLA picture may be ignored (removed from the bitstream and discarded), as they are not specified for output and have no effect on the decoding process of any other pictures that are specified for output. However, in HRD operations as specified in Annex Error: Reference source not found, RASL access units may need to be taken into consideration in derivation of CPB arrival and removal times.

When this process is invoked, the following applies.

- For each RefPicSetStFoll[ i ], with i in the range of 0 to NumPocStFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause Error: Reference source not found, and the following applies.

- The value of PicOrderCntVal for the generated picture is set to PocStFoll[ i ].

- The value of PicOutputFlag for the generated picture is set to 0.

- The generated picture is marked as "used for short-term reference".

- RefPicSetStFoll[ i ] is set to be the generated reference picture.

- The value of nuh_reserved_zero_6bits for the generated picture is set equal to the value of nuh_reserved_zero_6bits of the current picture.

- For each RefPicSetLtFoll[ i ], with i in the range of 0 to NumPocLtFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause Error: Reference source not found, and the following applies.

- The value of PicOrderCntVal for the generated picture is set to PocLtFoll[ i ].

- The value of pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtFoll[ i ] & ( MaxPicOrderCntLsb – 1 ) )

- The value of PicOutputFlag for the generated picture is set to 0.

- The generated picture is marked as "used for long-term reference".

- RefPicSetLtFoll[ i ] is set to the generated reference picture.

- The value of nuh_reserved_zero_6bits for the generated picture is set equal to LayerIdLtFoll[ i ].

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).