| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 October 2012 | Document: JCTVC-K1003_v10 |
| --- | --- |

| Title: | High Efficiency Video Coding (HEVC) text specification draft 9 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Draft 9 of High efficiency video coding.

Ed. Notes (D9):

- General editorial cleanup and consistency improvements (including some section restructuring)

- Cleanup and correction of colour space specification aspects (e.g. w.r.t. UHDTV)

- Improvement of Annex A examples for level limits on picture sizes and frame rates

- Correction of missing condition on LongTermRefPic in spatial MVP derivation relative to action recorded for JCTVC-J0071 and JCTVC-J0121 (#647)

- Improved definition of "inter prediction" (#342)

- Correcting usage of "disable_deblocking_filter_flag" (#654)

- Corrected use of Log2CtbSize versus Log2CtbSizeY in Annex A (#655)

- Corrected use of I_PCM mode (#634)

- Removed apparently-unnecessary checks for "&&  NumPocTotalCurr > 1" in ref_pic_list_modification( ) syntax, as it does not seem to make sense for the encoder to set ref_pic_list_modification_flag_lX to 1 when NumPocTotalCurr is equal to 1

- Modifying cbf_cb[ x0 ][ y0 ][ trafoDepth ] and cbf_cr[ x0 ][ y0 ][ trafoDepth ] semantics in regard to luma versus chroma (TB versus TU) location specification (Dzung Hoang JCT-VC experts reflector email 3 Aug 2012)

- Fixed typos/trivial issues #697, #667, #658, #652, #726, #725, #722, #694, #688, #682, #674, #721, #719, #718, #708, #705, #695, #693, #691, #689, #683, #677, #672, #663, #706, #668, #656, #653

- Fixed spatial merge candidate horizontal position xBk modification (#702)

- Fixed 8 bit WP chroma offset (#699)

- Fixed scaling_list_dc_coef_minus8 has no default value (#679)

- Fixed missing condition on constrained_intra_pred_flag (#665)

- Fixed typos/trivial issues #698, #741, #752, #469, #764, #765, #766, #767, #769, #773, #774, #775

- Renamed ref_pic_list_modification() to ref_pic_lists_modification()

- Replaced Sign2() by Sign3() and renamed Sign3() to Sign()

- Fixed NOTE formating by using Note 2 style for notes in definitions and Note 1 style for all other notes

- Renamed CtbAddrRS and CtbAddrTS in CtbAddrInRS and CtbAddrInTS to not be prefix of the arrays CtbAddrRStoTS and CtbAddrTStoRS anymore.

- Added default values for num_tile_columns_minus1, num_tile_rows_minus1 and added tiles_enable_flag to the constraint (#649)

- video_format table added to VUI parameters semantics (#730)

- fixed chroma deblocking grid (#711)

- Clarified binarization of cu_qp_delta_abs with prefix and suffix (#759)

- Fixed part_mode ctx derivation for binIdx equal to 2 (#720)

- Improved chroma_weight_l1_flag parsing (#779)

- Fixed default scaling list data issue (#762)

- Fixed using chroma bit depth in chroma edge deblocking (#744)

- Improved semantics of loop_filter_across_slices_enabled_flag and slice_loop_filter_across_slices_enabled_flag (#772)

- Added description of quantization group (#736)

- Fixed qPY_PREV derivation (#742)

- Replaced abs(⋅) with Abs(⋅)

- Added editorial comments to parts that need to be updated from AVC spec.

- Insert line breaks in the nal_unit_type table to align the number and the names.

- Fix for RPS restriction of TSA to reflect the original intent.

- Fixed the problem with the precedence in the derivation / usage of NumPocTotalCurr.

- Editorial fix for the marking process of the bumping decoder in Annex C.

- Renamed TFD and DLP pictures as random access skipped leading (RASL) picture and random access decodable leading (RADL) picture, respectively.

- Incorporated cleanup and complexity reduction for spatial merge candidate derivation according to JCTVC-K0197.

- Incorporated MV line buffer cleanup (JCTVC-K0101).

- Incorporated the contouring artefact fix (JCTVC-K0139)

- Incorporated inverse ctb raster scan fix and grouping of pcm sps syntax elements (JCTVC-K0217)

- Changed Table A1, A3 and A4 for Level limits in Level 6, 6.1 and 6.2 (JCTVC-K0377)

- Modified number of slice constraints (JCTVC-K0201)

- Added number of tiles constraints (JCTVC-K0202)

- Incorporated CTU bit size limit (JCTVC-K0176)

- Delete the 2/3 case of MaxDpbSize derivation (JCTVC-K0189)

- Incorporated Main 10 profile (JCTVC-K0109)

- Removed num_subsequent_pcm (JCTVC-K0258)

- Remove entropy slices (JCTVC-K0288)

- Add Main Still Picture profile (as agreed jointly in VCEG+MPEG+JPEG discussion).

- Incorporated 4x4 default scaling matrix as flat (JCTVC-K0203)

- Incorporated CABAC termination for the end of tile/WPP (JCTVC-K0182)

- Incorporated CABAC cleanup and fixes

- Fixed PCM mode definition (#796)

- Fixed infinite loop in intra mode parsing (#793)

- Disabled luma DC, hor. and ver. intra filtering for 32x32 blocks (JCTVC-K0380)

- Incorporated use of picture-level chroma QP offsets in deblocking (JCTVC-K0220)

- Dependent slices clarification, including the use of the term "slice segment" (JCTVC-K0314)

- Replaced the chroma subsampling figures, fixed and added CABAC decoding flowcharts

- Removed the slice (segment) extension data (JCTVC-K0361)

- Integrated re-allocation of NAL unit types per decisions noted under JCTVC-K0159

- Integrated the following decisions noted under JCTVC-K0120:

  - Move sps_temporal_nesting_flag to an earlier place to replace sps_reserved_zero_bit (later cleanup #864)

  - Clarification of semantics of end of sequence RBSP

  - Specify activation of VPS and SPS by APS SEI message and not specify SPS activation by the SPS ID in the BP SEI message

  - 2^24 constraint on POC difference for LTRP per BoG report (JCTVC-K0339)

  - Alignment of the range restriction on SPS ID (make it 16) and PPS ID (make it 64) for semantics and profile specification. We can simply remove this constraint in the profile specification

  - Removal of constraint on position of persistent SEI messages from 7.4.1.4.2

  - Specify a suffix SEI message NUT – with payloadType = 132 for the decoded picture hash SEI message

  - Added an informative table describing the scope of each type of SEI message

  - POC temporal relationship syntax based on JCTVC-K0343. timing_info_present_flag should have an inferred value of 0 when not present.

- Integrated SEI payload extension mechanism noted under JCTVC-K0371

- Integrated changes related to CRA and BLA pictures noted under JCTVC-K0122

- Integrated RPS changes noted under JCTVC-K0123

- Integrated VPS changes noted under JCTVC-K0125

- Integrated operation-point changes relating to JCTVC-K0204

- Integrated TL0 index SEI message under JCTVC-K0205

- Integrated addition of extra reserved slice header bits relating to JCTVC-K0210

- Integrated changes relating to temporal ID nesting relating to JCTVC-K0173

- Integrated VPS reserved bits changes relating to JCTVC-K0254

- Integrated sub-layering presence change for VPS and SPS and note on VPS discarding relating to JCTVC-K0330

- Integrated change of dependent slice segments flag locations relating to JCTVC-K0184

- Integrated moving of restricted_ref_pic_lists_flag and lists_modification_present_flag relating to JCTVC-K0170

- Integrated changes to ref_pic_list_modification( ) relating to proposal #1 of JCTVC-K0224

- Integrated change to not send inter_ref_pic_set_prediction_flag for index 0 relating to JCTVC-K0136

- Integrated change to slice_temporal_mvp_enable_flag in slice header noted under JCTVC-K0251

- Fixed several issues (#808, #809, #811, #812, #814, #815, #823, #825, #839, #842, #844, #846).

- Fixed several issues (#802, #685, #847, #849, #850, #851, #852, #853, #855, #856, #857, #783).

- Integrated decisions on FPA SEI messages and display/crop windows relating to JCTVC-K0382.

- Fixed several issues (#858, #859, #860, #863).

- Integrated  VPS changes, scalable nesting SEI message, and HRD and related changes under JCTVC-K0180 and JCTVC-K0126

- Integrated sub-picture CPB size under JCTVC-K0221

- Integrated fixed_pic_rate_cvs_flag under JCTVC-K0140

- Integrated gradual decoding refresh information SEI message under JCTVC-K0128

Draft 8 of High efficiency video coding.

Ed. Notes (D8):

- prev_intra_pred_flag should be prev_intra_luma_pred_flag (#624)

- fixed level 2 MaxBR and Max CPB Size (#625)

- Revised the Annex A tables based on JCTVC-J0558, JCTVC-J0154, JCTVC-J0151.

- Renamed no_residual_data_flag to no_residual_syntax_flag (editorial only). (JCTVC-J0336)

- Added limit to the minimum CTB size to 32x32 for level 5 and higher while still requiring decoding of lower level bitstreams (JCTVC-J0334)

- Integrated step wise increase of MaxDpbSize in spirit of JCTVC-J0496

- Bug fix for recovery point SEI so that it can point to a point before the recovery point

- Remove rap_pic_id and move no_output_of_prior_pic_flag to the top before any ue(v) (JCTVC-J0108)

- Added editorial suggestions from JCTVC-J0345

- Added picture timing SEI message syntax changes and corresponding text to Annex C CPB operation (JCTVC-J0306 and JCTVC-J0136)

- Added text for sub-picture based CPB removal timing (JCTVC-J0569)

- Added GTLA (STSA) related text (JCTVC-J0305)

- Added TFD related constraints on the RPS that are missing (JCTVC-J0229)

- Integrated RAP NUT according to (JCTVC-J0344) and GTLA (STSA) NUT according to (JCTVC-J0305)

- Remove nal_ref_flag and related text, added 3 new non-reference NUT and the RPS constraint (JCTVC-J0549)

- prev_intra_pred_flag should be prev_intra_luma_pred_flag (#624)

- Fixed level 2 MaxBR and Max CPB Size (#625)

- Integrated Inter-RPS complexity reduction (JCTVC-J0234) and restriction on delta_idx_minus1 (JCTVC-J0185r2)

- Reverted definition of prevRefPic to have TemporalId equal to 0 (JCTVC-J0248)

- Integrated changes of NAL unit header, VPS, HRD parameters and other changes per adoptions noted under JCTVC-J0550, JCTVC-J0548, and JCTVC-J0562 (JCTVC-J0550, JCTVC-J0548, and JCTVC-J0562)

- Incorporated removal of zigzag scan from scaling list coding (JCTVC-J0150)

- Incorporated removal of the 8x2/2x8 coefficient groups (JCTVC-J0256)

- Incorporated SAO syntax changes (JCTVC-J0563)

- Incorporated greater1 and greater2 counter removal (JCTVC-J0408)

- Incorporated split_transform_flag ctx derivation cleanup (JCTVC-J0133)

- Incorporated bypass bins for reference index coding (JCTVC-J0098)

- Incorporated bin reduction for delta QP coding (JCTVC-J0089)

- Incorporated coeff_abs_level_remaining bin reduction (JCTVC-J0142)

- Incorporated simplification on context derivation of cbf_luma syntax element (JCTVC-J0303)

- Incorporated chroma QP range extension (JCTVC-J0342)

- More flexible syntax for tile and WPP combination using tiles_enabled_flag, entropy_coding_sync_enabled_flag, and entropy_slice_enabled_flag (JCTVC-J0558).

- Incorporated end of bitstream and end of sequence NUTs, with end of bitstream indicating HRD discontinuity (response to JCTVC-J0290 and JCTVC-J0347)

- Removed ALF

- Removed LM Chroma

- Removed fine-granularity slices

- Removed NSRQT

- Incorporated transform simplification for 4x4 luma intra transform block (JCTVC-J0021)

- Moved transform_skip_enabled_flag to PPS (JCTVC-J0184)

- Moved seq_loop_filter_across_slices_enabled_flag to PPS (JCTVC-J0288)

- Incorporated inter transform skipping changes (JCTVC-J0237)

- Use of one bit from profile_space for indication of the level tier

- A clarification of intended tolerance of decoders for reserved values (JCTVC-J0112)

- POC value range to 32 bits (JCTVC-J0084)

- Modification of POC definition in Clause 3 and the POC constraint in Annex C.4 (JCTVC-J0110)

- Changing SEI NAL unit to be allowed to follow the first VCL NAL unit in an AU, added sub-picture timing SEI message, and clarified hat existing SEI messages that have whole-picture scope shall appear before the first slice in the picture (JCTVC-J0255)

- Modification of the definitions of reference pictures and related (related to JCTVC-J0118)

- Clarification of semantics of vps_temporal_id_nesting_flag and sps_temporal_id_nesting_flag (JCTVC-J0183)

- Slice header syntax clean-up (JCTVC-J0300)

- Change of definitions of slice and tile scan (JCTVC-J0209)

- Integrated long-term reference pictures in SPS (JCTVC-J0116)

- Support of UHDTV colorimetry (JCTVC-0577, JCTVC-J0477)

- Change on signalling of luminance dynamic range in tone mapping information SEI (JCTVC-J0149)

- Change on CU QP delta enabling syntax (JCTVC-J0220)

- Inclusion of motion related hooks (JCTVC-J0568, JCTVC-J0071, JCTVC-J0121)

- Modified sub-bitstream extraction process (JCTVC-J0074)

- Modified prediction weight table syntax and semantics for slice header parsing overhead reduction and improved value range for chroma weight offset (JCTVC-J0571 and JCTVC-J0221)

- Changed example time interval for tc to a frame-based example rather than a field coding example (JCTVC-J0136)

- Changed semantics of restricted_ref_pic_lists_flag (JCTVC-J0290)

- Incorporated disallow bi-predictive mode for 8x4 and 4x8 inter PUs (JCTVC-J0086)

- Incorporated various limits and modulo MV interpretation (response to JCTVC-J0579 BoG report)

Draft 7 of High efficiency video coding.

Ed. Notes (D7):

- RQT related issues (#459)

- Log2MaxTrafoSize constraint (#348)

- log2TrafoWidth1 and log2TrafoHeight1 calculation for NSQT (#458)

- non_square_quadtree_enabled_flag (#403)

- Wrong 'else if...' for 'if (skip_flag)' in CU syntax (#452)

- use_delta_flag[ j ] is not decoded when used_by_curr_pic_flag[ j ] is 1 (#451)

- combined_inter_pred_ref_idx does not exist anymore (#448)

- Typos in reference picture list combination (#446)

- Text cleanup of QP prediction / derivation (#492)

- Equation (7-59) and (7-60) for RPS derivation do not match HM6.0 (#445)

- Chroma NSRQT fixes (#505,#506)

- Missing decoding process for AMP fixed (#361)

- Deblocking processing order fixed (#412)

- Derivation of coefficient group scan fixed (#372)

- Phrase "one of the following conditions is true" fixed (#540)

- Non-normative aspects in scaling process removed (#544)

- scaling_list_present_flag issues fixed (#407)

- CTB,CB,PB,TB/CTU,CU,PU,TU defined

- beta_offset_div2 and tc_offset_div2 semantics fixed (#353)

- pixel replaced by sample (#406)

- references in HRD Annex C fixed (#557)

- sao_type_idx binarization fixed (#367)

- raster scan to tile scan order fixed (#376)

- error in tileId[] derivation loop fixed (#558)

- CtbAddrRS update in slice data syntax fixed (#345)

- SAO process issues fixed (#504)

- SAO clipping added (#517)

- fixed-length (FL) binarization process fixed (#518)

- intra transform skipping shift with high bit-depth fixed (#560)

- missing syntax element sao_type_idx added in sao_param syntax table (#576)

- residual coding semantics typo fixed (#333)

- PCM alignment at PU syntax fixed (#346)

- non-break hyphen and minus fixed (#347)

- uneccessary shared_pps_info_enabled_flag semantics removed (#351)

- skip_flag semantics fixed (#355)

- POC usage description fixed (#359)

- replaced coding tree depth variables cuDepth and cbDepth with ctDepth (#385)

- PicWidthInSamples fixed (#443)

- RefPicList naming fixed (#478)

- References to StCurr0/1 changed to StCurrBefore/After (#482)

- two references in 8.1 fixed (#479)

- one reference in 8.1 fixed (#509)

- replaced 'nal_ref_idc' by 'nal_ref_flag' in subclause 8.3.6 (#510)

- removed non-breaking space between log2_min_coding_block_size_minus and 3 (#386)

- removed the "+1" in clause C in description of DPB size (#522)

- moved the restriction on the relationship between tiles and slices to subclause 6.3 (#331)

- Subclause 0.7 text reference issue fixed (#572)

- cu_qp_delta binarization fixed (#588)

- typo fixed (#480)

- indices of nonsquare scaling matrix fixed (#535)

- intra_chroma_pred_mode bypass-coded bin fixed (#570)

- typos fixed and definition of picture added (#378)

- uneccesary input paramter in luma block edge deblocking removed (#583)

- missing index i for sao_offset_abs in sao_param syntax added (#592)

- fixed the Sign() function for SAO edgeIdx calculation (#591)

- picture constrution process added and embedded properly (#364)

- removed SignalledAsChromaDC because it not used anymore (#371)

- CABAC cross references fixed (#462)

- recovery_frame_cnt semantics removed (#600)

- nal_unit_type values updated in temporal_id semantics (#601)

- bugs, wordings, typos fixed in inter prediction process (#433)

- removed redundant equations in WP process (#416)

- fixed clipping values in MV scaling (#341)

- fixed 14-bit issues in WP (#594)

- fixed significant_coeff_group_flag for 2x8 and 8x2 CGs (#609)

- fixed inter_pred_idc binarization (#585)

- clarified condition in "6.4.5 Up-right diagonal scanning array initialization process" (#612)

- revision of weighted prediction process (#606)

- added transform_skip_enabled_flag semantics (#613)

- removed old alf aps syntax (#615)

- fixed decoding process for intra blocks for chroma (#330)

- fixed filtering process for neighbouring samples for Intra_FromLuma (#404)

- IntraPredMode derivation improvement (#360)

- vui_parameters_present_flag semantics added (#477)

- fixed Cr pcm_sample_chroma location in PCM intra decoding (#526)

- renamed unused intraPreModeN to candIntraPreModeN (#581)

- fixed sig flag semantics for 8x2 / 2x8 coefficient groups (#617)

- fixed out of bounds index computation in Angular Intra prediction (#339)

- fixed table for invAngle (#363)

- intra decoding process cleanup and fixes (#417)

- fixed %32 error for luma intra prediction mode derivation process (#493)

- generalized angular prediction process (#537)

- fixed aps_extension_flag semantics (#621)

- fixed ctxIdxInc for cbf_cb and cbf_cr overflow (#619)

- fixed mismatch between WD and HM on end_of_slice_flag (#598)

- fixed typo in 8.3.2 Decoding process for reference picture set (#622)

- fixed coeff_abs_level_greater1/2_flag ctx derivation (#384)

- updated CABAC init values to match HM (#473)

- fixed cRiceParam mismatch (#636)

- fixed transform issues (#524)

- fixed typos in 9.3 CABAC parsing process (#637)

- fixed hor and ver intra filtering (#639)

- Incorporated CBF coding without derivation process (JCTVC-I0152)

- Incorporated Unified CBFU and CBFV Coding in RQT (JCTVC-I0332)

- Incorporated BoG on I_PCM / lossless deblocking unification (JCTVC-I0586)

- Incorporated transform and quantization bypass (JCTVC-I0529)

- Incorporated intra 4x4 transform skipping (JCTVC-I0408)

- Incorporated modifed deblocking threshold derivation table (JCTVC-I0258)

- Incorporated constrained motion data compression (JCTVC-I0182)

- Removed SAO parameters from APS (JCTVC-I0021)

- Incorporated SAO offset signaling with magnitude and sign (JCTVC-I0168)

- Incorporated SAO offset magnitude TU binarization (JCTVC-I0066)

- Incorporated no SAO merge at tile boundaries (JCTVC-I0172)

- Incorporated reordering of slice type values (JCTVC-I0500)

- Incorporated having tile syntax only in PPS and reordering of pic_parameter_set_id in slice header to solve the slice header parsing issue (JCTVC-I0113)

- Incorporated moving list scaling syntax as well as deblocking filter parameters from APS to SPS and PPS (JCTVC-I0465)

- Incorporated a note on the presense of required parameter sets for random access (JCTVC-I0067)

- Incorporated an additional constraint on RPS for TLA (TSA) pictures (JCTVC-I0236)

- Incorporated changing the derivation of the variable prevRefPic used in derivation of picture order count (JCTVC-I0345)

- Incorporated high-level syntax clean-ups on TMVP enabling as well as signalling of collocated picture, CU QP delta, entropy slice header, and slice header syntax (JCTVC-I0127, JCTVC-I0266, and JCTVC-I0420)

- Incorporated mandating nal_ref_flag to be 1 for CRA pictures (JCTVC-I0143)

- Incorporated broken link access (BLA) pictures, signaling of leading/TFD pictures, signalling of presence of leading/TFD pictures, and allocation of NAL unit types (JCTVC-I0275, JCTVC-I0278, JCTVC-I0404 and JCTVC-I0607)

- Incorporated removal of combined list (JCTVC-I0125)

- Incorporated entropy slice enabling in PPS, support of dependent slice, a constraint on prevRefPic, slice header byte alignment, entropy slice header, relationship between TLA (TSA) and temporal_id_nesting_flag, tile and WPP byte alginment, semantics of num_reorder_pics[ i ], and semantics of temporal_id (JCTVC-I0138, JCTVC-I0229, JCTVC-I0330 and JCTVC-I0600)

- Incorporated WPP simplification and a restriction on coexistence of WPP and slices (JCTVC-I0360 and JCTVC-I0361)

- Incorporated removal of entry point markers, addition of VUI flag tiles_fixed_structure_flag, mandating entry point signalling for each tile and WPP sub-stream, and entry point offsets being relative to end of slice header (JCTVC-I0159, JCTVC-I0233, JCTVC-I0237 and JCTVC-I0357)

- Incorporated a fix for an unhandled LTRP case, a fix to the POC MSB cycle coding, coding of LTRP POC LSB directly as u(v), and no MVP scaling for LTRPs (JCTVC-I0234, JCTVC-I0340, JCTVC-I0422)

- Incorporated HRD buffering for CRA/BLA pictures and sub-picture based CPB operation (JCTVC-I0277 and JCTVC-I0588)

- Incorporated introduction of video parameter set, and extension mechanisms for slice header and slice layer RBSP (JCTVC-I0230 and JCTVC-I0235)

- Incorporated increasing of POC range to 64 bits, a limit of POC difference between the current picture and a long-term reference picture, changes to scaling list syntax, and changed signalling for profile and level (JCTVC-I0045, JCTVC-I0059, and JCTVC-I0499)

- Incorporated a bug fix for the recovery point SEI message and a change to the decoded picture hash SEI message (JCTVC-I0044 and JCTVC-I0218)

- Incorporated a change to the field indication SEI message (JCTVC-I0393)

- Incorporated adaptive loop filter text (JCTVC-I0603)

- Removed implicit weighted prediction (JCTVC-I0589)

- Incorporated simplified merge TMVP refIdx derivation (JCTVC-I0116)

- Incorporated AMVP and merge zero MV candidate list completing (JCTVC-I0314/JCTVC-I0134)

- Incorporated removal of number of combined merge candidate restriction (JCTVC-I0414)

- Removed inter 4x4 partitions

- Incorporated maxNumMergeCand signaling fix (JCTVC-I0256)

- Incorporated no bi-prediction for luma prediction blocks smaller than 8x8 (JCTVC-I0297)

- Incorporated cu_qp_delta parsing to enable CU-level processing (JCTVC-I0219)

- Incorporated simplification for multiple sign bit hiding  (JCTVC-I0156)

- Incorporated restricting the range of coeff_abs_level_remaining (JCTVC-I0254)

- Incorporated table removal in contexts assignment of last significant position coding (JCTVC-I0331)

- Incorporated simplified context derivation for significance map (JCTVC-I0296)

- Incorporated context derivation clean-up for significance map (JCTVC-I0373)

- Incorporated simplified coeff_abs_level_remaining binarization (JCTVC-I0487/I0124)

- Incorporated a maximum bound on slices per picture  (JCTVC-I0238)

- Incorporated Main Profile coding tool decisions

- Incorporated LM (intra chroma prediction based on luma) mode clean-up (JCTVC-I0148)

- Incorporated LM mode with uniform bit-width multipliers (JCTVC-I0151)

- Incorporated LM mode with uniform bit-width multipliers and reduced look-up table for division approximation (JCTVC-I0166)

- Incorporated LM mode with simplified alpha bit-depth restriction (JCTVC-I0178)

- Incorporated grouping of intra mode bypass bins for NxN intra prediction blocks (JCTVC-I0302)

- Incorporated level 2.1 and modified level 3 and 3.1 (JCTVC-I0472/I0455)

Ed. Notes (D6):

- Incorporated limiting dynamic range when qmatrix is used (JCTVC-H0541)

- Incorporated simplified intra horizontal and vertical modes (JCTVC-H0238)

- Incorporated DC mode as a default mode (JCTVC-H0242)

- Incorporated compatible QP prediction with RC and AQ (JCTVC-H0204)

- Incorporated burst transmission of I_PCM (JCTVC-H0051)

- SAO syntax fix (Ticket #308)

- Unused semantics removal related to reference picture list modification (Ticket #293)

- hPos and vPos table fix in SAO EO (Ticket #300)

- Typo in weighted prediction fix (Ticket #309)

- Incorporated clipping operation in strong deblocking (JCTVC-H0275)

- Incorporated removing sign of SAO offset (JCTVC-H0434)

- Fix QP’Cb and QP’Cr to consider QpBdOffsetC (Ticket #313)

- Fix padding issue of LM mode

- Incorporated deblocking filter simplification (JCTVC-H0473)

- Fix ALF syntax mismatch

- Incorporated ALF with single filter type (JCTVC-H0068)

- Incorporated intra mode coding clean-up and simplification (JCTVC-H0712)

- Fix clipping in DST (Ticket #307)

- Considering pcm_loop_filter_disable_flag in SAO (Ticket #301)

- Fix wrong geometry of sub-pel interpolation filter (Ticket #318)

- Fix max value of num_ref_idx_l0/l1_default_active_minus1 (Ticket #281)

- Fix 16x16 and 32x32 quantization matrices (Ticket #320)

- Incorporated quantization matrix signalling (JCTVC-H0237)

- Incorporated deblocking parameter signalling (JCTVC-H0424/H0398)

- Incorporated chroma mode signalling (JCTVC-H0475/H0326)

- Incorporated 4x4 and 8x8 default quantization matrices (JCTVC-H0461)

- Incorporated lossless mode (JCTVC-H0530)

- Incorporated downsampling of q-matrix (JCTVC-H0230)

- Fix ALF chroma coefficients prediction (Ticket #321)

- Incorporated two stage design ALF with LCU-based syntax (JCTVC-H0274)

- Incorporated SAO with LCU-based syntax (JCTVC-H0273)

- Incorporated change to add a condition for presence of ref_idx_list_curr (JCTVC-H0137 proposal #1)

- Incorporated unification of reference picture list modification processes (JCTVC-H0138)

- Incorporated change to NAL unit header and output flag (part of JCTVC-H0388)

- Incorporated coding treeblock and coding block scanning and address derivation.

- Incorporated changes relating to allowing the bitstream to start with a CRA picture (JCTVC-H0496)

- Incorporated multiple sign bits hiding (JCTVC-H0481)

- Incorporated 8x8 diagonal scan by 4x4 diagonal sub-scans (JCTVC-H0526/H0399)

- Incorporated abs_greater1 and abs_greater2 context reduction (JCTVC-H0130)

- Incorporated 8 bit codeword, change the Rice parameter to 4 (JCTVC-H0498)

- Incorporated high throughput binarization for CABAC (JCTVC-H0554)

- Incorporated sharing sig_coeff_flag cxtCnt=0 at high frequency area (JCTVC-H0095)

- Incorporated unified sig_coeff_flag context selection for 16x16 and 32x32 (JCTVC-H0290)

- Incorporated simplification on sig_coeff_group_flag coding (JCTVC-H0131)

- Incorporated Profiles and Levels (JCTVC-H0738)

- Incorporated CABAC initialization process (JCTVC-H0535)

- Incorporated part_mode context reduction (JCTVC-H0545)

- Incorporated merge index context reduction (JCTVC-H0251)

- Incorporated CABAC bit to bin expantion ratio limit (JCTVC-H0450)

- Incorporated CABAC_init_flag (JCTVC-H0540)

- Incorporated last_sig_coeff_position_prefix for luma context reduction (JCTVC-H0537)

- Incorporated last_sig_coeff_position_prefix for chroma context reduction (JCTVC-H0514)

- Incorporated unified transform and coefficient tree (JCTVC-H0123)

- Fix chroma cbf syntax mismatch (Ticket #295)

- Incorporated decisions relating to picture size and cropping parameters (response to JCTVC-H0485)

- Incorported adoptions on tiles, WPP and entropy slices documented in JCTVC-H0737 (response to JCTVC-H0439, H0463, H0513, H0517, and H0556 )

- Incorported decisions on long-term reference picture signalling (response to JCTVC-H0200 and JCTVC-H0531) and a resriction on POC values (response to JCTVC-H0449)

- Merge estimation region syntax (JCTVC-H0082)

- Setting the merge TMVP refidx to 0 for the non-first partition (JCTVC-H0278 / JCTVC-H0199)

- One merge candidate list for all partitions inside a 8x8 CU (JCTVC-H0240 Variant 2 conditioned on log2_parallel_merge_level_minus2 > 0 as described in H0082 section 6)

- Removing non-scaled bi-predictive merging candidates (JCTVC-H0250 /JCTVC-H0164)

- Removing the list empty check and the duplicate check in AMVP for zero motion vector (JCTVC-H0239)

- Removing redundant spatial candidates check in AMVP (JCTVC-H0316 first part)

- Clipping scaled MV to 16 bit and adjust it according to profile/level decisions (JCTVC-H0216 / JCTVC-H0555)

- Fix AMVP non-scaled/scaled candidate WD/HM mismatch by correcting the WD (JCTVC-H0462)

- Motion prediction at entropy slice boundary (JCTVC-H0362)

- Limiting collocated temporal reference to one per picture (JCTVC-H0442)

- Modification of bi-prediction syntax CE9 BP08 (JCTVC-H0111)

- CE9 SP (JCTVC-H0252)

- Incorported adoptions related to reference picture set, SPS syntax and HRD (JCTVC-H0568, H0566, H0567, H0423 and H0412).

- Rewrote SAO and ALF syntax and semantics and part of the processes.

- Incorporated harmonization of number of ALF classes between RA and BA modes  (JCTVC-H0409)

- Incorporated harmonization of ALF luma and chroma center coefficient prediction (JCTVC-H0483)

- Incorporated JCTVC-H0174-B with modification of number of bands and coding of the offset band according to JCTVC-H0406 (5b FLC/bypass coding)..

- Imported VUI from AVC with HEVC modifications (based upon JCTVC-F289)

- Imported SEI messages from AVC with HEVC semantic restrictions (based upon JCTVC-E346)

- Incorporated display orientation SEI (VCEG-AR12_r2)

- Incorporated temporal structure SEI (JCTVC-H0423)

- Incorporated decoded picture hash SEI (JCTVC-E490)

- Incorporated field indication SEI and VUI (JCTVC- H0720)

- Incorporated inheriting the QP prediction value at the left edge from the slice header in which the LCU belongs (JCTVC-H0226)

- Fixed missing definition of "cu_qp_delta_enabled_flag" (Ticket #310)

- Incorporated start-code based markers for signalling of tile entry points (JCTVC-F594)

- Made the following clean-up changes:

  - Addressing editing notes and cross references in subclauses 0.2, 0.3, 0.6, 0.7 and Clause 3

  - Various changes to the definitions, including

    - Correcting of the definition of "IDR picture", as marking all reference pictures as "unused for reference" is not anymore immediately after the decoding of the IDR picture due to the reference picture set based picture buffering mechanism

    - Resolving editing notes related to the definitions of "leading picture" and "output order"

    - Resolving editing notes related to the definitions of "reference picture list (X)" (X being 0 or 1)

    - Correcting for the indention of some bullet items in subclause 8.3.3 (Decoding process for generating unavailable reference pictures)

    - Improving definitions of "tile", "tree"

    - Added a definition of "z-scan"

  - Removing mentioning of redundant pictures and changing "primary (coded) picture" to "(coded) picture", auxiliary (coded) picture, and data partitioning

  - Moving of the sentence "The first coded picture in a bitstream shall be an IDR picture or a CRA picture." from the decoder conformance subclause to the bitstream conformance subclause

- Incorporated the ability, at the slice level, to disable loop filtering across slice boundaries (in response to JCTVC-H0391)

Working Draft 5 of High Efficiency Video Coding.

Ed. Notes (WD5):

- Incorporated weighted prediction (JCTVC-F265)

- Removed CAVLC

- Incorporated wavefront parallel processing (JCTVC-F274)

- Incorporated wavefront CABAC flush (JCTVC-F275)

- Incorporated tiles (JCTVC-F335)

- Removed ClipMv (JCTVC-G134)

- Removed merge partition redundancy check (JCTVC-G681)

- Incorporated simplified merge pruning (JCTVC-G1006)

- Incorporated extend scaling factor clipping to 16 (JCTVC-G223)

- Incorporated amvp position dependency removal (JCTVC-G542)

- Incorporated simplified TMVP refidx derivation (JCTVC-G163)

- Incorporated MaxNumMergeCand signalling in slice header (JCTVC-G091)

- Incorporated modified H and center TMVP positions (JCTVC-G082)

- Incorporated intra smoothing for horizontal and vertical directions (G457)

- Incorporated removal of ALF DC offset (JCTVC-G445)

- Incorporated line buffer elimination (JCTVC-G145)

- Incorporated simplified intra mode mapping (JCTVC-G418/G109/G144)

- Incorporated modified intra mode coding (JCTVC-G119)

- Incorporated luma interpolation filter (JCTVC-G778)

- Incorporated chroma interpolation filter (JCTVC-G778)

- Incorporated simplified intra padding (JCTVC-G812)

- Incorporated modified cRiceParam update (JCTVC-G700)

- Incorporated 8bit init values for CABAC (JCTVC-G633)

- Incorporated harmonized pred and part mode binarization (JCTVC-G1042)

- Incorporated significant map context reduction (JCTVC-G1015)

- Incorporated level chroma context reduction (JCTVC-G783)

- Incorporated diagonal sub-block scan for residual coding (JCTVC-G323)

- Removed NSQT remapping and transform reordering (JCTVC-G1038)

- Incorporated modified last_significant_coeff_x/y coding (JCTVC-G201/G704)

- Incorporated shared chroma CBF contexts (JCTVC-G718)

- Incorporated luma intra mode bypass coding (JCTVC-G767)

- Incorporated multi-level significant map (JCTVC-G644)

- Incorporated WD and HM mismatch for LM prediction (JCTVC-G1034)

- Revert the slice boundary padding for adaptive loop filter to WD4

- Incorporated 4x4 BA classification (JCTVC-G609)

- Incorporated virtual boundary processing (JCTVC-G212)

- Incorporated fixed K-table for ALF (JCTVC-G610)

- Incorporated removing 15th merge flag for BA mode in ALF (JCTVC-G216)

- Incorporated prediction of ALF coefficients (JCTVC-G665)

- Incorporated deblocking clean-up (JCTVC-G1035/G620)

- Revert the deblocking decision to HM3 (JCTVC-G088)

- Incorporated support of varying QP in deblocking (JCTVC-G1031)

- Incorporated reducing motion data line buffers (JCTVC-G229)

- Incorporated reference picture set (RPS) (JCTVC-G1002)

- Incorporated reference picture set prediction (JCTVC-G198)

- Incorporated separate decisions for each half (4 lines) of a length 8 block boundaries (JCTVC-G590)

- Incorporated BoG on deblocking fix (JCTVC-G1035)

- Incorporated core transform (JCTVC-G495)

- Incorporated clipping at the output of the first inverse transform (JCTVC-G782)

- Incorporated forbidding level values outside of 16b (JCTVC-G719)

- Incorporated reducing cbf flag signalling redundancy (JCTVC-G444)

- Incorporated changing luma/chroma coefficient interleaving from CU to TU level (JCTVC-G381)

- Incorporated defining MaxIPCMCUSize, MinChromaTrafoSize (JCTVC-G112)

- Incorporated harmonization of implicit TU, AMP and NSQT (JCTVC-G519)

- Incorporated improved weighted prediction (JCTVC-G065)

- Incorporated redundancy removal of explicit weighted prediction syntax (JCTVC-G441)

- Incorporated non-cross-tiles loop filtering for independent tiles (JCTVC-G194)

- Incorporated low latency CABAC initialization for dependent tiles (JCTVC-G197)

- Incorporated AVC-based quantization matrices syntax (JCTVC-G434)

- Incorporated HVS-based quantization matrices (JCTVC-G880)

- Incorporated APS quantization matrices and parameter set extension syntax (JCTVC-G1016)

- Incorporated nal_unit_type value of 14 for APS

- Incorporated SPS syntax for chroma_format_idc from AVC

- Incorporated pure VLC for SAO and ALF (JCTVC-G220)

- Moved slice address and put slice_type and cabac_init_idc into slice and entropy slice header. (JCTVC-G1025)

- Incorporated picture width and height coding using ue(v) rather than u(16) (JCTVC-G325)

- Incorporated ALF and SAO flags in slice header (JCTVC-G566)

- Incorporated marking process for non-TMVP pictures (JCTVC-G398)

- Incorporated max_dec_frame_buffering, num_reorder_frames, and use max_latency_increase (JCTVC-G546)

- Incorporated high level syntax clean up (JCTVC-G507)

- Incorporated chroma QP offset (JCTVC-G509)

Working Draft 4 of High Efficiency Video Coding.

Ed. Notes (WD4):

- Removed inferred merge (JCTVC-F082)

- Incorporated slice header flag to disable 4x4 inter partitions (JCTVC-F744)

- Incorporated modified rounding in MV scaling (JCTVC-F142)

- Removed intermediate amvp spatial candidates redundancy check (JCTVC-F050)

- Incorporated reducing the number of spatial mv scalings to 1 (JCTVC-F088)

- Incorporated spatial merge candidate positions unification (JCTVC-F419)

- Incorporated one reference list check for temporal mvp (JCTVC-F587)

- Incorporated AMVP/merge parsing robustness with simplifications (JCTVC-F470)

- Incorporated unified availability check for intra (JCTVC-F477)

- Incorporated generic interpolation filter (JCTVC-F537)

- Incorporated non-square quadtree transform NSQT (JCTVC-F412)

- Incorporated asymmetric motion partitions AMP (JCTVC-F379)

- Incorporated CBF redundancy reduction (JCTVC-C277)

- Incorporated modified delta QP binarization (JCTVC-F745)

- Incorporated diagonal coefficient scanning in CABAC (JCTVC-F129)

- Incorporated parallel context processing for coefficient levels in CABAC (JCTVC-F130)

- Incorporated context sharing for significant_coeff_flag of 16x16 and 32x32 transforms (JCTVC-F132)

- Incorporated unified scans (JCTVC-F288)

- Incorporated sample adaptive offset (JCTVC-E049)

- Incorporated sample adaptive offset for chroma (JCTVC-F057)

- Incorporated sample adaptive offset offset accuracy (JCTVC-F396)

- Incorporated updated ALF slice padding (JCTVC-D128)

- Incorporated updated ALF slice padding due to ALF filter shape change (JCTVC-F303/F042)

- Incorporated updated ALF slice padding due to unified luma and chroma filter shapes (JCTVC-F157)

- Incorporated ALF filter using subset of pixels (JCTVC-F301)

- Incorporated modified deblocking process for luma (JCTVC-F118)

- Incorporated modified tc_offset in deblocking process (JCTVC-F143)

- Incorporated MDIS and pixel position change of planar (JCTVC-F483)

- Incorporated availability check removal for intra DC filtering (JCTVC-F178)

- Incorporated size-independent intra DC filtering (JCTVC-F252)

- Incorporated modified MDIS table (JCTVC-F126)

- Incorporated simplified intra_FromLuma prediction (JCTVC-F760)

- Incorporated fixed number of MPM (JCTVC-F765)

- Incorporated SAO boundary processing (JCTVC-F232)

- Minor bug in deriving sample positions in SAO process was fixed

- Bug in coding tree syntax table related to the initialization of variable IsCuQpDeltaCoded was fixed

- Incorporated modified last significant coefficient position coding in CABAC (JCTVC-F375)

- Incorporated modified mvd coding in CABAC (JCTVC-F455)

- Incorporated reduced number of contexts in CABAC (JCTVC-F746)

- Incorporated high-level syntax cleanup (JCTVC-F714)

- Incorporated NAL unit type and CDR (CRA) (JCTVC-F462/464)

- Incorporated adaptation parameter set (APS) (JCTVC-F747)

Ed. Notes (WD3):

- Added Residual coding CABAC syntax and semantics

- Added Zig-zag scanning process

- Added CABAC Binarization processes

- Incorporated MV coding (JCTVC-E481)

- Incorporated Compression of reference indices (JCTVC-E059)

- Incorporated Zero merge candidate (JCTVC-E146)

- Incorporated Intra mode coding (JCTVC-E088/E131) (Inserted by TK 31/3/2011 with notes)

- Fixed the CABAC coefficients syntax, semantics and inverse scanning process

- Incorporated CABAC coeffs (JCTVC-E253)

- Moved the EGk binarization from the UEGk subclause in a separate subclause

- Added text representing CABAC entropy coding context initialization

- Added text representing CABAC entropy coding context derivation.

- Mode-dependent 3- scan for intra (JCTVC-D393)

- Incorporated CABAC: Context size reduction (JCTVC-E227/E489)

- Incorporated CABAC: significance map coding simplification (JCTVC-E227/E338/E344/E494)

- Incorporated CABAC: Contexts for MVD (JCTVC-E324)

- Incorporated initial draft of CAVLC text

- CAVLC for 16x16 & 32x32 (JCTVC-E383)

- CAVLC table size reduction (JCTVC-E384)

- CAVLC for RQT  (JCTVC-E404)

- CAVLC: counters (JCTVC-E143)

- CAVLC: Intra prediction mode coding in LCEC (JCTVC-D366)

- CAVLC: Inter prediction mode coding in LCEC (JCTVC- D370)

- CAVLC: 4x4 and 8x8 transform coefficient coding in LCEC (JCTVC- D374)

- Block-based ALF (JCTVC-E046/E323)

- ALF parameters to PPS (JCTVC-E045)

- Parallel deblocking (JCTVC-E496/E181/E224)

- Clipping for bi-pred averaging (JCTVC-E242)

- Reference sample padding (JCTVC-E488)

- Transformation processes are replaced by [TBD] mark (meeting note, JCTVC-E243)

- Sub-LCU-level dQP (JCTVC-E051/E220)

- Temporal layer switching and reference list management based on temporal_id (JCTVC-E279/D081/D200)

- Improved text of entropy slice (JCTVC-D070)

- Slice independent deblocking and adaptive loop filtering (JCTVC-D128)

- Fine-granularity slices (JCTVC-E483)

- PCM mode (JCTVC-E057 and JCTVC-E192)

- CAVLC: Inter pred coding (JCTVC-E381)

- CAVLC: Combined coding of inter prediction direction and reference frame index (JCTVC-D141)

- 4x4 DST (JCTVC-E125)

- Planar mode (JCTVC-E321)

- Luma-based chroma intra prediction (JCTVC-E266)

- Modification of DC predictor (JCTVC-E069)

- Bug in mapping table and the corresponding text for mostProbableIntra was fixed. (64x64 uses 3-directions, but the table was specified for 5-directions)

- Non-existing cases of Intra_DC, Intra_Planar and Intra_FromLuma are removed (due to reference sample padding, JCTVC-E488)

Ed. Notes (WD2):

- Incorporated Partial Merging according to JCTVC-D441

  - removed direct mode

  - moved merge to prediction_unit and added candidates

  - added partial merge restrictions

  - inter NxN partitioning only for smallest coding_unit

- Updated transform_tree and transform_coeff syntax

- Added transform_coeff to coding_unit syntax (Fix)

- Incorporated intra NxN partitioning only for smallest coding_unit according to JCTVC-D432

- Incorporated modified temporal motion vector predition according to JCTVC-D164

- Incorporated simplified motion vector prediction according to JCTVC-D231

  - removed median

  - removed pruning process

  - changed the selection manner of left/top predictor

- 8-tap luma interpolation filter according to JCTVC-D344

- 4-tap chroma interpolation filter according to JCTVC-D347

- Improved deblocking filter text according to JCTVC-D395

- IBDI syntax is removed

- Updated syntax and semantics

  - Two tool-enabling flags (adaptive_loop_filter_enabled_flag and cu_qp_delta_enabled_flag) are added in SPS according to software. However, low_delay_coding_enabled_flag is not added – it could be handled by more general reference management scheme. merging_enabled_flag is not added – partial merging (JCTVC-D441) was adopted thus merging cannot be turned off any more. amvp_mode[] is not added since amvp cannot be turned off any more due to absence of median predictor (JCTVC-D231). Note that software has all switches.

  - cu_qp_delta (coding unit layer), syntax and semantics are added. (JCTVC-D258)

  - collocated_from_l0 (slice header), syntax and semantics are added.

- Clean decoding refresh (CDR) (JCTVC-D234).

- Temporal motion vector memory compression (JCTVC-D072)

- Constrained intra prediction (JCTVC-D086)

- Mode-dependent intra smoothing (JCTVC-D282)

- Merging chroma intra prediction process into luma intra prediction process

- Combined reference list (JCTVC-D421)

- Chroma intra prediction mode reordering (JCTVC-D255/D278/D166)

- Adaptive loop filter text is added

- Entropy slice is added (JCTVC-D070)

- High precision bi-directional averaging (JCTVC-D321)

- Reduction of number of intra prediction modes for 64x64 blocks (JCTVC-D100)

- Misc.

  - TPE bits are reduced from 4 to 2

  - Clipping is applied to (temporally) scaled mv – revisit

Ed. Notes (WD1):

- Incorporated the decisions on high-level syntax according to JCTVC-B121

- Incorporated text from JCTVC-B205revision7

- Incorporated text from JCTVC-C319 (as found to be stable)

- Revised coding tree, coding unit and prediction unit syntaxes (coding tree syntax is newly added. needs to be confirmed)

- Initial drafting of decoding process of coding units in intra prediction mode (luma part, JCTVC-B100 and JCTVC-C042)

- Initial drafting of decoding process of coding units in inter prediction mode

- Initial drafting of scaling and transformation process

- Added text, transform 16T and 32T

- Initial drafting of deblocking process

- Improving the text, derivation process for motion vector components and reference indices

- Added text, boundary filtering strength

Open issues:

- Should support for monochrome, 4:2:2 and 4:4:4 (with and w/o separate colour planes) be included from the start? Currently, it has been left in the text as it doesn't seem to affect much text.

- Use of bin string and bit string should be consistent.

- Improve text quality by considering: strict use of terms "unit" and "block" – block = a rectangular 2D array (one component), unit = collective term for specifying information for both luma and chroma. Don’t use ther term ‘unit’ by itself – always use the term ‘unit’ with prefix – coding unit, prediction unit or transform unit.

- Both variables IntraPredMode and IntraPredModeC are used in the syntax table but the actual derivations are specified in the decoding process. Maybe it’s better to move them to the semantics section.

- Binarization of intra_chroma_pred_mode reflecting codeword switching and luma-based chroma intra prediction (JCTVC-E266) is missing.

- Clarification on the use of intra_chroma_pred_mode and IntraPredModeC is needed. The former specifies the syntax item to indicate how to determine the chroma intra prediction mode (IntraPredModeC) as 0 (to Intra_FromLuma), 1 (to Intra_Vertical), 2 (to Intra_Horizontal), 3 (Intra_DC or Intra_Planar) or 4 (re-use luma mode). The latter specifies chroma intra prediction mode, which is actually mapped to the specific prediction process.

- Software-text mismatch of JCTVC-F252: SW applies DC filtering to 64x64 intra block but WD does not.

- There are input/output parameter inconsistencies between function call and actual functions of intra prediction. Function call uses sample position, block, size and chroma index while some functions do not use some parameters.

- CONTENTS

Page

Abstract	i

0	Introduction	1

0.1	Prologue	1

0.2	Purpose	1

0.3	Applications	1

0.4	Publication and versions of this Specification	1

0.5	Profiles, tiers and levels	2

0.6	Overview of the design characteristics	2

0.7	How to read this Specification	2

1	Scope	3

2	Normative references	3

2.1	General	3

2.2	Identical Recommendations | International Standards	3

2.3	Paired Recommendations | International Standards equivalent in technical content	3

2.4	Additional references	3

3	Definitions	3

4	Abbreviations	11

5	Conventions	12

5.1	Arithmetic operators	12

5.2	Logical operators	13

5.3	Relational operators	13

5.4	Bit-wise operators	13

5.5	Assignment operators	13

5.6	Range notation	14

5.7	Mathematical functions	14

5.8	Order of operation precedence	14

5.9	Variables, syntax elements, and tables	15

5.10	Text description of logical operations	16

5.11	Processes	17

6	Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships	17

6.1	Bitstream formats	17

6.2	Source, decoded, and output picture formats	17

6.3	Spatial subdivision of pictures, slices, slice segments, and tiles	20

6.4	Availability processes	22

6.4.1	Derivation process for z-scan order block availability	22

6.4.2	Derivation process for prediction block availability	22

6.5	Scanning processes	23

6.5.1	Coding tree block raster and tile scanning conversion process	23

6.5.2	Z-scan order array initialization process	25

6.5.3	Up-right diagonal scan order array initialization process	25

6.5.4	Horizontal scan order array initialization process	25

6.5.5	Vertical scan order array initialization process	26

7	Syntax and semantics	26

7.1	Method of specifying syntax in tabular form	26

7.2	Specification of syntax functions and descriptors	27

7.3	Syntax in tabular form	28

7.3.1	NAL unit syntax	28

7.3.1.1	General NAL unit syntax	28

7.3.1.2	NAL unit header syntax	29

7.3.2	Raw byte sequence payloads, trailing bits, and byte alignment syntax	29

7.3.2.1	Video parameter set RBSP syntax	29

7.3.2.2	Sequence parameter set RBSP syntax	30

7.3.2.3	Picture parameter set RBSP syntax	31

7.3.2.4	Supplemental enhancement information RBSP syntax	33

7.3.2.5	Access unit delimiter RBSP syntax	33

7.3.2.6	End of sequence RBSP syntax	33

7.3.2.7	End of bitstream RBSP syntax	33

7.3.2.8	Filler data RBSP syntax	33

7.3.2.9	Slice segment layer RBSP syntax	33

7.3.2.10	RBSP slice segment trailing bits syntax	34

7.3.2.11	RBSP trailing bits syntax	34

7.3.2.12	Byte alignment syntax	34

7.3.3	Profile, tier and level syntax	35

7.3.4	Bit rate and picture rate information syntax	35

7.3.5	Operation point layer identifier flags syntax	36

7.3.6	Scaling list data syntax	36

7.3.7	Supplemental enhancement information message syntax	37

7.3.8	Slice segment header syntax	38

7.3.8.1	General slice segment header syntax	38

7.3.8.2	Short-term reference picture set syntax	41

7.3.8.3	Reference picture list modification syntax	41

7.3.8.4	Weighted prediction parameters syntax	42

7.3.9	Slice segment data syntax	43

7.3.9.1	General slice segment data syntax	43

7.3.9.2	Coding tree unit syntax	43

7.3.9.3	Sample adaptive offset syntax	44

7.3.9.4	Coding quadtree syntax	45

7.3.9.5	Coding unit syntax	46

7.3.9.6	Prediction unit syntax	48

7.3.9.7	PCM sample syntax	48

7.3.9.8	Transform tree syntax	49

7.3.9.9	Motion vector difference syntax	49

7.3.9.10	Transform unit syntax	50

7.3.9.11	Residual coding syntax	51

7.4	Semantics	53

7.4.1	NAL unit semantics	53

7.4.1.1	General NAL unit semantics	53

7.4.1.2	NAL unit header semantics	54

7.4.1.3	Encapsulation of an SODB within an RBSP (informative)	57

7.4.1.4	Order of NAL units and association to coded pictures, access units, and video sequences	58

7.4.2	Raw byte sequence payloads, trailing bits, and byte alignment semantics	61

7.4.2.1	Video parameter set RBSP semantics	61

7.4.2.2	Sequence parameter set RBSP semantics	63

7.4.2.3	Picture parameter set RBSP semantics	67

7.4.2.4	Supplemental enhancement information RBSP semantics	70

7.4.2.5	Access unit delimiter RBSP semantics	71

7.4.2.6	End of sequence RBSP semantics	71

7.4.2.7	End of bitstream RBSP semantics	71

7.4.2.8	Filler data RBSP semantics	71

7.4.2.9	Slice segment layer RBSP semantics	71

7.4.2.10	RBSP slice segment trailing bits semantics	71

7.4.2.11	RBSP trailing bits semantics	71

7.4.2.12	Byte alignment semantics	72

7.4.3	Profile, tier and level semantics	72

7.4.4	Bit rate and picture rate information semantics	72

7.4.5	Operation point layer identifier flags semantics	74

7.4.6	Scaling list data semantics	74

7.4.7	Supplemental enhancement information message semantics	77

7.4.8	Slice segment header semantics	77

7.4.8.1	General slice segment header semantics	77

7.4.8.2	Short-term reference picture set semantics	81

7.4.8.3	Reference picture list modification semantics	83

7.4.8.4	Weighted prediction parameters semantics	84

7.4.9	Slice segment data semantics	85

7.4.9.1	General slice segment data semantics	85

7.4.9.2	Coding tree unit semantics	85

7.4.9.3	Sample adaptive offset semantics	85

7.4.9.4	Coding quadtree semantics	87

7.4.9.5	Coding unit semantics	87

7.4.9.6	Prediction unit semantics	89

7.4.9.7	PCM sample semantics	90

7.4.9.8	Transform tree semantics	90

7.4.9.9	Motion vector difference semantics	91

7.4.9.10	Transform unit semantics	91

7.4.9.11	Residual coding semantics	92

8	Decoding process	94

8.1	General decoding process	94

8.2	NAL unit decoding process	95

8.3	Slice decoding process	96

8.3.1	Decoding process for picture order count	96

8.3.2	Decoding process for reference picture set	97

8.3.3	Decoding process for generating unavailable reference pictures	101

8.3.3.1	General decoding process for generating unavailable reference pictures	101

8.3.3.2	Generation of one unavailable picture	101

8.3.4	Decoding process for reference picture lists construction	102

8.4	Decoding process for coding units coded in intra prediction mode	102

8.4.1	General decoding process for coding units coded in intra prediction mode	102

8.4.2	Derivation process for luma intra prediction mode	103

8.4.3	Derivation process for chroma intra prediction mode	105

8.4.4	Decoding process for intra blocks	106

8.4.4.1	General decoding process for intra blocks	106

8.4.4.2	Intra sample prediction	107

8.5	Decoding process for coding units coded in inter prediction mode	113

8.5.1	General decoding process for coding units coded in inter prediction mode	113

8.5.2	Inter prediction process	113

8.5.3	Decoding process for prediction units in inter prediction mode	116

8.5.3.1	Derivation process for motion vector components and reference indices	116

8.5.3.2	Decoding process for inter prediction samples	130

8.5.4	Decoding process for the residual signal of coding units coded in inter prediction mode	139

8.5.4.1	Decoding process for luma residual blocks	140

8.5.4.2	Decoding process for chroma residual blocks	140

8.6	Scaling, transformation and array construction process prior to deblocking filter process	141

8.6.1	Derivation process for quantization parameters	141

8.6.2	Scaling and transformation process	143

8.6.3	Scaling process for transform coefficients	144

8.6.4	Transformation process for scaled transform coefficients	144

8.6.4.1	Transformation process	145

8.6.5	Picture construction process prior to in-loop filter process	147

8.7	In-loop filter process	147

8.7.1	General	147

8.7.2	Deblocking filter process	147

8.7.2.1	Derivation process of transform block boundary	149

8.7.2.2	Derivation process of prediction block boundary	150

8.7.2.3	Derivation process of boundary filtering strength	150

8.7.2.4	Edge filtering process	151

8.7.3	Sample adaptive offset process	160

8.7.3.1	General	160

8.7.3.2	Coding tree block modification process	160

9	Parsing process	162

9.1	Parsing process for 0-th order Exp-Golomb codes	162

9.1.1	Mapping process for signed Exp-Golomb codes	163

9.2	CABAC parsing process for slice segment data	165

9.2.1	Initialization process	166

9.2.1.1	Initialization process for context variables	168

9.2.1.2	Memorization process for context variables	175

9.2.1.3	Synchronization process for context variables	176

9.2.1.4	Initialization process for the arithmetic decoding engine	176

9.2.2	Binarization process	176

9.2.2.1	Unary (U) binarization process	181

9.2.2.2	Truncated unary (TU) binarization process	181

9.2.2.3	Truncated Rice (TR) binarization process	181

9.2.2.4	k-th order Exp-Golomb (EGk) binarization process	182

9.2.2.5	Fixed-length (FL) binarization process	182

9.2.2.6	Binarization process for cu_qp_delta_abs	182

9.2.2.7	Binarization process for part_mode	182

9.2.2.8	Binarization process for coeff_abs_level_remaining	183

9.2.2.9	Binarization process for intra_chroma_pred_mode	183

9.2.2.10	Binarization process for inter_pred_idc	184

9.2.3	Decoding process flow	184

9.2.3.1	Derivation process for ctxIdx	185

9.2.3.2	Arithmetic decoding process	190

9.2.4	Arithmetic encoding process (informative)	197

9.2.4.1	Initialization process for the arithmetic encoding engine (informative)	197

9.2.4.2	Encoding process for a binary decision (informative)	198

9.2.4.3	Renormalization process in the arithmetic encoding engine (informative)	199

9.2.4.4	Bypass encoding process for binary decisions (informative)	201

9.2.4.5	Encoding process for a binary decision before termination (informative)	202

9.2.4.6	Byte stuffing process (informative)	204

10	Specification of bitstream subsets	204

10.1	Sub-bitstream extraction process	204

Annex A  Profiles, tiers and levels	205

A.1	Overview of profiles, tiers and levels	205

A.2	Requirements on video decoder capability	205

A.3	Profiles	205

A.3.1	General	205

A.3.2	Main profile	205

A.3.3	Main 10 profile	206

A.3.4	Main Still Picture profile	206

A.4	Tiers and levels	207

A.4.1	General tier and level limits	207

A.4.2	Profile-specific level limits for the Main and Main 10 profiles	209

A.4.3	Effect of level limits on picture rate for the Main and Main 10 profiles (informative)	211

Annex B  Byte stream format	215

B.1	Byte stream NAL unit syntax and semantics	215

B.1.1	Byte stream NAL unit syntax	215

B.1.2	Byte stream NAL unit semantics	215

B.2	Byte stream NAL unit decoding process	216

B.3	Decoder byte-alignment recovery (informative)	216

Annex C  Hypothetical reference decoder	217

C.1	General	217

C.2	Operation of coded picture buffer (CPB)	219

C.2.1	Timing of bitstream arrival	219

C.2.2	Timing of decoding unit removal and decoding of decoding unit	221

C.3	Operation of the decoded picture buffer (DPB)	222

C.3.1	Removal of pictures from the DPB	222

C.3.2	Picture output	223

C.3.3	Current decoded picture marking and storage	223

C.4	Bitstream conformance	223

C.5	Decoder conformance	225

C.5.1	Operation of the output order DPB	226

C.5.2	Removal of pictures from the DPB	226

C.5.2.1	"Bumping" process	227

C.5.3	Picture decoding, marking and storage	227

Annex D  Supplemental enhancement information	228

D.1	SEI payload syntax	229

D.1.1	General SEI message syntax	229

D.1.2	Buffering period SEI message syntax	230

D.1.3	Picture timing SEI message syntax	231

D.1.4	Pan-scan rectangle SEI message syntax	231

D.1.5	Filler payload SEI message syntax	231

D.1.6	User data registered by Rec. ITU-T T.35 SEI message syntax	231

D.1.7	User data unregistered SEI message syntax	231

D.1.8	Recovery point SEI message syntax	231

D.1.9	Scene information SEI message syntax	231

D.1.10	Full-frame snapshot SEI message syntax	231

D.1.11	Progressive refinement segment start SEI message syntax	232

D.1.12	Progressive refinement segment end SEI message syntax	232

D.1.13	Film grain characteristics SEI message syntax	232

D.1.14	Post-filter hint SEI message syntax	232

D.1.15	Tone mapping information SEI message syntax	233

D.1.16	Frame packing arrangement SEI message syntax	234

D.1.17	Display orientation SEI message syntax	234

D.1.18	SOP description SEI message syntax	235

D.1.19	Field indication SEI message syntax	235

D.1.20	Decoded picture hash SEI message syntax	235

D.1.21	Active parameter sets SEI message syntax	236

D.1.22	Sub-picture timing SEI message syntax	236

D.1.23	Temporal level zero index SEI message syntax	236

D.1.24	Reserved SEI message syntax	236

D.2	SEI payload semantics	236

D.2.1	General SEI payload semantics	236

D.2.2	Buffering period SEI message semantics	237

D.2.3	Picture timing SEI message semantics	238

D.2.4	Pan-scan rectangle SEI message semantics	239

D.2.5	Filler payload SEI message semantics	239

D.2.6	User data registered by ITU-T Rec. T.35 SEI message semantics	239

D.2.7	User data unregistered SEI message semantics	239

D.2.8	Recovery point SEI message semantics	239

D.2.9	Scene information SEI message semantics	241

D.2.10	Full-frame snapshot SEI message semantics	241

D.2.11	Progressive refinement segment start SEI message semantics	241

D.2.12	Progressive refinement segment end SEI message semantics	241

D.2.13	Film grain characteristics SEI message semantics	241

D.2.14	Post-filter hint SEI message semantics	241

D.2.15	Tone mapping information SEI message semantics	241

D.2.16	Frame packing arrangement SEI message semantics	244

D.2.17	Display orientation SEI message semantics	255

D.2.18	SOP description SEI message semantics	256

D.2.19	Field indication SEI message semantics	257

D.2.20	Decoded picture hash SEI message semantics	260

D.2.21	Active parameter sets SEI message semantics	261

D.2.22	Sub-picture timing SEI message semantics	262

D.2.23	Temporal level zero index SEI message semantics	262

D.2.24	Reserved SEI message semantics	263

Annex E  Video usability information	264

E.1	VUI syntax	265

E.1.1	VUI parameters syntax	265

E.1.2	HRD parameters syntax	267

E.1.3	Sub-layer HRD parameters syntax	267

E.2	VUI semantics	268

E.2.1	VUI parameters semantics	268

E.2.2	HRD parameters semantics	278

E.2.3	Sub-layer HRD parameters semantics	280

Bibliography	281

- List of figures

Figure 6-1 – Nominal vertical and horizontal locations of 4:2:0 luma and chroma samples in a picture	19

Figure 6-2 – Nominal vertical and horizontal locations of 4:2:2 luma and chroma samples in a picture	19

Figure 6-3 – Nominal vertical and horizontal locations of 4:4:4 luma and chroma samples in a picture	20

Figure 6-4 – A picture with 11 by 9 luma coding tree blocks that is partitioned into two slices, the first of which is partitioned into three slice segments (informative)	21

Figure 6-5 – A picture with 11 by 9 luma coding tree blocks that is partitioned into two tiles and one slice (left) or is partitioned into two tiles and three slices (right) (informative)	21

Figure 7-1 – Structure of an access unit not containing any NAL units with nal_unit_type equal to FD_NUT, VPS_NUT, SPS_NUT, PPS_NUT, or in the ranges of RSV_RAP_VCL22..RSV_RAP_VCL23, RSV_VCL24..RSV_VCL31, RSV_NVCL40..RSV_NVCL47, or UNSPEC48..UNSPEC63	61

Figure 8-1 – Intra prediction mode directions (informative)	104

Figure 8-2 – Intra prediction angle definition (informative)	111

Figure 8-3 – Spatial motion vector neighbours (informative)	126

Figure 8-4 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation	133

Figure 8-5 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for eighth sample chroma interpolation	135

Figure 9-1 – Illustration of CABAC parsing process for a syntax element SE (informative)	166

Figure 9-2 – Spatial neighbour T that is used to invoke the coding tree block availability derivation process relative to the current coding tree block (informative)	167

Figure 9-3 – Illustration of CABAC initialization process (informative)	168

Figure 9-4 – Illustration of CABAC memorization process (informative)	176

Figure 9-5 – Overview of the arithmetic decoding process for a single bin (informative)	191

Figure 9-6 – Flowchart for decoding a decision	193

Figure 9-7 – Flowchart of renormalization	196

Figure 9-8 – Flowchart of bypass decoding process	196

Figure 9-9 – Flowchart of decoding a decision before termination	197

Figure 9-10 – Flowchart for encoding a decision	199

Figure 9-11 – Flowchart of renormalization in the encoder	200

Figure 9-12 – Flowchart of PutBit(B)	201

Figure 9-13 – Flowchart of encoding bypass	202

Figure 9-14 – Flowchart of encoding a decision before termination	203

Figure 9-15 – Flowchart of flushing at termination	204

Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks [Ed. (KJS): Text renders poorly on screen – redraw the figure.]	217

Figure C-2 – HRD buffer model	218

Figure D-1 – Rearrangement and upconversion of checkerboard interleaving  (frame_packing_arrangement_type equal to 0)	250

Figure D-2 – Rearrangement and upconversion of column interleaving  with frame_packing_arrangement_type equal to 1, quincunx_sampling_flag equal to 0, and (x, y) equal to (0, 0) or (4, 8) for both constituent frames	250

Figure D-3 – Rearrangement and upconversion of column interleaving with frame_packing_arrangement_type equal to 1, quincunx_sampling_flag equal to 0,  (x, y) equal to (0, 0) or (4, 8) for constituent frame 0 and (x, y) equal to (12, 8) for constituent frame 1	251

Figure D-4 – Rearrangement and upconversion of row interleaving with frame_packing_arrangement_type equal to 2, quincunx_sampling_flag equal to 0, and (x, y) equal to (0, 0) or (8, 4) for both constituent frames	251

Figure D-5 – Rearrangement and upconversion of row interleaving with frame_packing_arrangement_type equal to 2, quincunx_sampling_flag equal to 0,  (x, y) equal to (0, 0) or (8, 4) for constituent frame 0, and (x, y) equal to (8, 12) for constituent frame 1	252

Figure D-6 – Rearrangement and upconversion of side-by-side packing arrangement with frame_packing_arrangement_type equal to 3, quincunx_sampling_flag equal to 0, and (x, y) equal to (0, 0) or (4, 8) for both constituent frames	252

Figure D-7 – Rearrangement and upconversion of side-by-side packing arrangement with frame_packing_arrangement_type equal to 3, quincunx_sampling_flag equal to 0, (x, y) equal to (12, 8) for constituent frame 0, and (x, y) equal to (0, 0) or (4, 8) for constituent frame 1	253

Figure D-8 – Rearrangement and upconversion of top-bottom packing arrangement with frame_packing_arrangement_type equal to 4, quincunx_sampling_flag equal to 0, and (x, y) equal to (0, 0) or (8, 4) for both constituent frames	253

Figure D-9 – Rearrangement and upconversion of top-bottom packing arrangement with frame_packing_arrangement_type equal to 4, quincunx_sampling_flag equal to 0, (x, y) equal to (8, 12) for constituent frame 0, and (x, y) equal to (0, 0) or (8, 4) for constituent frame 1	254

Figure D-10 – Rearrangement and upconversion of side-by-side packing arrangement with quincunx sampling (frame_packing_arrangement_type equal to 3 with quincunx_sampling_flag equal to 1)	254

Figure D-11 – Rearrangement of a temporal interleaving frame arrangement  (frame_packing_arrangement_type equal to 5)	255

Figure D-12 – Rearrangement and upconversion of rectangular region frame packing arrangement (frame_packing_arrangement_type equal to 7 with quincunx_sampling_flag equal to 0)	255

Figure D-13 – Nominal vertical and horizontal sampling locations of 4:2:0 samples in top and bottom fields	259

Figure D-14 – Nominal vertical and horizontal sampling locations of 4:2:2 samples in top and bottom fields	259

Figure D-15 – Nominal vertical and horizontal sampling locations of 4:4:4 samples in top and bottom fields	260

Figure E-1 – Location of chroma samples for top and bottom fields for chroma_format_idc equal to 1 (4:2:0 chroma format) as a function of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field	276

- List of tables

Table 5-1 – Operation precedence from highest (at top of table) to lowest (at bottom of table)	15

Table 6-1 – SubWidthC, and SubHeightC values derived from  chroma_format_idc and separate_colour_plane_flag	18

Table 7-1 – NAL unit type codes and NAL unit type classes	55

Table 7-2 – Interpretation of pic_type	71

Table 7-3 – Specification of sizeId	75

Table 7-4 – Specification of matrixId according to sizeId, prediction mode and colour component	75

Table 7-5 – Specification of default values of ScalingList[ 0 ][ matrixId ][ i ] with i = 0..15	75

Table 7-6 – Specification of default values of ScalingList[ 1..3 ][ matrixId ][ i ] with i = 0..63	76

Table 7-7 – Name association to slice_type	78

Table 7-8 – Specification of the SAO type	86

Table 7-9 – Specification of the SAO edge offset class	87

Table 7-10 – Name association to prediction mode and partitioning type	89

Table 7-11 – Name association to inter prediction mode	89

Table 8-1 – Specification of intra prediction mode and associated names	104

Table 8-2 – Specification of IntraPredModeC	106

Table 8-3 – Specification of intraHorVerDistThres[ nT ] for various transform block sizes	109

Table 8-4 – Specification of intraPredAngle	111

Table 8-5 – Specification of invAngle	111

Table 8-6 – Specification of l0CandIdx and l1CandIdx	123

Table 8-7 – Assignment of the luma prediction sample predSampleLXL[ xL, yL ]	134

Table 8-8 – Assignment of the chroma prediction sample predSampleLXC[ xC, yC ] for ( X, Y ) being replaced by ( 1, b ), ( 2, c ), ( 3, d ), ( 4, e ), ( 5, f ), ( 6, g ), and ( 7, h ), respectively	136

Table 8-9 – Specification of QPC as a function of qPi	143

Table 8-10 – Derivation of threshold variables β′ and tC′ from input Q	156

Table 8-11 – Specification of hPos and vPos according to the sample adaptive offset class	162

Table 9-1 – Bit strings with "prefix" and "suffix" bits and assignment to codeNum ranges (informative)	163

Table 9-2 – Exp-Golomb bit strings and codeNum in explicit form and used as ue(v) (informative)	163

Table 9-3 – Assignment of syntax element to codeNum for signed Exp-Golomb coded syntax elements se(v)	164

Table 9-4 – Association of ctxIdx and syntax elements for each initializationType in the initialization process	170

Table 9-5 – Values of variable initValue for sao_merge_left_flag and sao_merge_up_flagctxIdx	171

Table 9-6 – Values of variable initValue for sao_type_idx_luma and sao_type_idx_chroma ctxIdx	171

Table 9-7 – Values of variable initValue for split_cu_flag ctxIdx	171

Table 9-8 – Values of variable initValue for cu_transquant_bypass_flag ctxIdx	171

Table 9-9 – Values of variable initValue for skip_flag ctxIdx	171

Table 9-10 – Values of variable initValue for cu_qp_delta_abs ctxIdx	171

Table 9-11 – Values of variable initValue for pred_mode_flag	172

Table 9-12 – Values of variable initValue for part_mode	172

Table 9-13 – Values of variable initValue for prev_intra_luma_pred_flag ctxIdx	172

Table 9-14 – Values of variable initValue for intra_chroma_pred_mode ctxIdx	172

Table 9-15 – Value of variable initValue for merge_flag ctxIdx	172

Table 9-16 – Values of variable initValue for merge_idx ctxIdx	172

Table 9-17 – Values of variable initValue for inter_pred_idc ctxIdx	173

Table 9-18 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx	173

Table 9-19 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx	173

Table 9-20 – Values of variable initValue for mvp_l0_flag, mvp_l1_flag ctxIdx	173

Table 9-21 – Values of variable initValue for no_residual_syntax_flag ctxIdx	173

Table 9-22 – Values of variable initValue for split_transform_flag ctxIdx	173

Table 9-23 – Values of variable initValue for cbf_luma ctxIdx	174

Table 9-24 – Values of variable initValue for cbf_cb and cbf_cr ctxIdx	174

Table 9-25 – Values of variable initValue for transform_skip_flag ctxIdx	174

Table 9-26 – Values of variable initValue for last_significant_coeff_x_prefix ctxIdx	174

Table 9-27 – Values of variable initValue for last_significant_coeff_y_prefix ctxIdx	174

Table 9-28 – Values of variable initValue for coded_sub_block_flag ctxIdx	174

Table 9-29 – Values of variable initValue for significant_coeff_flag ctxIdx	175

Table 9-30 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx	175

Table 9-31 – Values of variable initValue for coeff_abs_level_greater2_flag ctxIdx	175

Table 9-32 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset	178

Table 9-33 – Bin string of the unary binarization (informative)	181

Table 9-34 – Binarization for part_mode	183

Table 9-35 – Specification of prefix and suffix part for intra_chroma_pred_mode binarization	184

Table 9-36 – Binarization for inter_pred_idc	184

Table 9-37 – Assignment of ctxIdxInc to syntax elements with context coded bins	186

Table 9-38 – Specification of ctxIdxInc using left and above syntax elements	187

Table 9-39 – Specification of ctxIdxMap[ i ]	189

Table 9-40 – Specification of rangeTabLPS depending on pStateIdx and qCodIRangeIdx	194

Table 9-41 – State transition table	195

Table A-1 – General tier and level limits	209

Table A-2 – Tier and level limits for the Main and Main 10 profiles	211

Table A-3 – Maximum picture rates (pictures per second) at level 1 to 4.3 for some example picture sizes when MinCbSizeY is equal to 64	212

Table A-4 – Maximum picture rates (pictures per second) at level 5 to 6.2 for some example picture sizes when MinCbSizeY is equal to 64	213

Table D-1 – Persistence scope of prefix SEI messages (informative)	237

Table D-2 – Persistence scope of suffix SEI messages (informative)	237

Table D-3 – Mapping of camera_iso_sensitivity_idc and exposure_index_idc to ISO numbers	243

Table D-4 – Indicated interpretation of field_pic_flag and progressive_source_flag	258

Table D-5 – Interpretation of hash_type	261

Table E-1 – Interpretation of sample aspect ratio indicator	268

Table E-2 – Meaning of video_format	269

Table E-3 – Colour primaries	270

Table E-4 – Transfer characteristics	271

Table E-5 – Matrix coefficients	275

Foreword

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardising telecommunications on a world-wide basis. The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups that, in turn, produce Recommendations on these topics. The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1. In some areas of information technology that fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for world-wide standardization. National Bodies that are members of ISO and IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work. In the field of information technology, ISO and IEC have established a joint technical committee, ISO/IEC JTC 1. Draft International Standards adopted by the joint technical committee are circulated to national bodies for voting. Publication as an International Standard requires approval by at least 75% of the national bodies casting a vote.

This Recommendation | International Standard was prepared jointly by ITU-T SG 16 Q.6, also known as VCEG (Video Coding Experts Group), and by ISO/IEC JTC 1/SC 29/WG 11, also known as MPEG (Moving Picture Experts Group). VCEG was formed in 1997 to maintain prior ITU-T video coding standards and develop new video coding standard(s) appropriate for a wide range of conversational and non-conversational services. MPEG was formed in 1988 to establish standards for coding of moving pictures and associated audio for various applications such as digital storage media, distribution, and communication.

In this Recommendation | International Standard Annexes Error: Reference source not found through Error: Reference source not found contain normative requirements and are an integral part of this Recommendation | International Standard.

INTERNATIONAL  STANDARD

ITU-T  RECOMMENDATION

High efficiency video coding

# Introduction

This clause does not form an integral part of this Recommendation | International Standard.

## Prologue

This subclause does not form an integral part of this Recommendation | International Standard.

As the costs for both processing power and memory have reduced, network support for coded video data has diversified, and advances in video coding technology have progressed, the need has arisen for an industry standard for compressed video representation with substantially increased coding efficiency and enhanced robustness to network environments. Toward these ends the ITU-T Video Coding Experts Group (VCEG) and the ISO/IEC Moving Picture Experts Group (MPEG) formed a Joint Collaborative Team on Video Coding (JCT-VC) in 2010 for development of a new Recommendation | International Standard. This Recommendation | International Standard was developed in the JCT-VC.

## Purpose

This subclause does not form an integral part of this Recommendation | International Standard.

This Recommendation | International Standard was developed in response to the growing need for higher compression of moving pictures for various applications such as videoconferencing, digital storage media, television broadcasting, internet streaming, and communication. It is also designed to enable the use of the coded video representation in a flexible manner for a wide variety of network environments as well as to enable the use of multi-core parallel encoding and decoding devices. The use of this Recommendation | International Standard allows motion video to be manipulated as a form of computer data and to be stored on various storage media, transmitted and received over existing and future networks and distributed on existing and future broadcasting channels.

## Applications

This subclause does not form an integral part of this Recommendation | International Standard.

This Recommendation | International Standard is designed to cover a broad range of applications for video content including but not limited to the following:

CATV	Cable TV on optical networks, copper, etc.

DBS	Direct broadcast satellite video services

DSL 	Digital subscriber line video services

DTTB	Digital terrestrial television broadcasting

ISM	Interactive storage media (optical disks, etc.)

MMM	Multimedia mailing

MSPN	Multimedia services over packet networks (video streaming, etc.)

RTC	Real-time conversational services (videoconferencing, videophone, etc.)

RVS	Remote video surveillance

SSM	Serial storage media (digital VTR, etc.)

[Ed. (TK): Should we update this list of application areas? Perhaps with the list from the Requirements document? (GJS): Probably a good idea. Which Requirements document?]

## Publication and versions of this Specification

This subclause does not form an integral part of this Recommendation | International Standard.

This Specification has been jointly developed by ITU-T Video Coding Experts Group (VCEG) and the ISO/IEC Moving Picture Experts Group (MPEG). It is published as technically-aligned twin text in both organizations ITU-T and ISO/IEC. As the basis text has been drafted to become both an ITU-T Recommendation and an ISO/IEC International Standard, the term "Specification" (with capitalization to indicate that it refers to the whole of the text) is used herein when the text refers to itself.

## Profiles, tiers and levels

This subclause does not form an integral part of this Recommendation | International Standard.

This Recommendation | International Standard is designed to be generic in the sense that it serves a wide range of applications, bit rates, resolutions, qualities, and services. Applications should cover, among other things, digital storage media, television broadcasting and real-time communications. In the course of creating this Specification, various requirements from typical applications have been considered, necessary algorithmic elements have been developed, and these have been integrated into a single syntax. Hence, this Specification will facilitate video data interchange among different applications.

Considering the practicality of implementing the full syntax of this Specification, however, a limited number of subsets of the syntax are also stipulated by means of "profiles", "tiers", and "levels". These and other related terms are formally defined in clause 3.

A "profile" is a subset of the entire bitstream syntax that is specified by this Recommendation | International Standard. Within the bounds imposed by the syntax of a given profile it is still possible to require a very large variation in the performance of encoders and decoders depending upon the values taken by syntax elements in the bitstream such as the specified size of the decoded pictures. In many applications, it is currently neither practical nor economic to implement a decoder capable of dealing with all hypothetical uses of the syntax within a particular profile.

In order to deal with this problem, "tiers" and "levels" are specified within each profile. A level of a tier is a specified set of constraints imposed on values of the syntax elements in the bitstream. These constraints may be simple limits on values. Alternatively they may take the form of constraints on arithmetic combinations of values (e.g. picture width multiplied by picture height multiplied by number of pictures decoded per second). A level specified for a lower tier is more constrained than a level specified for a higher tier.

Coded video content conforming to this Recommendation | International Standard uses a common syntax. In order to achieve a subset of the complete syntax, flags, parameters, and other syntax elements are included in the bitstream that signal the presence or absence of syntactic elements that occur later in the bitstream.

## Overview of the design characteristics

This subclause does not form an integral part of this Recommendation | International Standard.

The coded representation specified in the syntax is designed to enable a high compression capability for a desired image or video quality. The algorithm is typically not lossless, as the exact source sample values are typically not preserved through the encoding and decoding processes. A number of techniques may be used to achieve highly efficient compression. Encoding algorithms (not specified in this Recommendation | International Standard) may select between inter and intra coding for block-shaped regions of each picture. Inter coding uses motion vectors for block-based inter prediction to exploit temporal statistical dependencies between different pictures. Intra coding uses various spatial prediction modes to exploit spatial statistical dependencies in the source signal for a single picture. Motion vectors and intra prediction modes may be specified for a variety of block sizes in the picture. The prediction residual is then further compressed using a transform to remove spatial correlation inside the transform block before it is quantized, producing an irreversible process that typically discards less important visual information while forming a close approximation to the source samples. Finally, the motion vectors or intra prediction modes are combined with the quantized transform coefficient information and encoded using arithmetic coding.

## How to read this Specification

This subclause does not form an integral part of this Recommendation | International Standard.

It is suggested that the reader starts with clause 1 (Scope) and moves on to clause 3 (Definitions). Clause 6 should be read for the geometrical relationship of the source, input, and output of the decoder. Clause 7 (Syntax and semantics) specifies the order to parse syntax elements from the bitstream. See subclauses 7.1–7.3 for syntactical order and see subclause 7.4 for semantics; i.e. the scope, restrictions, and conditions that are imposed on the syntax elements. The actual parsing for most syntax elements is specified in clause Error: Reference source not found (Error: Reference source not found). Finally, clause 8 (Decoding process) specifies how the syntax elements are mapped into decoded samples. Throughout reading this Specification, the reader should refer to clauses 2 (Normative references), 4 (Abbreviations), and 5 (Conventions) as needed. Annexes Error: Reference source not found through Error: Reference source not found also form an integral part of this Recommendation | International Standard.

Annex Error: Reference source not found specifies profiles each being tailored to certain application domains, and defines the so-called tiers and levels of the profiles. Annex Error: Reference source not found specifies syntax and semantics of a byte stream format for delivery of coded video as an ordered stream of bytes. Annex Error: Reference source not found specifies the hypothetical reference decoder, bitstream conformance, decoder conformance, and the use of the hypothetical reference decoder to check bitstream and decoder conformance. Annex Error: Reference source not found specifies syntax and semantics for supplemental enhancement information message payloads. Annex Error: Reference source not found specifies syntax and semantics of the video usability information parameters of the sequence parameter set.

Throughout this Specification, statements appearing with the preamble "NOTE –" are informative and are not an integral part of this Recommendation | International Standard.

# Scope

This document specifies High efficiency video coding.

# Normative references

## General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None.

## Paired Recommendations | International Standards equivalent in technical content

–	Rec. ITU-T H.264 (in force), Advanced video coding for generic audiovisual services.

ISO/IEC 23008-02: in force, Information technology – High efficiency coding and media delivery in heterogeneous environments – Part 2: High Efficiency Video Coding.

[Ed. (TK): This should be changed to the appropriate ITU-T & ISO/IEC standard for HEVC]

## Additional references

–	ISO 11664-1, Colorimetry  — Part 1: CIE standard colorimetric observers.

# Definitions

[Ed. (TW) Needs more work including turning them into 1 sentence each.]

For the purposes of this Recommendation | International Standard, the following definitions apply:

  - access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain exactly one coded base picture.

NOTE 1 – In addition to containing the coded slice segment NAL units of the coded base picture, an access unit may also contain other NAL units not containing slice segments of the coded base picture. The decoding of an access unit always results in a decoded picture.

  - AC transform coefficient: Any transform coefficient for which the frequency index in one or both dimensions is non-zero.

  - associated non-VCL NAL unit: An assoicated non-VCL NAL unit of a VCL NAL unit is one of the non-VCL NAL units for which the VCL NAL unit is their associated VCL NAL unit.

  - associated VCL NAL unit: The most recent preceding VCL NAL unit in decoding order for non-VCL NAL units with nal_unit_type equal to EOS_NUT, EOB_NUT, FD_NUT, or SUFFIX_SEI_NUT, or in the range of RSV_NVCL45..RSV_NVCL47, or in the range of UNSPEC48..UNSPEC63, or the first subsequent VCL NAL unit in decoding order for non-VCL NAL units with nal_unit_type equal to other values.

  - associated RAP picture: The previous RAP picture in decoding order within the same layer (if present).

  - bin: One bit of a bin string.

  - binarization: A set of bin strings for all possible values of a syntax element.

  - binarization process: A unique mapping process of all possible values of a syntax element onto a set of bin strings.

  - bin string: An intermediate binary representation of values of syntax elements from the binarization of the syntax element.

  - bi-predictive (B) slice: A slice that may be decoded using intra prediction or inter prediction using at most two motion vectors and reference indices to predict the sample values of each block.

  - bitstream: A sequence of bits that forms the representation of coded pictures and associated data forming one or more coded video sequences. Bitstream is a collective term used to refer either to a NAL unit stream or a byte stream. [Ed. Note (GJS): Multi-sentence.]

  - block: An MxN (M-column by N-row) array of samples, or an MxN array of transform coefficients.

  - broken link: A location in a bitstream at which it is indicated that some subsequent pictures in decoding order may contain serious visual artefacts due to unspecified operations performed in the generation of the bitstream.

  - broken link access (BLA) access unit: An access unit in which the coded base picture is a BLA picture.

  - broken link access (BLA) picture: A RAP picture for which each slice segment has nal_unit_type equal to BLA_W_LP, BLA_W_DLP or BLA_N_LP.

NOTE 2 – A BLA picture contains only I slices, and may be the first picture in the bitstream in decoding order, or may appear later in the bitstream. Each BLA picture begins a new coded video sequence, and has the same effect on the decoding process as an IDR picture. However, a BLA picture contains syntax elements that, if it had been CRA picture instead, would specify a non-empty reference picture set. When a BLA picture is encountered in a bitstream, these syntax elements are ignored and the reference picture set is instead initialized as an empty set. When a BLA picture has nal_unit_type equal to BLA_W_LP, it may have associated RASL pictures, which are not output by the decoder and may not be decodable, as they may contain references to reference pictures that are not present in the bitstream. When a BLA picture has nal_unit_type equal to BLA_W_LP, it may also have associated RADL pictures, which are specified to be decoded. When a BLA picture has nal_unit_type equal to BLA_W_DLP, it does not have associated RASL pictures but may have associated RADL pictures, which are specified to be decoded. When a BLA picture has nal_unit_type equal to BLA_N_LP, it does not have any associated leading pictures.

  - byte: A sequence of 8 bits, written and read with the most significant bit on the left and the least significant bit on the right. When represented in a sequence of data bits, the most significant bit of a byte is first.

  - byte-aligned: A position in a bitstream is byte-aligned when the position is an integer multiple of 8 bits from the position of the first bit in the bitstream. A bit or byte or syntax element is said to be byte-aligned when the position at which it appears in a bitstream is byte-aligned.

  - byte stream: An encapsulation of a NAL unit stream containing start code prefixes and NAL units as specified in Annex Error: Reference source not found.

  - can: A term used to refer to behaviour that is allowed, but not necessarily required.

  - chroma: An adjective specifying that a sample array or single sample is representing one of the two colour difference signals related to the primary colours. The symbols used for a chroma array or sample are Cb and Cr.

NOTE 3 – The term chroma is used rather than the term chrominance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term chrominance.

  - clean random access (CRA) access unit: An access unit in which the coded base picture is a CRA picture.

  - clean random access (CRA) picture: A RAP picture for which each slice segment has nal_unit_type equal to CRA_NUT.

NOTE 4 – A CRA picture contains only I slices, and may be the first picture in the bitstream in decoding order, or may appear later in the bitstream. A CRA picture may have associated RADL and RASL pictures. When a CRA picture is the first picture in the bitstream in decoding order, the CRA picture is the first picture of a coded video sequence in decoding order, and any associated RASL pictures are not output by the decoder and may not be decodable, as they may contain references to reference pictures that are not present in the bitstream.

  - coded base picture: A coded picture comprising VCL NAL units with reserved_zero_6bits equal to 0.

  - coded picture: A coded representation of a picture comprising VCL NAL units with a particular value of reserved_zero_6bits and containing all coding tree units of the picture.

  - coded picture buffer (CPB): A first-in first-out buffer containing  decoding units in decoding order specified in the hypothetical reference decoder in Annex Error: Reference source not found.

  - coded representation: A data element as represented in its coded form.

  - coded slice segment NAL unit: A NAL unit that has nal_unit_type in the range of 1 to 14, inclusive, which indicates that the NAL unit contains a coded slice segment.

  - coded video sequence: A sequence of access units that consists, in decoding order, of a CRA access unit that is the first access unit in the bitstream, an IDR access unit or a BLA access unit, followed by zero or more non-IDR and non-BLA access units including all subsequent access units up to but not including any subsequent IDR or BLA access unit.

  - coding block: An NxN block of samples for some value of N. The division of a coding tree block into coding blocks is a partitioning. [Ed. (GJS): Multi-sentence.]

  - coding tree block: An NxN block of samples for some value of N. The division of one of the arrays that compose a picture that has three sample arrays or of the array that compose a picture in monochrome format or a picture that is coded using three separate colour planes into coding tree blocks is a partitioning. [Ed. (GJS): Multi-sentence.]

  - coding tree unit: A coding tree block of luma samples, two corresponding coding tree blocks of chroma samples of a picture that has three sample arrays, or a coding tree block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to code the samples. The division of a slice into coding tree units is a partitioning.

  - coding unit: A coding block of luma samples, two corresponding coding blocks of chroma samples of a picture that has three sample arrays, or a coding block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to code the samples. The division of a coding tree unit into coding units is a partitioning.

  - column: An integer number of coding tree blocks. Columns are delineated from one another by vertical boundaries that extend from the top boundary to the bottom boundary of the picture and are ordered consecutively from left to right in the picture. The division of each picture into columns is a partitioning.

  - component: An array or single sample from one of the three arrays (luma and two chroma) that compose a picture in 4:2:0, 4:2:2, or 4:4:4 colour format or the array or a single sample of the array that compose a picture in monochrome format.

  - context variable: A variable specified for the adaptive binary arithmetic decoding process of a bin by an equation containing recently decoded bins.

  - decoded picture: A decoded picture is derived by decoding a coded picture.

  - decoded picture buffer (DPB): A buffer holding decoded pictures for reference, output reordering, or output delay specified for the hypothetical reference decoder in Annex Error: Reference source not found.

  - decoder: An embodiment of a decoding process.

  - decoder under test (DUT): A decoder that is tested for conformance to this Specification by operating the hypothetical stream scheduler to deliver a conforming bitstream to the decoder and to the hypothetical reference decoder and comparing the values and timing (or order) of the output of the two decoders.

  - decoding order: The order in which syntax elements are processed by the decoding process.

  - decoding process: The process specified in this Specification that reads a bitstream and derives decoded pictures from it.

  - decoding unit: An access unit if SubPicCpbFlag is equal to 0 or a subset of an access unit otherwise, consisting of one or more VCL NAL units in an access unit and the associated non-VCL NAL units.

  - dependent slice segment: A slice segment for which the values of some syntax elements of the slice segment header are inferred from the values for the preceding independent slice segment in decoding order.

  - display process: A process not specified in this Specification having, as its input, the cropped decoded pictures that are the output of the decoding process.

  - elementary stream: A sequence of one or more bitstreams.

NOTE 5 – An elementary stream that consists of two or more bitstreams would typically have been formed by splicing together two or more bitstreams (or parts thereof).

  - emulation prevention byte: A byte equal to 0x03 that may be present within a NAL unit. The presence of emulation prevention bytes ensures that no sequence of consecutive byte-aligned bytes in the NAL unit contains a start code prefix.

  - encoder: An embodiment of an encoding process.

  - encoding process: A process, not specified in this Specification, that produces a bitstream conforming to this Specification.

  - filler data NAL units: NAL units with nal_unit_type equal to FD_NUT.

  - flag: A variable that can take one of the two possible values 0 and 1.

  - frequency index: A one-dimensional or two-dimensional index associated with a transform coefficient prior to an inverse transform part of the decoding process.

  - hypothetical reference decoder (HRD): A hypothetical decoder model that specifies constraints on the variability of conforming NAL unit streams or conforming byte streams that an encoding process may produce.

  - hypothetical stream scheduler (HSS): A hypothetical delivery mechanism for the timing and data flow of the input of a bitstream into the hypothetical reference decoder. The HSS is used for checking the conformance of a bitstream or a decoder.

  - independent slice segment: A slice segment for which the values of the syntax elements of the slice segment header are not inferred from the values for a preceding slice segment.

  - informative: A term used to refer to content provided in this Specification that is not an integral part of this Specification. Informative content does not establish any mandatory requirements for conformance to this Specification.

  - instantaneous decoding refresh (IDR) access unit: An access unit in which each coded picture is an IDR picture.

  - instantaneous decoding refresh (IDR) picture: A RAP picture for which each slice segment has nal_unit_type equal to IDR_W_DLP or IDR_N_LP.

NOTE 6 – An IDR picture contains only I slices, and may be the first picture in the bitstream in decoding order, or may appear later in the bitstream. Each IDR picture is the first picture of a coded video sequence in decoding order. When an IDR picture has nal_unit_type equal to IDR_W_DLP, it may have associated RADL pictures. When an IDR picture has nal_unit_type equal to IDR_N_LP, it does not have any associated leading pictures.

  - inter coding: Coding of a coding block, slice, or picture that uses inter prediction.

  - inter prediction: A prediction derived in a manner that is dependent on data elements (e.g. sample values or motion vectors) of reference pictures other than the current decoded picture.

  - intra coding: Coding of a coding block, slice, or picture that uses intra prediction.

  - intra prediction: A prediction derived from only data elements (e.g. sample values) of the same decoded slice.

  - intra (I) slice: A slice that is decoded using intra prediction only.

  - inverse transform: A part of the decoding process by which a set of transform coefficients are converted into spatial-domain values.

  - layer: A set of VCL NAL units that all have a particular value of nuh_reserved_zero_6bits and the associated non-VCL NAL units, or one of a set of syntactical structures in a non-branching hierarchical relationship.

NOTE 7 – Depending on the context, either the first layer concept or the second layer concept applies, The first layer concept is also referred to as scalable layer, wherein a layer may be a spatial scalable layer, a quality scalable layer, a view, etc. A temporal true subset of a scalable layer, or a sub-layer, is not a layer. The second layer concept is also referred to as coding layer, wherein higher layers contain lower layers, and the coding layers are the coded video sequence, picture, slice, slice segment, and coding tree unit layers.

  - leading picture: A picture that is in the same layer as the assocated RAP picture and precedes the associated RAP picture in output order.

  - leaf: A terminating node of a tree that is a root node of a tree of depth 0.

  - level: A defined set of constraints on the values that may be taken by the syntax elements and variables of this Specification. The same set of levels is defined for all profiles, with most aspects of the definition of each level being in common across different profiles. Individual implementations may, within specified constraints, support a different level for each supported profile. In a different context, level is the value of a transform coefficient prior to scaling.

  - list 0 (list 1) motion vector: A motion vector associated with a reference index pointing into reference picture list 0 (list 1).

  - list 0 (list 1) prediction: Inter prediction of the content of a slice using a reference index pointing into reference picture list 0 (list 1).

  - long-term reference picture: A picture that is marked as "used for long-term reference".

  - luma: An adjective specifying that a sample array or single sample is representing the monochrome signal related to the primary colours. The symbol or subscript used for luma is Y or L.

NOTE 8 – The term luma is used rather than the term luminance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term luminance. The symbol L is sometimes used instead of the symbol Y to avoid confusion with the symbol y as used for vertical location.

  - may: A term used to refer to behaviour that is allowed, but not necessarily required. In some places where the optional nature of the described behaviour is intended to be emphasized, the phrase "may or may not" is used to provide emphasis.

  - motion vector: A two-dimensional vector used for inter prediction that provides an offset from the coordinates in the decoded picture to the coordinates in a reference picture.

  - must: A term used in expressing an observation about a requirement or an implication of a requirement that is specified elsewhere in this Specification. This term is used exclusively in an informative context.

  - network abstraction layer (NAL) unit: A syntax structure containing an indication of the type of data to follow and bytes containing that data in the form of an RBSP interspersed as necessary with emulation prevention bytes.

  - network abstraction layer (NAL) unit stream: A sequence of NAL units.

  - non-reference picture: A picture that is marked as "unused for reference".

NOTE 9 – A non-reference picture contains samples that cannot be used for inter prediction in the decoding process of subsequent pictures in decoding order.

  - note: A term used to prefix informative remarks. This term is used exclusively in an informative context.

  - operation point: A bitstream created from another bitstream by operation of the sub-bitstream extraction process.

  - operation point set: A set of operation points that have the same set of nuh_reserved_zero_6bits values in the corresponding bitstreams.

  - output order: The order in which the decoded pictures are output from the decoded picture buffer in case the decoded pictures are to be output from the decoded picture buffer. The output order of a picture is specified by the picture order count value, regardless of whether the picture is to be output. [Ed. (YK): Check the discussion related to K0120 and see whether any editorial changes herein are desirable.]

  - parameter: A syntax element of a video parameter set, sequence parameter set or picture parameter set. Parameter is also used as part of the defined term quantization parameter.

  - partitioning: The division of a set into subsets such that each element of the set is in exactly one of the subsets.

  - picture: An array of luma samples in monochrome format or an array of luma samples and two corresponding arrays of chroma samples in 4:2:0, 4:2:2, and 4:4:4 colour format.

  - picture parameter set (PPS): A syntax structure containing syntax elements that apply to zero or more entire coded pictures as determined by the pic_parameter_set_id syntax element found in each slice segment header.

  - picture order count: A variable that is associated with each picture that indicates the position of the associated picture in output order relative to the output order positions of the other pictures in the same coded video sequence. [Ed. (YK): Check the discussion related to K0120 and see whether any editorial changes herein are desirable.]

  - prediction: An embodiment of the prediction process.

  - prediction block: A rectangular MxN block of samples on which the same prediction is applied. The division of a coding block into prediction blocks is a partitioning.

  - prediction process: The use of a predictor to provide an estimate of the data element (e.g. sample value or motion vector) currently being decoded.

  - prediction unit: A prediction block of luma samples, two corresponding prediction blocks of chroma samples of a picture that has three sample arrays, or a prediction block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to predict the prediction block samples.

  - predictive (P) slice: A slice that may be decoded using intra prediction or inter prediction using at most one motion vector and reference index to predict the sample values of each block.

  - predictor: A combination of specified values or previously decoded data elements (e.g. sample value or motion vector) used in the decoding process of subsequent data elements.

  - prefix SEI NAL unit: An SEI NAL unit that has nal_unit_type equal to PREFIX_SEI_NUT.

  - profile: A specified subset of the syntax of this Specification.

  - quadtree: A tree in which a parent node can be split into four child nodes. A child node may become parent node for another split into four child nodes.

  - quantization parameter: A variable used by the decoding process for scaling of transform coefficient levels.

  - random access: The act of starting the decoding process for a bitstream at a point other than the beginning of the stream.

  - random access decodable leading (RADL) picture: A coded picture for which each slice segment has nal_unit_type equal to RADL_NUT.

NOTE 10 – All RADL pictures are leading pictures. RADL pictures are not used as reference pictures for the decoding process of trailing pictures of the same associated RAP picture. When present, all RADL pictures precede, in decoding order, all trailing pictures of the same associated RAP picture.

  - random access point (RAP) access unit: An access unit in which the coded picture is a RAP picture.

  - random access point (RAP) picture: A coded picture for which each slice segment has nal_unit_type in the range of 7 to 12, inclusive.

NOTE 11 – A RAP picture contains only I slices, and may be a BLA picture, a CRA picture or an IDR picture. The first picture in the bitstream must be a RAP picture. Provided the necessary parameter sets are available when they are activated, the RAP picture and all subsequent non-RASL pictures in decoding order can be correctly decoded without performing the decoding process of any pictures that precede the RAP picture in decoding order. There may be pictures in a bitstream that contain only I slices that are not RAP pictures.

  - random access skipped leading (RASL) access unit: An access unit in which the coded picture is a RASL picture.

  - random access skipped leading (RASL) picture: A coded picture for which each slice segment has nal_unit_type equal to RASL_NUT.

NOTE 12 – All RASL pictures are leading pictures of an associated BLA or CRA picture. When the associated RAP picture is a BLA picture or is the first coded picture in the bitstream, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to reference pictures that are not present in the bitstream. RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures. When present, all RASL pictures precede, in decoding order, all trailing pictures of the same associated RAP picture.

  - raster scan: A mapping of a rectangular two-dimensional pattern to a one-dimensional pattern such that the first entries in the one-dimensional pattern are from the first top row of the two-dimensional pattern scanned from left to right, followed similarly by the second, third, etc., rows of the pattern (going down) each scanned from left to right.

  - raw byte sequence payload (RBSP): A syntax structure containing an integer number of bytes that is encapsulated in a NAL unit. An RBSP is either empty or has the form of a string of data bits containing syntax elements followed by an RBSP stop bit and followed by zero or more subsequent bits equal to 0. [Ed. (GJS): Multi-sentence definition.]

  - raw byte sequence payload (RBSP) stop bit: A bit equal to 1 present within a raw byte sequence payload (RBSP) after a string of data bits. The location of the end of the string of data bits within an RBSP can be identified by searching from the end of the RBSP for the RBSP stop bit, which is the last non-zero bit in the RBSP. [Ed. (GJS): Multi-sentence definition.]

  - recovery point: A point in the bitstream at which the recovery of an exact or an approximate representation of the decoded pictures represented by the bitstream is achieved after a random access or broken link.

  - reference index: An index into a reference picture list.

  - reference picture: A picture that is a short-term reference picture or a long-term reference picture.

NOTE 13 – A reference picture contains samples that may be used for inter prediction in the decoding process of subsequent pictures in decoding order.

  - reference picture list: A list of reference pictures that is used for inter prediction of a P or B slice. For the decoding process of a P slice, there is one reference picture list – reference picture list 0. For the decoding process of a B slice, there are two reference picture lists  – reference picture list 0 and reference picture list 1. [Ed. (GJS): Multi-sentence definition.]

  - reference picture list 0: A reference picture list used for inter prediction of a P or B slice. All inter prediction used for P slices uses reference picture list 0. Reference picture list 0 is one of two reference picture lists used for bi-prediction for a B slice, with the other being reference picture list 1. [Ed. (GJS): This definition refers to bi-prediction in italics, but there is no definition of that term.] [Ed. (GJS): It may be beneficial to add a definition of "uni-prediction" or "single-list prediction" to refer to inter prediction that is not bi-prediction.] [Ed. (GJS): The definition should avoid giving the impression that all inter prediction in a B slice is bi-prediction.] [Ed. (GJS): Multi-sentence definition.]

  - reference picture list 1: A reference picture list used for inter prediction of a B slice. Reference picture list 1 is one of two reference picture lists used for bi-prediction for a B slice, with the other being reference picture list 0. [Ed. (GJS): This definition refers to bi-prediction in italics, but there is no definition of that term.] [Ed. (GJS): It may be beneficial to add a definition of "uni-prediction" or "single-list prediction" to refer to inter prediction that is not bi-prediction.] [Ed. (GJS): The definition should avoid giving the impression that all inter prediction in a B slice is bi-prediction.] [Ed. (GJS): The definition should avoid giving the impression that reference picture list 1 is only used for bi-prediction.] [Ed. (GJS): Multi-sentence definition.]

  - reference picture set: A set of reference pictures associated with a picture, consisting of all reference pictures that are prior to the associated picture in decoding order, that may be used for inter prediction of the associated picture or any picture following the associated picture in decoding order.

  - reserved: The term reserved, when used in the clauses specifying some values of a particular syntax element, are for future use by ITU-T | ISO/IEC. These values shall not be used in bitstreams conforming to this Specification, but may be used in future extensions of this Specification by ITU-T | ISO/IEC. [Ed. (GJS): Multi-sentence definition.]

  - residual: The decoded difference between a prediction of a sample or data element and its decoded value.

  - row: An integer number of coding tree blocks. Rows are delineated from one another by horizontal boundaries that extend from the left boundary to the right boundary of the picture and are ordered consecutively from top to bottom in the picture. The division of each picture into rows is a partitioning. [Ed. (GJS): Multi-sentence.]

  - sample aspect ratio: Specifies, for assisting the display process, which is not specified in this Specification, the ratio between the intended horizontal distance between the columns and the intended vertical distance between the rows of the luma sample array in a picture. Sample aspect ratio is expressed as h:v, where h is horizontal width and v is vertical height (in arbitrary units of spatial distance). [Ed. (GJS): Multi-sentence.]

  - scaling: The process of multiplying transform coefficient levels by a factor, resulting in transform coefficients.

  - sequence parameter set (SPS): A syntax structure containing syntax elements that apply to zero or more entire coded video sequences as determined by the content of a seq_parameter_set_id syntax element found in the picture parameter set referred to by the pic_parameter_set_id syntax element found in each slice segment header.

  - shall: A term used to express mandatory requirements for conformance to this Specification. When used to express a mandatory constraint on the values of syntax elements or on the results obtained by operation of the specified decoding process, it is the responsibility of the encoder to ensure that the constraint is fulfilled. When used in reference to operations performed by the decoding process, any decoding process that produces identical results to the decoding process described herein conforms to the decoding process requirements of this Specification. [Ed. (GJS): Multi-sentence definition.]

  - short-term reference picture: A picture that is marked as "used for short-term reference".

  - should: A term used to refer to behaviour of an implementation that is encouraged to be followed under anticipated ordinary circumstances, but is not a mandatory requirement for conformance to this Specification.

  - slice: An integer number of coding tree units contained in one independent slice segment and all subsequent dependent slice segments (if any) that precede the next independent slice segment (if any) within the same access unit.

  - slice header: The slice segment header of the independent slice segment that is a current slice segment or is the independent slice segment that precedes a current dependent slice segment.

  - slice segment: An integer number of coding tree units ordered consecutively in the tile scan and contained in a single NAL unit; the division of each picture into slice segments is a partitioning.

  - slice segment header: A part of a coded slice segment containing the data elements pertaining to the first or all coding tree units represented in the slice segment.

  - source: Term used to describe the video material or some of its attributes before encoding.

  - start code prefix: A unique sequence of three bytes equal to 0x000001 embedded in the byte stream as a prefix to each NAL unit. The location of a start code prefix can be used by a decoder to identify the beginning of a new NAL unit and the end of a previous NAL unit. Emulation of start code prefixes is prevented within NAL units by the inclusion of emulation prevention bytes. [Ed. (GJS): Multi-sentence definition – just move the 2nd and 3rd sentences somewhere else.]

  - step-wise temporal sub-layer access (STSA) access unit: An access unit in which the coded picture is an STSA picture.

  - step-wise temporal sub-layer access (STSA) picture: A coded picture for which each slice segment has nal_unit_type equal to STSA_R or STSA_N.

NOTE 14 – [Ed. (GJS): Explain more intiutively in note].

  - string of data bits (SODB): A sequence of some number of bits representing syntax elements present within a raw byte sequence payload prior to the raw byte sequence payload stop bit. Within an SODB, the left-most bit is considered to be the first and most significant bit, and the right-most bit is considered to be the last and least significant bit.

  - sub-bitstream extraction process: A specified process by which a set of NAL units is selected from a bitstream based on the associated TemporalId variable and nuh_reserved_zero_6bits syntax element values.

  - sub-layer: A temporal scalable layer of a temporal scalable bitstream consisting of VCL NAL units with a particular value of the TemporalId variable, and the associated non-VCL NAL units.

  - sub-layer non-reference picture: A picture that contains samples that cannot be used for inter prediction in the decoding process of subsequent pictures of the same and lower sub-layers in decoding order.

  - sub-layer representation: A subset of the bitstream consisting of NAL units of a particular sub-layer and the lower sub-layers.

  - suffix SEI NAL unit: An SEI NAL unit that has nal_unit_type equal to SUFFIX_SEI_NUT.

  - supplemental enhancement information (SEI) NAL unit: A NAL unit that has nal_unit_type equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT.

  - syntax element: An element of data represented in the bitstream.

  - syntax structure: Zero or more syntax elements present together in the bitstream in a specified order.

  - temporal sub-layer access (TSA) access unit: An access unit in which the coded picture is a TSA picture.

  - temporal sub-layer access (TSA) picture: A coded picture for which each slice segment has nal_unit_type equal to TSA_R or TSA_N.

NOTE 15 – [Ed. (GJS): Explain more intiutively in note].

  - temporal sub-layer: A temporal scalable layer of a temporal scalable bitstream consisting of VCL NAL units with a particular value of TemporalId and the associated non-VCL NAL units. [Ed. (GJS): I don't like having a variable name in a definition.]

  - tile: An integer number of coding tree blocks co-occurring in one column and one row, ordered consecutively in coding tree block raster scan of the tile. The division of each picture into tiles is a partitioning. Tiles in a picture are ordered consecutively in tile raster scan of the picture.

  - tile scan: A specific sequential ordering of coding tree blocks partitioning a picture. The tile scan traverses the coding tree blocks in coding tree block raster scan within a tile and traverses tiles in tile raster scan within a picture. Although a slice contains coding tree blocks that are consecutive in coding tree block raster scan of a tile, these coding tree blocks are not necessarily consecutive in coding tree block raster scan of the picture.

  - trailing picture: A picture that is in the same layer as the assocated RAP picture and follows the associated RAP picture in output order.

  - transform block: A rectangular MxN block of samples on which the same transform is applied. The division of a coding block into transform blocks is a partitioning.

  - transform coefficient: A scalar quantity, considered to be in a frequency domain, that is associated with a particular one-dimensional or two-dimensional frequency index in an inverse transform part of the decoding process.

  - transform coefficient level: An integer quantity representing the value associated with a particular two-dimensional frequency index in the decoding process prior to scaling for computation of a transform coefficient value.

  - transform unit: A transform block of luma samples, two corresponding transform blocks of chroma samples of a picture that has three sample arrays, or a transform block of samples of a monochrome picture or a picture that is coded using three separate colour planes and syntax structures used to transform the transform block samples.

  - tree: A tree is a finite set of nodes with a unique root node. A terminating node is called a leaf.

  - universal unique identifier (UUID): An identifier that is unique with respect to the space of all universal unique identifiers.

  - unspecified: The term unspecified, when used in the clauses specifying some values of a particular syntax element, indicates that the values have no specified meaning in this Specification and will not have a specified meaning in the future as an integral part of this Specification.

  - video coding layer (VCL) NAL unit: A collective term for coded slice segment NAL units and the subset of NAL units that have reserved values of nal_unit_type that are classified as VCL NAL units in this Specification.

  - video parameter set (VPS): A syntax structure containing syntax elements that apply to zero or more entire coded video sequences as determined by the content of a video_parameter_set_id syntax element found in the sequence parameter set referred to by the seq_parameter_set_id syntax element, which is found in the picture parameter set referred to by the pic_parameter_set_id syntax element found in each slice segment header.

  - z-scan order: A specific sequential ordering of blocks partitioning a picture. When the blocks are of the same size as coding tree blocks, the z-scan order is identical to coding tree block raster scan of the picture. When the blocks are of a smaller size than coding tree blocks, i.e. coding tree blocks are further partitioned into smaller coding blocks, the z-scan order traverses from coding tree block to coding tree block in coding tree block raster scan of the picture, and inside each coding tree block, which may be divided into quadtrees hierachically to lower levels, the z-scan order traverses from quadree to quadree of a particular level in quadtree-of-the-particular-level raster scan of the quadtree of the immediately higher level. [Ed. (YK): Multi-sentence definition.]

# Abbreviations

For the purposes of this Recommendation | International Standard, the following abbreviations apply:

B	Bi-predictive

BLA	Broken Link Access

CABAC	Context-based Adaptive Binary Arithmetic Coding

CB	Coding Block

CBR	Constant Bit Rate

CRA	Clean Random Access

CPB	Coded Picture Buffer

CTB	Coding Tree Block

CTU	Coding Tree Unit

CU	Coding Unit

DPB	Decoded Picture Buffer

DUT	Decoder Under Test

FIFO	First-In, First-Out

GDR	Gradual Decoding Refresh

HRD	Hypothetical Reference Decoder

HSS	Hypothetical Stream Scheduler

I	Intra

IDR	Instantaneous Decoding Refresh

LSB	Least Significant Bit

MSB	Most Significant Bit

NAL	Network Abstraction Layer

P	Predictive

PB	Prediction Block

PPS	Picture Parameter Set

PU	Prediction Unit

RADL	Random Access Decodable Leading (Picture)

RAP	Random Access Point

RASL	Random Access Skipped Leading (Picture)

RBSP	Raw Byte Sequence Payload

SEI	Supplemental Enhancement Information

SODB	String Of Data Bits

SPS	Sequence Parameter Set

STSA	Step-wise Temporal Sub-layer Access

TB	Transform Block

TSA	Temporal Sub-layer Access

TU	Transform Unit

UUID	Universal Unique Identifier

VBR	Variable Bit Rate

VCL	Video Coding Layer

VPS	Video Parameter Set

VUI	Video Usability Information

# Conventions

NOTE – The mathematical operators used in this Specification are similar to those used in the C programming language. However, the results of integer division and arithmetic shift operations are defined more precisely, and additional operations are defined, such as exponentiation and real-valued division. Numbering and counting conventions generally begin from 0.

## Arithmetic operators

The following arithmetic operators are defined as follows:

	Addition

−	Subtraction (as a two-argument operator) or negation (as a unary prefix operator)

*	Multiplication, including matrix multiplication

x y	Exponentiation. Specifies x to the power of y. In other contexts, such notation is used for superscripting not intended for interpretation as exponentiation.

/	Integer division with truncation of the result toward zero. For example, 7/4 and −7/−4 are truncated to 1 and −7/4 and 7/−4 are truncated to −1.

	Used to denote division in mathematical equations where no truncation or rounding is intended.

xy	Used to denote division in mathematical equations where no truncation or rounding is intended.

i=xyf(i)	The summation of f( i ) with i taking all integer values from x up to and including y.

x % y	Modulus. Remainder of x divided by y, defined only for integers x and y with x >= 0 and y > 0.

## Logical operators

The following logical operators are defined as follows:

x  &&  y	Boolean logical "and" of x and y.

x  | |  y	Boolean logical "or" of x and y.

!	Boolean logical "not".

x ? y : z	If x is TRUE or not equal to 0, evaluates to the value of y; otherwise, evaluates to the value of z.

## Relational operators

The following relational operators are defined as follows:

	Greater than.

	Greater than or equal to.

	Less than.

	Less than or equal to.

 	Equal to.

!	Not equal to.

When a relational operator is applied to a syntax element or variable that has been assigned the value "na" (not applicable), the value "na" is treated as a distinct value for the syntax element or variable. The value "na" is considered not to be equal to any other value.

## Bit-wise operators

The following bit-wise operators are defined as follows:

&	Bit-wise "and". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

|	Bit-wise "or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

^	Bit-wise "exclusive or". When operating on integer arguments, operates on a two's complement representation of the integer value. When operating on a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding more significant bits equal to 0.

x >> y	Arithmetic right shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the MSBs as a result of the right shift have a value equal to the MSB of x prior to the shift operation.

x << y	Arithmetic left shift of a two's complement integer representation of x by y binary digits. This function is defined only for non-negative integer values of y. Bits shifted into the LSBs as a result of the left shift have a value equal to 0.

## Assignment operators

The following arithmetic operators are defined as follows:

	Assignment operator.

 	Increment, i.e. x  is equivalent to x  x  1; when used in an array index, evaluates to the value of the variable prior to the increment operation.

− −	Decrement, i.e. x−  − is equivalent to x  x − 1; when used in an array index, evaluates to the value of the variable prior to the decrement operation.

+=	Increment by amount specified, i.e. x += 3 is equivalent to x = x + 3, and x += (−3) is equivalent to x = x + (−3).

−=	Decrement by amount specified, i.e. x −= 3 is equivalent to x = x − 3, and x −= (−3) is equivalent to x = x − (−3).

## Range notation

The following notation is used to specify a range of values:

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers.

## Mathematical functions

The following mathematical functions are defined as follows:

Abs( x )  x; x>=0−x; x<0		(5-1)

Ceil( x ) 	the smallest integer greater than or equal to x.	(5-2)

Clip1Y( x ) = Clip3( 0, ( 1 << BitDepthY ) − 1, x )		(5-3)

Clip1C( x ) = Clip3( 0, ( 1 << BitDepthC ) − 1, x )		(5-4)

Clip3( x, y, z ) = x; z<xy; z>yz; otherwise		(5-5)

Floor( x )	the greatest integer less than or equal to x.		(5-6)

Log2( x ) returns the base-2 logarithm of x.		(5-7)

Log10( x ) returns the base-10 logarithm of x.		(5-8)

Min( x, y ) = x; x<=yy; x>y		(5-9)

Max( x, y ) = x; x>=yy; x<y		(5-10)

Round( x ) = Sign( x ) * Floor( Abs( x ) + 0.5 )		(5-11)

Sign( x ) = 1;x>00;x=0−1;x<0		(5-12)

Sqrt( x ) = x		(5-13)

Swap( x, y ) = ( y, x )		(5-14)

## Order of operation precedence

When order of precedence in an expression is not indicated explicitly by use of parentheses, the following rules apply:

–	operations of a higher precedence are evaluated before any operation of a lower precedence,

–	operations of the same precedence are evaluated sequentially from left to right.

Table 5-1 specifies the precedence of operations from highest to lowest; a higher position in the table indicates a higher precedence.

NOTE – For those operators that are also used in the C programming language, the order of precedence used in this Specification is the same as used in the C programming language.

Table 5-1 – Operation precedence from highest (at top of table) to lowest (at bottom of table)

operations (with operands x, y, and z)

"x++", "x− −"

"!x", "−x" (as a unary prefix operator)

xy

"x * y", "x / y", "x  y""xy", "x % y"

"x + y", "x − y" (as a two-argument operator), "i=xyf(i)"

"x << y", "x >> y"

"x < y", "x <= y", "x > y", "x >= y"

"x = = y", "x != y"

"x & y"

"x ¦ y"

"x && y"

"x ¦ ¦ y"

"x ? y : z"

"x..y"

"x = y", "x += y", "x −= y"

## Variables, syntax elements, and tables

Syntax elements in the bitstream are represented in bold type. Each syntax element is described by its name (all lower case letters with underscore characters), and one descriptor for its method of coded representation. The decoding process behaves according to the value of the syntax element and to the values of previously decoded syntax elements. When a value of a syntax element is used in the syntax tables or the text, it appears in regular (i.e. not bold) type.

In some cases the syntax tables may use the values of other variables derived from syntax elements values. Such variables appear in the syntax tables, or text, named by a mixture of lower case and upper case letter and without any underscore characters. Variables starting with an upper case letter are derived for the decoding of the current syntax structure and all depending syntax structures. Variables starting with an upper case letter may be used in the decoding process for later syntax structures without mentioning the originating syntax structure of the variable. Variables starting with a lower case letter are only used within the subclause in which they are derived.

In some cases, "mnemonic" names for syntax element values or variable values are used interchangeably with their numerical values. Sometimes "mnemonic" names are used without any associated numerical values. The association of values and names is specified in the text. The names are constructed from one or more groups of letters separated by an underscore character. Each group starts with an upper case letter and may contain more upper case letters.

NOTE – The syntax is described in a manner that closely follows the C-language syntactic constructs.

Functions that specify properties of the current position in the bitstream are referred to as syntax functions. These functions are specified in subclause 7.2 and assume the existence of a bitstream pointer with an indication of the position of the next bit to be read by the decoding process from the bitstream. Syntax functions are described by their names, which are constructed as syntax element names and end with left and right round parentheses including zero or more variable names (for definition) or values (for usage), separated by commas (if more than one variable).

Functions that are not syntax functions (including mathematical functions specified in subclause 5.7) are described by their names, which start with an upper case letter, contain a mixture of lower and upper case letters without any underscore character, and end with left and right parentheses including zero or more variable names (for definition) or values (for usage) separated by commas (if more than one variable).

A one-dimensional array is referred to as a list. A two-dimensional array is referred to as a matrix. Arrays can either be syntax elements or variables. Subscripts or square parentheses are used for the indexing of arrays. In reference to a visual depiction of a matrix, the first subscript is used as a row (vertical) index and the second subscript is used as a column (horizontal) index. The indexing order is reversed when using square parentheses rather than subscripts for indexing. Thus, an element of a matrix s at horizontal position x and vertical position y may be denoted either as s[ x, y ] or as syx. An array construction is denoted by { {...} {...} }, where each inner pair of brackets denote a row of an array in the increasing order of rows. Each inner bracket contains elements of the corresponsing row of an array in the increasing order of columns. [Ed. (GJS): Try to improve the phrasing of this paragraph.]

Binary notation is indicated by enclosing the string of bit values by single quote marks. For example, '01000001' represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Hexadecimal notation, indicated by prefixing the hexadecimal number by "0x", may be used instead of binary notation when the number of bits is an integer multiple of 4. For example, 0x41 represents an eight-bit string having only its second and its last bits (counted from the most to the least significant bit) equal to 1.

Numerical values not enclosed in single quotes and not prefixed by "0x" are decimal values.

A value equal to 0 represents a FALSE condition in a test statement. The value TRUE is represented by any value different from zero.

## Text description of logical operations

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0 )
  statement 0
else if( condition 1 )
  statement 1
…
else /* informative remark on remaining condition */
  statement n

may be described in the following manner:

... as follows / ... the following applies.

–	If condition 0, statement 0

–	Otherwise, if condition 1, statement 1

–	…

–	Otherwise (informative remark on remaining condition), statement n

Each "If ... Otherwise, if ... Otherwise, ..." statement in the text is introduced with "... as follows" or "... the following applies" immediately followed by "If ... ". The last condition of the "If ... Otherwise, if ... Otherwise, ..." is always an "Otherwise, ...". Interleaved "If ... Otherwise, if ... Otherwise, ..." statements can be identified by matching "... as follows" or "... the following applies" with the ending "Otherwise, ...".

In the text, a statement of logical operations as would be described mathematically in the following form

if( condition 0a  &&  condition 0b )
  statement 0
else if( condition 1a  | |  condition 1b )
  statement 1
…
else
  statement n

may be described in the following manner:

... as follows / ... the following applies.

–	If all of the following conditions are true, statement 0

–	condition 0a

–	condition 0b

–	Otherwise, if one or more of the following conditions are true, statement 1

–	condition 1a

–	condition 1b

–	…

–	Otherwise, statement n

In the text, a statement of logical operations as would be described mathematically in the following form:

if( condition 0 )
  statement 0
if( condition 1 )
  statement 1

may be described in the following manner:

When condition 0, statement 0

When condition 1, statement 1

## Processes

Processes are used to describe the decoding of syntax elements. A process has a separate specification and invoking. All syntax elements and upper case variables that pertain to the current syntax structure and depending syntax structures are available in the process specification and invoking. A process specification may also have a lower case variable explicitly specified as the input. Each process specification has explicitly specified an output. The output is a variable that can either be an upper case variable or a lower case variable.

When invoking a process, the assignment of variables is specified as follows.

–	If the variables at the invoking and the process specification do not have the same name, the variables are explicitly assigned to lower case input or output variables of the process specification.

–	Otherwise (the variables at the invoking and the process specification have the same name), assignment is implied.

In the specification of a process, a specific coding block may be referred to by the variable name having a value equal to the address of the specific coding block.

# Source, coded, decoded and output data formats, scanning processes, and neighbouring relationships

## Bitstream formats

This subclause specifies the relationship between the NAL unit stream and byte stream, either of which are referred to as the bitstream.

The bitstream can be in one of two formats: the NAL unit stream format or the byte stream format. The NAL unit stream format is conceptually the more "basic" type. It consists of a sequence of syntax structures called NAL units. This sequence is ordered in decoding order. There are constraints imposed on the decoding order (and contents) of the NAL units in the NAL unit stream.

The byte stream format can be constructed from the NAL unit stream format by ordering the NAL units in decoding order and prefixing each NAL unit with a start code prefix and zero or more zero-valued bytes to form a stream of bytes. The NAL unit stream format can be extracted from the byte stream format by searching for the location of the unique start code prefix pattern within this stream of bytes. Methods of framing the NAL units in a manner other than use of the byte stream format are outside the scope of this Recommendation | International Standard. The byte stream format is specified in Annex Error: Reference source not found.

## Source, decoded, and output picture formats

This subclause specifies the relationship between source and decoded pictures that is given via the bitstream.

The video source that is represented by the bitstream is a sequence of pictures in decoding order.

The source and decoded pictures are each comprised of one or more sample arrays:

–	Luma (Y) only (monochrome).

–	Luma and two chroma (YCbCr or YCgCo).

–	Green, Blue and Red (GBR, also known as RGB).

–	Arrays representing other unspecified monochrome or tri-stimulus colour samplings (for example, YZX, also known as XYZ).

For convenience of notation and terminology in this Specification, the variables and terms associated with these arrays are referred to as luma (or L or Y) and chroma, where the two chroma arrays are referred to as Cb and Cr; regardless of the actual colour representation method in use. The actual colour representation method in use can be indicated in syntax that is specified in Annex Error: Reference source not found.

The variables SubWidthC, and SubHeightC are specified in Table 6-2, depending on the chroma format sampling structure, which is specified through chroma_format_idc and separate_colour_plane_flag. An entry marked as "-" in Table 6-2 denotes an undefined value for SubWidthC or SubHeightC. Other values of chroma_format_idc, SubWidthC, and SubHeightC may be specified in the future by ITU-T | ISO/IEC.

Table 6-2 – SubWidthC, and SubHeightC values derived from 
chroma_format_idc and separate_colour_plane_flag

| chroma_format_idc | separate_colour_plane_flag | Chroma Format | SubWidthC | SubHeightC |
| --- | --- | --- | --- | --- |
| 0 | 0 | monochrome | - | - |
| 1 | 0 | 4:2:0 | 2 | 2 |
| 2 | 0 | 4:2:2 | 2 | 1 |
| 3 | 0 | 4:4:4 | 1 | 1 |
| 3 | 1 | 4:4:4 | - | - |

In monochrome sampling there is only one sample array, which is nominally considered the luma array.

In 4:2:0 sampling, each of the two chroma arrays has half the height and half the width of the luma array.

In 4:2:2 sampling, each of the two chroma arrays has the same height and half the width of the luma array.

In 4:4:4 sampling, depending on the value of separate_colour_plane_flag, the following applies.

–	If separate_colour_plane_flag is equal to 0, each of the two chroma arrays has the same height and width as the luma array.

–	Otherwise (separate_colour_plane_flag is equal to 1), the three colour planes are separately processed as monochrome sampled pictures.

The number of bits necessary for the representation of each of the samples in the luma and chroma arrays in a video sequence is in the range of 8 to 14, inclusive, and the number of bits used in the luma array may differ from the number of bits used in the chroma arrays.

When the value of chroma_format_idc is equal to 1, the nominal vertical and horizontal relative locations of luma and chroma samples in pictures are shown in Figure 6-1. Alternative chroma sample relative locations may be indicated in video usability information (see Annex Error: Reference source not found).

Figure 6-1 – Nominal vertical and horizontal locations of 4:2:0 luma and chroma samples in a picture

When the value of chroma_format_idc is equal to 2, the chroma samples are co-sited with the corresponding luma samples and the nominal locations in a picture are as shown in Figure 6-2.

Figure 6-2 – Nominal vertical and horizontal locations of 4:2:2 luma and chroma samples in a picture

When the value of chroma_format_idc is equal to 3, all array samples are co-sited for all cases of pictures and the nominal locations in a picture are as shown in Figure 6-3.

Figure 6-3 – Nominal vertical and horizontal locations of 4:4:4 luma and chroma samples in a picture

## Spatial subdivision of pictures, slices, slice segments, and tiles

This subclause specifies how a picture is partitioned into slices, slice segments, tiles and coding tree blocks. Pictures are divided into slices and tiles. A slice is a sequence of one or more slice segments starting with an independent slice segment and containing all subsequent dependent slice segments (if any) that precede the next independent slice segment (if any) within the same access unit. A slice segment is a sequence of coding tree units. [Ed. (GJS): Clarify w.r.t. coding tree blocks vs. coding tree units vs. luma coding tree blocks vs. Cb or Cr coding tree blocks.] Likewise, a tile is a sequence of coding tree units.

For example, a picture may be divided into two slices as shown in Figure 6-4. In this example, the first slice is composed of an independent slice segment containing 4 coding tree units, a dependent slice segment containing 32 coding tree units, and another dependent slice segment containing 24 coding tree units; and the second slice consists of a single indpendent slice segment containing the remaining 39 coding tree units of the picture.

As another example, a picture may be divided into two tiles separated by a horizontal tile boundary as shown in Figure 6-5. The left side of the figure illustrates a case in which the picture only contains one slice, starting with an independent slice segment and followed by four dependent slice segments. The right side of the figure illustrates an alternative case in which the picture contains two slices in the first tile and one slice in the second tile.

Unlike slices, tiles are always rectangular. A tile always contains an integer number of coding tree units, and may consist of coding tree units contained in more than one slice. Similarly, a slice may consist of coding tree units contained in more than one tile.

One or both of the following conditions shall be fulfilled for each slice and tile:

–	All coding tree units in a slice belong to the same tile.

–	All coding tree units in a tile belong to the same slice.

NOTE 16 – Within the same picture, there may be both slices that contain multiple tiles and tiles that contain multiple slices.

One or both of the following conditions shall be fulfilled for each slice segment and tile:

–	All coding tree units in a slice segment belong to the same tile.

–	All coding tree units in a tile belong to the same slice segment.

When a picture is coded using three separate colour planes (separate_colour_plane_flag is equal to 1), a slice contains only coding tree blocks of one colour component being identified by the corresponding value of colour_plane_id, and each colour component array of a picture consists of slices having the same colour_plane_id value. Coded slices with different values of colour_plane_id within an access unit can be interleaved with each other under the constraint that for each value of colour_plane_id, the coded slice segment NAL units with that value colour_plane_id shall be in the order of increasing coding tree block address in tile scan order for the first coding tree block of each coded slice segment NAL unit.

NOTE 17 – When separate_colour_plane_flag is equal to 0, each coding tree block of a picture is contained in exactly one slice. When separate_colour_plane_flag is equal to 1, each coding tree block of a colour component is contained in exactly one slice (i.e. information for each coding tree block of a picture is present in exactly three slices and these three slices have different values of colour_plane_id).

Figure 6-4 – A picture with 11 by 9 luma coding tree blocks that is partitioned into two slices, the first of which is partitioned into three slice segments (informative)

Figure 6-5 – A picture with 11 by 9 luma coding tree blocks that is partitioned into two tiles and one slice (left) or is partitioned into two tiles and three slices (right) (informative)

[Ed. (BB): Consider adding a new section for the following text describing the block and quadtree structures.]

The samples are processed in units of coding tree blocks. The array size for each luma coding tree block in both width and height is CtbSizeY in units of samples. The width and height of the array for each chroma coding tree block are CtbWidthC and CtbHeightC, respectively, in units of samples.

Each coding tree block is assigned a partition signalling to identify the block sizes for intra or inter prediction and for transform coding. The partitioning is a recursive quadtree partitioning. The root of the quadtree is associated with the coding tree block. The quadtree is split until a leaf is reached, which is referred to as the coding block. When the picture width is not an integer number of coding tree block sizes, the coding tree blocks at the right picture boundary are incomplete. When the picture height is not an integer number of coding tree block sizes, the coding tree blocks at the bottom picture boundary are incomplete.

The coding block is the root node of two trees, the prediction tree and the transform tree. The prediction tree specifies the position and size of prediction blocks. The transform tree specifies the position and size of transform blocks. The splitting information for luma and chroma is identical for the prediction tree and may or may not be identical for the transform tree.

The blocks and associated syntax structures are encapsulated in a "unit" as follows.

–	One prediction block (monochrome picture or separate_colour_plane_flag is equal to 1) or three prediction blocks (luma and chroma) and associated prediction syntax structures units are encapsulated in a prediction unit.

–	One transform block (monochrome picture or separate_colour_plane_flag is equal to 1) or three transform blocks (luma and chroma) and associated transform syntax structures units are encapsulated in a transform unit.

–	One coding block (monochrome picture or separate_colour_plane_flag is equal to 1) or three coding blocks (luma and chroma), the associated coding syntax structures and the associated prediction and transform units are encapsulated in a coding unit.

–	One coding tree block (monochrome picture or separate_colour_plane_flag is equal to 1) or three coding tree blocks (luma and chroma), the associated coding tree syntax structures and the associated coding units are encapsulated in a coding tree unit.

## Availability processes

### Derivation process for z-scan order block availability

Inputs to this process are:

- the luma location ( xCurr, yCurr ) of the top-left sample of the current block relative to the top-left luma sample of the current picture,

- the luma location ( xN, yN ) covered by a neighbouring block relative to the top-left luma sample of the current picture.

Output of this process is the availability of the neighbouring block covering the location ( xN, yN ), denoted as availableN.

The minimum luma block address in z-scan order minBlockAddrCurr of the current block is derived as follows.

minBlockAddrCurr = MinTbAddrZS[ xCurr >> Log2MinTrafoSize ][ yCurr >> Log2MinTrafoSize ]	(6-15)

The minimum luma block address in z-scan order minBlockAddrN of the neighbouring block covering the location ( xN,  yN ) is derived as follows.

–	If one or more of the following conditions are true, minBlockAddrN is set to −1.

–	xN is less than 0

–	yN is less than 0

–	xN is greater than pic_width_in_luma_samples

–	yN is greater than pic_height_in_luma_samples

–	Otherwise (xN and yN are inside picture boundaries),

minBlockAddrN = MinTbAddrZS[ xN >> Log2MinTrafoSize ][ yN >> Log2MinTrafoSize ]	(6-16)

The neighbouring block availability availableN is derived as follows.

–	If one or more of the following conditions are true, availableN is set to FALSE:

–	minBlockAddrN is less than 0,

–	minBlockAddrN is greater than minBlockAddrCurr,

–	the variable SliceAddrRS associated with the slice segment containing the neighbouring block with the minimum luma block address minBlockAddrN differs in value from the variable SliceAddrRS associated with the slice segment containing the current block with the minimum luma block address minBlockAddrCurr.

–	the neighbouring block with the minimum luma block address minBlockAddrN is contained in a different tile than the current block with the minimum luma block address minBlockAddrCurr.

–	Otherwise, availableN is set to TRUE.

### Derivation process for prediction block availability

Inputs to this process are:

- the luma location ( xC, yC ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable nCbS specifying the size of the current luma coding block,

- the luma location ( xP, yP ) of the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture,

- variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

- a variable partIdx specifying the partition index of the current prediction unit within the current coding unit.

- the luma location ( xN, yN ) covered by a neighbouring prediction block relative to the top-left luma sample of the current picture.

Output of this process is the availability of the neighbouring prediction block covering the location ( xN, yN ), denoted as availableN is derived as follows.

The variable sameCb specifying whether the current luma prediction block and the neighbouring luma prediction block cover the same luma coding block.

–	If all of the following conditions are true, sameCb is set equal to TRUE.

–	xC is less or equal than xN,

–	yC is less or equal than yN,

–	( xC + nCbS ) is greater than xN,

–	( yC + nCbS ) is greater than yN.

–	Otherwise, sameCb is set equal to FALSE.

The neighbouring prediction block availability availableN is derived as follows.

–	If sameCb is equal to FALSE, the derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to ( xP, yP ) and the luma location ( xN, yN ) as the input and the output is assigned to availableN.

–	Otherwise, if all of the following conditions are true, availableN is set equal to FALSE.

–	( nPbW << 1 ) is equal to nCbS,

–	( nPbH << 1 ) is equal to nCbS,

–	partIdx is equal to 1,

–	(yC + nPbH) is less or equal to yN,

–	(xC + nPbW) is greater than xN.

–	Otherwise, availableN is set equal to TRUE.

When availableN is equal to TRUE and PredMode[ xN ][ yN ] is equal to MODE_INTRA, availableN is set equal to FALSE.

## Scanning processes

### Coding tree block raster and tile scanning conversion process

The list colWidth[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of CTBs, is derived as follows.

if( uniform_spacing_flag )
	for( i = 0; i <= num_tile_columns_minus1; i++ )
		colWidth[ i ] = ( ( i + 1 ) * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 ) − 
						( i * PicWidthInCtbsY ) / ( num_tile_columns_minus1 + 1 )
else {
	colWidth[ num_tile_columns_minus1 ] = PicWidthInCtbsY
	for( i = 0; i < num_tile_columns_minus1; i++ ) {
		colWidth[ i ] = column_width_minus1[ i ] + 1
		colWidth[ num_tile_columns_minus1 ] −= colWidth[ i ]
	}
}

The list rowHeight[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of CTBs, is derived as follows.

if( uniform_spacing_flag )
	for( j = 0; j <= num_tile_rows_minus1; j++ )
		rowHeight[ j ]  = ( ( j + 1 ) * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1 ) − 
						( j * PicHeightInCtbsY ) / ( num_tile_rows_minus1 + 1) 
else {
	rowHeight[ num_tile_rows_minus1 ] = PicHeightInCtbsY
	for( j = 0; j < num_tile_rows_minus1; j++ )
		rowHeight[ j ] = row_height_minus1[ j ] + 1
		rowHeight[ num_tile_rows_minus1 ] −= rowHeight[ i ]
	}
}

The list colBd[ i ] for i ranging from 0 to num_tile_columns_minus1 + 1, inclusive, specifying the location of the i-th column boundary in units of coding tree blocks, is derived as follows.

for( colBd[ 0 ] = 0, i = 0; i <= num_tile_columns_minus1; i++ )
		colBd[ i + 1 ] = colBd[ i ] + colWidth[ i ]

The list rowBd[ j ] for j ranging from 0 to num_tile_rows_minus1 + 1, inclusive, specifying the location of the j-th row boundary in units of coding tree blocks, is derived as follows.

for( rowBd[ 0 ] = 0, j = 0; j <= num_tile_rows_minus1; j++ )
	rowBd[ j + 1 ] = rowBd[ j ] + rowHeight[ j ]

The list CtbAddrRStoTS[ ctbAddrRS ] for ctbAddrRS ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan, is derived as follows.

for( ctbAddrRS = 0; ctbAddrRS < PicSizeInCtbsY; ctbAddrRS++ ) {
		tbX = ctbAddrRS % PicWidthInCtbsY
		tbY = ctbAddrRS / PicWidthInCtbsY
		for( i = 0; i <= num_tile_columns_minus1; i++ )
			if( tbX >= colBd[ i ] )
				tileX = i
		for( j = 0; j <= num_tile_rows_minus1; j++ )
			if( tbY >= rowBd[ j ] )
				tileY = j
		CtbAddrRStoTS[ ctbAddrRS ] = 0
		for( i = 0; i < tileX; i++ )
			CtbAddrRStoTS[ ctbAddrRS ] += rowHeight[ tileY ] * colWidth[ i ]
		for( j = 0; j < tileY; j++ )
			CtbAddrRStoTS[ ctbAddrRS ] += PicWidthInCtbsY * rowHeight[ j ]
		CtbAddrRStoTS[ ctbAddrRS ] += ( tbY − rowBd[ tileY ] ) * colWidth[ tileX ] + tbX − colBd[ tileX ]
	}

The list CtbAddrTStoRS[ ctbAddrTS ] for ctbAddrTS ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture, is derived as follows.

for( ctbAddrRS = 0; ctbAddrRS < PicSizeInCtbsY; ctbAddrRS++ )
		CtbAddrTStoRS[ CtbAddrRStoTS[ ctbAddrRS ] ] = ctbAddrRS

The list TileId[ ctbAddrTS ] for ctbAddrTS ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID, is derived as follows.

for( j = 0, tIdx = 0; j <= num_tile_rows_minus1; j++ )
		for( i = 0; i <= num_tile_columns_minus1; i++, tIdx++ )
			for( y = rowBd[ j ]; y < rowBd[ j + 1 ]; y++ )
				for( x = colBd[ i ]; x < colBd[ i + 1 ]; x++ )
						TileId[ CtbAddrRStoTS[ y*PicWidthInCtbsY+ x ] ] = tIdx

The values of ColumnWidthInLumaSamples[ i ], specifying the width of the i-th tile column in units of luma samples, are set equal to colWidth[ i ] << Log2CtbSizeY for i ranging from 0 to num_tile_columns_minus1, inclusive.

The values of RowHeightInLumaSamples[ j ], specifying the height of the j-th tile row in units of luma samples, are set equal to rowHeight[ j ] << Log2CtbSizeY for j ranging from 0 to num_tile_rows_minus1, inclusive.

### Z-scan order array initialization process

The array MinTbAddrZS with elements MinTbAddrZS[ x ][ y ] for x ranging from 0 to ( PicWidthInCtbsY << ( Log2CtbSizeY − Log2MinTrafoSize ) ) − 1, inclusive, and y ranging from 0 to ( PicHeightInCtbsY << ( Log2CtbSizeY − Log2MinTrafoSize ) ) − 1, specifying the conversion from a location ( x, y ) in units of minimum blocks to a minimum block address in z-scan order, inclusive is derived as follows.

for( y = 0; y < ( PicHeightInCtbsY << ( Log2CtbSizeY − Log2MinTrafoSize ) ); y++ )
		for( x = 0; x < ( PicWidthInCtbsY << ( Log2CtbSizeY − Log2MinTrafoSize ) ); x++) {
			tbX = ( x << Log2MinTrafoSize ) >> Log2CtbSizeY
			tbY = ( y << Log2MinTrafoSize ) >> Log2CtbSizeY
			ctbAddrRS = PicWidthInCtbsY * tbY + tbX
			MinTbAddrZS[ x ][ y ] = CtbAddrRStoTS[ ctbAddrRS ] << (( Log2CtbSizeY − Log2MinTrafoSize ) * 2)
			for( i = 0, p = 0; i < ( Log2CtbSizeY − Log2MinTrafoSize ); i++ ) {
				m = 1 << i
				p += ( m & x ? m * m : 0 ) + ( m & y ? 2 * m * m : 0 )
			}
			MinTbAddrZS[ x ][ y ] += p
		}

### Up-right diagonal scan order array initialization process

Inputs to this process is a block size blkSize.

Output of this process is the array diagScan[ sPos ][ sComp ]. The array index sPos specify the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. Depending on the value of blkSize, the array diagScan is derived as follows.

i  =  0
x  =  0
y  =  0
stopLoop  =  FALSE
while( !stopLoop ) {
	while( y  >=  0 ) {
		if( x  <  blkSize  &&  y  <  blkSize ) {
			diagScan[ i ][ 0 ]  =  x
			diagScan[ i ][ 1 ]  =  y
			i++
		}
		y− −
		x++
	}
	y  =  x
	x  =  0
	if( i  >=  blkSize  *  blkSize ) 
		stopLoop  =  TRUE
}

### Horizontal scan order array initialization process

Inputs to this process is a block size blkSize.

Output of this process is the array horScan[ sPos ][ sComp ]. The array index sPos specifies the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. Depending on the value of blkSize, the array horScan is derived as follows.

i  =  0
for( y = 0; y  < blkSize; y++ )
	for( x = 0; x  <  blkSize; x++ ) {
		horScan[ i ][ 0 ]  = x
		horScan[ i ][ 1 ]  = y
		i++
	}

### Vertical scan order array initialization process

Inputs to this process is a block size blkSize.

Output of this process is the array verScan[ sPos ][ sComp ]. The array index sPos specifies the scan position ranging from 0 to ( blkSize * blkSize ) − 1. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. Depending on the value of blkSize, the array verScan is derived as follows.

i  =  0
for( x = 0; x  < blkSize; x++ )
	for( y = 0; y  <  blkSize; y++ ) {
		verScan[ i ][ 0 ]  = x
		verScan[ i ][ 1 ]  = y
		i++
	}

# Syntax and semantics

## Method of specifying syntax in tabular form

The syntax tables specify a superset of the syntax of all allowed bitstreams. Additional constraints on the syntax may be specified, either directly or indirectly, in other clauses.

NOTE – An actual decoder should implement some means for identifying entry points into the bitstream and some means to identify and handle non-conforming bitstreams. The methods for identifying and handling errors and other such situations are not specified in this Specification.

The following table lists examples of pseudo code used to describe the syntax. When syntax_element appears, it specifies that a syntax element is parsed from the bitstream and the bitstream pointer is advanced to the next position beyond the syntax element in the bitstream parsing process.

|  | Descriptor |
| --- | --- |
| /* A statement can be a syntax element with an associated descriptor or can be an expression used to specify conditions for the existence, type, and quantity of syntax elements, as in the following two examples */ |  |
| syntax_element | ue(v) |
| conditioning statement |  |
| /* A group of statements enclosed in curly brackets is a compound statement and is treated functionally as a single statement. */ |  |
| { |  |
| statement |  |
| statement |  |
| … |  |
| } |  |
| /* A "while" structure specifies a test of whether a condition is true, and if true, specifies evaluation of a statement (or compound statement) repeatedly until the condition is no longer true */ |  |
| while( condition ) |  |
| statement |  |
| /* A "do … while" structure specifies evaluation of a statement once, followed by a test of whether a condition is true, and if true, specifies repeated evaluation of the statement until the condition is no longer true */ |  |
| do |  |
| statement |  |
| while( condition ) |  |
| /* An "if … else" structure specifies a test of whether a condition is true, and if the condition is true, specifies evaluation of a primary statement, otherwise, specifies evaluation of an alternative statement. The "else" part of the structure and the associated alternative statement is omitted if no alternative statement evaluation is needed */ |  |
| if( condition ) |  |
| primary statement |  |
| else |  |
| alternative statement |  |
| /* A "for" structure specifies evaluation of an initial statement, followed by a test of a condition, and if the condition is true, specifies repeated evaluation of a primary statement followed by a subsequent statement until the condition is no longer true. */ |  |
| for( initial statement; condition; subsequent statement ) |  |
| primary statement |  |

## Specification of syntax functions and descriptors

The functions presented here are used in the syntactical description. These functions are expressed in terms of the value of a bitstream pointer that indicates the position of the next bit to be read by the decoding process from the bitstream.

byte_aligned( ) is specified as follows:

–	If the current position in the bitstream is on a byte boundary, i.e. the next bit in the bitstream is the first bit in a byte, the return value of byte_aligned( ) is equal to TRUE.

–	Otherwise, the return value of byte_aligned( ) is equal to FALSE.

more_data_in_byte_stream( ), which is used only in the byte stream NAL unit syntax structure specified in Annex Error: Reference source not found, is specified as follows:

–	If more data follow in the byte stream, the return value of more_data_in_byte_stream( ) is equal to TRUE.

–	Otherwise, the return value of more_data_in_byte_stream( ) is equal to FALSE.

more_data_in_payload( ) is specified as follows:

–	If byte_aligned( ) is equal to TRUE and the current position in the sei_payload( ) syntax structure is 8 * payloadSize bits from the beginning of the sei_payload( ) syntax structure, the return value of more_data_in_payload( ) is equal to FALSE.

–	Otherwise, the return value of more_data_in_payload( ) is equal to TRUE.

more_rbsp_data( ) is specified as follows:

–	If there is no more data in the RBSP, the return value of more_rbsp_data( ) is equal to FALSE.

–	Otherwise, the RBSP data is searched for the last (least significant, right-most) bit equal to 1 that is present in the RBSP. Given the position of this bit, which is the first bit (rbsp_stop_one_bit) of the rbsp_trailing_bits( ) syntax structure, the following applies.

–	If there is more data in an RBSP before the rbsp_trailing_bits( ) syntax structure, the return value of more_rbsp_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_data( ) is equal to FALSE.

The method for enabling determination of whether there is more data in the RBSP is specified by the application (or in Annex Error: Reference source not found for applications that use the byte stream format).

more_rbsp_trailing_data( ) is specified as follows.

–	If there is more data in an RBSP, the return value of more_rbsp_trailing_data( ) is equal to TRUE.

–	Otherwise, the return value of more_rbsp_trailing_data( ) is equal to FALSE.

payload_extension_present( ) is specified as follows:

–	If the current position in the sei_payload( ) syntax structure is not the position of the last (least significant, right-most) bit that is equal to 1 that is less than 8 * payloadSize bits from the beginning of the syntax structure (i.e. the position of the bit_equal_to_one syntax element), the return value of payload_extension_present( ) is equal to TRUE.

–	Otherwise, the return value of payload_extension_present( ) is equal to FALSE.

next_bits( n ) provides the next bits in the bitstream for comparison purposes, without advancing the bitstream pointer. Provides a look at the next n bits in the bitstream with n being its argument. When used within the byte stream format as specified in Annex Error: Reference source not found and fewer than n bits remain within the byte stream, next_bits( n ) returns a value of 0.

read_bits( n ) reads the next n bits from the bitstream and advances the bitstream pointer by n bit positions. When n is equal to 0, read_bits( n ) is specified to return a value equal to 0 and to not advance the bitstream pointer.

The following descriptors specify the parsing process of each syntax element.

–	ae(v): context-adaptive arithmetic entropy-coded syntax element. The parsing process for this descriptor is specified in subclause Error: Reference source not found.

–	b(8): byte having any pattern of bit string (8 bits). The parsing process for this descriptor is specified by the return value of the function read_bits( 8 ).

–	f(n): fixed-pattern bit string using n bits written (from left to right) with the left bit first. The parsing process for this descriptor is specified by the return value of the function read_bits( n ).

–	se(v): signed integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in subclause Error: Reference source not found.

–	u(n): unsigned integer using n bits. When n is "v" in the syntax table, the number of bits varies in a manner dependent on the value of other syntax elements. The parsing process for this descriptor is specified by the return value of the function read_bits( n ) interpreted as a binary representation of an unsigned integer with most significant bit written first.

–	ue(v): unsigned integer 0-th order Exp-Golomb-coded syntax element with the left bit first. The parsing process for this descriptor is specified in subclause Error: Reference source not found.

## Syntax in tabular form

### NAL unit syntax

#### General NAL unit syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| nal_unit_header( ) |  |
| NumBytesInRBSP = 0 |  |
| for( i = 2; i < NumBytesInNALunit; i++ ) |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_reserved_zero_6bits | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

### Raw byte sequence payloads, trailing bits, and byte alignment syntax

#### Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bit_rate_pic_rate_info( 0, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| vps_op_sets_num_minus1 | ue(v) |
| for( i = 1; i <= vps_num_op_sets_minus1; i++ ) { |  |
| operation_point_set( i ) |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_set_idx[ i ] |  |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disable_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| strong_intra_smoothing_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| nonbase_long_term_ref_pics_present_flag | u(1) |
| if( nonbase_long_term_ref_pics_present_flag ) { |  |
| num_nonbase_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_nonbase_long_term_ref_pics_sps; i++ ) { |  |
| nonbase_lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_nonbase_lt_sps_flag[ i ] | u(1) |
| lt_ref_layer_id_sps[ i ] | u(6) |
| } |  |
| } |  |
| } |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| sign_data_hiding_flag | u(1) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| if ( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pic_cb_qp_offset | se(v) |
| pic_cr_qp_offset | se(v) |
| pic_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| tiles_enabled_flag | u(1) |
| entropy_coding_sync_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pic_disable_deblocking_filter_flag | u(1) |
| if( !pic_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pic_scaling_list_data_present_flag | u(1) |
| if( pic_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| lists_modification_present_flag | u(1) |
| log2_parallel_merge_level_minus2 | ue(v) |
| num_extra_slice_header_bits | u(3) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Supplemental enhancement information RBSP syntax

| sei_rbsp( ) { | Descriptor |
| --- | --- |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Access unit delimiter RBSP syntax

| access_unit_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| pic_type | u(3) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### End of sequence RBSP syntax

| end_of_seq_rbsp( ) { | Descriptor |
| --- | --- |
| } |  |

#### End of bitstream RBSP syntax

| end_of_bitstream_rbsp( ) { | Descriptor |
| --- | --- |
| } |  |

#### Filler data RBSP syntax

| filler_data_rbsp( ) { | Descriptor |
| --- | --- |
| while( next_bits( 8 ) = = 0xFF ) |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### Slice segment layer RBSP syntax

| slice_segment_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_segment_header( ) |  |
| slice_segment_data( ) |  |
| rbsp_slice_segment_trailing_bits( ) |  |
| } |  |

#### RBSP slice segment trailing bits syntax

| rbsp_slice_segment_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_trailing_bits( ) |  |
| while( more_rbsp_trailing_data( ) ) |  |
| cabac_zero_word /* equal to 0x0000 */ | f(16) |
| } |  |

#### RBSP trailing bits syntax

| rbsp_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_stop_one_bit /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |

#### Byte alignment syntax

| byte_alignment( ) { | Descriptor |
| --- | --- |
| alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| alignment_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |

### Profile, tier and level syntax

| profile_tier_level( ProfilePresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( ProfilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( i = 0; i < 32; i++ ) |  |
| general_profile_compatibility_flag[ i ] | u(1) |
| general_reserved_zero_16bits [Ed. (GJS): Adjust semantics accordingly.] | u(16) |
| } |  |
| general_level_idc | u(8) |
| for( i = 0; i < MaxNumSubLayersMinus1; i++ ) { |  |
| if( ProfilePresentFlag ) |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( ProfilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

### Bit rate and picture rate information syntax

| bit_rate_pic_rate_info( TempLevelLow, TempLevelHigh ) { | Descriptor |
| --- | --- |
| for( i = TempLevelLow; i <= TempLevelHigh; i++ ) { |  |
| bit_rate_info_present_flag[ i ] | u(1) |
| pic_rate_info_present_flag[ i ] | u(1) |
| if( bit_rate_info_present_flag[ i ] ) { |  |
| avg_bit_rate[ i ] | u(16) |
| max_bit_rate [ i ] | u(16) |
| } |  |
| if( pic_rate_info_present_flag[ i ] ) { |  |
| constant_pic_rate_idc[ i ] | u(2) |
| avg_pic_rate[ i ] | u(16) |
| } |  |
| } |  |
| } |  |

### Operation point set syntax

| operation_point_set( opsIdx ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= vps_max_nuh_reserved_zero_layer_id; i++ ) |  |
| layer_id_included_flag[ opsIdx ][ i ] | u(1) |
| } |  |

### Scaling list data syntax

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( sizeId = 0; sizeId < 4; sizeId++ ) |  |
| for( matrixId = 0; matrixId < ( ( sizeId = = 3 ) ? 2 : 6 ); matrixId++ ) { |  |
| scaling_list_pred_mode_flag[ sizeId ][ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ sizeId ][ matrixId ] ) |  |
| scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] | ue(v) |
| else { |  |
| nextCoef = 8 |  |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeId << 1) ) ) ) |  |
| if( sizeId > 1 ) { |  |
| scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] + 8 |  |
| } |  |
| for( i = 0; i < coefNum; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ sizeId ][ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Supplemental enhancement information message syntax

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadType += 255 |  |
| } |  |
| last_payload_type_byte | u(8) |
| payloadType += last_payload_type_byte |  |
| payloadSize = 0 |  |
| while( next_bits( 8 ) = = 0xFF ) { |  |
| ff_byte /* equal to 0xFF */ | f(8) |
| payloadSize += 255 |  |
| } |  |
| last_payload_size_byte | u(8) |
| payloadSize += last_payload_size_byte |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

### Slice segment header syntax

#### General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id [Ed. (GJS): Violates syntax element naming convention by havin the same name as a syntax element in the picture parameter set.] | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if(dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| for ( i = 0; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_undetermined_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) [Ed. (GJS): Does the logic of this condition check make sense when the slice is not a B slice?] |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pic_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_disable_deblocking_filter_flag | u(1) |
| if( !slice_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| if( slice_segment_header_extension_length > 0 ) |  |
| slice_segment_header_extension( slice_segment_header_extension_length ) |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

##### Slice segment header extension syntax

| slice_segment_header_extension( numBytes ) { | Descriptor |
| --- | --- |
| extension_struct_idc | ue(v) |
| if( extension_struct_idc = = 0 ) |  |
| if( nonbase_long_term_ref_pics_present_flag ) { |  |
| if( num_nonbase_long_term_ref_pics_sps > 0 ) |  |
| num_nonbase_long_term_sps | ue(v) |
| num_nonbase_long_term_pics | ue(v) |
| for( i = 0; i < num_nonbase_long_term_sps + num_nonbase_long_term_pics; i++ ) { |  |
| if( i < num_nonbase_long_term_sps ) |  |
| nonbase_lt_idx_sps[ i ] | u(v) |
| else { |  |
| nonbase_poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_nonbase_lt_flag[ i ] | u(1) |
| layer_id_lt[ i ] | u(6) |
| } |  |
| nonbase_delta_poc_msb_present_flag[ i ] | u(1) |
| if( nonbase_delta_poc_msb_present_flag[ i ] ) |  |
| nonbase_delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| while( more_data_in_extension( ) ) // [Ed. to be specified similarly to more_data_in_payload( ) ] |  |
| slice_segment_header_extension_bit_equal_to_one | u(1) |
| } |  |

#### Short-term reference picture set syntax

[Ed. This appears in the SPS and SH. It should probably get its own level 3 heading.]

| short_term_ref_pic_set( idxRps ) { | Descriptor |
| --- | --- |
| if( idxRps != 0 ) |  |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| if( idxRps = = num_short_term_ref_pic_sets ) |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| used_by_curr_pic_flag[ j ] | u(1) |
| if( !used_by_curr_pic_flag[ j ] ) |  |
| use_delta_flag[ j ] | u(1) |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

#### Reference picture list modification syntax

| ref_pic_lists_modification( ) { | Descriptor |
| --- | --- |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| list_entry_l0[ i ] | u(v) |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| list_entry_l1[ i ] | u(v) |
| } |  |
| } |  |

#### Weighted prediction parameters syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( chroma_format_idc != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( chroma_format_idc != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l0_active_minus1; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( slice_type = = B ) { |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( chroma_format_idc != 0 ) |  |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i <= num_ref_idx_l1_active_minus1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### Slice segment data syntax

#### General slice segment data syntax

| slice_segment_data( ) { | Descriptor |
| --- | --- |
| do { |  |
| coding_tree_unit( ) |  |
| end_of_slice_segment_flag | ae(v) |
| CtbAddrInTS++ |  |
| CtbAddrInRS = CtbAddrTStoRS[ CtbAddrInTS ] |  |
| if( !end_of_slice_segment_flag && ( ( tiles_enabled_flag && TileId[ CtbAddrInTS ] != TileId[ CtbAddrInTS − 1 ] ) ¦ ¦ (entropy_coding_sync_enabled_flag && CtbAddrInTS % PicWidthInCtbsY = = 0) ) ) { |  |
| end_of_sub_stream_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| } |  |
| } while( !end_of_slice_segment_flag ) |  |
| } |  |

#### Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrRS % PicWidthInCtbsY ) << Log2CtbSizeY |  |
| yCtb = ( CtbAddrRS / PicWidthInCtbsY ) << Log2CtbSizeY |  |
| CtbAddrInSliceSeg = CtbAddrInRS − slice_segment_address |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> Log2CtbSizeY, yCtb >> Log2CtbSizeY ) |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| } |  |

#### Sample adaptive offset syntax

| sao( rx, ry ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSliceSeg = CtbAddrInSliceSeg > 0 |  |
| leftCtbInTile = TileId[ CtbAddrInTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrInRS − 1 ] ] |  |
| if( leftCtbInSliceSeg && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSliceSeg = ( CtbAddrInRS − PicWidthInCtbsY ) >= slice_segment_address |  |
| upCtbInTile = TileId[ CtbAddrInTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrInRS − PicWidthInCtbsY ] ] |  |
| if( upCtbInSliceSeg && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| if( ( slice_sao_luma_flag && cIdx = = 0 ) ¦ ¦ ( slice_sao_chroma_flag && cIdx > 0 ) ) { |  |
| if( cIdx = = 0 ) |  |
| sao_type_idx_luma | ae(v) |
| else if( cIdx = = 1 ) |  |
| sao_type_idx_chroma | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] != 0 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 1 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } else { |  |
| if( cIdx = = 0 ) |  |
| sao_eo_class_luma | ae(v) |
| if( cIdx = = 1 ) |  |
| sao_eo_class_chroma | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && log2CbSize > Log2MinCbSizeY ) |  |
| split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDelta = 0 |  |
| } |  |
| if( split_cu_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| } else |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| } |  |

#### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] [Ed. (GJS): Syntax element name convention violation – subset of name transform_skip_flag.] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### PCM sample syntax

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } |  |

#### Transform tree syntax

| transform_tree( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < MaxTrafoDepth && !(IntraSplitFlag && trafoDepth = = 0) ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize > 2 ) { |  |
| if( trafoDepth = = 0 ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( trafoDepth = = 0 ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_tree( x0, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, x0, y0, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| transform_unit (x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

#### Transform unit syntax

| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign | ae(v) |
| } |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSize, 0 ) |  |
| if( log2TrafoSize > 2 ) { |  |
| if( cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSize − 1, 1 ) |  |
| if( cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| residual_coding( x0, y0, log2TrafoSize − 1, 2 ) |  |
| } else if( blkIdx = = 3 ) { |  |
| if( cbf_cb[ xBase ][ yBase ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase, log2TrafoSize, 1 ) |  |
| if( cbf_cr[ xBase ][ yBase ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase, log2TrafoSize, 2 ) |  |
| } |  |
| } |  |
| } |  |

#### Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| lastScanPos = 16 |  |
| lastSubBlock = ( 1 << ( log2TrafoSize − 2 ) ) * ( 1 << ( log2TrafoSize − 2 ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| } |  |
| firstSigScanPos = 16 |  |
| lastSigScanPos = −1 |  |
| numGreater1Flag = 0 |  |
| lastGreater1ScanPos = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numGreater1Flag < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numGreater1Flag++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && lastGreater1ScanPos = = −1 ) |  |
| lastGreater1ScanPos = n |  |
| } |  |
| if( lastSigScanPos = = −1) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 && !cu_transquant_bypass_flag ) |  |
| if( lastGreater1ScanPos != −1 ) |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && ( !sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstSigScanPos ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbsLevel += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstSigScanPos && ( ( sumAbsLevel % 2 ) = = 1) ) |  |
| TransCoeffLevel[x0][y0][cIdx][xC][yC] = − TransCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Semantics

Semantics associated with the syntax structures and with the syntax elements within these structures are specified in this subclause. When the semantics of a syntax element are specified using a table or a set of tables, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this Specification.

### NAL unit semantics

#### General NAL unit semantics

NumBytesInNALunit specifies the size of the NAL unit in bytes. This value is required for decoding of the NAL unit. Some form of demarcation of NAL unit boundaries is necessary to enable inference of NumBytesInNALunit. One such demarcation method is specified in Annex Error: Reference source not found for the byte stream format. Other methods of demarcation may be specified outside of this Specification.

NOTE 18 – The VCL is specified to efficiently represent the content of the video data. The NAL is specified to format that data and provide header information in a manner appropriate for conveyance on a variety of communication channels or storage media. All data are contained in NAL units, each of which contains an integer number of bytes. A NAL unit specifies a generic format for use in both packet-oriented and bitstream systems. The format of NAL units for both packet-oriented transport and byte stream is identical except that each NAL unit can be preceded by a start code prefix and extra padding bytes in the byte stream format specified in Annex Error: Reference source not found.

rbsp_byte[ i ] is the i-th byte of an RBSP. An RBSP is specified as an ordered sequence of bytes as follows.

The RBSP contains an SODB as follows.

–	If the SODB is empty (i.e. zero bits in length), the RBSP is also empty.

–	Otherwise, the RBSP contains the SODB as follows:

1)	The first byte of the RBSP contains the (most significant, left-most) eight bits of the SODB; the next byte of the RBSP contains the next eight bits of the SODB, etc., until fewer than eight bits of the SODB remain. [Ed. (GJS): Generally, there are way too many places where the word "shall" is used. If it is not a testable constraint on the content of a conforming bitstream and is not a testable constraint on the output behaviour of the decoder, it should not be expressed using a "shall".]

2)	rbsp_trailing_bits( ) are present after the SODB as follows:

i)	The first (most significant, left-most) bits of the final RBSP byte contains the remaining bits of the SODB (if any).

ii)	The next bit consists of a single rbsp_stop_one_bit equal to 1.

iii)	When the rbsp_stop_one_bit is not the last bit of a byte-aligned byte, one or more rbsp_alignment_zero_bit is present to result in byte alignment.

3)	One or more cabac_zero_word 16-bit syntax elements equal to 0x0000 may be present in some RBSPs after the rbsp_trailing_bits( ) at the end of the RBSP.

Syntax structures having these RBSP properties are denoted in the syntax tables using an "_rbsp" suffix. These structures shall be carried within NAL units as the content of the rbsp_byte[ i ] data bytes. The association of the RBSP syntax structures to the NAL units shall be as specified in NOTE 22 – In HRD operations as specified in Annex Error: Reference source not found, NAL units with reserved values of nal_unit_type may need to be taken into consideration in derivation of CPB arrival and removal times. However, during the decoding process, NAL units with reserved values of nal_unit_type can be safely ignored (removed and discarded) and must be ignored..

NOTE 19 – When the boundaries of the RBSP are known, the decoder can extract the SODB from the RBSP by concatenating the bits of the bytes of the RBSP and discarding the rbsp_stop_one_bit, which is the last (least significant, right-most) bit equal to 1, and discarding any following (less significant, farther to the right) bits that follow it, which are equal to 0. The data necessary for the decoding process is contained in the SODB part of the RBSP.

emulation_prevention_three_byte is a byte equal to 0x03. When an emulation_prevention_three_byte is present in the NAL unit, it shall be discarded by the decoding process.

The last byte of the NAL unit shall not be equal to 0x00.

Within the NAL unit, the following three-byte sequences shall not occur at any byte-aligned position:

–	0x000000

–	0x000001

–	0x000002

Within the NAL unit, any four-byte sequence that starts with 0x000003 other than the following sequences shall not occur at any byte-aligned position:

–	0x00000300

–	0x00000301

–	0x00000302

–	0x00000303

#### NAL unit header semantics

forbidden_zero_bit shall be equal to 0.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in NOTE 22 – In HRD operations as specified in Annex Error: Reference source not found, NAL units with reserved values of nal_unit_type may need to be taken into consideration in derivation of CPB arrival and removal times. However, during the decoding process, NAL units with reserved values of nal_unit_type can be safely ignored (removed and discarded) and must be ignored..

NAL units that use nal_unit_type in the range of UNSPEC48..UNSPEC63, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 20 – NAL unit types in the range of UNSPEC48..UNSPEC63 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Specification. Since different applications might use NAL unit types in the range of UNSPEC48..UNSPEC63 for different purposes, particular care must be exercised in the design of encoders that generate NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC63, and in the design of decoders that interpret the content of NAL units with nal_unit_type in the range of UNSPEC48..UNSPEC63.

During the decoding process, decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 21 – This requirement allows future definition of compatible extensions to this Specification.

NOTE 22 – In HRD operations as specified in Annex Error: Reference source not found, NAL units with reserved values of nal_unit_type may need to be taken into consideration in derivation of CPB arrival and removal times. However, during the decoding process, NAL units with reserved values of nal_unit_type can be safely ignored (removed and discarded) and must be ignored.

Table 7-3 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0, 1 | TRAIL_N, TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2, 3 | TSA_N, TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4, 5 | STSA_N, STSA_R | Coded slice segment of an STSA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | RADL_N, RADL_R | Coded slice segment of a RADL picture slice_layer_rbsp( ) | VCL |
| 8, 9 | RASL_N, RASL_R, | Coded slice segment of a RASL picture slice_layer_rbsp( ) | VCL |
| 10, 12, 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved // reserved non-RAP non-reference VCL NAL unit types | VCL |
| 11, 13, 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved // reserved non-RAP reference VCL NAL unit types | VCL |
| 16, 17, 18 | BLA_W_LP BLA_W_DLP BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) [Ed. (YK): BLA_W_DLP -> BLA_W_RADL?] | VCL |
| 19, 20 | IDR_W_DLP IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) [Ed. (YK: IDR_W_DLP -> IDR_W_RADL?] | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22, 23 | RSV_RAP_VCL22.. RSV_RAP_VCL23 | Reserved // reserved RAP VCL NAL unit types [Ed.(YK): Some RAP related restrictions need to be specified for these 2 NUTs] | VCL |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved // reserved non-RAP VCL NAL unit types [Ed.(YK): Some non-RAP related restrictions may need to be specified for these NUTs] | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitsteam end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39, 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

- NOTE 23 – A CRA picture may have associated RASL pictures present in the bitstream.

NOTE 24 – A BLA picture having nal_unit_type equal to BLA_W_LP may have associated RASL or RADL pictures present in the bitstream. A BLA picture having nal_unit_type equal to BLA_W_DLP does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream. A BLA picture having nal_unit_type equal to BLA_N_LP does not have associated leading pictures present in the bitstream.

NOTE 25 – An IDR picture having nal_unit_type equal to IDR_N_LP does not have associated leading pictures present in the bitstream. An IDR picture having nal_unit_type equal to IDR_W_DLP does not have associated RASL pictures present in the bitstream, but may have associated RADL pictures in the bitstream.

NOTE 26 – When the value of nal_unit_type is equal to TRAIL_N, TSA_N or STSA_N, the decoded picture is not included in any of RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of any picture with the same value of TemporalId. A coded picture with nal_unit_type equal to TRAIL_N, TSA_N or STSA_N may be discarded without affecting the decodability of other pictures with the same value of TemporalId.

The variable IdrPicFlag is specified as

IdrPicFlag = ( nal_unit_type  = =  IDR_W_DLP  | |  nal_unit_type  = =  IDR_N_LP )	(7-17)

The variable RapPicFlag is specified as

RapPicFlag = ( nal_unit_type  >=  16  &&  nal_unit_type  <=  23 )	(7-18)

All coded slice segment NAL units of an access unit shall have the same value of nal_unit_type.

Each picture, other than the first picture in the bitstream, is considered to be associated with the previous RAP picture in decoding order.

When a picture is a leading picture, it shall be a RADL or RASL picture.

When a picture is a trailing picture, it shall not be a RADL or RASL picture.

When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures that are associated with the same RAP picture.

When a picture has PicOutputFlag equal to 1 and precedes a RAP picture in decoding order, it shall also precede the RAP picture in output order.

No RASL pictures shall be present in the bitstream that are associated with a BLA picture having nal_unit_type equal to BLA_W_DLP or BLA_N_LP.

No RASL pictures shall be present in the bitstream that are associated with an IDR picture.

No RADL pictures shall be present in the bitstream that are associated with a BLA picture having nal_unit_type equal to BLA_N_LP or that are associated with an IDR picture having nal_unit_type equal to IDR_N_LP.

- NOTE 27 – It is possible to perform random access at the position of a RAP access unit by discarding all access units before the RAP access unit (and to correctly decode the RAP picture and all the subsequent non-RASL pictures in decoding order), provided each parameter set is available (either in the bitstream or by external means) when it needs to be activated.

Any picture that has PicOutputFlag equal to 1 that precedes a CRA or BLA picture in decoding order shall precede any RADL picture associated with the CRA or BLA picture in output order.

Any RASL picture associated with a CRA or BLA picture shall precede any RADL picture associated with the CRA or BLA picture in output order. [Ed. (YK): Check the discussion related to K0120 and see whether any editorial changes herein are desirable, e.g. include mentioning of IDR picture here too such that only RAP picture needs to be mentioned, and combine this restriction with the above output order restriction "When a picture has PicOutputFlag equal to 1 and precedes a RAP picture in decoding order, it shall also precede the RAP picture in output order."]

Any RASL picture associated with a CRA picture shall follow, in output order, any RAP picture that precedes the CRA picture in decoding order. [Ed. (TK) Why do we need this restriction? (GJS): Just so that each RAP-to-RAP chunk is a roughly-sensible segment of the video content, I suppose. We have other such "unnecessary" constraints, such as the constraint that RASL's need to be leading pictures and the constraints on relative decoding order of RASL, RADL and trailing pictures.]

When sps_temporal_id_nesting_flag is equal to 1 and TemporalId is greater than 0, the nal_unit_type shall be equal to TSA_R, TSA_N, RADL_R, RADL_N, RASL_R, or RASL_NUT.

nuh_reserved_zero_6bits shall be equal to 0. Other values of nuh_reserved_zero_6bits may be specified in the future by ITU-T | ISO/IEC. During the decoding process, decoders shall ignore (i.e. remove from the bitstream and discard) all NAL units with values of nuh_reserved_zero_6bits not equal to 0.

- NOTE 28 – It is anticipated that in future scalable video coding or extensions of this specification, this syntax element will be used to identify additional layers that may be present in the coded video sequence, wherein a layer may be, e.g. a spatial scalable layer, a quality scalable layer, a texture view or a depth view.

NOTE 29 – In HRD operations as specified in Annex Error: Reference source not found, NAL units with reserved values of nuh_reserved_zero_6bits may need to be take into consideration in derivation of CPB arrival and removal timing. However, during the decoding process, NAL units with reserved values of nuh_reserved_zero_6bits can be safely ignored (removed and discarded) and must be ignored.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The value of nuh_temporal_id_plus1 shall be greater than 0.

The variable TemporalId is specified as

TemporalId = nuh_temporal_id_plus1 − 1		(7-19)

If nal_unit_type is in the range of 16 to 23 (coded slice segment of a RAP picture), inclusive, TemporalId shall be equal to 0; otherwise, when nal_unit_type is equal to TSA_R, TSA_N, STSA_R, or STSA_N, TemporalId shall not be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of an access unit is the value of the TemporalId of the VCL NAL units of the access unit.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If nal_unit_type is equal to VPS_NUT, SPS_NUT, EOS_NUT, or EOB_NUT, TemporalId shall be equal to 0.

–	Otherwise, if nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId shall be equal to the TemporalId of the access unit containing the non-VCL NAL unit.

–	Otherwise, when nal_unit_type is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId shall be greater or equal to the TemporalId of the access unit containing the NAL unit.

- NOTE 30 – When the NAL unit is a non-VCL NAL unit, the value of TemporalId is equal to the minimum value of the TemporalId values of all access units that the non-VCL NAL unit applies to. When nal_unit_type is equal to VPS_NUT or SPS_NUT, TemporalId must be equal to 0, as a sequence parameter set applies at least to one RAP access unit. When nal_unit_type is equal to AUD_NUT or FD_NUT, TemporalId must be equal to the TemporalId of the access unit containing the non-VCL NAL unit, as access unit delimiter or filler data only applies to the containing access unit. When nal_unit_type is equal to PPS_NUT, TemporalId may be less than, equal to, or greater than the TemporalId of the containing access unit, as a picture parameter set may be repeated in access units not referring to the picture parameter set (for error resilience purposes), and all picture parameter sets may be included in the beginning of a bitstream, wherein the first coded picture has TemporalId equal to 0. When nal_unit_type is equal to PREFIX_SEI_NUT or SUFFIX_SEI_NUT, TemporalId may be greater than or equal to the TemporalId of the containing access unit, as an SEI NAL unit may contain information, e.g. in a buffering period SEI message or a picture timing SEI message, that applies to a bitstream subset that includes access units for which the TemporalId values are greater than the TemporalId of the access unit containing the SEI NAL unit.

#### Encapsulation of an SODB within an RBSP (informative)

This subclause does not form an integral part of this Specification.

The form of encapsulation of an SODB within an RBSP and the use of the emulation_prevention_three_byte for encapsulation of an RBSP within a NAL unit is described for the following purposes:

–	To prevent the emulation of start codes within NAL units while allowing any arbitrary SODB to be represented within a NAL unit,

–	To enable identification of the end of the SODB within the NAL unit by searching the RBSP for the rbsp_stop_one_bit starting at the end of the RBSP,

–	To enable a NAL unit to have a size larger than that of the SODB under some circumstances (using one or more cabac_zero_word syntax elements).

The encoder can produce a NAL unit from an RBSP by the following procedure:

- The RBSP data is searched for byte-aligned bits of the following binary patterns:

'00000000 00000000 000000xx'  (where 'xx' represents any two-bit pattern: '00', '01', '10', or '11'),

and a byte equal to 0x03 is inserted to replace the bit pattern with the pattern:

'00000000 00000000 00000011 000000xx',

and finally, when the last byte of the RBSP data is equal to 0x00 (which can only occur when the RBSP ends in a cabac_zero_word), a final byte equal to 0x03 is appended to the end of the data. The last zero byte of a byte-aligned three-byte sequence 0x000000 in the RBSP (which is replaced by the four-byte sequence 0x00000300) is taken into account when searching the RBSP data for the next occurrence of byte-aligned bits with the binary patterns specified above.

- The resulting sequence of bytes is then prefixed as follows.

–	The sequence of bytes is prefixed with the NAL unit header, within which the nal_unit_type indicates the type of RBSP data structure in the NAL unit.

The process specified above results in the construction of the entire NAL unit.

This process can allow any SODB to be represented in a NAL unit while ensuring that

–	no byte-aligned start code prefix is emulated within the NAL unit,

- no sequence of 8 zero-valued bits followed by a start code prefix, regardless of byte-alignment, is emulated within the NAL unit.

#### Order of NAL units and association to coded pictures, access units, and video sequences

This subclause specifies constraints on the order of NAL units in the bitstream.

Any order of NAL units in the bitstream obeying these constraints is referred to in the text as the decoding order of NAL units. Within a NAL unit, the syntax in subclauses 7.3, Error: Reference source not found, and Error: Reference source not found specifies the decoding order of syntax elements. Decoders shall be capable of receiving NAL units and their syntax elements in decoding order.

##### Order of video, sequence, and picture parameter set RBSPs and their activation

This subclause specifies the activation process of video, sequence and picture parameter sets.

NOTE 31 – The video, sequence, and picture parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded block data. Video, sequence, and picture parameter sets may, in some applications, be conveyed "out-of-band".

- A picture parameter set RBSP includes parameters that can be referred to by the coded slice segment NAL units of one or more coded pictures. Each picture parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one picture parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular picture parameter set RBSP results in the deactivation of the previously-active picture parameter set RBSP (if any).

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not active and it is referred to by a coded slice segment NAL unit (using that value of pic_parameter_set_id), it is activated. This picture parameter set RBSP is called the active picture parameter set RBSP until it is deactivated by the activation of another picture parameter set RBSP. A picture parameter set RBSP, with that particular value of pic_parameter_set_id, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the picture parameter set NAL unit, unless the picture parameter set is provided through external means.

- Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active picture parameter set RBSP for a coded picture shall have the same content as that of the active picture parameter set RBSP for the coded picture unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing an active sequence parameter sets SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP results in the deactivation of the previously-active sequence parameter set RBSP (if any).

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing an active parameter sets SEI message (using that value of seq_parameter_set_id), it is activated. This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0, unless the sequence parameter set is provided through external means. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 32 – Because an IDR or BLA access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by an active sequence parameter sets SEI message when the active sequence parameter sets SEI message is part of an IDR or BLA access unit.

- Any sequence parameter set NAL unit containing the value of seq_parameter_set_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing an active sequence parameter sets SEI message (when present) of another coded video sequence.

- A video parameter set RBSP includes parameters that can be referred to by one or more sequence parameter set RBSPs or one or more SEI NAL units containing an active parameter sets SEI message. Each video parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one video parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular video parameter set RBSP results in the deactivation of the previously-active video parameter set RBSP (if any).

- When a video parameter set RBSP (with a particular value of video_parameter_set_id) is not already active and it is referred to by activation of a sequence parameter set RBSP (using that value of video_parameter_set_id) or is referred to by an SEI NAL unit containing an active parameter sets SEI message (using that value of seq_parameter_set_id), it is activated. This video parameter set RBSP is called the active video parameter set RBSP until it is deactivated by the activation of another video parameter set RBSP. A video parameter set RBSP, with that particular value of video_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0, unless the video parameter set is provided through external means. An activated video parameter set RBSP shall remain active for the entire coded video sequence.

- Any video parameter set NAL unit containing the value of video_parameter_set_id for the active video parameter set RBSP for a coded video sequence shall have the same content as that of the active video parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit, the first sequence parameter set NAL unit, and the first SEI NAL unit containing an active parameter sets SEI message (when present) of another coded video sequence.

- NOTE 33 – If video parameter set RBSP, sequence parameter set RBSP, or picture parameter set RBSP are conveyed within the bitstream, these constraints impose an order constraint on the NAL units that contain the video parameter set RBSP, sequence parameter set RBSP, or picture parameter set RBSP respectively. Otherwise (video parameter set RBSP, sequence parameter set RBSP, or picture parameter set RBSP are conveyed by other means not specified in this Specification), they must be available to the decoding process in a timely fashion such that these constraints are obeyed.

All constraints that are expressed on the relationship between the values of the syntax elements (and the values of variables derived from those syntax elements) in video parameter sets, sequence parameter sets, and picture parameter sets and other syntax elements are expressions of constraints that apply only to the active video parameter set, the active sequence parameter set, and the active picture parameter set. If any video parameter set RBSP, sequence parameter set RBSP, and picture parameter set RBSP is present that is not ever activated in the bitstream, its syntax elements shall have values that would conform to the specified constraints if it were activated by reference in an otherwise conforming bitstream.

During operation of the decoding process (see clause 8), the values of parameters of the active video parameter set, the active sequence parameter set, and the active picture parameter set RBSP shall be considered in effect. For interpretation of SEI messages, the values of the active video parameter set, the active sequence parameter set, and the active picture parameter set RBSP for the operation of the decoding process for the VCL NAL units of the coded picture in the same access unit shall be considered in effect unless otherwise specified in the SEI message semantics.

##### Order of access units and association to coded video sequences

A bitstream conforming to this Specification consists of one or more coded video sequences.

A coded video sequence consists of one or more access units. The order of NAL units and coded pictures and their association to access units is described in subclause 7.4.1.4.3.

If a coded video sequence is the first coded video sequence in the bitstream, the first access unit of the coded video sequence is a RAP access unit, which may be an IDR, BLA or CRA access unit; otherwise, the first access unit of the coded video sequence is an IDR or BLA access unit. All access units in a coded video sequence that are not the first access unit in the coded video sequence are non-IDR and non-BLA access units.

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of sequence NAL unit shall be an IDR or BLA access unit.

It is a requirement of bitstream conformance that, when present, the next access unit after an access unit that contains an end of bitstream NAL unit shall be a RAP access unit, which may be an IDR, BLA or CRA access unit.

##### Order of NAL units and coded pictures and association to access units

This subclause specifies the order of NAL units and coded pictures and association to access unit for coded video sequences that conform to one or more of the profiles specified in Annex Error: Reference source not found that are decoded using the decoding process specified in clauses 2–Error: Reference source not found.

An access unit consists of one coded picture and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in subclause 7.4.1.4.4.

The first access unit in the bitstream starts with the first NAL unit of the bitstream.

The first of any of the following NAL units after the last VCL NAL unit of a coded picture specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	video parameter set NAL unit (when present),

–	sequence parameter set NAL unit (when present),

–	picture parameter set NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL units with nal_unit_type in the range of RSV_NVCL41..RSV_NVCL44 (when present),

–	first VCL NAL unit of a coded picture (always present).

The following constraints shall be obeyed by the order of the coded pictures and non-VCL NAL units within an access unit:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	When any prefix SEI NAL units are present, they shall not follow the last VCL NAL unit of the access unit. [Ed. (GJS): Can the currently-specified prefix SEI NAL units (that have a whole-picture scope) be placed between the VCL NAL units of the picture?]

–	When an prefix SEI NAL unit having nuh_reserved_zero_6bits equal to 0 and containing a buffering period SEI message is present, the prefix SEI NAL unit shall also contain an active parameter sets SEI message, and the active parameter sets SEI message and the buffering period SEI message shall be the first and the second SEI message payloads, respectively, of the first SEI NAL unit in the access unit. [Ed. (YK): This restriction should only apply to SEI NAL units with nuh_reserved_zero_6bits equal to 0. Since nuh_reserved_zero_6bits is required to be equal to 0 in this version of this Specification, now we don't have to mention nuh_reserved_zero_6bits equal to 0, but it may be safer if we do say it now, though it is not needed. Also, if the parsing dependency of picture timing SEI message and decoding unit information SEI message can be resolved, we don't need to have this restriction at all.]

–	NAL units having nal_unit_type equal to FD_NUT, SUFFIX_SEI_NUT, in the range of RSV_NVCL45..RSV_NVCL47, or in the range of UNSPEC48..UNSPEC63 shall not precede the first VCL NAL unit of the coded picture.

–	When an end of sequence NAL unit is present, it shall be the last NAL unit in the access unit other than an end of bitstream NAL unit (if present).

–	When an end of bitstream NAL unit is present, it shall be the last NAL unit in the access unit.

NOTE – Video parameter set NAL units, sequence parameter set NAL units, or picture parameter set NAL units may be present in an access unit, but cannot follow the last VCL NAL unit of the coded picture within the access unit, as this condition would specify the start of a new access unit.

The structure of access units not containing any NAL units with nal_unit_type equal to FD_NUT, VPS_NUT, SPS_NUT, PPS_NUT, or in the ranges of RSV_RAP_VCL22..RSV_RAP_VCL23, RSV_VCL24..RSV_VCL31, RSV_NVCL40..RSV_NVCL47, or UNSPEC48..UNSPEC63 is shown in Figure 7-6.

Figure 7-6 – Structure of an access unit not containing any NAL units with nal_unit_type equal to FD_NUT, VPS_NUT, SPS_NUT, PPS_NUT, or in the ranges of RSV_RAP_VCL22..RSV_RAP_VCL23, RSV_VCL24..RSV_VCL31, RSV_NVCL40..RSV_NVCL47, or UNSPEC48..UNSPEC63

##### Order of VCL NAL units and association to coded pictures

This subclause specifies the order of VCL NAL units and association to coded pictures.

Each VCL NAL unit is part of a coded picture.

The order of the VCL NAL units within a coded picture is constrained as follows [Ed (YK): May need to improve to support the case when separate_colour_plane_flag  = =  1.]:

–	The first VCL NAL unit of the coded picture shall have first_slice_segment_in_pic_flag equal to 1.

–	Let sliceSegAddrA and sliceSegAddrB be the slice_segment_address values of any two coded slice segment NAL units A and B within the same coded picture. When one or more of the following conditions are true, coded slice segment NAL unit A shall precede the coded slice segment NAL unit B.

–	TileId[ CtbAddrRStoTS[ sliceSegAddrA ] ] is less than TileId[ CtbAddrRStoTS[ sliceSegAddrB] ].

–	TileId[ CtbAddrRStoTS[ sliceSegAddrA] ] is equal to TileId[ CtbAddrRStoTS[ sliceSegAddrB ] ] and CtbAddrRStoTS[ sliceSegAddrA ] is less than CtbAddrRStoTS[ sliceSegAddrB ].

### Raw byte sequence payloads, trailing bits, and byte alignment semantics

#### Video parameter set RBSP semantics

NOTE 34 – Encoders conforming to this version of this Specification are required to include video parameter set NAL units in the bitstream, as specified in subclause 7.4.1.4.1. However, the video parameter set RBSP contains information that is not necessary for operation of the decoding process of this version of this Specification. Decoders conforming to this version of this Specification may ignore (remove from the bitstream and discard) the content of all video parameter set NAL units.

Any two instances of the syntax structure hrd_parameters( ) included in a video parameter set RBSP shall not have the same content.

Any two instances of the syntax structure operation_point_set( ) included in a video parameter set RBSP shall not have the same content.

video_parameter_set_id identifies a video parameter set.

vps_reserved_three_2bits shall be equal to 3 in bitstreams conforming to this Specification. Other values for vps_reserved_three_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of vps_reserved_three_2bits.

vps_reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this Specification. Other values for vps_reserved_zero_6bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of vps_reserved_zero_6bits.

NOTE 35 – It is anticipated that in future scalable and/or 3D video extensions of this Specification, this field specifies the maximum number of layers that may be present in the coded video sequence, wherein a layer may e.g. be a spatial scalable layer, a quality scalable layer, a texture view or a depth view.

vps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in the bitstream. The value of vps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

vps_temporal_id_nesting_flag, when vps_max_sub_layers_minus1 is greater than 0,  specifies whether inter prediction is additionally restricted for coded video sequences referring to the video parameter set. When vps_max_sub_layers_minus1 is equal to 0, vps_temporal_id_nesting_flag has no meaning and may have any value.

NOTE 36 – The syntax element vps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e. switching from decoding of up to a specific TemporalId tIdN to decoding up to a TemporalId tIdM that is greater than tIdN, is always possible.

vps_reserved_0xffff_16bits shall be equal to 0xFFFF in bitstreams conforming to this Specification. Other values for vps_reserved_0xffff_16bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of vps_reserved_0xffff_16bits.

NOTE 37 – It is anticipated that in future scalable and/or 3D video extensions of this Specification, this field specifies the byte offset of the next set of fixed-length coded information in the video parameter set NAL unit, starting from the beginning of the NAL unit. Video parameter set information for non-base layer or view starts from a byte-aligned position of the video parameter set NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset would then help to locate and access those essential information in the video parameter set NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only the information in the video parameter set that is essential for session negotiation and/or capability exchange.

- vps_sub_layer_ordering_info_present_flag equal to 1 specifies that vps_max_dec_pic_buffering[ i ], vps_max_num_reorder_pics[ i ], and vps_max_latency_increase[ i ] are present for vps_max_sub_layers_minus1 + 1 sub-layers, vps_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of vps_max_dec_pic_buffering[ vps_max_sub_layers_minus1 ], vps_max_num_reorder_pics[ vps_max_sub_layers_minus1 ], and vps_max_latency_increase[ vps_max_sub_layers_minus1 ] apply to all sub-layers.

- vps_max_dec_pic_buffering[ i ] specifies the required size of the decoded picture buffer in units of picture storage buffers when HighestTid is equal to i. The value of vps_max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive. When i is greater than 0, vps_max_dec_pic_buffering[ i ] shall be greater than or equal to vps_max_dec_pic_buffering[ i − 1 ]. When vps_max_dec_pic_buffering[ i ] is not present for i in the range of 0 to vps_max_sub_layers_minus1 − 1 due to vps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_dec_pic_buffering[ vps_max_sub_layers_minus1 ].

- vps_max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures preceding any picture in decoding order and succeeding that picture in output order when HighestTid is equal to i. The value of vps_max_num_reorder_pics[ i ] shall be in the range of 0 to vps_max_dec_pic_buffering[ i ], inclusive. When i is greater than 0, vps_max_num_reorder_pics[ i ] shall be greater than or equal to vps_max_num_reorder_pics[ i − 1 ]. When vps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to vps_max_sub_layers_minus1 − 1 due to vps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_num_reorder_pics[ vps_max_sub_layers_minus1 ].

- vps_max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by setting MaxLatencyPictures[ i ] to vps_max_num_reorder_pics[ i ] + vps_max_latency_increase[ i ]. When vps_max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order when HighestTid is equal to i. When vps_max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of vps_max_latency_increase[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_latency_increase[ i ] is not present for i in the range of 0 to vps_max_sub_layers_minus1 − 1 due to vps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_latency_increase[ vps_max_sub_layers_minus1 ].

vps_max_nuh_reserved_zero_layer_id specifies the maximum allowed value nuh_reserved_zero_6bits of all NAL units in the coded video sequence.

vps_num_op_sets_minus1 plus 1 specifies the number of operation point sets that are specified by the video parameter set. In bitstreams conforming to this Specification, the value of vps_num_op_sets_minus1 shall be equal to 0. Although the value of vps_num_op_sets_minus1 is required to be equal to 0 in this version of this Specification, decoders shall allow other values of vps_num_op_sets_minus1 in the range of 0 to 1023 to appear in the syntax.

Each operation point is identified by a set OpLayerIdset, which includes and only includes the set of nuh_reserved_zero_6bits values of all NAL unit included in the operation point, and a variable OpTid, which is equal to the greatest TemporalId of NAL unit included in the operation point. The bitstream subset associated with the operation point identified by OpLayerIdSet and OpTid refers to the output of the sub-bitstream extraction process as specified in subclause Error: Reference source not found with OpTid and OpLayerIdSet as inputs. The OpLayerIdSet and OpTid that identify an operation point are also referred to the OpLayerIdSet and OpTid of the operation point, respectively.

Each operation point set consists of all operation points for which the included NAL units have the same set OpLayerIdSet and different values of OpTid. An operation point is thus also referred to as identified by the particular value of OpLayerIdSet and considered to have OplayerIdSet equal to the particular value. The OpLayerIdSet that identifies an operation point set is also referred to as the OpLayerIdSet of the operation point set.

The 0-th operation point set specified by the video parameter set is identified by the OpLayerIdSet that includes the value 0 only. The i-th operation point set, with i greater than 0, specified by the video parameter set is specified by the syntax structure operation_point_set( i ).

- vps_num_hrd_parameters specifies the number of hrd_parameters( ) syntax structures present in the video parameter set RBSP. In bitstreams conforming to this Specification, the value of vps_num_hrd_parameters shall be less than or equal to 1. Although the value of vps_num_hrd_parameters is required to be less than or equal to 1 in this version of this Specification, decoders shall allow other values of vps_num_hrd_parameters in the range of 0 to 1024 to appear in the syntax.

hrd_op_set_idx[ i ] specifies the index, to the list of operation point sets specified by the video parameter set, of the operation point set to which the i-th hrd_parameters() syntax structure in the video parameter set applies. In bitstreams conforming to this Specification, the value of hrd_op_idx[ i ] shall be less than or equal to 1. Although the value of hrd_op_set_idx[ i ] is required to be less than or equal to 1 in this version of this Specification, decoders shall allow other values of hrd_op_set_idx[ i ] in the range of 0 to 1023 to appear in the syntax.

cprms_present_flag[ i ] equal to 1 specifies that the HRD parameters that are common for all sub-layers are present in the i-th hrd_parameters() syntax structure in the video parameter set. cprms_present_flag[ i ] equal to 0 specifies that the HRD parameters that are common for all sub-layers are not present in the i-th hrd_parameters() syntax structure in the video parameter set and are derived to be the same as the ( i – 1 )-th hrd_parameters() syntax structure in the video parameter set. cprms_present_flag[ 0 ] is inferred to be equal to 1.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the video parameter set RBSP syntax structure. vps_extension_flag shall be equal to 0 in bitstreams conforming to this Specification. The value of 1 for vps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for vps_extension_flag in a video parameter set NAL unit.

vps_extension_data_flag may have any value. It shall not affect the conformance to profiles specified in this Specification.

#### Sequence parameter set RBSP semantics

video_parameter_set_id refers to the active video parameter set. The value of video_parameter_set_id shall be in the range of 0 to 15, inclusive.

sps_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each coded video sequence referring to the sequence parameter set. The value of sps_max_sub_layers_minus1 shall be in the range of 0 to 6, inclusive.

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for coded video sequences referring to the sequence parameter set. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag shall be equal to 1.

NOTE 38 – The syntax element sps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e. switching from decoding up to a specific TemporalId tIdN to decoding up to a TemporalId tIdM that is greater than tIdN, is always possible in the coded video sequence.

sequence_parameter_set_id identifies a sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause 6.2. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive.

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it is inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb or Cr) that each use the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value.

NOTE 39 – There is no dependency in decoding processes between the colour planes having different colour_plane_id values. For example, the decoding process of a monochrome picture with one value of colour_plane_id does not use any data from monochrome pictures having different values of colour_plane_id for inter prediction.

Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the sequence parameter set. conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present.

conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset specify the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When conformance_window_flag is equal to 0, the values of conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset are inferred to be equal to 0.

The variables WinUnitX and WinUnitY are derived as follows:

–	If ChromaArrayType is equal to 0, WinUnitX and WinUnitY are derived as:

WinUnitX = 1		(7-20)
WinUnitY = 1		(7-21)

–	Otherwise (ChromaArrayType is equal to 1, 2, or 3), WinUnitX and WinUnitY are derived as:

WinUnitX = SubWidthC		(7-22)
WinUnitY = SubHeightC		(7-23)

The conformance cropping window contains the luma samples with horizontal picture coordinates from WinUnitX * conf_win_left_offset to pic_width_in_luma_samples − ( WinUnitX * conf_win_right_offset + 1 ) and vertical picture coordinates from WinUnitY * conf_win_top_offset to pic_height_in_luma_samples − ( WinUnitY * conf_win_bottom_offset + 1 ), inclusive. It is a requirement of bitstream conformance that the value of conf_win_left_offset shall be in the range of 0 to ( pic_width_in_luma_samples / WinUnitX ) − ( conf_win_right_offset + 1 ), inclusive; and that the value of conf_win_top_offset shall be in the range of 0 to ( pic_height_in_luma_samples / WinUnitY ) − ( conf_win_bottom_offset + 1 ), inclusive.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE 40 – The conformance cropping window offset parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

bit_depth_luma_minus8 + 8 specifies the bit depth of the samples of the luma array and the value of the luma quantization parameter range offset QpBdOffsetY, as specified by

BitDepthY      = 8 + bit_depth_luma_minus8		(7-24)
QpBdOffsetY = 6 * bit_depth_luma_minus8		(7-25)

bit_depth_luma_minus8 shall be in the range of 0 to 6, inclusive.

bit_depth_chroma_minus8 + 8 specifies the bit depth of the samples of the chroma arrays and the value of the chroma quantization parameter range offset QpBdOffsetC, as specified by

BitDepthC      = 8 + bit_depth_chroma_minus8		(7-26)
QpBdOffsetC = 6 * bit_depth_chroma_minus8		(7-27)

bit_depth_chroma_minus8 shall be in the range of 0 to 6, inclusive.

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )	(7-28)

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

sps_sub_layer_ordering_info_present_flag equal to 1 specifies that sps_max_dec_pic_buffering[ i ], sps_max_num_reorder_pics[ i ], and sps_max_latency_increase[ i ] are present for sps_max_sub_layers_minus1 + 1 sub-layers, sps_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ], sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ], and sps_max_latency_increase[ sps_max_sub_layers_minus1 ] apply to all sub-layers.

- sps_max_dec_pic_buffering[ i ] specifies the maximum required size of the decoded picture buffer in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering[ i ] shall be in the range of 0 to MaxDpbSize (as specified in subclause Error: Reference source not found), inclusive. When i is greater than 0, sps_max_dec_pic_buffering[ i ] shall be greater than or equal to sps_max_dec_pic_buffering[ i − 1 ]. The value of sps_max_dec_pic_buffering[ i ] shall be less than or equal to vps_max_dec_pic_buffering[ i ] for each value of i. When sps_max_dec_pic_buffering[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1 due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ].

- sps_max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures preceding any picture in decoding order and succeeding that picture in output order when HighestTid is equal to i. The value of sps_max_num_reorder_pics[ i ] shall be in the range of 0 to sps_max_dec_pic_buffering[ i ], inclusive. When i is greater than 0, sps_max_num_reorder_pics[ i ] shall be greater than or equal to sps_max_num_reorder_pics[ i − 1 ]. The value of sps_max_num_reorder_pics[ i ] shall be less than or equal to vps_max_num_reorder_pics[ i ] for each value of i. When sps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1 due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ].

sps_max_latency_increase[ i ] not equal to 0 is used to compute the value of MaxLatencyPictures[ i ] as specified by setting MaxLatencyPictures[ i ] equal to sps_max_num_reorder_pics[ i ] + sps_max_latency_increase[ i ]. When sps_max_latency_increase[ i ] is not equal to 0, the value of MaxLatencyPictures[ i ] specifies the maximum number of pictures that can precede any picture in the coded video sequence in output order and follow that picture in decoding order when HighestTid is equal to i. When sps_max_latency_increase[ i ] is equal to 0, no corresponding limit is expressed. The value of sps_max_latency_increase[ i ] shall be in the range of 0 to 232 − 2, inclusive. The value of sps_max_latency_increase[ i ] shall be less than or equal to vps_max_latency_increase[ i ] for each value of i. When sps_max_latency_increase[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1 due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_latency_increase[ sps_max_sub_layers_minus1 ].

log2_min_luma_coding_block_size_minus3 specifies the minimum size of a luma coding block.

log2_diff_max_min_luma_coding_block_size specifies the difference between the maximum and minimum luma coding block size.

The variables Log2MinCbSizeY, Log2CtbSizeY, MinCbSizeY, CtbSizeY, PicWidthInMinCbsY, PicWidthInCtbsY, PicHeightInMinCbsY, PicHeightInCtbsY, PicSizeInMinCbsY, PicSizeInCtbsY, and PicSizeInSamplesY are set as follows.

Log2MinCbSizeY = log2_min_luma_coding_block_size_minus3 + 3	(7-29)
Log2CtbSizeY = Log2MinCbSizeY + 	log2_diff_max_min_luma_coding_block_size	(7-30)
MinCbSizeY = 1 << Log2MinCbSizeY		(7-31)
CtbSizeY = 1 << Log2CtbSizeY		(7-32)
PicWidthInMinCbsY = pic_width_in_luma_samples / MinCbSizeY	(7-33)
PicWidthInCtbsY = Ceil( pic_width_in_luma_samples ÷ CtbSizeY )	(7-34)
PicHeightInMinCbsY = pic_height_in_luma_samples / MinCbSizeY	(7-35)
PicHeightInCtbsY = Ceil( pic_height_in_luma_samples ÷ CtbSizeY )	(7-36)
PicSizeInMinCbsY = PicWidthInMinCbsY * PicHeightInMinCbsY	(7-37)
PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY	(7-38)
PicSizeInSamplesY = pic_width_in_luma_samples * pic_height_in_luma_samples	(7-39)

[Ed. (GJS): Variable names MinCbSizeY and CtbSizeY violate the editorial convention by being substrings of other variable names. (BB): Consider rename of Log2MinCbSizeY, Log2CtbSizeY and log2TrafoSize to MinCbLog2SizeY, CtbLog2SizeY, and trafoLog2Size (and other similar names, if any).]

The variables CtbWidthC and CtbHeightC, which specify the width and height, respectively, of the array for each chroma coding tree block, are derived as follows.

–	If chroma_format_idc is equal to 0 (monochrome) or separate_colour_plane_flag is equal to 1, CtbWidthC and CtbHeightC are both equal to 0.

–	Otherwise, CtbWidthC and CtbHeightC are derived as

CtbWidthC = CtbSizeY / SubWidthC		(7-40)
CtbHeightC = CtbSizeY / SubHeightC		(7-41)

log2_min_transform_block_size_minus2 specifies the minimum transform block size.

The variable Log2MinTrafoSize is set equal to log2_min_transform_block_size_minus2 + 2. The bitstream shall not contain data that result in Log2MinTrafoSize greater than or equal to Log2MinCbSizeY.

log2_diff_max_min_transform_block_size specifies the difference between the maximum and minimum transform block size.

The variable Log2MaxTrafoSize is set equal to log2_min_transform_block_size_minus2 + 2 + log2_diff_max_min_transform_block_size.

The bitstream shall not contain data that result in Log2MaxTrafoSize greater than Min( Log2CtbSizeY, 5 ).

The array ScanOrder[ log2BlockSize ][ scanIdx ][ sPos ][ sComp ] specifies the mapping of the scan position sPos, ranging from 0 to ( ( 1 << log2BlockSize ) * ( 1 << log2BlockSize ) ) − 1, inclusive, to horizontal and vertical components of the scan-order matrix. The array index scanIdx equal to 0 specifies an up-right diagonal scan order, scanIdx equal to 1 specifies a horizontal scan order, and scanIdx equal to 2 specifies a vertical scan order. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. The array ScanOrder is derived as follows.

For the variable log2BlockSize ranging from Min( 2, Log2MinTrafoSize − 2) to 3, inclusive, the scanning order array ScanOrder is derived as follows.

[Ed. (BB): If the signalling of scaling matrices is restricted to the allowed transform sizes, the 3 needs to be changed to Max( 2, Log2MaxTrafoSize − 2 ). ]

–	The up-right diagonal scan order array initialization process as specified in 6.5.3 is invoked with 1 << log2BlockSize as input and the output is assigned to ScanOrder[ log2BlockSize ][ 0 ].

–	The horizontal scan order array initialization process as specified in 6.5.4 is invoked with 1 << log2BlockSize as input and the output is assigned to ScanOrder[ log2BlockSize ][ 1 ].

–	The vertical scan order array initialization process as specified in 6.5.5 is invoked with 1 << log2BlockSize as input and the output is assigned to ScanOrder[ log2BlockSize ][ 2 ].

max_transform_hierarchy_depth_inter specifies the maximum hierarchy depth for transform units of coding units coded in inter prediction mode. The value of max_transform_hierarchy_depth_inter shall be in the range of 0 to Log2CtbSizeY − Log2MinTrafoSize, inclusive.

max_transform_hierarchy_depth_intra specifies the maximum hierarchy depth for transform blocks of coding blocks coded in intra prediction mode. The value of max_transform_hierarchy_depth_intra shall be in the range of 0 to Log2CtbSizeY − Log2MinTrafoSize, inclusive.

scaling_list_enable_flag equal to 1 specifies that a scaling list is used for the scaling process for transform coefficients. scaling_list_enable_flag equal to 0 specifies that scaling list is not used for the scaling process for transform coefficients.

sps_scaling_list_data_present_flag equal to 1 specifies that scaling list data are present in the sequence parameter set. sps_scaling_list_data_present_flag equal to 0 specifies that scaling list data are not present in the sequence parameter set. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0. When scaling_list_enable_flag is equal to 1 and sps_scaling_list_data_present_flag is equal to 0, the default scaling list data is used to derive the array ScalingFactor as described in the scaling list data semantics 7.4.6.

[Ed. (BB): Check whether this description is still OK when the transform_skip_enabled_flag dependency issue (#786) is fixed.]

amp_enabled_flag equal to 1 specifies that asymmetric motion partitions, i.e. PartMode equal to PART_2NxnU, PART_2NxnD, PART_nLx2N, or PART_nRx2N, may be used in coding tree blocks; amp_enabled_flag equal to 0 specifies that asymmetric motion partitions cannot be used in coding tree blocks.

sample_adaptive_offset_enabled_flag equal to 1 specifies that the sample adaptive offset process is applied to the reconstruced picture after the deblocking filter process. sample_adaptive_offset_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstruced picture after the deblocking filter process.

pcm_enabled_flag equal to 0 specifies that PCM data shall not be present in the video sequence.

NOTE 41 – When Log2MinCbSizeY is equal to 6, PCM data is not present in the video sequence even if pcm_enabled_flag is equal to 1. The maximum size of coding block with pcm_enabled_flag equal to 1 is restricted to be equal to or less than Min( Log2CtbSizeY, 5 ). Encoders are encouraged to use an appropriate combination of log2_min_luma_coding_block_size_minus3, log2_min_pcm_luma_coding_block_size_minus3, and log2_diff_max_min_pcm_luma_coding_block_size values when sending PCM data in the video sequence.

pcm_sample_bit_depth_luma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of luma component. The value of pcm_sample_bit_depth_luma_minus1 + 1 shall be less than or equal to the value of BitDepthY.

PCMBitDepthY = 1 + pcm_sample_bit_depth_luma_minus1	(7-42)

pcm_sample_bit_depth_chroma_minus1 + 1 specifies the number of bits used to represent each of PCM sample values of chroma components. The value of pcm_sample_bit_depth_chroma_minus1 + 1 shall be less than or equal to the value of BitDepthC.

PCMBitDepthC = 1 + pcm_sample_bit_depth_chroma_minus1	(7-43)

log2_min_pcm_luma_coding_block_size_minus3 + 3 specifies the minimum size of coding blocks with pcm_flag equal to 1.

The variable Log2MinIpcmCbSizeY is set equal to log2_min_pcm_luma_coding_block_size_minus3 + 3. The variable Log2MinIpcmCbSizeY shall be in the range of Log2MinCbSizeY to Min( Log2CtbSizeY, 5 ), inclusive.

log2_diff_max_min_pcm_luma_coding_block_size specifies the difference between the maximum and minimum size of coding blocks with pcm_flag equal to 1.

The variable Log2MaxIpcmCbSizeY is set equal to log2_min_pcm_luma_coding_block_size_minus3 + 3 + log2_diff_max_min_pcm_luma_coding_block_size. The variable Log2MaxIpcmCbSizeY shall be equal or less than Min( Log2CtbSizeY, 5 ).

pcm_loop_filter_disable_flag specifies whether the loop filter process is disabled on reconstructed samples in a coding unit with pcm_flag equal to 1. If the pcm_loop_filter_disable_flag value is equal to 1, deblocking filter and sample adaptive offset filter processes on the reconstructed samples in a coding unit with pcm_flag equal to 1 are disabled; otherwise if the pcm_loop_filter_disable_flag value is equal to 0, deblocking filter and sample adaptive offset filter processes on the reconstructed samples in a coding unit with pcm_flag equal to 1 are not disabled. When pcm_loop_filter_disable_flag is not present, it is inferred to be equal to 0.

[Ed. (WJ): select one expression – enabled_flag or disable_flag]

num_short_term_ref_pic_sets specifies the number of short-term reference picture sets that are specified in the sequence parameter set. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE 42 – A decoder must allocate space for a total number of num_short_term_ref_pic_sets + 1 short-term reference picture sets since a coded video sequence may contain a short-term reference picture set explicitly signalled in the slice headers of a current picture. An explicitly signalled short-term reference picture set will always have an index equal to num_short_term_ref_pic_sets in the list of short-term reference picture sets.

[Ed. constrain the semantics of the following syntax elements apply only to pictures with nuh_layer_id equal to 0.]

long_term_ref_pics_present_flag equal to 0 specifies that no long-term reference picture is used for inter prediction of any coded picture in the coded video sequence. long_term_ref_pics_present_flag equal to 1 specifies that long-term reference pictures may be used for inter prediction of one or more coded pictures in the coded video sequence.

num_long_term_ref_pics_sps specifies the number of candidate long-term reference pictures that are specified in the sequence parameter set. The value of num_long_term_ref_pics_sps shall be in the range of 0 to 32, inclusive.

lt_ref_pic_poc_lsb_sps[ i ] specifies the least significant bits of the picture order count of the i-th candidate long-term reference picture specified in the sequence parameter set. The number of bits used to represent lt_ref_pic_poc_lsb_sps[ i ] is equal to log2_max_pic_order_cnt_lsb_minus4 + 4.

used_by_curr_pic_lt_sps_flag[ i ] equal to 0 specifies that the i-th candidate long-term reference picture specified in the sequence parameter set is not used for reference by a picture that includes in its reference picture set the i-th candidate long-term reference picture specified in the sequence parameter set.

sps_temporal_mvp_enable_flag equal to 1 specifies that slice_temporal_mvp_enable_flag is present in the slice headers of pictures with IdrPicFlag equal to 0. sps_temporal_mvp_enable_flag equal to 0 specifies that slice_temporal_mvp_enable_flag is not present in slice headers and that temporal motion vector predictors is not used.

strong_intra_smoothing_enable_flag equal to 1 specifies that bi-linear interpolation is conditionally used in the filtering process as specified in subclause Error: Reference source not found. strong_intra_smoothing_enable_flag equal to 0 specifies that that the bi-linear interpolation is not used.

vui_parameters_present_flag equal to 1 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is present. vui_parameters_present_flag equal to 0 specifies that the vui_parameters( ) syntax structure as specified in Annex Error: Reference source not found is not present.

sps_extension_flag equal to 0 specifies that [Ed. list of syntax elements] are not present. sps_extension_flag equal to 1 specifies that [Ed. list of syntax elements] are present.

nonbase_layer_long_term_ref_pics_present_flag equal to 0 specifies that no long-term reference picture with nuh_layer_id greater than 0 is used for inter prediction of any coded picture in the coded video sequence. long_term_ref_pics_present_flag equal to 1 specifies that long-term reference pictures with nuh_layer_id greater than 0 may be used for inter prediction of one or more coded pictures in the coded video sequence.

nonbase_layer_long_term_ref_pics_present_flag, nonbase_long_term_ref_pics_present_flag, num_nonbase_long_term_ref_pics_sps, nonbase_lt_ref_pic_poc_lsb_sps[ i ] ,and used_by_curr_pic_nonbase_lt_sps_flag[ i ] are specified identically to long_term_ref_pics_present_flag, long_term_ref_pics_present_flag, num_long_term_ref_pics_sps, lt_ref_pic_poc_lsb_sps[ i ] ,and used_by_curr_pic_lt_sps_flag[ i ] but apply only to pictures with nuh_layer_id greater than 0.

lt_ref_layer_id_sps[ i ] specifies a nuh_layer_id value of the i-th candidate long-term reference picture with nuh_layer_id greater than 0 specified in the sequence parameter set. If not present, the value of lt_ref_layer_id_sps[ i ] is inferred to be equal to 0.

sps_extension2_flag equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the sequence parameter set RBSP syntax structure. sps_extension_flag shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for sps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all sps_extension_data_flag syntax elements that follow the value 1 for sps_extension_flag in a sequence parameter set NAL unit.

sps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore (remove from the bitstream and discard) all sps_extension_data_flag syntax elements.

#### Picture parameter set RBSP semantics

pic_parameter_set_id identifies the picture parameter set that is referred to in the slice segment header. The value of pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

seq_parameter_set_id refers to the active sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

dependent_slice_segments_enabled_flag equal to 1 specifies the presence of the syntax element dependent_slice_segment_flag in the slice segment headers for coded pictures referring to the picture parameter set. dependent_slice_segments_enabled_flag equal to 0 specifies the absence of the syntax element dependent_slice_segment_flag in the slice segment headers for coded pictures referring to the picture parameter set.

sign_data_hiding_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled.

cabac_init_present_flag equal to 1 specifies that cabac_init_flag is present in slice headers referring to the picture parameter set. cabac_init_present_flag equal to 0 specifies that cabac_init_flag is not present in slice headers referring to the picture parameter set.

num_ref_idx_l0_default_active_minus1 specifies the inferred value of num_ref_idx_l0_active_minus1 for P and B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l0_default_active_minus1 shall be in the range of 0 to 15, inclusive.

num_ref_idx_l1_default_active_minus1 specifies the inferred value of num_ref_idx_l1_active_minus1 for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_l1_default_active_minus1 shall be in the range of 0 to 15, inclusive.

pic_init_qp_minus26 specifies the initial value minus 26 of SliceQPY for each slice. The initial value is modified at the slice segment layer when a non-zero value of slice_qp_delta is decoded, and is modified further when a non-zero value of cu_qp_delta_abs is decoded at the coding unit layer. The value of pic_init_qp_minus26 shall be in the range of −(26 + QpBdOffsetY ) to +25, inclusive.

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring coding blocks coded using either intra or inter prediction modes. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction, in which case intra prediction only uses residual data and decoded samples from neighboring coding blocks coded using intra prediction modes.

transform_skip_enabled_flag equal to 1 specifies that transform_skip_flag may be present in the residual coding syntax. transform_skip_enabled_flag equal to 0 specifies that transform_skip_flag is not present in the residual coding syntax.

cu_qp_delta_enabled_flag equal to 1 specifies that the diff_cu_qp_delta_depth syntax element is present in the picture parameter set and that cu_qp_delta_abs can be present in transform unit syntax. cu_qp_delta_enabled_flag equal to 0 specifies that the diff_cu_qp_delta_depth syntax element is not present in the picture parameter set and that cu_qp_delta_abs is not present in transform unit syntax.

diff_cu_qp_delta_depth specifies the granularity for QPY values within a picture. The value of diff_cu_qp_delta_depth shall be in the range of 0 to log2_diff_max_min_luma_coding_block_size, inclusive.

The variable Log2MinCuQpDeltaSize, specifying the minimum luma coding block size of coding units that convey cu_qp_delta_abs and cu_qp_delta_sign, is derived as follows.

Log2MinCuQpDeltaSize = Log2CtbSizeY − diff_cu_qp_delta_depth	(7-44)

pic_cb_qp_offset and pic_cr_qp_offset specify offsets to the luma quantization parameter QP′Y used for deriving QP′Cb and QP′Cr, respectively. The values of pic_cb_qp_offset and pic_cr_qp_offset shall be in the range of −12 to +12, inclusive.

pic_slice_chroma_qp_offsets_present_flag equal to 1 indicates that the slice_cb_qp_offset and slice_cr_qp_offset syntax elements are present in the associated slice headers. pic_slice_chroma_qp_offsets_present_flag equal to 0 indicates that these syntax elements are not present in the associated slice headers.

weighted_pred_flag equal to 0 specifies that weighted prediction is not applied to P slices. weighted_pred_flag equal to 1 specifies that weighted prediction is applied to P slices.

weighted_bipred_flag equal to 0 specifies that the default weighted prediction is applied to B slices. weighted_bipred_flag equal to 1 specifies that weighted prediction is applied to B slices.

output_flag_present_flag equal to 1 indicates that the pic_output_flag syntax element is present in the associated slice headers. output_flag_present_flag equal to 0 indicates that the pic_output_flag syntax element is not present in the associated slice headers.

transquant_bypass_enable_flag equal to 1 specifies that cu_transquant_bypass_flag is present. transquant_bypass_enable_flag equal to 0 specifies that cu_transquant_bypass_flag is not present.

tiles_enabled_flag equal to 1 specifies that there is more than one tile in each picture referring to the picture parameter set. tiles_enabled_flag equal to 0 specifies that there is only one tile in each picture referring to the picture parameter set.

entropy_coding_sync_enabled_flag equal to 1 specifies that a specific synchronization process for context variables is invoked before decoding the first coding tree block of a row of coding tree blocks in each tile in each picture referring to the picture parameter set, and a specific memorization process for context variables is invoked after decoding two coding tree blocks of a row of coding tree blocks in each tile in each picture referring to the picture parameter set. entropy_coding_sync_enabled_flag equal to 0 specifies that no specific synchronization process for context variables is required to be invoked before decoding the first coding tree block of a row of coding tree blocks in each tile in each picture referring to the picture parameter set, and no specific memorization process for context variables is required to be invoked after decoding two coding tree blocks of a row of coding tree blocks in each tile in each picture referring to the picture parameter set. [Ed. (GJS): Consider using a different word than "memorization".]

It's a requirement of bitstream conformance that the values of tiles_enabled_flag and entropy_coding_sync_enabled_flag shall be the same, respectively, for all picture parameter sets that are activated within a coded video sequence.

When entropy_coding_sync_enabled_flag is equal to 1 and the first coding tree block in a slice is not the first coding tree block of a row of coding tree blocks in a tile, it is a requirement of bitstream conformance that the last coding tree block in the slice shall belong to the same row of coding tree blocks as the first coding tree block in the slice.

When entropy_coding_sync_enabled_flag is equal to 1 and the first coding tree block in a slice segment is not the first coding tree block of a row of coding tree blocks in a tile, it is a requirement of bitstream conformance that the last coding tree block in the slice segment shall belong to the same row of coding tree blocks as the first coding tree block in the slice segment.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture. num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of num_tile_columns_minus1 is inferred to be equal to 0.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture. num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of num_tile_rows_minus1 is inferred to be equal to 0.

When tiles_enable_flag is equal to 1 and num_tile_columns_minus1 is equal to 0, num_tile_rows_minus1 shall not be equal to 0.

uniform_spacing_flag equal to 1 specifies that column boundaries and likewise row boundaries are distributed uniformly across the picture. uniform_spacing_flag equal to 0 specifies that column boundaries and likewise row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements column_width_minus1[ i ] and row_height_minus1[ i ].

column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of coding tree blocks.

row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of coding tree blocks.

The following variables are derived by invoking the coding tree block raster and tile scanning conversion process as specified in subclause 6.5.1.

–	The list CtbAddrRStoTS[ ctbAddrRS ] for ctbAddrRS ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in CTB raster scan of a picture to a CTB address in tile scan,

–	the list CtbAddrTStoRS[ ctbAddrTS ] for ctbAddrTS ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a CTB address in CTB raster scan of a picture,

–	the list TileId[ ctbAddrTS ] for ctbAddrTS ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in tile scan to a tile ID,

–	the list ColumnWidthInLumaSamples[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, specifying the width of the i-th tile column in units of luma samples,

–	the list RowHeightInLumaSamples[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, specifying the height of the j-th tile row in units of luma samples.

The values of ColumnWidthInLumaSamples[ i ] for i ranging from 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] for j ranging from 0 to num_tile_rows_minus1, inclusive, shall all be greater than 0.

The array MinTbAddrZS with elements MinTbAddrZS[ x ][ y ] for x ranging from 0 to ( PicWidthInCtbsY << ( Log2CtbSizeY − Log2MinTrafoSize ) ) − 1, inclusive, and y ranging from 0 to ( PicHeightInCtbsY << ( Log2CtbSizeY − Log2MinTrafoSize ) ) − 1, inclusive, specifying the conversion from a location ( x, y ) in units of minimum transform blocks to a transform block address in z-scan order, is derived by invoking the z-scan order array initialization process as specified in subclause 6.5.2.

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across tile boundaries. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries. The in-loop filtering operations include the deblocking filter and sample adaptive offset filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

loop_filter_across_slices_enabled_flag equal to 1 specifies that the slice_loop_filter_across_slices_enabled_flag determines whether in-loop filtering operations are performed across left and upper boundaries of the current slice; otherwise, the in-loop filtering operations are not applied across left and upper boundaries of the current slice. The in-loop filtering operations include the deblocking filter and sample adaptive offset filter.

deblocking_filter_control_present_flag equal to 1 specifies the presence of deblocking filter control syntax elements in the picture parameter set and in the slice headers for pictures referring to the picture parameter set. deblocking_filter_control_present_flag equal to 0 specifies the absence of deblocking filter control syntax elements in the picture parameter set and in the slice headers for pictures referring to the picture parameter set.

deblocking_filter_override_enabled_flag equal to 1 specifies the presence of deblocking_filter_overriding_flag in the slice headers for pictures referring to the picture parameter set. deblocking_filter_override_enabled_flag equal to 0 specifies the absence of deblocking_filter_overriding_flag in the slice headers for pictures referring to the picture parameter set. When not present, the value of deblocking_filter_override_enabled_flag is inferred to be equal to 0.

pic_disable_deblocking_filter_flag equal to 1 specifies that the operation of deblocking filter shall not be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. pic_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be applied for pictures referring to the picture parameter set when deblocking_filter_override_enabled_flag is equal to 0. When not present, the value of pic_disable_deblocking_filter_flag is inferred to be equal to 0.

beta_offset_div2 and tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied for pictures referring to the picture parameter set unless the default deblocking parameter offsets are overriden by the deblocking parameter offsets present in the slice headers for pictures referring to the picture parameter set. The values of beta_offset_div2 and tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the value of beta_offset_div2 and tc_offset_div2 is inferred to be equal to 0.

pic_scaling_list_data_present_flag equal to 1 specifies that parameters are present in the picture parameter set to modify the scaling lists specified in the active sequence parameter set. pic_scaling_list_data_present_flag equal to 0 specifies that the scaling lists used for the pictures referring to the picture parameter set is inferred to be equal to those specified by the active sequence parameter set. When scaling_list_enable_flag is equal to 0, the value of pic_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enable_flag is equal to 1 and pic_scaling_list_data_present_flag is equal to 0, the default scaling list data is used to derive the array ScalingFactor as described in the scaling list data semantics 7.4.6.

[Ed. (BB): Check whether this description is still OK when the transform_skip_enabled_flag dependency issue (#786) is fixed.]

lists_modification_present_flag equal to 1 specifies that the syntax structure ref_pic_lists_modification( ) is present in the slice segment header. lists_modification_present_flag equal to 0 specifies that the syntax structure ref_pic_lists_modification( ) is not present in the slice segment header.

log2_parallel_merge_level_minus2 specifies the parallel processing level of merge/skip mode. The value of log2_parallel_merge_level_minus2 shall be in the range of 0 to log2_min_luma_coding_block_size_minus3 + 1 + log2_diff_max_min_luma_coding_block_size, inclusive.

num_extra_slice_header_bits equal to 0 specifies that no extra slice header bits are present in the slice header RBSP for coded pictures referring to the picture parameter set. num_extra_slice_header_bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for num_extra_slice_header_bits are reserved for future use by ITU-T | ISO/IEC. However, decoders shall allow num_extra_slice_header_bits to have any value.

slice_segment_header_extension_present_flag equal to 0 specifies that no slice segment header extension syntax elements are present in the slice segment headers for coded pictures referring to the picture parameter set. slice_segment_header_extension_present_flag shall be equal to 0 in bitstreams conforming to this Specification. The value of 1 for slice_segment_header_extension_present_flag is reserved for future use by ITU-T | ISO/IEC.

pps_extension_flag equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the picture parameter set RBSP syntax structure. pps_extension_flag shall be equal to 0 in bitstreams conforming to this Specification. The value of 1 for pps_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follow the value 1 for pps_extension_flag in a picture parameter set NAL unit.

pps_extension_data_flag may have any value. Its value does not affect decoder conformance to profiles specified in this Specification.

#### Supplemental enhancement information RBSP semantics

Supplemental Enhancement Information (SEI) contains information that is not necessary to decode the samples of coded pictures from VCL NAL units. An SEI RBSP contains one or more SEI messages.

#### Access unit delimiter RBSP semantics

The access unit delimiter may be used to indicate the type of slices present in a coded picture and to simplify the detection of the boundary between access units. There is no normative decoding process associated with the access unit delimiter.

pic_type indicates that the slice_type values for all slices of the coded picture are members of the set listed in Table 7-4 for the given value of pic_type.

Table 7-4 – Interpretation of pic_type

| pic_type | slice_type values that may be present in the coded picture |
| --- | --- |
| 0 | I |
| 1 | P, I |
| 2 | B, P, I |

#### End of sequence RBSP semantics

When the next subsequent NAL unit in decoding order is not an end of bitstream NAL unit, the end of sequence RBSP specifies that the next subsequent access unit in the bitstream in decoding order (if any) is an IDR or BLA access unit. The syntax content of the SODB and RBSP for the end of sequence RBSP are empty.

#### End of bitstream RBSP semantics

The end of bitstream RBSP indicates that no additional NAL units are present in the bitstream that are subsequent to the end of bitstream RBSP in decoding order. The syntax content of the SODB and RBSP for the end of bitstream RBSP are empty.

NOTE – When an elementary stream contains more than one bitstream, the last NAL unit of the last access unit of a bitstream contains an end of bitstream NAL unit and the first access unit of the subsequent bitstream must be a RAP access unit. This RAP access unit may be a CRA, BLA, or IDR access unit.

#### Filler data RBSP semantics

The filler data RBSP contains bytes whose value shall be equal to 0xFF. No normative decoding process is specified for a filler data RBSP.

ff_byte is a byte equal to 0xFF.

#### Slice segment layer RBSP semantics

The slice segment layer RBSP consists of a slice segment header and slice segment data.

#### RBSP slice segment trailing bits semantics

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInVclNALunits be the sum of the values of NumBytesInNALunit for all VCL NAL units of a coded picture.

Let BinCountsInNALunits be the number of times that the parsing process function DecodeBin( ), specified in subclause Error: Reference source not found, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as

RawMinCuBits = MinCbSizeY * MinCbSizeY * ( BitDepthY + BitDepthC / 2 )	(7-45)

The value of BinCountsInNALunits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNALunits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the slice segment NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNALunits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

#### RBSP trailing bits semantics

rbsp_stop_one_bit shall be equal to 1.

rbsp_alignment_zero_bit shall be equal to 0.

#### Byte alignment semantics

alignment_bit_equal_to_one shall be equal to 1.

alignment_bit_equal_to_zero shall be equal to 0.

### Profile, tier and level semantics

general_profile_space specifies the context for the interpretation of general_profile_idc and general_profile_combatibility_flag[ i ] for all values of i in the range of 0 to 31, inclusive. The value of general_profile_space shall be equal to 0 in bitstreams conforming to this Specification. Other values for general_profile_space are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the coded video sequence if general_profile_space is not equal to 0. [Ed. (GJS): Ignore the CVS or ignore the VPS/SPS? Think about this.]

general_tier_flag specifies the tier context for the interpretation of general_level_idc as specified in Annex Error: Reference source not found.

general_profile_idc, when general_profile_space is equal to 0, indicates a profile to which the coded video sequence conforms as specified in Annex Error: Reference source not found. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex Error: Reference source not found. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

[Ed. (DS): We might prefer not to use the general_profile_idc value zero, or reserve it to mean "no profile signalled, bitstream is unconstrained"; this gives us one spare bit in the profile compatibility flags array.]

general_profile_compatibility_flag[ i ] equal to 1, when general_profile_space is equal to 0, indicates that the coded video sequence conforms to the profile indicated by general_profile_idc equal to i as specified in Annex Error: Reference source not found. When general_profile_space is equal to 0, general_profile_compatibility_flag[ general_profile_idc ] shall be equal to 1. The value of general_profile_compatibility_flag[ i ] shall be equal to 0 for any value of i that is not specified as an allowed value of general_profile_idc in Annex Error: Reference source not found.

general_reserved_zero_16bits shall be equal to 0 in bitstreams conforming to this Specification. Other values for general_reserved_zero_16bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_16bits.

general_level_idc indicates a level to which the coded video sequence conforms as specified in Annex Error: Reference source not found.

NOTE 43 – A greater value of general_level_idc indicates a higher level. The maximum level signalled in the video parameter set for a coded video sequence may be higher than the level signalled in the sequence parameter set for the same coded video sequence.

NOTE 44 – For bitstreams compatible with more than one profile, the value of general_profile_idc should be set to the "best viewed as" profile.

NOTE 45 – The general_reserved_zero_16bits can be used to indicate the respect of further constraints on the bitstream (e.g. that a selected tool, permitted by the profiles signalled, is nonetheless not used). These flags are ideally profile independent, but unlike general_level_idc, it is allowed to be contextual on general_profile_idc.

sub_layer_profile_present_flag[ i ] equal to 1, when ProfilePresentFlag is equal to 1, specifies that profile information is present in the profile_tier_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i. sub_layer_profile_present_flag[ i ] equal to 0 specifies that profile information is not present in the profile_tier_level( ) syntax structure for the representations of the sub-layer with TemporalId equal to i. When not present, the value of sub_layer_profile_present_flag[ i ] is inferred to be equal to 0.

sub_layer_level_present_flag[ i ] equal to 1 specifies that level information is present in the profile_tier_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i. sub_layer_level_present_flag[ i ] equal to 0 specifies that level information is not present in the profile_tier_level( ) syntax structure for the representation of the sub-layer with TemporalId equal to i.

sub_layer_profile_space[ i ], sub_layer_tier_flag[ i ], sub_layer_profile_idc[ i ], sub_layer_profile_compatibility_flag[ i ][ j ], sub_layer_reserved_zero_16bits[ i ], and sub_layer_level_idc[ i ] have the same semantics as general_profile_space, general_tier_flag, general_profile_idc, general_profile_compatibility_flag[ j ], general_reserved_zero_16bits, and general_level_idc, respectively, but apply to the representation of the sub-layer with TemporalId equal to i.

When not present, the value of sub_layer_tier_flag[ i ] is inferred to be equal to 0.

### Bit rate and picture rate information semantics

bit_rate_info_present_flag[ i ] equal to 1 specifies that the bit rate information for the i-th sub-layer is present. bit_rate_info_present_flag[ i ] equal to 0 specifies that the bit rate information for the i-th sub-layer is not present.

pic_rate_info_present_flag[ i ] equal to 1 specifies that picture rate information for the i-th sub-layer is present. pic_rate_info_present_flag[ i ] equal to 0 specifies that picture rate information for the i-th sub-layer is not present.

avg_bit_rate[ i ] indicates the average bit rate of the representation of the i-th sub-layer. The average bit rate for the representation of the i-th sub-layer in bits per second is given by BitRateBPS( avg_bit_rate[ i ] ) with the function BitRateBPS( ) being specified by

BitRateBPS( x ) = ( x & ( 214 − 1 ) ) * 10( 2 + ( x >> 14 ) )	(7-46)

The average bit rate is derived according to the access unit removal time specified in Annex Error: Reference source not found of this Specification. In the following, bTotal is the number of bits in all NAL units of the representation of the i-th sub-layer, t1 is the removal time (in seconds) of the first access unit to which the video parameter set applies, and t2 is the removal time (in seconds) of the last access unit (in decoding order) to which the video parameter set applies.

With x specifying the value of avg_bit_rate[ i ], the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

( x & ( 214 − 1 ) )  = =  Round( bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x >> 14 ) ) ) )	(7-47)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

( x & ( 214 − 1 ) )  = =  0		(7-48)

max_bit_rate_layer[ i ] indicates an upper bound for the bit rate of the representation of the i-th sub-layer in any one-second time window of access unit removal time as specified in Annex Error: Reference source not found. The upper bound for the bit rate in bits per second is given by BitRateBPS( max_bit_rate_layer[ i ] ). The bit rate values are derived according to the access unit removal time specified in Annex Error: Reference source not found of this Specification. In the following, t1 is any point in time (in seconds), t2 is set equal to t1 + 1 ÷ 100, and bTotal is the number of bits in all NAL units of access units with a removal time greater than or equal to t1 and less than t2. With x specifying the value of max_bit_rate_layer[ i ], the following condition shall be obeyed for all values of t1:

( x & ( 214 − 1 ) )  >=  bTotal ÷ ( ( t2 − t1 ) * 10( 2 + ( x >> 14 ) ) )	(7-49)

constant_pic_rate_idc[ i ] indicates whether the picture rate of the representation of the i-th sub-layer is constant. In the following, a temporal segment tSeg is any set of two or more consecutive access units, in decoding order, of the representation of the i-th sub-layer, fTotal( tSeg ) is the number of pictures in the temporal segment tSeg, t1( tSeg ) is the removal time (in seconds) of the first access unit (in decoding order) of the temporal segment tSeg, t2( tSeg ) is the removal time (in seconds) of the last access unit (in decoding order) of the temporal segment tSeg, and avgFR( tSeg ) is the average frame rate in the temporal segment tSeg, which is given by:

avgFR( tSeg)  = =  Round( fTotal( tSeg ) * 256 ÷ ( t2( tSeg ) − t1( tSeg ) ) )	(7-50)

If the representation of the i-th sub-layer only contains one access unit or the value of avgFR( tSeg ) is constant over all temporal segments of the representation of the i-th sub-layer, the picture rate is constant; otherwise, the picture rate is not constant.

constant_pic_rate_idc[ i ] equal to 0 indicates that the picture rate of the representation of the i-th sub-layer is not constant. constant_pic_rate_idc[ i ] equal to 1 indicates that the picture rate of the representation of the i-th sub-layer is constant. constant_pic_rate_idc[ i ] equal to 2 indicates that the picture rate of the representation of the i-th sub-layer may or may not be constant. [Ed. (GJS): If equal to 1, is the bitstream non-conforming if the picture rate happens to turn out to be constant?] [Ed. (GJS): If there are only two pictures, is the picture rate constant?],  The value of constant_pic_rate_idc[ i ] shall be in the range of 0 to 2, inclusive.

avg_pic_rate[ i ] indicates the average picture rate, in units of picture per 256 seconds, of representation of the i-th sub-layer. With fTotal being the number of pictures in the representation of the i-th sub-layer, t1 being the removal time (in seconds) of the first access unit to which the video parameter set applies, and t2 being the removal time (in seconds) of the last access unit (in decoding order) to which the video parameter set applies, the following applies:

–	If t1 is not equal to t2, the following condition shall be true:

avg_pic_rate[ i ]  = =  Round( fTotal * 256 ÷ ( t2 − t1 ) )	(7-51)

–	Otherwise (t1 is equal to t2), the following condition shall be true:

avg_pic_rate[ i ]  = =  0		(7-52)

### Operation point set semantics

The operation_point_set( opsIdx ) syntax structure specifies the OpLayerIdSet of the opsIdx-th operation point set specified by the video parameter set.

layer_id_included_flag[ opsIdx ][ i ] equal to 0 specifies that the value of nuh_reserved_zero_6bits equal to i is not included in the OpLayerIdSet of the opsIdx-th operation point set specified by the video parameter set. layer_id_included_flag[ opsIdx ][ i ] equal to 1 specifies that the value of nuh_reserved_zero_6bits equal to i is included in the OpLayerIdSet of the opsIdx-th operation point set specified by the video parameter set. The sum of all layer_id_included_flag[ opsIdx ][ i ] values for i from 0 to vps_max_nuh_reserved_zero_layer_id, inclusive, shall be in the range of 1 to vps_reserved_zero_6bits + 1, inclusive.

The variable numLayerIdsMinus1[ opsIdx ] and the variables layerId[ opsIdx ][ i ], for i in the range of 0 to numLayerIdsMinus1[ opsIdx ], inclusive, are derived as follows.

j = 0
for( i = 0; i <= vps_max_nuh_reserved_zero_layer_id; i++ )
	if( layer_id_included_flag[ opsIdx ][ i ] )		(7-53)
		layerId[ opsIdx ][ j++ ] = i
numLayerIdsMinus1[ opsIdx ] = j − 1

The values of numLayerIdsMinus1[ 0 ] and layerId[ 0 ][ 0 ] are defined as follows:

NumOpLayerIdsMinus1[ 0 ] = 0		(7-54)
OpLayerId[ 0 ][ 0 ] = 0			(7-55)

Any two sets layerId[ opsIdx1 ] and layerId[ opsIdx2 ], where opsIdx1 is not equal to opsIdx2, shall not include the same set of nuh_reserved_zero_6bits values.

The OpLayerIdSet of the opsIdx-th operation point set specified in the video parameter set is set to the set of nuh_reserved_zero_6bits values equal to OpLayerId[ opsIdx ][ i ], for i in the range of 0 to numLayerIdsMinus1[ opsIdx ], inclusive.

### Scaling list data semantics

scaling_list_pred_mode_flag[ sizeId ][ matrixId ] equal to 0 specifies that the values of the scaling list are the same as the values of a reference scaling list. The reference scaling list is specified by scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ]. scaling_list_pred_mode_flag[ sizeId ][ matrixId ] equal to 1 specifies that the values of the scaling list are explicitly signalled.

scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] specifies the reference scaling list used to derive ScalingList[ sizeId ][ matrixId ] as follows.

–	If scaling_list_pred_matrix_id_delta is equal to 0, the scaling list is inferred from the default scaling list ScalingList[ sizeId ][ matrixId ][ i ] as specified in Table 7-7 and Table 7-8 for i = 0..Min( 64, ( 1 << ( 4 + ( sizeId << 1) ) ) ).

–	Otherwise, the scaling list is inferred from the reference scaling list as follows.

refMatrixId = matrixId − scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ]	(7-56)

ScalingList[ sizeId ][ matrixId ][ i ] = ScalingList[ sizeId ][ refMatrixId ][ i ] 
with i =0 .. Min( 64, ( 1 << ( 4 + ( sizeId << 1) ) ) )	(7-57)

The value of scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] shall be in the range of 0 to matrixId, inclusive.

Table 7-5 – Specification of sizeId

| - Size of quantization matrix | - sizeId |
| --- | --- |
| - 4x4 | - 0 |
| - 8x8 | - 1 |
| - 16x16 | - 2 |
| - 32x32 | - 3 |

Table 7-6 – Specification of matrixId according to sizeId, prediction mode and colour component

| - sizeId | - PredMode | - cIdx (colour component) | - matrixId |
| --- | --- | --- | --- |
| - 0, 1, 2 | - MODE_INTRA | - 0 (Y) | - 0 |
| - 0, 1, 2 | - MODE_INTRA | - 1 (Cb) | - 1 |
| - 0, 1, 2 | - MODE_INTRA | - 2 (Cr) | - 2 |
| - 0, 1, 2 | - MODE_INTER | - 0 (Y) | - 3 |
| - 0, 1, 2 | - MODE_INTER | - 1 (Cb) | - 4 |
| - 0, 1, 2 | - MODE_INTER | - 2 (Cr) | - 5 |
| - 3 | - MODE_INTRA | - 0 (Y) | - 0 |
| - 3 | - MODE_INTER | - 0 (Y) | - 1 |

scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] plus 8 specifies the DC value of the scaling list for 16x16 size when sizeId is equal to 2 and specifies the DC value of the scaling list for 32x32 size when sizeId is equal to 3. The value of scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] shall be in the range of −7 to 247, inclusive. When scaling_list_dc_coef_minus8 is not present, it is inferred to be equal to 8.

scaling_list_delta_coef specifies the difference between the current matrix coefficient ScalingList[ sizeId ][ matrixId ][ i ] and the previous matrix coefficient ScalingList[ sizeId ][ matrixId ][ i − 1 ], when scaling_list_pred_mode_flag[ sizeId ][ matrixId ] is equal to 1. The value of scaling_list_delta_coef shall be in the range of −128 to 127, inclusive. The value of ScalingList[ sizeId ][ matrixId ][ i ] shall be greater than 0.

Table 7-7 – Specification of default values of ScalingList[ 0 ][ matrixId ][ i ] with i = 0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 |
| - ScalingList[0][3..5][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 |

Table 7-8 – Specification of default values of ScalingList[ 1..3 ][ matrixId ][ i ] with i = 0..63

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 16 | - 17 | - 16 | - 17 | - 18 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 17 | - 17 | - 18 |
| - i − 16 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 17 | - 18 | - 18 | - 17 | - 18 | - 21 | - 19 | - 20 | - 21 | - 20 | - 19 | - 21 | - 24 | - 22 | - 22 | - 24 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 18 | - 18 | - 18 | - 18 | - 18 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 24 | - 24 | - 24 | - 24 |
| - i − 32 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 24 | - 22 | - 22 | - 24 | - 25 | - 25 | - 27 | - 30 | - 27 | - 25 | - 25 | - 29 | - 31 | - 35 | - 35 | - 31 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 24 | - 24 | - 24 | - 24 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 28 | - 28 | - 28 | - 28 | - 28 |
| - i − 48 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 29 | - 36 | - 41 | - 44 | - 41 | - 36 | - 47 | - 54 | - 54 | - 47 | - 65 | - 70 | - 65 | - 88 | - 88 | - 115 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 28 | - 33 | - 33 | - 33 | - 33 | - 33 | - 41 | - 41 | - 41 | - 41 | - 54 | - 54 | - 54 | - 71 | - 71 | - 91 |

The four-dimensional array ScalingFactor[ sizeId ][ matrixId ][ x ][ y ], with x, y = 0..( 1 << ( 2 + sizeId ) ) − 1, specifies the array of scaling factors according to the variables sizeId specified in Table 7-5 and matrixId specified in Table 7-6.

The elements of the quantization matrix of size 4x4, ScalingFactor[ 0 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 0 ][ matrixId ][ x ][ y ] = ScalingList[ 0 ][ matrixId ][ i ]	(7-58)
with i = 0..15 and matrixId = 0..5

where x=ScanOrder[ 2 ][ 0 ][ i ][ 0 ] and y=ScanOrder[ 2 ][ 0 ][ i ][ 1 ]

The elements of the quantization matrix of size 8x8, ScalingFactor[ 1 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 1 ][ matrixId ][ x ][ y ] = ScalingList[ 1 ][ matrixId ][ i ]	(7-59)
with i = 0..63 and matrixId = 0..5

where x=ScanOrder[ 3 ][ 0 ][ i ][ 0 ] and y=ScanOrder[ 3 ][ 0 ][ i ][ 1 ]

The elements of the quantization matrix of size 16x16, ScalingFactor[ 2 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 2 ][ matrixId ][ x *2 + k ][ y * 2 + j ] = ScalingList[ 2 ][ matrixId ][ i ]	(7-60)
with i = 0..63, j = 0..1, k = 0..1 and matrixId = 0..5

where x=ScanOrder[ 3 ][ 0 ][ i ][ 0 ] and y=ScanOrder[ 3 ][ 0 ][ i ][ 1 ]

ScalingFactor[ 2 ][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 0 ][ matrixId ] + 8	(7-61)
with matrixId = 0..5

The elements of the quantization matrix of size 32x32, ScalingFactor[ 3 ][ matrixId ][ ][ ], are derived as follows:

ScalingFactor[ 3 ][ matrixId ][ x * 4 + k ][ y * 4 + j ] = ScalingList[ 3 ][ matrixId ][ i ]	(7-62)
with i = 0..63, j = 0..3, k = 0..3 and matrixId = 0..1

where x=ScanOrder[ 3 ][ 0 ][ i ][ 0 ] and y=ScanOrder[ 3 ][ 0 ][ i ][ 1 ]

ScalingFactor[ 3 ][ matrixId ][ 0 ][ 0 ] = scaling_list_dc_coef_minus8[ 1 ][ matrixId ] + 8	(7-63)
with matrixId = 0..1

### Supplemental enhancement information message semantics

Each SEI message consists of the variables specifying the type payloadType and size payloadSize of the SEI message payload. SEI message payloads are specified in Annex Error: Reference source not found. The derived SEI message payload size payloadSize is specified in bytes and shall be equal to the number of RBSP bytes in the SEI message payload.

NOTE – The NAL unit byte sequence containing the SEI message might include one or more emulation prevention bytes (represented by emulation_prevention_three_byte syntax elements). Since the payload size of an SEI message is specified in RBSP bytes, the quantity of emulation prevention bytes is not included in the size payloadSize of an SEI payload.

ff_byte is a byte equal to 0xFF identifying a need for a longer representation of the syntax structure that it is used within.

last_payload_type_byte is the last byte of the payload type of an SEI message.

last_payload_size_byte is the last byte of the payload size of an SEI message.

### Slice segment header semantics

#### General slice segment header semantics

When present, the value of the slice segment header syntax elements pic_parameter_set_id, pic_output_flag, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_sps_flag, short_term_ref_pic_set_idx, num_long_term_sps, num_long_term_pics, and slice_temporal_mvp_enable_flag shall be the same in all slice segment headers of a coded picture. When present, the value of the slice segment header syntax elements lt_idx_sps[ i ], poc_lsb_lt[ i ], used_by_curr_pic_lt_flag[ i ], delta_poc_msb_present_flag[ i ], and delta_poc_msb_cycle_lt[ i ] shall be the same in all slice segment headers of a coded picture for each possible value of i.

first_slice_segment_in_pic_flag equal to 1 specifies that the slice segment is the first slice segment of the picture in decoding order. first_slice_segment_in_pic_flag equal to 0 specifies that the slice segment is not the first slice segment of the picture in decoding order..

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture. See Annex Error: Reference source not found. When the IDR or BLA picture is the first picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the picture in the bitstream and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ HighestTid ] derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or sps_max_dec_pic_buffering[ HighestTid ] derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag. [Ed. (GJS): Check why this had not been moved previously.] [Ed. (GJS): Use a variable here.]

pic_parameter_set_id specifies the picture parameter set in use. The value of pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

dependent_slice_segment_flag equal to 1 specifies that the value of each slice segment header syntax element that is not present is inferred to be equal to the value of the corresponding slice segment header syntax element in the slice header. When not present, the value of dependent_slice_segment_flag is inferred to be equal to 0.

The variable SliceAddrRS is derived as follows.

- If dependent_slice_segment_flag is equal to 0, SliceAddrRS is set equal to slice_segment_address.

- Otherwise, SliceAddrRS is set equal to SliceAddrRS of the preceding slice segment containing the coding tree block for which the coding tree block address is ctbAddrTStoRS[ ctbAddrRStoTS[ slice_segment_address ] − 1 ].

slice_segment_address specifies the address of the first coding tree block in the slice segment, in coding tree block raster scan of a picture. The length of the slice_segment_address syntax element is Ceil( Log2( PicSizeInCtbsY ) ) bits. The value of slice_segment_address shall be in the range of 1 to PicSizeInCtbsY − 1, inclusive and the value of slice_segment_address shall not be equal to the value of slice_segment_address of any other coded slice segment NAL unit of the same coded picture. When slice_segment_address is not present, it is inferred to be equal to 0.

The variable CtbAddrInRS, specifying a coding tree block address in coding tree block raster scan of a picture, is set equal to slice_segment_address. The variable CtbAddrInTS, specifying a coding tree block address in tile scan, is set equal to CtbAddrRStoTS[ CtbAddrInRS ]. The variable CuQpDelta, specifying the difference between a luma quantization parameter for the coding unit containing cu_qp_delta_abs and its prediction, is set equal to 0.

slice_reserved_undetermined_flag[ i ] has semantics and values that are reserved for future specification by ITU-T | ISO/IEC. Decoders shall ignore the presence and value of slice_reserved_undetermined_flag[ i ].

slice_type specifies the coding type of the slice according to Table 7-9.

Table 7-9 – Name association to slice_type

| - slice_type | - Name of slice_type |
| --- | --- |
| - 0 | - B (B slice) |
| - 1 | - P (P slice) |
| - 2 | - I (I slice) |

When nal_unit_type has a value in the range of 16 to 23, inclusive (RAP picture), slice_type shall be equal to 2.

When sps_max_dec_pic_buffering[ TemporalId ] is equal to 0, slice_type shall be equal to 2.

pic_output_flag affects the decoded picture output and removal processes as specified in Annex Error: Reference source not found. When pic_output_flag is not present, it is inferred to be equal to 1.

colour_plane_id specifies the colour plane associated with the current slice RBSP when separate_colour_plane_flag is equal to 1. The value of colour_plane_id shall be in the range of 0 to 2, inclusive. colour_plane_id equal to 0, 1, and 2 correspond to the Y, Cb, and Cr planes, respectively.

NOTE 46 – There is no dependency between the decoding processes of pictures having different values of colour_plane_id.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb is inferred to be equal to 0, except as specified in subclause 8.3.3.1. [Ed. (GJS): This syntax element name violates the convention against having a syntax element name that is a sub-string of another syntax element name.]

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term reference picture set of the current picture is created using syntax elements in the active sequence parameter set. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term reference picture set of the current picture is created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active sequence parameter set that is used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx is represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active sequence parameter set.

The variable StRpsIdx is derived as follows:

–	If short_term_ref_pic_set_sps_flag is equal to 1, StRpsIdx is set equal to short_term_ref_pic_set_idx.

–	Otherwise, StRpsIdx is set equal to num_short_term_ref_pic_sets.

num_long_term_sps specifies the number of candidate long-term reference pictures specified in the active sequence parameter set that are included in the long-term reference picture set of the current picture. The value of num_long_term_sps shall be in the range of 0 to Min( num_long_term_ref_pics_sps, sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] −NumNegativePics[ StRpsIdx ] − NumPositivePics[ StRpsIdx ] ), inclusive. When not present, the value of num_long_term_sps is inferred to be equal to 0.

num_long_term_pics specifies the number of candidate long-term reference pictures specified in the slice header that are included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] − NumNegativePics[ StRpsIdx ] − NumPositivePics[ StRpsIdx ] − num_long_term_sps, inclusive. When not present, the value of num_long_term_pics is inferred to be equal to 0.

lt_idx_sps[ i ] specifies an index into the list of candidate long-term reference pictures specified in the active sequence parameter set for identification of the picture that is included in the long-term reference pictur set of the current picture. The value of lt_idx_sps[ i ] shall be in the range of 0 to num_long_term_ref_pics_sps − 1, inclusive. The number of bits used to represent lt_idx_sps[ i ] is equal to Ceil( Log2( num_long_term_ref_pics_sps ) ).

poc_lsb_lt[ i ] specifies the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. The length of the poc_lsb_lt[ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th candidate long-term reference picture to be included in the long-term reference picture set of the current picture is not used for reference by the current picture.

The variables PocLsbLt[ i ] and UsedByCurrPicLt[ i ] are derived as follows:

–	If i is less than num_long_term_sps, PocLsbLt[ i ] is set equal to lt_ref_pic_poc_lsb_sps[ lt_idx_sps[ i ] ] and UsedByCurrPicLt[ i ] is set equal to used_by_curr_pic_lt_sps_flag[ lt_idx_sps[ i ] ].

–	Otherwise, PocLsbLt[ i ] is set equal to poc_lsb_lt[ i ] and UsedByCurrPicLt[ i ] is set equal to used_by_curr_pic_lt_flag[ i ].

delta_poc_msb_present_flag[ i ] equal to 1 specifies that delta_poc_msb_cycle_lt[ i ] is present. delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt[ i ] is not present. delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one reference picture in the decoded picture buffer with the least significant bits of the picture order count value equal to poc_lsb_lt[ i ].

delta_poc_msb_cycle_lt[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. When delta_poc_msb_cycle_lt[ i ] is not present, it is inferred to be equal to 0.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows:

if( i  = =  0  | |  i = =  num_long_term_sps )
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ]
else							(7-64)
	DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt[ i ] + DeltaPocMSBCycleLt[ i − 1 ]

The value of DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb + pic_order_cnt_lsb − PocLsbLt[ i ] shall be in the range of 1 to 224 − 1, inclusive.

slice_temporal_mvp_enable_flag specifies whether temporal motion vector predictors can be used for inter prediction. If slice_temporal_mvp_enable_flag is equal to 0, temporal motion vector predictors shall not be used in decoding of the current picture. If slice_temporal_mvp_enable_flag is equal to 1, temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of slice_temporal_mvp_enable_flag is inferred to be equal to 0. [Ed. (GJS): Check the use of "shall" in this paragraph. Is that a bitstream constraint or a decoding process specification? Is it duplicating a prescription expressed elsewhere?]

When both slice_temporal_mvp_enable_flag and TemporalId are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vectors from any picture that precedes the current picture in decoding order. [Ed. (GJS): Check the use of "shall" in this paragraph.]

slice_sao_luma_flag equal to 1 specifies that SAO is enabled for the luma component in the current slice; slice_sao_luma_flag equal to 0 specifies that SAO is disabled for the luma component in the current slice. When slice_sao_luma_flag is not present, it is inferred to be equal to 0.

slice_sao_chroma_flag equal to 1 specifies that SAO is enabled for the chroma component in the current slice; slice_sao_chroma_flag equal to 0 specifies that SAO is disabled for the chroma component in the current slice. When slice_sao_chroma_flag is not present, it is inferred to be equal to 0.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_l0_active_minus1 is present for P and B slices and that the syntax element num_ref_idx_l1_active_minus1 is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are not present.

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that may be used to decode the slice. num_ref_idx_l0_active_minus1 shall be in the range of 0 to 15, inclusive. When the current slice is a P or B slice and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 is inferred to be equal to num_ref_idx_l0_default_active_minus1.

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice. When the current slice is a B slice and num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 is inferred to be equal to num_ref_idx_l1_default_active_minus1.The range of num_ref_idx_l1_active_minus1 is constrained as specified in the semantics for num_ref_idx_l0_active_minus1 with l0 and list 0 replaced by l1 and list 1, respectively.

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to 0 for compIdx = 0..1. mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed.

cabac_init_flag specifies the method for determining the initialization table used in the initialization process for context variables. When cabac_init_flag is not present, it is inferred to be 0.

collocated_from_l0_flag equal to 1 specifies the picture that contains the collocated partition is derived from list 0, otherwise the picture is derived from list 1.When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

collocated_ref_idx specifies the reference index of the picture that contains the collocated partition. When the current slice is a P slice, collocated_ref_idx refers to a picture in list 0. When the current slice is a B slice, collocated_ref_idx refers to a picture in list 0 if collocated_from_l0 is 1, otherwise it refers to a picture in list 1. collocated_ref_idx shall always refer to a valid list entry, and the resulting picture shall be the same for all slices of a coded picture. When collocated_ref_idx is not present, it is inferred to be equal to 0.

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as

MaxNumMergeCand = 5 − five_minus_max_num_merge_cand	(7-65)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 1 to 5, inclusive.

slice_qp_delta specifies the initial value of QPY to be used for the coding blocks in the slice until modified by the value of CuQpDelta in the coding unit layer. The initial value of the QPY quantization parameter for the slice, SliceQPY, is derived as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-66)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

slice_cb_qp_offset specifies a difference to be added to the value of pic_cb_qp_offset when determining the value of the QP′Cb quantization parameter. The value of slice_cb_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cb_qp_offset is not present, it is inferred to be equal to 0. The value of pic_cb_qp_offset + slice_cb_qp_offset shall be in the range of −12 to +12, inclusive.

slice_cr_qp_offset specifies a difference to be added to the value of pic_cr_qp_offset when determining the value of the QP′Cr quantization parameter. The value of slice_cr_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cr_qp_offset is not present, it is inferred to be equal to 0. The value of pic_cr_qp_offset + slice_cr_qp_offset shall be in the range of −12 to +12, inclusive.

deblocking_filter_override_flag equal to 0 specifies that deblocking parameters from the active picture parameter set are used for deblocking the current slice. deblocking_filter_override_flag equal to 1 specifies that deblocking parameters from the slice header are used for deblocking the current slice. When not present, the value of deblocking_filter_override_flag is inferred to be equal to 0.

slice_disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter is not applied for the current slice. slice_disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter is applied for the current slice. When slice_disable_deblocking_filter_flag is not present, it is inferred to be equal to pic_disable_deblocking_filter_flag.

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice. The values of beta_offset_div2 and tc_offset_div2 shall be in the range of −6 to 6, inclusive.

slice_loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations are performed across left and upper boundaries of the current slice; otherwise, the in-loop operations are not applied across left and upper boundaries of the current slice. The in-loop filtering operations include the deblocking filter and sample adaptive offset filter. When slice_loop_filter_across_slices_enabled_flag is not present, it is inferred to be equal to loop_filter_across_slices_enabled_flag.

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

The value of num_entry_point_offsets shall be constrained as follows:

–	If tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive.

–	Otherwise, if tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive.

–	Otherwise, when tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * PicHeightInCtbsY − 1, inclusive.

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

entry_point_offset[ i ] specifies the i-th entry point offset in bytes, and is represented by offset_len_minus1 plus 1 bits. The slice segment data that follows the slice segment header consists of num_entry_point_offsets + 1 subsets, with subset index values ranging from 0 to num_entry_point_offsets, inclusive. The first byte of the slice segment data is considered byte 0. When present, emulation prevention bytes that appear in the slice segment data portion of the coded slice segment NAL unit are counted as part of the slice segment data for purposes of subset identification. Subset 0 consists of bytes 0 to entry_point_offset[ 0 ] − 1, inclusive, of the coded slice segment data, subset k, with k in the range of 1 to num_entry_point_offsets − 1, inclusive, consists of bytes firstByte[ k ] to lastByte[ k ], inclusive, of the coded slice segment data with firstByte[ k ] and lastByte[ k ] defined as:

firstByte[ k ] = ∑n( entry_point_offset[ n − 1 ] ) with n = 1..k	(7-67)

lastByte[ k ] = firstByte[ k ] − 1+entry_point_offset[ k ]	(7-68)

The last subset (with subset index equal to num_entry_point_offsets) consists of the remaining bytes of the coded slice segment data.

When tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, each subset shall consist of all coded bits of all coding tree units in the slice segment that are within the same tile. In this case, the number of subsets (i.e. the value of num_entry_point_offsets + 1) shall be equal to the number of tiles that contain coding tree units that are in the coded slice segment.

NOTE 47 – When tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 0, each slice must include either a subset of the coding tree units of one tile (in which case the syntax element entry_point_offset[ i ] is not present) or must include all coding tree units of an integer number of complete tiles.

When tiles_enabled_flag is equal to 0 and entropy_coding_sync_enabled_flag is equal to 1, each subset k with k in the range of 0 to num_entry_point_offsets, inclusive, shall consist of all coded bits of all coding tree units in the slice semgnet that are within the same row of coding tree units in the picture. In this case, the number of subsets (i.e. the value of num_entry_point_offsets + 1) shall be equal to the number of rows of coding tree units of the picture that contain coding tree units that are in the coded slice segment.

NOTE 48 – The last subset (i.e. subset k for k equal to num_entry_point_offsets) may or may not contain all coding tree units that are within a row of coding tree units in the picture.

When tiles_enabled_flag is equal to 1 and entropy_coding_sync_enabled_flag is equal to 1, each subset k with k in the range of 0 to num_entry_point_offsets − 1, inclusive, shall consist of all coded bits of all coding tree units in the slice segment that are within the same row of coding tree units in a tile. In this case, the number of subsets (i.e. the value of num_entry_point_offsets + 1) shall be equal to the number of rows of coding tree units of tiles that contain coding tree units that are in the coded slice segment.

[Ed. (RS): Search/fix uses of "coding tree blocks" vs. "coding tree units" and "coding blocks" vs. "coding units".]

slice_segment_header_extension_length specifies the length of the slice segment header extension data in bytes, not including the bits used for signalling slice_segment_header_extension_length itself. The value of slice_segment_header_extension_length shall be in the range of 0 to 256, inclusive.

slice_segment_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this Specification.

##### Slice segment header extension semantics

extension_struct_idc specifies the presence of syntax elements in the slice segment header extension.

[Ed. num_nonbase_long_term_sps, num_nonbase_long_term_pics, nonbase_lt_idx_sps[ i ], nonbase_poc_lsb_lt[ i ], etc. are specified identically to the respective syntax elements in slice segment header but apply to pictures with nuh_layer_id greater than 0]

layer_id_lt[ i ] specifies that the i-th candidate long-term reference picture with nuh_reserved_zero_6bits greater than 0 to be included in the long-term reference picture set of the current picture has nuh_reserved_zero_6bits equal to layer_id_lt[ i ]. If not present, layer_id_lt[ i ] is inferred to be equal to 0.

#### Short-term reference picture set semantics

A short-term reference picture set may be present in a sequence parameter set or in a slice header. When a short-term reference picture set is present in a slice header, the content of the short-term reference picture set syntax structure shall be the same in all slice headers of a picture and the value of idxRps shall be equal to the syntax element num_short_term_ref_pic_sets from the active sequence parameter set. In the following, when the short_term_ref_pic_set( idxRps ) syntax structure is included in the sequence parameter set, the term "the current picture" applies to all pictures in the coded video sequence with short_term_ref_pic_set_idx equal to idxRps.

inter_ref_pic_set_prediction_flag equal to 1 specifies that the reference picture set of the current picture is predicted using another reference picture set in the active sequence parameter set. When inter_ref_pic_set_prediction_flag is not present, it is inferred to be equal to 0.

delta_idx_minus1 plus 1 specifies the difference between the index of the reference picture set of the current picture and the index of the reference picture set used for inter reference picture set prediction. The value of delta_idx_minus1 shall be in the range of 0 to idxRps − 1, inclusive. When delta_idx_minus1 is not present, it is inferred to be equal to 0.

The variable RIdx is derived as follows.

RIdx = idxRps − ( delta_idx_minus1 + 1 )		(7-69)

delta_rps_sign and abs_delta_rps_minus1 together specify the value of the variable DeltaRPS as follows

DeltaRPS = (1 − 2 * delta_rps_sign ) * ( abs_delta_rps_minus1 + 1 )	(7-70)

The variable DeltaRPS represents the value to be added to picture order count difference values of the reference picture set used for inter reference picture set prediction to obtain the picture order count difference values of the current reference picture set.

used_by_curr_pic_flag[ j ] equal to 0 specifies that the j-th picture is not used for reference by the current picture.

use_delta_flag[ j ] equal to 1 specifies that the j-th picture is included in the reference picture set of the current picture. use_delta_flag[ j ] equal to 0 specifies that the j-th picture is not included in the reference picture set of the current picture. When use_delta_flag[ j ] is not present, its value is inferred to be equal to 1.

When inter_ref_pic_set_prediction_flag is equal to 1, the variables DeltaPocS0[ idxRps ][ i ], UsedByCurrPicS0[ idxRps ][ i ], and NumNegativePics[ idxRps ] are derived as follows.

i = 0
for( j = NumPositivePics[ RIdx ] − 1; j  >=  0; j−  − ) {
	dPoc = DeltaPocS1[ RIdx ][ j ] + DeltaRPS
	if( dPoc < 0  &&  use_delta_flag[ NumNegativePics[ RIdx ] + j ] ) {
		DeltaPocS0[ idxRps ][ i ] = dPoc
		UsedByCurrPicS0[ idxRps ][ i++ ] = used_by_curr_pic_flag[ NumNegativePics[ RIdx ] + j ]
	}
}
if( DeltaRPS < 0  &&  use_delta_flag[ NumDeltaPocs[ RIdx ] ] ) {
	DeltaPocS0[ idxRps ][ i ] = DeltaRPS
	UsedByCurrPicS0[ idxRps ][ i++ ] = used_by_curr_pic_flag[ NumDeltaPocs[ RIdx ] ]	(7-71)
}
for( j = 0; j < NumNegativePics[ RIdx ]; j++ ) {
	dPoc = DeltaPocS0[ RIdx ][ j ] + DeltaRPS
	if( dPoc < 0  &&  use_delta_flag[ j ] ) {
		DeltaPocS0[ idxRps ][ i ] = dPoc
		UsedByCurrPicS0[ idxRps ][ i++ ] = used_by_curr_pic_flag[ j ]
	}
}
NumNegativePics[ idxRps ] = i

When inter_ref_pic_set_prediction_flag is equal to 1, the variables DeltaPocS1[ idxRps ][ i ], UsedByCurrPicS1[ idxRps ][ i ], and NumPositivePics[ idxRps ] are derived as follows.

i = 0
for( j = NumNegativePics[ RIdx ] − 1; j  >=  0; j−  − ) {
	dPoc = DeltaPocS0[ RIdx ][ j ] + DeltaRPS
	if( dPoc > 0  &&  use_delta_flag[ j ] ) {
		DeltaPocS1[ idxRps ][ i ] = dPoc
		UsedByCurrPicS1[ idxRps ][ i++ ] = used_by_curr_pic_flag[ j ]
	}
}
if( DeltaRPS > 0  &&  use_delta_flag[ NumDeltaPocs[ RIdx ] ] ) {
	DeltaPocS1[ idxRps ][ i ] = DeltaRPS
	UsedByCurrPicS1[ idxRps ][ i++ ] = used_by_curr_pic_flag[ NumDeltaPocs[ RIdx ] ]	(7-72)
}
for( j = 0; j < NumPositivePics[ RIdx ]; j++) {
	dPoc = DeltaPocS1[ RIdx ][ j ] + DeltaRPS
	if( dPoc > 0  &&  use_delta_flag[ NumNegativePics[ RIdx ] + j ] ) {
		DeltaPocS1[ idxRps ][ i ] = dPoc
		UsedByCurrPicS1[ idxRps ][ i++ ] = used_by_curr_pic_flag[ NumNegativePics[ RIdx ] + j ]
	}
}
NumPositivePics[ idxRps ] = i

num_negative_pics, when present, specifies the number of the following delta_poc_s0_minus1[ i ] and used_by_curr_pic_s0_flag[ i ] syntax elements. The value of num_negative_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ], inclusive.

num_positive_pics, when present, specifies the number of the following delta_poc_s1_minus1[ i ] and used_by_curr_pic_s1_flag1[ i ] syntax elements. The value of num_positive_pics shall be in the range of 0 to sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] − num_negative_pics, inclusive.

delta_poc_s0_minus1[ i ] plus 1, when delta_poc_s0_minus1[ i ] is present, specifies an absolute difference between two picture order count values. The value of delta_poc_s0_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

used_by_curr_pic_s0_flag[ i ] equal to 0, when used_by_curr_pic_s0_flag[ i ] is present, specifies that the i-th reference picture that has picture order count less than that of the current picture is not used for reference by the current picture.

delta_poc_s1_minus1[ i ] plus 1, when delta_poc_s1_minus1[ i ] is present specifies an absolute difference between two picture order count values. The value of delta_poc_s1_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

used_by_curr_pic_s1_flag[ i ] equal to 0, when used_by_curr_pic_s1_flag[ i ]  is present, specifies that the i-th reference picture that has picture order count greater than that of the current picture is not used for reference by the current picture.

When inter_ref_pic_set_prediction_flag is equal to 0, the variables NumNegativePics[ idxRps ], NumPositivePics[ idxRps ], UsedByCurrPicS0[ idxRps ][ i ], UsedByCurrPicS1[ idxRps ][ i ], DeltaPocS0[ idxRps ][ i ], and DeltaPocS1[ idxRps ][ i ] are derived as follows.

NumNegativePics[ idxRps ] = num_negative_pics		(7-73)
NumPositivePics[ idxRps ] = num_positive_pics		(7-74)
UsedByCurrPicS0[ idxRps ][ i ] = used_by_curr_pic_s0_flag[ i ]	(7-75)
UsedByCurrPicS1[ idxRps ][ i ] = used_by_curr_pic_s1_flag[ i ]	(7-76)

–	If the variable i is equal to 0, the following applies.

DeltaPocS0[ idxRps ][ i ] = − ( delta_poc_s0_minus1[ i ] + 1 ) 	(7-77)
DeltaPocS1[ idxRps ][ i ] = delta_poc_s1_minus1[ i ] + 1	(7-78)

–	Otherwise, the following applies.

DeltaPocS0[ idxRps ][ i ] = DeltaPocS0[ idxRps ][ i − 1 ] − ( delta_poc_s0_minus1[ i ] + 1 ) 	(7-79)
DeltaPocS1[ idxRps ][ i ] = DeltaPocS1[ idxRps ][ i − 1 ] + ( delta_poc_s1_minus1[ i ] + 1 ) 	(7-80)

The variable NumDeltaPocs[ idxRps ] is derived as follows.

NumDeltaPocs[ idxRps ] = NumNegativePics[ idxRps ] + NumPositivePics[ idxRps ]	(7-81)

#### Reference picture list modification semantics

ref_pic_list_modification_flag_l0 equal to 1 indicates that reference picture list 0 is specified explicitly as a list of list_entry_l0[ i ] values. ref_pic_list_modification_flag_l0 equal to 0 indicates that reference picture list 0 is determined implicitly. When ref_pic_list_modification_flag_l0 is not present, it is inferred to be equal to 0.

list_entry_l0[ i ] specifies the index of the reference picture in RefPicListTemp0 to be placed at the current position of reference picture list 0. The length of the list_entry_l0[ i ] syntax element is Ceil( Log2( NumPocTotalCurr) ) bits. The value of list_entry_l0[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive. If the syntax element list_entry_l0[ i ] is not present, it is inferred to be equal to 0.

The variable NumPocTotalCurr is derived as follows.

NumPocTotalCurr = 0;
	for( i = 0; i < NumNegativePics[ StRpsIdx ]; i++)
		if(UsedByCurrPicS0[ StRpsIdx ][ i ] = = 1)
			NumPocTotalCurr++
	for( i = 0; i < NumPositivePics[ StRpsIdx ]; i++)	(7-82)
		if(UsedByCurrPicS1[ StRpsIdx ][ i ] = = 1)
			NumPocTotalCurr++
	for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
		if( UsedByCurrPicLt[ i ] = = 1)
			NumPocTotalCurr++

ref_pic_list_modification_flag_l1 equal to 1 indicates that reference picture list 1 is specified explicitly as a list of list_entry_l1[ i ] values. ref_pic_list_modification_flag_l1 equal to 0 indicates that reference picture list 1 is determined implicitly. When ref_pic_list_modification_flag_l1 is not present, it is inferred to be equal to 0.

list_entry_l1[ i ] specifies the index of the reference picture in RefPicListTemp1 to be placed at the current position of reference picture list 1. The length of the list_entry_l1[ i ] syntax element is Ceil( Log2( NumPocTotalCurr ) ) bits. The value of list_entry_l1[ i ] shall be in the range of 0 to NumPocTotalCurr − 1, inclusive. If the syntax element list_entry_l1[ i ] is not present, it is inferred to be equal to 0.

#### Weighted prediction parameters semantics

luma_log2_weight_denom is the base 2 logarithm of the denominator for all luma weighting factors. The value of luma_log2_weight_denom shall be in the range of 0 to 7, inclusive.

delta_chroma_log2_weight_denom is the difference of the base 2 logarithm of the denominator for all chroma weighting factors.

The variable ChromaLog2WeightDenom is specified by luma_log2_weight_denom + delta_chroma_log2_weight_denom and it shall be in the range of 0 to 7, inclusive.

luma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 0 prediction using RefPicList0[ i ] are present. luma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present.

chroma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the chroma prediction values of list 0 prediction using RefPicList0[ i ] are present. chroma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present.  When chroma_weight_l0_flag[ i ] is not present, it is inferred to be equal to 0.

delta_luma_weight_l0[ i ] is the difference of the weighting factor applied to the luma prediction value for list 0 prediction using RefPicList0[ i ].

The variable LumaWeightL0[ i ] is specified by (1 << luma_log2_weight_denom ) + delta_luma_weight_l0[ i ]. When luma_weight_l0_flag[ i ] is equal to 1, the value of delta_luma_weight_l0[ i ] shall be in the range of −128 to 127, inclusive. When luma_weight_l0_flag[ i ] is equal to 0, LumaWeightL0[ i ] is inferred to be equal to 2luma_log2_weight_denom.

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. The value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive.  When luma_weight_l0_flag[ i ] is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0.

delta_chroma_weight_l0[ i ][ j ] is the difference of the weighting factor applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaWeightL0[ i ][ j ] is specified by ( 1 << ChromaLog2WeightDenom ) + delta_chroma_weight_l0[ i ][ j ]. When chroma_weight_l0_flag[ i ] is equal to 1, the value of delta_chroma_weight_l0[ i ][ j ] shall be in the range of −128 to 127, inclusive.  When chroma_weight_l0_flag[ i ] is equal to 0, ChromaWeightL0[ i ][ j ] is inferred to be equal to 2ChromaLog2WeightDenom.

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr.

The variable ChromaOffsetL0[ i ][ j ] is specified as follows:

ChromaOffsetL0[ i ][ j ] = Clip3( −128, 127, ( delta_chroma_offset_l0[ i ][ j ] − 	(7-83)
				( ( 128 * ChromaWeightL0[ i ][ j ] ) >> ChromaLog2WeightDenom ) + 128 ) )

The value of delta_chroma_offset_l0[ i ][ j ] shall be in the range of −512 to 511, inclusive. When chroma_weight_l0_flag[ i ] is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0.

luma_weight_l1_flag[ i ], chroma_weight_l1_flag[ i ], delta_luma_weight_l1[ i ], luma_offset_l1[ i ], delta_chroma_weight_l1[ i ][ j ], and delta_chroma_offset_l1[ i ][ j ] have the same semantics as luma_weight_l0_flag[ i ], chroma_weight_l0_flag[ i ], delta_luma_weight_l0[ i ], luma_offset_l0[ i ], delta_chroma_weight_l0[ i ][ j ], and delta_chroma_offset_l0[ i ][ j ], respectively, with l0, list 0, and List0 replaced by l1, list 1, and List1, respectively.

The variable sumWeightL0Flags is specified as the sum of luma_weight_l0_flag[ i ] + 2 * chroma_weight_l0_flag[ i ], for i = 0..num_ref_idx_l0_active_minus1.

When slice_type is equal to B, the variable sumWeightL1Flags is specified as the sum of luma_weight_l1_flag[ i ] + 2 * chroma_weight_l1_flag[ i ], for i = 0..num_ref_idx_l1_active_minus1.

It is a requirement of bitstream conformance that, when slice_type is equal to P, sumWeightL0Flags shall be less than or equal to 24, and when slice_type is equal to B, the sum of sumWeightL0Flags and sumWeightL1Flags shall be less than or equal to 24.

### Slice segment data semantics

#### General slice segment data semantics

end_of_slice_segment_flag equal to 0 specifies that another coding tree unit is following in the slice. end_of_slice_segment_flag equal to 1 specifies the end of the slice segment and that no further coding tree unit follows.

end_of_sub_stream_one_bit shall be equal to 1.

#### Coding tree unit semantics

The coding tree unit is the root node of the coding quadtree structure.

#### Sample adaptive offset semantics

sao_merge_left_flag equal to 1 specifies that the syntax elements sao_type_idx_luma, sao_type_idx_chroma, sao_band_position, sao_eo_class_luma, sao_eo_class_chroma, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the left coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding tree block. When sao_merge_left_flag is not present, it is inferred to be equal to 0.

sao_merge_up_flag equal to 1 specifies that the syntax elements sao_type_idx_luma, sao_type_idx_chroma, sao_band_position, sao_eo_class_luma, sao_eo_class_chroma, sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the above coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the above coding tree block. When sao_merge_up_flag is not present, it is inferred to be equal to 0.

sao_type_idx_luma indicates the offset type for the luma component. The array SaoTypeIdx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-10 of current coding tree block at position rx and ry for the colour component cIdx. The value of SaoTypeIdx[ 0 ][ rx ][ ry ] is derived as follows.

- If sao_type_idx_luma is present, SaoTypeIdx[ 0 ][ rx ][ ry ] is set equal to sao_type_idx_luma.

- Otherwise (sao_type_idx_luma is not present), SaoTypeIdx[ 0 ][ rx ][ ry ] is inferred as follows.

- If sao_merge_left_flag is equal to 1, SaoTypeIdx[ 0 ][ rx ][ ry ] is set equal to SaoTypeIdx[ 0 ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, SaoTypeIdx[ 0 ][ rx ][ ry ] is set equal to SaoTypeIdx[ 0 ][ rx ][ ry − 1 ].

- Otherwise, SaoTypeIdx[ 0 ][ rx ][ ry ] is set equal to 0.

sao_type_idx_chroma indicates the offset type for the chroma components. The values of SaoTypeIdx[ cIdx ][ rx ][ ry ] are derived as follows for cIdx equal to 1..2.

- If sao_type_idx_chroma is present, SaoTypeIdx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx_chroma.

- Otherwise (sao_type_idx_chroma is not present), SaoTypeIdx[ cIdx ][ rx ][ ry ] is inferred as follows.

- If sao_merge_left_flag is equal to 1, SaoTypeIdx[ cIdx ][ rx ][ ry ] is set equal to SaoTypeIdx[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, SaoTypeIdx[ cIdx ][ rx ][ ry ] is set equal to SaoTypeIdx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, SaoTypeIdx[ cIdx ][ rx ][ ry ] is set equal to 0.

Table 7-10 – Specification of the SAO type

| - SaoTypeIdx[ cIdx ][ rx ][ ry ] | - SAO type (informative) |
| --- | --- |
| - 0 | - Not applied |
| - 1 | - Band offset |
| - 2 | - Edge offset |

sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] indicates the offset value of i-th category of current coding tree block at position rx and ry for the colour component cIdx.

When sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_abs[ cIdx ][ rx − 1 ][ ry ][ i ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_abs[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] is set equal to 0.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of sao_offset[ cIdx ][ rx ][ ry ][ i ] when SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 1.

When sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx − 1 ][ ry ][ i ]

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal 0.

The variable offsetSign is derived as follows.

- If SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and i is greater than 1,offsetSign is set equal to −1.

- Otherwise, if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and i is less than 2, offsetSign is set equal to 1.

- Otherwise, if sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is equal to 0, offsetSign is set to equal to 1.

- Otherwise, offsetSign is set to equal to −1.

The array SaoOffsetVal is derived as follows.

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0		(7-84)

SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = 
			offsetSign*sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] << ( bitDepth − Min( bitDepth, 10 ) ) 	(7-85)

sao_band_position[ cIdx ][ rx ][ ry ] indicates the displacement of the band offset of the sample range when SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 1.

When sao_band_position[ cIdx ][ rx ][ ry ] is not present it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx − 1 ][ ry ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to sao_band_position[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_band_position[ cIdx ][ rx ][ ry ] is set equal to 0.

sao_eo_class_luma indicates the edge offset class for the luma component. The array SaoEoClass[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-11 of current coding tree block at position rx and ry for the colour component cIdx. The value of SaoEoClass[ 0 ][ rx ][ ry ] is derived as follows.

- If sao_eo_class_luma is present, SaoEoClass[ 0 ][ rx ][ ry ] is set equal to sao_eo_class_luma.

- Otherwise (sao_eo_class_luma is not present), SaoEoClass[ 0 ][ rx ][ ry ] is inferred as follows.

- If sao_merge_left_flag is equal to 1, SaoEoClass[ 0 ][ rx ][ ry ] is set equal to SaoEoClass[ 0 ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, SaoEoClass[ 0 ][ rx ][ ry ] is set equal to SaoEoClass[ 0 ][ rx ][ ry − 1 ].

- Otherwise, SaoEoClass[ 0 ][ rx ][ ry ] is set equal to 0.

sao_eo_class_chroma indicates the edge offset class for the chroma components. The values of SaoEoClass[ cIdx ][ rx ][ ry ] are derived as follows for cIdx equal to 1..2.

- If sao_eo_class_chroma is present, SaoEoClass[ cIdx ][ rx ][ ry ] is set equal to sao_eo_class_chroma.

- Otherwise (sao_eo_class_chroma is not present), SaoEoClass[ cIdx ][ rx ][ ry ] is inferred as follows.

- If sao_merge_left_flag is equal to 1, SaoEoClass[ cIdx ][ rx ][ ry ] is set equal to SaoEoClass[ cIdx ][ rx − 1 ][ ry  ].

- Otherwise, if sao_merge_up_flag is equal to 1, SaoEoClass[ cIdx ][ rx ][ ry ] is set equal to SaoEoClass[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, SaoEoClass[ cIdx ][ rx ][ ry ] is set equal to 0.

Table 7-11 – Specification of the SAO edge offset class

| - SaoEoClass[ cIdx ][ rx ][ ry ] | - SAO edge offset class (informative) |
| --- | --- |
| - 0 | - 1D 0-degree edge offset |
| - 1 | - 1D 90-degree edge offset |
| - 2 | - 1D 135-degree edge offset |
| - 3 | - 1D 45-degree edge offset |

#### Coding quadtree semantics

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

–	If log2CbSize is greater than Log2MinCbSizeY, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	Otherwise (log2CbSize is equal to Log2MinCbSizeY), the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

#### Coding unit semantics

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.

skip_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, when decoding a P or B slice, no more syntax elements except the merging candidate index merge_idx[ x0 ][ y0 ] are parsed after skip_flag[ x0 ][ y0 ]. skip_flag[ x0 ][ y0 ] equal to 0 specifies that the coding unit is not skipped. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When skip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

pred_mode_flag equal to 0 specifies that the current coding unit is coded in inter prediction mode. pred_mode_flag equal to 1 specifies that the current coding unit is coded in intra prediction mode. The variable PredMode[ x ][ y ] is derived as follows for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS − 1.

- If pred_mode_flag is equal to 0,

  - PredMode[ x ][ y ] is set to MODE_INTER.

- Otherwise (pred_mode_flag is equal to 1),

  - PredMode[ x ][ y ] is set to MODE_INTRA.

When pred_mode_flag is not present, the variable PredMode[ x ][ y ] is derived as follows for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS − 1.

- If slice_type is equal to I,

  - PredMode[ x ][ y ] is inferred to be equal to MODE_INTRA

- Otherwise (slice_type is equal to P or B), if skip_flag[ x0 ][ y0 ] is equal to 1,

  - PredMode[ x ][ y ] is inferred to be equal to MODE_SKIP

part_mode specifies partitioning mode of the current coding unit. The semantics of part_mode depend on PredMode[ x0 ][ y0 ]. The variables PartMode and IntraSplitFlag are derived from the value of part_mode as defined in Table 7-12.

The value of part_mode is restricted as follows.

- If PredMode[ x0 ][ y0 ] is equal to MODE_INTRA, part_mode shall be equal to 0 or 1.

- Otherwise (PredMode[ x0 ][ y0 ] is equal to MODE_INTER), the following applies

- If log2CbSize is greater than Log2MinCbSizeY and amp_enabled_flag is equal to 1, part_mode shall be in the range of 0 to 2, inclusive and in the range of 4 to 7, inclusive.

- Otherwise, if log2CbSize is greater than Log2MinCbSizeY and amp_enabled_flag is equal to 0, part_mode shall be in the range of 0 to 2, inclusive.

- Otherwise, if log2CbSize is equal to 3, the value of part_mode shall be in the range of 0 to 2, inclusive.

- Otherwise (log2CbSize is greater than 3), the value of part_mode shall be in the range of 0 to 3, inclusive.

When part_mode is not present, the variables PartMode and IntraSplitFlag are derived as follows.

- PartMode is inferred to be equal to PART_2Nx2N,

- IntraSplitFlag is inferred to be equal to 0.

pcm_flag[ x0 ][ y0 ] equal to 1 specifies that pcm_sample() syntax is present and transform_tree() syntax is not present in the coding unit including the luma coding block at location ( x0, y0 ). pcm_flag[ x0 ][ y0 ] equal to 0 specifies that pcm_sample() syntax is not present. When pcm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The value of pcm_flag[ x0 + i ][ y0 + j ] with i = 1..nCbS − 1, j = 1..nCbS − 1 is inferred to be equal to pcm_flag[ x0 ][ y0 ].

pcm_alignment_zero_bit is a bit equal to 0.

Table 7-12 – Name association to prediction mode and partitioning type

| - PredMode[ x0 ][ y0 ] | - part_mode | - IntraSplitFlag | PartMode |
| --- | --- | --- | --- |
| - MODE_INTRA | - 0 | - 0 | - PART_2Nx2N |
|  | - 1 | - 1 | - PART_NxN |
| - MODE_INTER | - 0 | - 0 | PART_2Nx2N |
|  | - 1 | - 0 | PART_2NxN |
|  | - 2 | - 0 | PART_Nx2N |
|  | - 3 | - 0 | PART_NxN |
|  | - 4 | - 0 | PART_2NxnU |
|  | - 5 | - 0 | PART_2NxnD |
|  | - 6 | - 0 | PART_nLx2N |
|  | - 7 | - 0 | PART_nRx2N |

prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ], mpm_idx[ x0 + i ][ y0 + j ] and rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] specify the intra prediction mode for luma samples. The array indices x0 + i, y0 + j specify the location ( x0 + i, y0 + j ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. When prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit according to subclause Error: Reference source not found.

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

no_residual_syntax_flag equal to 1 specifies that transform_tree() syntax is not present for the current coding unit. no_residual_syntax_flag equal to 0 specifies that transform_tree() syntax is present for the current coding unit.

When no_residual_syntax_flag is not present, its value is inferred to be equal to 0.

#### Prediction unit semantics

mvp_l0_flag[ x0 ][ y0 ] specifies the motion vector predictor index of list 0 where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When mvp_l0_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mvp_l1_flag[ x0 ][ y0 ] has the same semantics as mvp_l0_flag, with l0 and list 0 replaced by l1 and list 1, respectively.

merge_flag[ x0 ][ y0 ] specifies whether the inter prediction parameters for the current prediction unit are inferred from a neighbouring inter-predicted partition. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When merge_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1 or bi-prediction is used for the current prediction unit according to Table 7-13. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

Table 7-13 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( nPbW + nPbH ) != 12 | ( nPbW + nPbH ) = = 12 |
| 0 | Pred_L0 | Pred_L0 |
| 1 | Pred_L1 | Pred_L1 |
| 2 | Pred_BI | na |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to Pred_L0.

ref_idx_l0[ x0 ][ y0 ] specifies the list 0 reference picture index for the current prediction unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When ref_idx_l0[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

ref_idx_l1[ x0 ][ y0 ] has the same semantics as ref_idx_l0, with l0 and list 0 replaced by l1 and list 1, respectively.

#### PCM sample semantics

pcm_sample_luma[ i ] represents a coded luma sample value in the raster scan within the coding unit. The number of bits used to represent each of these samples is PCMBitDepthY.

pcm_sample_chroma[ i ] represents a coded chroma sample value in the raster scan within the coding unit. The first half of the values represent coded Cb samples and the remaining half of the values represent coded Cr samples. The number of bits used to represent each of these samples is PCMBitDepthC.

#### Transform tree semantics

split_transform_flag[ x0 ][ y0 ][ trafoDepth ] specifies whether a block is split into four blocks with half horizontal and half vertical size for the purpose of transform coding. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

The variable interSplitFlag is derived as follows.

- If max_transform_hierarchy_depth_inter is equal to 0 and PredMode[ x0 ][ y0 ] is equal to MODE_INTER and PartMode is not equal to PART_2Nx2N and trafoDepth is equal to 0, interSplitFlag is set to 1.

- Otherwise, interSplitFlag is set to 0.

When split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred as follows:

- If one or more of the following conditions are true, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 1.

- log2TrafoSize is greater than Log2MaxTrafoSize

- IntraSplitFlag is equal to 1 and trafoDepth is equal to 0

- interSplitFlag is equal to 1

- Otherwise, the value of split_transform_flag[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

cbf_luma[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When cbf_luma[ x0 ][ y0 ][ trafoDepth ] is not present, it is inferred to be equal to 1.

cbf_cb[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cb transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When cbf_cb[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred as follows.

- If trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cb[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

cbf_cr[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the Cr transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the top-left location ( x0, y0 ) of the considered transform unit. The array index trafoDepth specifies the current subdivision level of a coding block into blocks for the purpose of transform coding. trafoDepth is equal to 0 for blocks that correspond to coding blocks.

When cbf_cr[ x0 ][ y0 ][ trafoDepth ] is not present, the value of cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred as follows.

- If trafoDepth is greater than 0 and log2TrafoSize is equal to 2, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ]

- Otherwise, cbf_cr[ x0 ][ y0 ][ trafoDepth ] is inferred to be equal to 0.

#### Motion vector difference semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus2[ compIdx ] is the absolute value of a motion vector component difference minus 2. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When abs_mvd_minus2[ compIdx ] is not present, it is inferred as follows.

- If abs_mvd_greater1_flag[ compIdx ] is equal to 0, abs_mvd_minus2[ compIdx ] is inferred to be equal to −1.

- Otherwise (abs_mvd_greater1_flag[ compIdx ] is equal to 1), abs_mvd_minus2[ compIdx ] is inferred to be equal to 0.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows.

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The motion vector difference lMvd[ compIdx ] for compIdx = 0..1 is derived as follows.

lMvd[ compIdx ] = abs_mvd_greater0_flag[ compIdx ] * 
			( abs_mvd_minus2[ compIdx ] + 2 ) *	(7-86)
			( 1 − 2 * mvd_sign_flag[ compIdx ] )

The variable MvdLX[ x0 ][ y0 ][ compIdx ] with X being either 0 or 1, specifies the difference between a list X vector component to be used and its prediction. The value of MvdLX[ x0 ][ y0 ][ compIdx ] shall be in the range of −215 to 215 − 1. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

#### Transform unit semantics

The transform coefficient levels are parsed into the arrays TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. The array indices xC, yC specify the transform coefficient location ( xC, yC ) within the current transform block. When the value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is not specified in subclause 7.3.9.11 residual coding syntax, it is inferred to be equal to 0.

cu_qp_delta_abs specifies the absolute value of the difference between a luma quantization parameter for the coding unit containing cu_qp_delta_abs and its prediction.

cu_qp_delta_sign specifies the sign of a CuQpDelta as follows.

- If cu_qp_delta_sign is equal to 0, the corresponding CuQpDelta has a positive value.

- Otherwise (cu_qp_delta_sign is equal to 1), the corresponding CuQpDelta has a negative value.

When cu_qp_delta_sign is not present, it is inferred to be equal to 0.

When cu_qp_delta_abs is present, the variables IsCuQpDeltaCoded and CuQpDelta are derived as follows.

IsCuQpDeltaCoded = 1	(7-87)

CuQpDelta = cu_qp_delta_abs * ( 1 − 2 * cu_qp_delta_sign )	(7-88)

The decoded value of CuQpDelta shall be in the range of −( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive.

#### Residual coding semantics

For intra prediction, different scanning orders are used. The variable scanIdx specifies which scan order is used where scanIdx equal to 0 specifies an up-right diagonal scan order, scanIdx equal to 1 specifies a horizontal scan order, and scanIdx equal to 2 specifies a vertical scan order. The value of scanIdx is derived as follows.

- If PredMode[ x0 ][ y0 ] is equal to MODE_INTRA and one or more of the following conditions are true,

- log2TrafoSize is equal to 2

- log2TrafoSize is equal to 3 and cIdx is equal to 0

scanIdx is dervied as follows.

- If IntraPredMode is in the range from 6 to 14, inclusive, scanIdx is set equal to 2.

- Otherwise, if IntraPredMode is in the range from 22 to 30, inclusive, scanIdx is set equal to 1.

- Otherwise, scanIdx is set equal to 0.

- Otherwise, scanIdx is set equal to 0.

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated transform block or not: The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1, specifies that no transform will be applied to the current transform block. When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

last_significant_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x_prefix shall be in the range from 0 to ( log2TrafoSize << 1 ) − 1, inclusive.

last_significant_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_y_prefix shall be in the range from 0 to ( log2TrafoSize << 1 ) − 1, inclusive.

last_significant_coeff_x_suffix specifies the suffix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_x_suffix shall be in the range from 0 to ( 1 << ( ( last_significant_coeff_x_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The column position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffX is derived as follows.

- If last_significant_coeff_x_suffix is not present, the following applies.

LastSignificantCoeffX  =  last_significant_coeff_x_prefix	(7-89)

- Otherwise (last_significant_coeff_x_suffix is present), the following applies.

LastSignificantCoeffX  =  (1 << ((last_significant_coeff_x_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_x_prefix & 1))  +  	(7-90)
				last_significant_coeff_x_suffix

last_significant_coeff_y_suffix specifies the suffix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_significant_coeff_y_suffix shall be in the range from 0 to ( 1 << (  ( last_significant_coeff_y_prefix >> 1 ) − 1 ) ) − 1, inclusive.

The row position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffY is derived as follows.

- If last_significant_coeff_y_suffix is not present, the following applies.

LastSignificantCoeffY  =  last_significant_coeff_y_prefix	(7-91)

- Otherwise (last_significant_coeff_y_suffix is present), the following applies.

LastSignificantCoeffY  =  (1 << ((last_significant_coeff_y_prefix >> 1) − 1))  *  	
				(2 + (last_significant_coeff_y_prefix & 1))  +  	(7-92) 
				last_significant_coeff_y_suffix

When scanIdx is equal to 2, the coordinates are swapped as follows.

( LastSignificantCoeffX, LastSignificantCoeffY ) = Swap( LastSignificantCoeffX, LastSignificantCoeffY )	(7-93)

coded_sub_block_flag[ xS ][ yS ] specifies the following for the sub-block at location ( xS, yS ) within the current transform block, where a sub-block is a (4x4) array of 16 transform coefficient levels.

- If coded_sub_block_flag[ xS ][ yS ] is equal to 0, the 16 transform coefficient levels of the sub-block at location ( xS, yS ) are inferred to be equal to 0;

- Otherwise (coded_sub_block_flag[ xS ][ yS ] is equal to 1), the following applies.

- If ( xS, yS ) is equal to ( 0, 0 ) and ( LastSignificantCoeffX, LastSignificantCoeffY ) is not equal to ( 0, 0 ), at least one of the 16 significant_coeff_flag syntax elements is present for the sub-block at location ( xS, yS ) .

- Otherwise, at least one of the 16 transform coefficient levels of the sub-block at location ( xS, yS ) has a non zero value.

When coded_sub_block_flag[ xS ][ yS ] is not present, it is inferred as follows.

- If one or more of the following conditions are true, coded_sub_block_flag[ xS ][ yS ] is inferred to be equal to 1.

- ( xS, yS ) is equal to ( 0, 0 )

- ( xS, yS ) is equal to ( LastSignificantCoeffX >> 2 , LastSignificantCoeffY >> 2 )

- Otherwise, coded_sub_block_flag[ xS ][ yS ] is inferred to be equal to 0.

significant_coeff_flag[ xC ][ yC ] specifies for the transform coefficient position ( xC, yC ) within the current transform block whether the corresponding transform coefficient level at location ( xC, yC ) is non-zero as follows.

- If significant_coeff_flag[ xC ][ yC ] is equal to 0, the transform coefficient level at location ( xC, yC ) is set equal to 0.

- Otherwise (significant_coeff_flag[ xC ][ yC ] is equal to 1), the transform coefficient level at location ( xC, yC ) has a non-zero value.

When significant_coeff_flag[ xC ][ yC ] is not present, it is inferred as follows.

- If ( xC, yC ) is the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order or all of the following conditions are true, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 1.

- ( xC & 3, yC & 3 ) is equal to ( 0, 0 )

- inferSbDcSigCoeffFlag is equal to 1

- coded_sub_block_flag[ xS ][ yS ] is equal to 1

- Otherwise, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.

coeff_abs_level_greater1_flag[ n ] specifies for the scanning position n whether there are transform coefficient levels greater than 1.

When coeff_abs_level_greater1_flag[ n ] is not present, it is inferred to be equal to 0.

coeff_abs_level_greater2_flag[ n ] specifies for the scanning position n whether there are transform coefficient levels greater than 2.

When coeff_abs_level_greater2_flag[ n ] is not present, it is inferred to be equal to 0.

coeff_sign_flag[ n ] specifies the sign of a transform coefficient level for the scanning position n as follows.

- If coeff_sign_flag[ n ] is equal to 0, the corresponding transform coefficient level has a positive value.

- Otherwise (coeff_sign_flag[ n ] is equal to 1), the corresponding transform coefficient level has a negative value.

When coeff_sign_flag[ n ] is not present, it is inferred to be equal to 0.

coeff_abs_level_remaining[ n ] is the remaining absolute value of a transform coefficient level that is coded with Golomb-Rice code at the scanning position n. The value of coeff_abs_level_remaining[ n ] is constrained that the corresponding value of TransCoeffLevel[x0][y0][cIdx][xC][yC] is in the range from −32768 to 32767, inclusive. When coeff_abs_level_remaining [ n ] is not present, it is inferred as 0.

# Decoding process

## General decoding process

The input of this process is a bitstream and the output is a list of decoded pictures.

The set TargetDecLayerIdSet, which specifies the set of nuh_reserved_zero_6bits values of  NAL units to be decoded, is specified as follows:

–	If some external means not specified in this Specification is available to set TargetDecLayerIdSet, TargetDecLayerIdSet is set by the external means.

–	Otherwise if the decoding process is invoked in a bitstream conformance test as specified in subclause Error: Reference source not found, TargetDecLayerIdSet is set as specified in subclause Error: Reference source not found.

–	Otherwise, TargetDecLayerIdSet contains only one nuh_reserved_zero_6bits vlaue that  is equal to 0.

The variable HighestTid, which identifies the highest temporal sub-layer to be decoded, is specified as follows:

–	If some external means not specified in this Specification is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise if the decoding process is invoked in a bitstream conformance test as specified in subclause Error: Reference source not found, HighestTid is set as specified in subclause Error: Reference source not found.

–	Otherwise, HighestTid is set to sps_max_sub_layers_minus1.

The sub-bitstream extraction process as specified in subclause Error: Reference source not found is applied with HighestTid and TargetDecLayerIdSet as inputs and the output is assigned to a bitstream referred to as BitstreamToDecode.

The following applies to each coded picture (referred to as the current picture, and denoted by the variable CurrPic) in BitstreamToDecode.

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as input the  syntax elements and upper-case variables from clause 7. When intepreting the semantics of each syntax element in each NAL unit, "the bitstream" or part thereof (e.g. a coded video sequecne) means BitstreamToDecode or part thereof.

When the current picture has nuh_reserved_zero_6bits equal to 0, the decoding process specified in subclause 8.1.1 is invoked.

### Decoding process for a coded base picture

When the current picture is a BLA picture that has nal_unit_type equal to BLA_W_LP or is a CRA picture, the following applies:

–	If some external means not specified in this Specification is available to set the variable UseAltCpbParamsFlag to a value, UseAltCpbParamsFlag is set to the value provided by the external means.

–	Otherwise, the value of UseAltCpbParamsFlag is set to 0.

When the current picture is a CRA picture, the following applies:

–	If some external means not specified in this Specification is available to set the variable HandleCraAsBlaFlag to a value, HandleCraAsBlaFlag is set to the value provided by the external means.

–	Otherwise, the value of HandleCraAsBlaFlag is set to 0.

When the current picture is a CRA picture and HandleCraAsBlaFlag is equal to 1, the value of no_output_of_prior_pics_flag is set to 1 and the following applies during the parsing and decoding processes for each coded slice segment NAL unit: [Ed. (GJS): Do not set a syntax element equal to a value. You can only do that with variables. Create a variable called something like NoOutputOfPriorPicsFlag and use that instead.]

–	If UseAltCpbParamsFlag is equal to 0, the value of nal_unit_type is set to BLA_W_LP. [Ed. (GJS): Do not set a syntax element equal to a value. You can only do that with variables. Create a variable called something like BlaPicActionFlag and use that instead.]

–	Otherwise, the value of nal_unit_type is set to BLA_W_DLP. [Ed. (GJS): Do not set a syntax element equal to a value. Create a variable called something like BlaPicActionFlag and use that instead.]

Each picture referred to in this clause is a complete coded picture.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a coded video sequence with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to the 3 sample arrays of the current picture with the NAL units with colour_plane_id equal to 0 being assigned to SL, the NAL units with colour_plane_id equal to 1 being assigned to SCb, and the NAL units with colour_plane_id equal to 2 being assigned to SCr.

NOTE – The variable ChromaArrayType is derived as 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures with chroma_format_idc being equal to 0.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause 8.2.

- The processes in subclause 8.3 specify decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause 8.3.1 (which only needs to be invoked for the first slice segment of a picture).

–	The decoding process for reference picture set in subclause 8.3.2 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference" (which only needs to be invoked for the first slice segment of a picture).

–	When the current picture is a BLA picture or is a CRA picture that is the first picture in the bitstream, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked (which only needs to be invoked for the first slice segment of a picture).

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and the previous RAP picture with nuh_reserved_zero_6bits equal to 0 in decoding order is a BLA picture or is a CRA picture that is the first coded picture in the bitstream, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause 8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

–	After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference".

- The processes in subclauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found specify decoding processes using syntax elements in the coding tree unit layer and above.

## NAL unit decoding process

Inputs to this process are NAL units.

Outputs of this process are the RBSP syntax structures encapsulated within the NAL units.

The decoding process for each NAL unit extracts the RBSP syntax structure from the NAL unit and then operates the decoding processes specified for the RBSP syntax structure in the NAL unit as follows.

Subclause 8.3 describes the decoding process for VCL NAL units.

NAL units with nal_unit_type equal to VPS_NUT, SPS_NUT, and PPS_NUT contain video parameter sets, sequence parameter sets, and picture parameter sets, respectively. Sequence parameter sets are used in the decoding processes of other NAL units as determined by reference to a sequence parameter set within the picture parameter sets or buffereing period SEI messages. Picture parameter sets are used in the decoding processes of other NAL units as determined by reference to a picture parameter set within the slice segment headers.

## Slice decoding process

### Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, to represent picture order differences between pictures for motion vector derivation, and for decoder conformance checking (see subclause Error: Reference source not found).

Each coded picture is associated with one picture order count, denoted as PicOrderCntVal.

- When none of the following conditions is true: [Ed. (GJS): Logical structure of sentence seems strange.]

- The current picture is an IDR

- The current picture is a BLA picture

- The current picture is a CRA picture and is the first coded picture in the bitstream

- the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows. Let prevTid0Pic be the previous reference picture in decoding order that has TemporalId equal to 0 and nuh_reserved_zero_6bits equal to nuh_reserved_zero_6bits of the current picture. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevTid0Pic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

- The variable PicOrderCntMsb of the current picture is derived as follows.

- If the current picture is an IDR or a BLA picture, or if the first coded picture in the bitstream is a CRA picture and the current picture is the first coded picture in the bitstream, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-94)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as

PicOrderCntVal = PicOrderCntMsb + pic_order_cnt_lsb	(8-95)

NOTE 49 – All IDR pictures will have PicOrderCntVal equal to 0 since pic_order_cnt_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one coded video sequence, the PicOrderCntVal values for any two coded pictures shall not be the same.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(8-96)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-97)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive. [Ed. (GJS/YKW): Include the limit in all places where such a POC difference is used in the decoding process (i.e. TMVP).]

NOTE 50 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

NOTE 51 – Many encoders assign PicOrderCntVal proportional to the sampling time of the corresponding picture relative to the sampling time of the previous IDR or BLA picture.

### Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction for the slice as specified in subclause 8.3.3. This process may result in one or more reference pictures in the DPB being marked as "unused for reference" or "used for long-term reference".

NOTE 52 – The reference picture set is an absolute description of the reference pictures used in the decoding process of the current and future coded pictures. The reference picture set signalling is explicit in the sense that all reference pictures included in the reference picture set are listed explicitly and there is no default reference picture set construction process in the decoder that depends on the status of the DPB. [Ed. (GJS): Search for line spacing exactly 9.95 pt and change to "single".]

A picture can be marked as "unused for reference", "used for short-term reference", or "used for long-term reference", but only one among these three. Assigning one of these markings to a picture implicitly removes another of these markings when applicable. When a picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both).

When the current picture is the first picture in the bitstream, the DPB is initialized to be an empty set of pictures.

When the current picture is an IDR picture with nuh_reserved_zero_6bits equal to 0 or a BLA picture, all reference pictures currently in the DPB (if any) are marked as "unused for reference".

Short-term reference pictures are identified by their PicOrderCntVal values. Long-term reference pictures are identified either by their PicOrderCntVal values or their pic_order_cnt_lsb values. When nonbase_layer_long_term_ref_pics_present_flag is equal to 1, long-term reference pictures are additionally identified by their nuh_reserved_zero_6bits values.

Five lists of picture order count values are constructed to derive the reference picture set; PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll with NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of elements, respectively. Two lists of nuh_reserved_6bits values are additionally constructed to derive the reference picture set; LayerIdLtCurr and LayerIdLtFoll with NumPocLtCurr and NumPocLtFoll number of elements, respectively.

- If the current picture is an IDR picture, PocStCurrBefore, PocStCurrAfter, PocStFoll, PocLtCurr, and PocLtFoll are all set to empty, and NumPocStCurrBefore, NumPocStCurrAfter, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set to 0.

- Otherwise, the following applies for derivation of the five lists of picture order count values and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ )
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		PocStCurrBefore[ j++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
NumPocStCurrBefore = j

for( i = 0, j = 0; i < NumPositivePics[ StRpsIdx ]; i++ )
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		PocStCurrAfter[ j++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
NumPocStCurrAfter = j
NumPocStFoll = k		(8-98)
for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics + num_nonbase_long_term_sps + num_nonbase_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt += PicOrderCntVal − 	DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb − pic_order_cnt_lsb
	if( i < num_long_term_sps + num_long_term_pics ) 
		layerIdLt = 0
	else if( i < num_long_term_sps + num_long_term_pics + num_nonbase_long_term_sps ) 
		layerIdLt = lt_ref_layer_id_sps[ i – (num_long_term_sps + num_long_term_pics ) ]
	else
		layerIdLt = lt_ref_layer_id[ i – (num_long_term_sps + num_long_term_pics + num_nonbase_long_term_sps ) ]
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt				LayerIdLtCurr[ j ] = layerIdLt
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		LayerIdLtFoll[ k ] = layerIdLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in subclause 8.3.1.

NOTE 53 – A value of StRpsIdx in the range from 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a short-term reference picture set from the active sequence parameter set is being used, where StRpsIdx is the index of the short-term reference picture set to the list of short-term reference picture sets in the order in which they are signalled in the sequence parameter set. StRpsIdx equal to num_short_term_ref_pic_sets indicates that a short-term reference picture set explicitly signalled in the slice header is being used.

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to PocLtCurr[ j ] and LayerIdLtCurr[ i ] is equal to LayerIdLtCurr[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to PocLtFoll[ j ] and LayerIdLtFoll[ i ] is equal to LayerIdLtFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to PocLtCurr[ j ] and LayerIdLtFoll[ i ] is equal to LayerIdLtCurr[ j ].

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtCurr[ i ] is equal to LayerIdLtCurr[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to nuh_reserved_zero_6bits.

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to ( PocLtFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to LayerIdLtFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ) and LayerIdLtFoll[ i ] is equal to LayerIdLtCurr[ j ].

The reference picture set consists of five lists of reference pictures; RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetStFoll, RefPicSetLtCurr and RefPicSetLtFoll. The variable NumPocTotalCurr is derived in 7.4.8.3, reference picture list modification semantics.

It is a requirement of bitstream conformance that the following applies to the value of NumPocTotalCurr:

- If the current picture is a BLA or CRA picture, the value of NumPocTotalCurr shall be equal to 0.

- Otherwise, when the current picture contains a P or B slice, the value of NumPocTotalCurr shall not be equal to 0.

NOTE 54 – RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr contains all reference pictures that may be used in inter prediction of the current picture and that may be used in inter prediction of one or more of the pictures following the current picture in decoding order. RefPicSetStFoll and RefPicSetLtFoll consists of all reference pictures that are not used in inter prediction of the current picture but may be used in inter prediction of one or more of the pictures following the current picture in decoding order.

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ )
	if( !CurrDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB [Ed. (JB): Should be made more precise. (GJS): Seems roughly OK to me.]
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] 
				and with nuh_reserved_zero_6bits equal to LayerIdLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
																						(8-99)
for( i = 0; i < NumPocLtFoll; i++ )
	if( !FollDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else 
				RefPicSetLtFoll[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
					with PicOrderCntVal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
			else if( there is a short-term reference picture picY in the DPB
					with PicOrderCntVal equal to PocLtFoll[ i ] 
					and with nuh_reserved_zero_6bits equal to LayerIdLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else
				RefPicSetLtFoll[ i ] = "no reference picture"

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference".

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrBefore[ i ]
			and with nuh_reserved_zero_6bits equal to nuh_reserved_zero_6bits of the current picture )
		RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrAfter[ i ]
			and with nuh_reserved_zero_6bits equal to nuh_reserved_zero_6bits of the current picture )
		RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(8-100)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStFoll[ i ]
			and with nuh_reserved_zero_6bits equal to nuh_reserved_zero_6bits of the current picture )
		RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures in the decoded picture buffer that have nuh_reserved_zero_6bits equal to nuh_reserved_zero_6bits of the current picture and are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

NOTE 55 – There may be one or more reference pictures that are included in the reference picture set but not present in the decoded picture buffer. Entries in RefPicSetStFoll or RefPicSetLtFoll that are equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr that is equal to "no reference picture".

It is a requirement of bitstream conformance that the reference picture set is restricted as follows:

- There shall be no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr for which one or more of the following are true.

  - The entry is equal to "no reference picture".

  - The entry is a picture that has nal_unit_type equal to TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12, or RSV_VCL_N14 and TemporalId equal to that of the current picture.

  - The entry is a picture that has TemporalId greater than that of the current picture.

- There shall no entry in RefPicSetLtCurr or RefPicSetLtFoll for which the difference between the picture order count value of the current picture and the picture order count value of the entry is equal to or greater than 224.

- There shall be no picture in RefPicSetLtCurr with LayerIdLtCurr[ i ] greater than nuh_reserved_zero_6bits of the current picture.

- When the current picture is a TSA picture, there shall be no picture included in the reference picture set with TemporalId greater than or equal to the TemporalId of the current picture.

- When the current picture is an STSA picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, an STSA picture that has TemporalId equal to that of the current picture and nuh_reserved_zero_6bits equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that preceded the STSA picture in decoding order.

- When the current picture is a CRA picture, there shall be no picture included in the reference picture set that precedes, in decoding order, any preceding RAP picture in decoding order having nuh_reserved_zero_6bits equal to that of the current picture (when present) or any preceding BLA picture in decoding order (when present).

- When the current picture is a trailing picture, there shall be no picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that was generated by the decoding process for generating unavailable reference pictures as specified in subclause 8.3.3.

- When the current picture is a trailing picture, there shall be no picture in the reference picture set that precedes the associated RAP picture in output order or decoding order.

- When the current picture is a RADL picture, there shall be no picture included in RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetLtCurr that is any of the following types of pictures.

  - A RASL picture.

  - A picture that was generated by the decoding process for generating unavailable reference pictures as specified in subclause 8.3.3.

  - A picture that precedes the associated RAP picture in decoding order.

- When the sps_temporal_id_nesting_flag is equal to 1, the following applies. Let tIdA be the value of TemporalId of the current picture picA. Any picture picB with TemporalId equal to tIdB that is less than or equal to tIdA shall not be included in RefPicSetStCurrBefore, RefPicSetStCurrAfter and RefPicSetLtCurr of picA when there exists a picture picC with TemporalId equal to tIdC that is less than tIdB, which follows the picture picB in decoding order and precedes the picture picA in decoding order.

NOTE 56 – A picture cannot be included in more than one of the five reference picture set lists.

### Decoding process for generating unavailable reference pictures

#### General decoding process for generating unavailable reference pictures

[Ed. (GJS): Scan the table of contents to look for any cases where headings do not appear there, as this is almost certainly due to having some style other than the ordinary heading style used on the headings of the missing sections.]

NOTE 1 – This process is invoked once the current picture when the current picture is a BLA picture or is a CRA picture that is the first picture in the bitstream.

NOTE 2 – This process is primarily specified only for the specification of syntax constraints for RASL pictures. The entire specification of the decoding process for RASL pictures associated with a CRA picture at the beginning of the bitstream or for RASL pictures associated with a BLA picture is only included herein for purposes of specifying constraints on the allowed syntax content of such RASL pictures. During the decoding process, any RASL pictures associated with a CRA picture at the beginning of the bitstream or any RASL pictures associated with a BLA picture may be ignored (removed from the bitstream and discarded), as they are not specified for output and have no effect on the decoding process of any other pictures that are specified for output. However, in HRD operations as specified in Annex Error: Reference source not found, RASL access units may need to be taken into consideration in derivation of CPB arrival and removal times.

When this process is invoked, the following applies.

- For each RefPicSetStFoll[ i ], with i in the range of 0 to NumPocStFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies.

- The value of PicOrderCntVal for the generated picture is set to PocStFoll[ i ].

- The value of PicOutputFlag for the generated picture is set to 0.

- The generated picture is marked as "used for short-term reference".

- RefPicSetStFoll[ i ] is set to be the generated reference picture.

- The value of nuh_reserved_zero_6bits for the generated picture is set equal to the value of nuh_reserved_zero_6bits of the current picture.

- For each RefPicSetLtFoll[ i ], with i in the range of 0 to NumPocLtFoll − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.3.2, and the following applies.

- The value of PicOrderCntVal for the generated picture is set to PocLtFoll[ i ].

- The value of pic_order_cnt_lsb for the generated picture is inferred to be equal to ( PocLtFoll[ i ] & ( MaxPicOrderCntLsb – 1 ) )

- The value of PicOutputFlag for the generated picture is set to 0.

- The generated picture is marked as "used for long-term reference".

- RefPicSetLtFoll[ i ] is set to the generated reference picture.

- The value of nuh_reserved_zero_6bits for the generated picture is set equal to LayerIdLtFoll[ i ].

#### Generation of one unavailable picture

When this process is invoked, an unavailable picture is generated as follows:

- The value of each element in the sample array SL for the picture is set to 1 << ( BitDepthY − 1 ).

- The value of each element in the sample arrays SCb and SCr for the picture is set to 1 << ( BitDepthC − 1 ).

- The prediction mode PredMode[ x ][ y ] is set to MODE_INTRA for x = 0..pic_width_in_luma_samples − 1, y = 0..pic_height_in_luma_samples − 1.

### Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause Error: Reference source not found. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(8-101)
		RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(8-102)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(8-103)
		RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(8-104)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]