| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0171 |
| --- | --- |

| Title: | SHVC HLS: support for unequal BL and EL GOP lengths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Antti Hallapuro, and Kemal Ugur Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The following changes are proposed in this contribution:

- To specify an access unit to contain VCL NAL units of a single value of nuh_layer_id only.

  - Consequently, to require that nuh_layer_id of the access unit delimiter NAL unit (if present) shall be equal to nuh_layer_id of the VCL NAL units contained in the access unit.

- To allow a decoding/bitstream order of coded pictures with a certain value of nuh_layer_id to differ from the decoding/bitstream order of coded pictures with another value of nuh_layer_id.

The second change is asserted to enable SHVC to have a different prediction structure of the enhancement layer (EL) from that of the base layer (BL). The claimed benefits of this are:

- Higher coding efficiency for EL can be achieved by having a longer GOP length than base layer. For example, a dBR of -2.7% was reached for EL luma when the EL GOP length is twice of the BL GOP length for 2X spatial scalability and random access common conditions when the inter-layer motion hook was turned on.

- Lower complexity and decoded picture buffer (DPB) memory use for EL could be achieved by using a shorter EL GOP length than the BL GOP length.

It is asserted that all proposals of this contribution apply to a reference index based (a.k.a. high-level syntax based) SHVC design and an SHVC design where low-level changes are made.

# Introduction

In this contribution, we propose that the GOP length of an enhancement layer of an SHVC bitstream could be different than the GOP length of the base layer, as was proposed in Nokia’s CfP response. We basically see no reason to couple the GOP lengths of both layers as it reduces the flexibility of SHVC codec unnecessarily. By allowing the GOP lengths to be different for an enhancement layer and the base layer, the following benefits could be achieved:

- Additional coding efficiency could be achieved for an enhancement layer by using a longer EL GOP length.

- Lower complexity and DPB memory use could be achieved for an enhancement layer by using a shorter EL GOP length.

- Coded EL pictures might be transmitted over a channel having different error characteristics than the channel used for transmitting the base layer. Moreover, if the EL is subject to transmission losses, the receiver can continue the decoding and output of the base layer. For example, a forward error correction might be used differently for BL pictures and EL pictures. Hence, different prediction structures might be beneficial for EL and BL to optimize the quality.

# Access unit to contain a coded picture of a single value of nuh_layer_id only

## Background

An access unit has been defined as follows in HEVC:

access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain exactly one coded picture.

NOTE 1 – In addition to containing the coded slice segment NAL units of the coded picture, an access unit may also contain other NAL units not containing slice segments of the coded picture. The decoding of an access unit always results in a decoded picture.

A design decision was made for the scalable extension (SVC) of H.264/AVC to consider all dependency representations and layer representations as part of the same access unit. We suspect that this design decision was made primarily due to the single-loop decoding property and the coded picture buffer (CPB) operation. Because of the single-loop decoding property, the decoding of syntax elements from any reference layer might be needed when decoding the target layer. Consequently, no reference layer data can be removed from the CPB before the decoding of the target dependency/layer representation has been completed. Furthermore, the decoding of an access unit resulted into one decoded picture (at the target layer).

As SHVC is envisioned to utilize multi-loop decoding, each coded picture (having a certain value of nuh_layer_id) can be removed from the CPB as soon as it has been decoded. Furthermore, due to multi-loop decoding, the decoding of each coded picture (having a certain value of nuh_layer_id) results into one decoded picture.

## Proposal

Having reviewed the HEVC v1 specification and considered the scalable extensions, it seems to be straightforward to define an access unit to contain one coded picture (having a certain value of nuh_layer_id). In fact, we believe that no changes in the definitions of an access unit and a coded picture are needed. On the contrary, it seems to us that defining an access unit to contain all VCL NAL units of a certain capture/output time would require re-defining some terms and processes of HEVC v1 for the scalable extensions.

Defining an access unit to contain one coded picture (having a certain value of nuh_layer_id) also enables layers having different decoding/bitstream orders without substantial additions into the SHVC specification text.

As a consequence of the proposed definition of an access unit, it is proposed that nuh_layer_id of the access unit delimiter NAL unit (if present) shall be equal to nuh_layer_id of the VCL NAL units contained in the access unit.

# Layers with different decoding orders

## Background

Nokia’s response JCTVC-K0047 allowed a different GOP length between layers to enable the encoder to have the flexibility to:

- Improve the compression efficiency by having longer GOPs in the enhancement layer than in the base layer.

- Reduce the DPB memory use by having shorter GOPs in the enhancement layer than in the base layer.

An example of an enhancement layer (EL) with twice the GOP size compared to that of the base layer (BL) is provided below.

The decoding order of pictures in this example is: ..., B4, B2, B1, B3, B8, E8, E4, E2, E1, E3, B6, E6, B5, E5, B7, E7, ... The additional DPB memory use is equal to 2 BL pictures (B1, B3) when compared to using the same GOP length (4) in both BL and EL.

Nokia’s CfP responses JCTVC-K0047 and JCTVC-K0040 were otherwise identical but in JCTVC-K0047 longer EL GOPs were used, whereas in JCTVC-K0040 the GOP lengths of BL and EL were identical. Hence, a comparison of the results in JCTVC-K0047 and JCTVC-K0040 provides an indication of the compression efficiency potential of the proposed method.

In section 4 of this contribution we provide results of unequal GOP lengths implemented on top of the common software for SHVC tool experiments.

## Proposal

It is proposed that the definition of the access unit is changed as proposed in section 2. Consequently, only the following changes are needed.

In addition to marking of pictures as “used for reference” and “unused for reference” for inter prediction within a layer, pictures can be marked as “used for inter-layer reference” and “unused for inter-layer reference”. A picture with nuh_layer_id equal to LID is marked as “used for inter-layer reference” immediately after its decoding, if LID is not a leaf layer within the target operation point. Otherwise (the picture is on a leaf layer within the target operation point), the picture is marked as “unused for inter-layer reference”.

A picture is subsequently marked as “unused for inter-layer reference”, when all pictures (of the same POC value and within the same target operation point) that use the picture as inter-layer reference (according to the dependencies provided in the active VPS) have been decoded.

The bumping process of the DPB is modified to remove a picture from the DPB when it is marked as “unused for reference”, “unused for inter-layer reference” and “not needed for output”.

It should be noted that the same reference index could point to a picture with a different POC value at the base and enhancement layers if their reference picture lists are not identical (e.g. when different reference picture list modification is allowed, when reference picture marking is different and/or when the BL and EL GOP lengths are different). Therefore, reference indices from the base layer cannot be directly used for an enhancement layer for inter-layer motion prediction. We use a table that maps the reference indices of the base layer (refIdxBase) to corresponding reference indices of the enhancement layer (refIdxEnh). The table is constructed in such a way that the POC value of the reference picture at index refIdxBase (POC_refIdxBase) in the base reference picture list corresponds to the same POC value of the reference picture at index refIdxEnh in the enhancement reference picture list. If any of the enhancement reference pictures list does not contain a reference picture with the same POC value (POC_refIdxBase) for the same list, then the corresponding base layer MVP is not placed in the merge candidate list.

# Simulation Results

In the performed simulations, the EL GOP was selected to be 16, while the BL configuration was kept unchanged compared to the random access (RA) common test conditions.

We first present the gain of using longer GOP using SHVC 0.1.1 configuration, where the anchor is SHVC 0.1.1 and all the hooks disabled.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3,2% | -6,4% | -7,1% |  |  |  | -3,0% | -5,6% | -6,2% |
| Class B | -1,3% | -6,7% | -8,5% | -1,1% | -6,9% | -8,1% | -0,7% | -4,9% | -6,5% |
| Overall (EL+BL) | -1,8% | -6,6% | -8,1% | -1,1% | -6,9% | -8,1% | -1,3% | -5,1% | -6,4% |
| Overall (EL) | -3,0% | -10,5% | -12,6% | -1,2% | -14,5% | -16,3% | -2,1% | -10,4% | -12,3% |

We also present the gains of using longer GOP when motion hook in SHVC 0.1.1 is enabled. In this case, the reference index from the base-layer is mapped as described in Section 3.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3,1% | -6,2% | -6,9% |  |  |  | -2,6% | -5,3% | -5,9% |
| Class B | -1,1% | -6,4% | -8,3% | -0,8% | -6,5% | -7,6% | -0,3% | -4,8% | -6,5% |
| Overall (EL+BL) | -1,7% | -6,4% | -7,9% | -0,8% | -6,5% | -7,6% | -0,9% | -4,9% | -6,3% |
| Overall (EL) | -2,7% | -9,9% | -12,0% | -0,4% | -13,3% | -15,1% | -1,3% | -10,1% | -12,0% |

The detailed results are provided with the excel sheet included in the zip package of the contribution.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).