| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0172 |
| --- | --- |

| Title: | Non-TE5.1: Cross-check of inter-layer intra prediction mode coding tool in JCTVC-L0260 from MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhijie Zhao Appelstr. 9A Hannover, Germany | Tel: Email: | 0049-511 762 5333 zhao@tnt.uni-hannover.de |
| Source: | Leibniz Universitaet Hannover |  |  |

_____________________________

# Abstract

The document reports the verification results of method 1 in the proposal JCTVC-L0260. It is reported that the BD-rate and PSNR results exactly match those provided by MediaTek.

# Introduction

In JCTVC-L0260  [1], MediaTek proposed to predict the intra mode at enhancement layer with the intra mode of the collocated block at base layer in order to exploit the correlation of intra modes from different layers. The algorithm is described as follows:

For each of modeBL, modeLeft and modeTop, if it is available and is not DC/Planar, it is put into MPM according to the priority (i.e., modeBL>modeLeft>modeTop) with the repetition removed. When three modes are available in MPM, {modeBL, modeLeft, modeTop} are directly used to predict the current intra mode at enhancement layer; otherwise, MPM is further generated as follows. Denote the number of modes available in MPM as numAvai.

- numAvai = 2, MPM = {MPM[0], MPM[1], DC}

- numAvai = 1, MPM = {MPM[0], MPM[0]-1, MPM[0]+1}

- numAvai = 0, MPM = {DC, VER, HOR}

# Simulation Results

The test is performed under the conditions defined in JCTVC-K1105 [2] with AI2x and AI1.5x configurations. The bit rate and PSNR results we got with MediaTek’s software and the results provided by MediaTek are same.

Table 1 Coding performance with MDCS

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.1% | 0.0% |  |  |  |
| Class B | -0.4% | -0.2% | -0.3% | -0.2% | 0.1% | 0.0% |
| Overall (EL+BL) | -0.4% | -0.2% | -0.2% | -0.2% | 0.1% | 0.0% |
| Overall (EL) | -0.6% | -0.3% | -0.3% | -0.4% | 0.2% | 0.2% |
| Enc Time[%] | 101.8% |  |  | 100.9% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2 Coding performance without MDCS

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.1% |  |  |  |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | 0.1% | 0.1% |
| Overall (EL+BL) | -0.2% | 0.0% | 0.0% | -0.2% | 0.1% | 0.1% |
| Overall (EL) | -0.4% | 0.0% | 0.1% | -0.4% | 0.2% | 0.3% |
| Enc Time[%] | 101.8% |  |  | 100.8% |  |  |
| Dec Time[%] | 98.7% |  |  | 99.0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Reference

| [1] | M. Guo, S. Liu, S. Lei, J. Park, J. Kim und B. Jeo, Non-TE5.1: Inter-layer Intra mode prediction, JCTVC-L0260. |
| --- | --- |
| [2] | V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim und H. Yang, „Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer,“ JCTVC-K1105, 2012. |

# Patent rights declaration(s)

Leibniz Universitaet Hannover does not have any current or pending patent rights relating to the technology described in this contribution.