| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0174 |
| --- | --- |

| Title: | Non-TE2: Inter-layer reference picture placement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peng Yin, Taoran Lu and Tao Chen (Dolby) 432 Lakeside Drive, Sunnyvale, CA, 94085, USA Xiaoyu Xiu, Yan Ye (InterDigital) 9710 Scranton R-D, #250 San Diego, CA 92121, USA | Tel: Email: | 1-408-330-3251 1-858-210-4803 pyin@dolby.com Yan.Ye@InterDigital.com |
| Source: | Dolby Laboratories Inc. and InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This contribution provides some additional data based on TE A2 3.2.1: interlayer reference picture placement.  The objective is to reduce the encoder run-time in the AI case. All the changes are simple non-normative encoder modification. Two sets of test results are presented, and compared to the best case in TE A2 3.2.1, RefIdx setting 2 zeroMV. The first set of test results reports a reduction of about 50% in encoder run-time with a BDrate of {0.3% and 0.8%} for Y for {AI 2x, AI 1.5x} in the enhancement layer. The second set of test results reports a reduction of about 25% in encoder run-time with a BDrate of {-0.1% and 0.0%} for Y for {AI 2x, AI 1.5x} in the enhancement layer.  The simulation results also report an improvement on chroma coding performance in all the test cases, AI, RA and LD-P.

# Introduction

The REF_IDX framework is under evaluation in TE A2 [1]. In the AI case, the current SMuC 0.1.1 encoder software requires a run-time that almost doubles the run-time of the simulcast encoding. This contribution presents some simple encoder non-normative changes to significantly reduce the encoder run-time in the AI case. These changes are also applicable to the other test cases in TE A2.

# Proposal

The following changes have been applied to the software used in TE A2 3.2.1 for CU compression. The setting is same as in the case of RefIdx setting 2 zeroMV [2].

- Add a mode decision module for 2Nx2N ILR (Inter-layer reference prediction) with uni-prediction and motion vectors forced to be zero.

- When BL is I_SLICE, two test methods have been configured. In Test Method 1, only intra mode and 2Nx2N ILR mode are tested, and all the other inter modes are not tested. In Test Method 2, only 2Nx2N merge mode, intra mode and 2Nx2N ILR mode are tested.

- When BL is not I_SLICE, two changes are applied. First, in the 2Nx2N inter mode, ME excludes the testing of inter-layer reference picture. Second, the 2Nx2N ILR mode is tested after intra mode.

When BL is I_SLICE and EL is configured to P_SLICE, there is only one reference picture for EL and motion vectors are fixed to ZeroMV. In this case, there is a lot of redundancy in the cost calculation for different inter partition modes and it is not necessary to try all the inter partition modes other than 2Nx2N. To eliminate redundant calculation, we disable all inter-partition search and mode decision, and add a mode decision module for the 2Nx2N ILR uni-prediction with zeroMV. The difference between the two test methods is the inclusion of merge modes. In Test Method 1, all the merge modes are disabled. In Test Method 2, only 2Nx2N merge mode is enabled. This is to show a tradeoff between computational complexity and coding efficiency.

The simulation results also suggest gains in chroma coding efficiency with use of the 2Nx2N ILR mode module. In the REF_IDX framework of current software, the decision for inter-layer texture prediction is made in ME. Since the current software does not consider chroma cost in ME but does that in MD, using the 2Nx2N ILR mode module can also improve chroma coding performance. In this proposal, we simply re-use the 2Nx2N ILR mode module and disable the search in 2Nx2N ME module to avoid redundancy.

# Simulation Results

The proposed algorithm is implemented in the TE A2 3.2.1 software [2]. RefIdx setting 2 zeroMV is used as an anchor. The BDrate results for Test Method 1 and Test Method 2 are shown in the following. Detailed performance data can be found in the accompanying excel files. In the AI case, the encoder run-time is reduced by about 50% in Test Method 1 and by about 25% in Test Method 2, respectively. The results of Test Method 1 show a BDrate of {0.3%, 0.8%} for Y in {AI2x and AI1.5} in the enhancement layer, while the results of Test Method 2 show a BDrate of {-0.1%, 0.0%} for Y in {AI2x and AI1.5} in the enhancement layer. In all the cases, e.g., AI, RA and LD-P, there are coding gains brought to the chroma components.

## Test Method 1

## Test Method 2

# Conclusion

The presented algorithm can significantly reduce the encoder run-time of REF_IDX framework in SMuC 0.1.1 in the AI case. The changes are also applicable to the other cases, and it is shown to improve chroma coding performance under the REF_IDX framework.

# Crosscheck

We would like to thank Liwei Guo from Qualcomm for crosschecking all the results.

# References

- “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Shanghai, China, October 2012.

- X. Xiu, Y. He, Y. He and Y. Ye, “Results of test 3.2.1 on inter-layer reference picture placement”, JCTVC-L0051, Geneva, Switzerland, January, 2013

# Patent rights declaration(s)

Dolby Laboratories Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).