| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Geneva, CH, 14-23 January, 2013 | Document: JCTVC-L0175r2 WG11 Doc: m27511 |
| --- | --- |

| Title: | AHG7: 4:4:4 Lite: A High-Level Syntax Approach to a Consumer 4:4:4 Codec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai, Madhu Krihnan, Pankaj Topiwala 750 N. Atlantic Ave., Cocoa Beach, FL 32931 | Tel: Email: | +1-410-730-6922 pankajtva@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

Techniques for integer color transforms and spatial resampling are developed for HEVC applications, using tools external in HM 9.1 (AHG7 branch).  Low-complexity coding in 4:4:4 format is performed starting/ending with RGB data, and coding performance is measured in terms of average PSNR in RGB. Our tools convert the RGB data to one of several (YUV-like) integer color spaces, and then down sample the data to 4:2:0 prior to encoding/decoding. The decoded data is converted back to RGB 4:4:4, where performance quality is measured. This approach for a baseline for 4:4:4 coding, and forms a low-complexity consumer-oriented coding method against which more sophisticated, native 4:4:4 techniques can be measured in their complexity/performance tradeoffs. Coding of 4:2:2 data is also treated as a by product.

# Spatial Sampling Considerations

Spatial resampling filters are in wide use in image and video processing applications, as there is often a mismatch between the native resolution of coded video data and that of target displays. Spatial resampling is even used within the context of video coding itself, such as in spatial scalability, where a mixed representation requiring multiple resolutions is an explicit part of the coding architecture.  In SVC for example, resampling filters are standardized (more specifically, the upsampling filter, needed in the decoder, is standardized). Note that no matter how well designed, the use of resampling filters nearly always involves a serious loss of information (when applied in contexts other than perfect reconstruction filter banks), which one naturally wishes to minimize. Resampling filters are also regularly used with little notice for color space representations, as most video codecs take YUV 4:2:0 as input, whereas the native representation of acquired video is typically RGB (4:4:4). Similarly to spatial resampling, there is information lost in the color space resampling to YUV 4:2:0 which is forever gone, so that that conversion needs to be of high quality. In fact, for certain applications, YUV 4:2:0 may be entirely inadequate, and working in fully 4:4:4 is required. In the development of HEVC, 4:4:4 coding is already part of the mandate, although an explicit timetable for work in this area has yet to be formulated. This disconnect between 4:4:4 and 4:2:0 has recently emerged as a possible issue on the reflector for the coding of screen content video in HEVC. Likewise, scalability is also of current interest in the HEVC development. In this proposal, we develop and discuss advanced spatial filters which can potentially provide improved quality for both spatial resampling, as well as chroma sampling.

In spatial resampling, one is often faced with having to select filters that perform well in generic situations, and applied in contexts where other processing elements may be unknown.  In particular, if downsampling and upsampling filters are independently developed, and in practice, unknown to each other, they must be both 'generic' low-pass filters.  While each may be optimized in this generic sense, the combination of such generically optimal filters may indeed be quite suboptimal. In this proposal, we present some FIR “nearly low-pass” filter designs for dyadic down-sampling and up-sampling filters, which, when properly co-designed, significantly outperform the generic low-pass filters. The proposed filters have their roots from the wavelet and spline interpolation theories, which has long been established to have excellent interpolation characteristics. Specifically, we propose an 11-tap downsampling filter that is designed to minimize aliasing artifacts in the sub-sampled images. The impulse response of this symmetric filter actually only contains 7 non-zero dyadic coefficients, so its computational complexity is quite modest. In contrast, the high-performance but generic downsampler we compare to is the 13-tap filter used in the SVC design.

On the upsampling side, we propose a dyadic symmetric designs with length 7, which is effectively only a 4-tap filter. It come from a family of interpolative splines called O-MOMS (optimal maximal-order minimal-support splines, designed for best least-square performances [2]). All of the proposed filters have integer coefficients; some with very low dynamic range, and hence, very hardware-friendly. This proposal also illustrates that coding efficiency does not necessarily have to be sacrificed by employing short low-complexity integer-coefficient filters.

# Spatial Sampling Filters

2.1. Downsampling Filter Design

Instead of optimizing the low-pass filter such that its frequency response has steep transition characteristics to match the ideal low-pass box filter, we opt for a smoother, slower-decaying frequency response. In general, we observe that filters which allow a little aliasing (to capture a bit more image information) seem to outperform filters with good anti-aliasing characteristics and good wavelet filters tend to perform well here. We frequently compare our 11-tap filter (h11) with the 13-tap svc filter.

h11 = [1  0  -3  0  10  16  10  0  -3  0  1] / 32.

svc13 = [ 2 0 -4 -3 5 19 26 19 5 -3 -4 0 2] / 64

2.2. Upsampling Filter Design

Generally, we found that filters with good anti-aliasing characteristics with smooth frequency responses (maxflat characteristic) tend to perform well in upsampling.

f7 = [-1     0     5     8     5     0    -1] / 8: 7-tap dyadic-coefficient filter; the equivalence of our length-4 interpolation based on order-3 O-MOMS spline. Again, it is actually a 4-tap,

f4 = [-1 5 5 -1] / 8.

These filters may be compared to the 4-tap filter standardized in the SVC design.

Svc4 = [ -3 19 19 -3] / 32.

Below, we refer to the down/up filter pair (h11, f4) as FV11d4u.

Next, we consider potential applications in improved chroma coding beyond 4:2:0.

# Chroma Sampling Considerations

These ideas in spatial sampling can be applied for chroma sampling directly.  While in principle one could specialize the filters for the different contexts to obtain optimal results, just to prove our point, we use the very same low complexity filters for our chroma sampling work as for spatial.

Our methods can be applied to higher chroma coding in several ways. Here are two:

a) Starting from a higher chroma format (e.g., 4:4:4), one can use our filters to sample down systematically to a lower chroma format (e.g.,  4:2:0), use HEVC v1 for coding, and resample back up to 4:4:4 in the decoder for a very low-complexity, moderate quality 4:4:4 coding method. With the current focus of 10-bit data in the AHG7 effort, we utilize the 10-bit version of HM9.1 in our study. Similar considerations apply for treating 4:2:2 and 4:2:0, wherein one subsamples in only one of two directions (horizontally), though we mainly focus on the 4:4:4 case.  We do not treat the interlace case at this time.

b) With the use of separate colour plane coding, one can sample down the chroma components flexibly as desired (on certain frames, in choice of directions, etc.), with complete freedom.  These methods provide complexity reductions to full chroma coding, but with significant improvements over method (a).

At a minimum, these low-complexity approaches set a clear bar for full chroma coding methods to outperform, and offer justification for their complexity surpluses.  The integer color transforms presented below can in any case be used in conjunction with any higher chroma coding technique.

3.1. RGB 4:4:4 to YUV 4:2:0

Numerous ITU recommendations and ISO/IEC standards begin with discussing chroma sampling, as most profiles in video standards use YUV 4:2:0 as input, while several more advanced profiles may work with 4:2:2 or 4:4:4. Such is the case with ITU/H.264 | ISO/IEC MPEG-4 Part 10 (“AVC”), and will likely be the case in the standardization of HEVC.  The current effort to develop “Part 1” is focused on the case of YUV 4:2:0 chroma sampling as input. However, the process of producing the various chroma samplings from RGB 4:4:4 has not been standardized to date (to the authors' knowledge). In fact, this process generally involves two steps: a color transform (say from RGB to some version of YUV), followed by a sub-sampling in the chroma components.  The example of 4:2:0 sampling is given below.

3.2. Chroma Sampling Experiments

A. Color Transform, Sampling Filters, and Higher Chroma Coding

1. We want to test 4 color transforms (referred generically as ColorSpace444):

a. YCrCb,

b. YCoCg (Microsoft), and

c. YFbFr (FastVDO).

d. YFbFr2 (FastVDO).

2. We also want to test 2 sets of down/upsampling filters.

a. Elementary decimation down, and repeat-sample up (as in the HM9.1/ahg7).

b. Our advanced filters (FV 11d 4u)

We list some key color transforms in a compact linear notation for ease of comparison.

YCbCr  = [.213, .715 .072; -.117, -.394, .511; .511, -.464, -.047]; (Per Rec. 709.)

YUV     = [.299 .587 .114; -.147 -.289 .436; .615 -.515 .1];

DCT      = [.5774 .5774 .5774;.7071 0 -.7071; .4082 -.8165 .4082];

YCoCg  = [.25 .5 .25; 1/2 0 -1/2;-.25 .5 -.25];

YFbFr    = [5/16 3/8 5/16; -1/2 1 -1/2;1 0 -1];

YFbFr2  = [3/16 5/8 3/16; -1/2 1 -1/2;1 0 -1];

These last three are actually lossless integer color transforms, and are implemented in a lossless manner with lifting steps, etc., rather than as matrix multiplies. But this table allows for an easy comparison of their linear, floating-point versions with standard color transforms. We remark that YCoCg is now well established, both in AVC, as well as anticipated in HEVC. The YFbFr and YFbFr2 are just minor variants.

Below, we will refer to any of these as ColorSpace444.

B. Experiments

There are many experiments that would be sensible to conduct in this setting. But we focus here on just one test.

1. RGB -> ColorSpace444 -> ColorSpace420 -> HM9.1/AHG7 Encode/Decode -> ColorSpace444 -> RGB, where the sampling is done with the two test filters (elementary, and FV11d4u), and ColorSpace444 is any of the examples in study.  This tests color transforms and sampling filters.

Here, the YCbCr color space, and the elementary resampling filters serve as anchors.  Any surplus in performance over these anchors indicates a gain available just by using a different colour space and resampling filters, which are both computationally very inexpensive.

C. Test results.

Our test results are provided in included spreadsheets for JCTVC-L0175. We show a consistent gain for all testing configurations, All Intra, Random Access, and Low Delay.

However, it is curious that the EBU test sequences uniquely do not appear to offer any gains.  On the other hand, high coding efficiency is available for these sequences under any of the test configurations, indicating that perhaps these sequences are not very rich in chroma (or at least in chroma differences) to begin with, so that little advantage can be gained by the use of advanced filters.

# Conclusions

Our tests indicate that for coding in the RGB domain with 4:4:4 chroma format, the combination of our colour transforms and resampling filters can permit the reuse of the baseline 4:2:0 codec to provide computationally efficient coding that can still be effective in the 4:4:4 domain.  Since our tools use the barest minimum of what a 4:4:4 codec would need, our approach provides a useful benchmark against which to measure the value of true 4:4:4 coding tools, for their effectiveness, weighed against their complexity surplus.

These same methods also provide an efficient way to deal with 4:2:2 data, by reusing a 4:2:0 codec.

# Draft Text as High-Level Syntax Proposal

We propose the following high level syntax features so that HEVC can provide a lite version of 444 codec via the baseline 420 codec:

- We propose a sequence level flag (lite_444_codec_flag), indicating 420 codec is used to encode the RGB444 input by first converting it to a 420 Chroma format.

- If lite_444_codec_flag presents, color_transform_id is used to specify which color space is used. Available color spaces include ycbcr, ycgco, yfbfr and yfbfr version2.

- If lite_444_codec_flag presents, resampling_filter_id is used to specify which down/up filter pair is used. Available down-up filter pair includes FV11d4u or can be user-defined.

# Proposed Syntax and Semantics

The additions in the sequence parameter set syntax (JVTVC-K1007) are indicated in red:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| if (nuh_layer_id == 0) |  |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| lite_444_codec_flag | u(1) |
| if(lite_444_codec_flag) |  |
| color_transform_id | u(2) |
| resampling_filter_id | u(1) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |

lite_444_codec_flag equal to 1 indicates coding 444 input via 420 codec. Lite_444_codec_flag equal to 0 indicates either of the following situations: 1) InputChromaFormat is not CHROMA_444; and 2) InputChromaFormat is CHROMA_444, and chroma_format_idc is CHROMA_444.

NOTE – When lite_444_codec_flag  is equal to 1, , chroma_format_idc should be CHROMA_420.

color_transform_id specifies color format used. Color_transform_id can be 0, 1, 2, or 3.  Color_transform_id equal to 0 indicates ycbcr. Color_transform_id equal to 1 indicates ycgco. Color_transform_id equal to 2 indicates yFbFr. Color_transform_id equal to 3 indicates yFbFr version 2.

NOTE – color_transform_id present if and only if input video is RGB444 format.

resampling_filter_id specifies down-sampling and up-sampling filters used to convert from CHROMA_444 format to CHROMA_420 format or vice versa. The value of resampling_filter_flag can be 0 or 1. If resampling_filter_id is equal to 0, then downsampling filter is: h11 = [1 0 -3 0 10 16 10 0 -3 0 1] / 32; and upsampling filter is f4 = [-1 5 5 -1] / 8. If resampling_filter_id is equal to 1, then the downsampling and upsampling filters are user-defined, and must be provided with additional input data (via a VUI/SEI).

- Draft Text as VUI/SEI

  - Color Transforms

For the new color transforms, YFbFr, and YFbFr2, we have the following draft text to add to the Draft Spec for HEVC (JCTVC-K1003), in Annex E, currently on page 282:

–	Otherwise, if matrix_coefficients is equal to 11, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-45)

t    = Round( B+(Cr >> 1))                                         	(E-46)

Cb = Round( G − t )        	                                                                                                                (E-47)

Y   = Round( t + (Cb*3) )>>3)		(E-48)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-49)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-50)

NOTE 4 – For purposes of the YFbFr nomenclature used in Table E-1, Cb and Cr of Equations E-49 and E-50 may be referred to as Fb and Fr, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y – (3*( Cb − ( 1 << ( BitDepthC − 1 ) ) )) >> 2	(E-51)

G = Clip1Y( t +2* ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )                                                                                     (E-52)

B = Clip1Y( t − ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-53)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-54)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-55)

t    = Round( B+(Cr >> 1))                                         	(E-56)

Cb = Round( G − t )                                                      	(E-57)

Y   = Round( t + (Cb*3) )>>3)		(E-58)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-59)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-60)

NOTE 5– For purposes of the YFbFr nomenclature used in Table E-1, Cb and Cr of Equations E-57 and E-55 may be referred to as Fb and Fr, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( 3*( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 3 )	(E-61)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-62)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-63)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-64)

–	Otherwise, if matrix_coefficients is equal to 12, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-65)

t    = Round( B+(Cr >> 1))                                         	(E-66)

Cb = Round( G − t )        	                                                                                                                (E-67)

Y   = Round( t + (Cb*5) )>>3)		(E-68)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-69)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-70)

NOTE 6 – For purposes of the YFb2Fr2 nomenclature used in Table E-1, Cb and Cr of Equations E-69 and E-70 may be referred to as Fb and Fr, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y – (5*( Cb − ( 1 << ( BitDepthC − 1 ) ) )) >> 2	(E-71)

G = Clip1Y( t +2* ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )                                                                                     (E-72)

B = Clip1Y( t − ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-73)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-74)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-75)

t    = Round( B+(Cr >> 1))                                         	(E-76)

Cb = Round( G − t )        	                                                                                                                (E-77)

Y   = Round( t + (Cb*5) )>>3)		(E-78)

Cb=  Cb + ( 1 << ( BitDepthC − 1 ) )                                         	(E-79)

Cr = Cr + ( 1 << ( BitDepthC − 1 ) )                                         	(E-80)

NOTE 7 – For purposes of the YFb2Fr2 nomenclature used in Table E-1, Cb and Cr of Equations E-79 and E-80 may be referred to as Fb2 and Fr2, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( 5*( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 3 )	(E-81)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-82)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-83)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-84)

Table E-1 – Matrix coefficients

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | GBR | Typically referred to as RGB; see Equations Error: Reference source not found to Error: Reference source not found IEC 61966-2-1 (sRGB) |
| 1 | KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 (sYCC) IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| 5 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.470-6 System B, G (historical) ITU-R Rec. BT.601-6 625 ITU-R Rec. BT.1358 625 ITU-R Rec. BT.1700 625 PAL and 625 SECAM IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) |
| 6 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.601-6 525 ITU-R Rec. BT.1358 525 ITU-R Rec. BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 5) |
| 7 | KR = 0.212; KB = 0.087 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | YCgCo | See Equations Error: Reference source not found to Error: Reference source not found |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020 non-constant luminance system; see Equations Error: Reference source not found to Error: Reference source not found |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020 constant luminance system see Equations Error: Reference source not found to Error: Reference source not found |
| 11 | YFbFr | See Equations E-45 to E-64 |
| 12 | YFb2Fr2 | See Equations E-65 to E-84 |
| 13..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

5.2.  Draft Text For Resampling Filters as VUI/SEI

Draft text for our resampling filters, presented as VUI and SEI, is included in the related document, JCTVC-L0077 [13].  Draft text for the use of resampling filters as a normative approach to range extensions will be developed upon request.

References

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “High-performance low-complexity re-sampling filters for SVC”, JVT-T041, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 20th meeting, Klagenfurt, AT, Jul. 2006.

- Thierry Blu, Philippe Thevenaz, and Michael Unser, “MOMS: Maximal-order interpolation of minimal support,” IEEE Trans. On Image Processing, vol. 10, No. 7, pp. 1069-1080, Jul. 2001.

- P. P. Vaidyanathan and Bojan Vrcelj, “Biorthogonal partners and applications”, IEEE Trans. on Signal Processing, vol. 49, No. 5, pp. 1013-1028, May 2001.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Extended Spatial Re-sampling Filters For SVC”, JVT-V030, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Dyadic Spatial Re-sampling Filters For SVC”, JVT-V031, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Gary Sullivan, “Resampling filters for SVC Upsampling”, JVT-R066, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 18th meeting, Bangkok, Jan. 2006.

- Pankaj Topiwala and Chengjie Tu, “New Invertible Integer Color Transforms Based on Lifting Steps and Coding of 4:4:4 Video”, JVT-I015r7, San Diego, Sept., 2003.

- Thomas Davies, Pankaj Topiwala, “AHG18: Adaptive Resolution Coding (ARC),” JCTVC-G264, Geneva, CH, 21-30 November, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, JCTVC-F618, Torino, IT, 14-22 July, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Advanced Resampling Filters for HEVC Applications”, JCTVC-G862, Geneva, CH, 14-22 November, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Integer Color Transforms and Resampling Filters for HEVC Extensions: Scalability, 3DV, and Higher Chroma Coding”, JCTVC-J0127, Stockholm, SE, 10-20 July, 2012.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “AHG7: Colour Spaces and Chroma Sampling Methods For Higher Chroma Format Coding”, JCTVC-K0211, Shanghai, CN, 10-19 October, 2012.

- Takeshi Chujoh, et al, “Additional VUI and SEI for chroma sampling filter,” JCTVC-L0077r2, Geneva, CH, 14-23 January, 2013.

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).