| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0182 |
| --- | --- |

| Title: | CE1: Test 1 - Rectangular transform units for 4:2:2 (and AHG7 benchmarks) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal and Information |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Limited |  |  |

_____________________________

# Abstract

This proposal describes the rectangular transform unit structure that is used for 4:2:2 processing in the HM Range Extension model. The model presented is the reference benchmark for CE1 (JCTVC-K1121) and AHG7 (JCTVC-K1006), and therefore, for information purposes, BD-rate comparisons are made against AVC JM18.4 for all formats and configurations. In addition, as requested during the Range Extension BoG JCTVC meeting, a comparison is made against MPEG4 SStP at professional operating points, the aim being to see what HEVC will offer over AVC for this market.

# Introduction

During the 11th JCT-VC conference, a BoG meeting (JCTVC-K0365) on Range Extensions to HM was held, where the C model (JCTVC-K0181), working draft of that C model (JCTVC-K0383), and test conditions and material (JCTVC-K1006) for the development of Range Extensions were agreed.

In addition, a CE (JCTVC-K1121) was set up to investigate the performance characteristics of using square transform units (TUs) for 4:2:2 chroma as an alternative to the rectangular TUs that are used in the Range Extension model.

Following the conference, multiple versions of the Range Extensions model were delivered to the community via the SVN server, as agreed in the development plan. At this time, it was indicated that CE1 should use SVN version 3055, which was based upon JCTVC-K0181, but included the changes necessary for migration to HM9.0.1, the changes in the default configurations, removal of macro-code that was no longer required in the standard model, and the configuration files for the new test sequences.

In parallel, the AHG7 chairman collated, prepared and released the new test conditions and source sequences. The new test conditions, as detailed in JCTVC-K1006, stated that two sets of QPs were to be used (Main-tier = {22,27,32,37} and High-tier = {17,22,27,32}) for the intra-he10, randomaccess_he10 and lowdelay_he10 profiles. An exception was made for the one 12-bit sequence, where the internal processing accuracy would be raised from the standard 10 to 12 bits. The test material is illustrated in Figure 1, Figure 2 and Figure 3.

Traffic (12-bit)	Kimono	EBULupoCandlelight	EBURainFruits

VenueVu (8-bit)	DucksAndLegs	OldTownCross	ParkScene

Figure 1 - First frames of AHG7 test material for RGB 4:4:4. Unless indicated, all are 10-bit source

Traffic	Kimono	EBULupoCandlelight	EBURainFruits

VenueVu	BirdsInCage	CrowdRun

Figure 2 - First frames of AHG7 test material for YCbCr 4:4:4. Unless indicated, all are 10-bit source

Traffic	Kimono	EBUHorse	EBUGraphics

EBUWaterRocksClose	EBUKidsSoccer	Seeking

Figure 3 - First frames of AHG7 test material for YCbCr 4:2:2. Unless indicated, all are 10-bit source

# Rectangular TUs for 4:2:2 Chroma

The use of rectangular TUs for 4:2:2 chroma allows the block processing to be similar to 4:2:0: the number of 4:2:2 chroma TUs is the same as for 4:2:0, the TU quad-tree depths are identical and no additional header syntax elements are required.

The differences between 4:2:2 and 4:2:0 chroma processing are:

- A TU is a rectangular sample array instead of square (but covers the same image region)

- The intra-angular-prediction scales the reference sample step intervals (‘intraPredAngle’ and ‘invAngle’) so as to account for the change in sample aspect ratio (this would also be needed for square TUs).

- The intra-planar-prediction applies a different shift to the top row, again, to account for the aspect ratio of the TU.

- The QP is incremented by 3 prior to quantisation/dequantisation, to account for a √2 factor from the 2D transforms.

- Nx2N scaling lists are formed from horizontally sub-sampled 2Nx2N scaling lists.

# Base Results

## AVC

The base AHG7 Range Extension model ‘HM9.0.1_RExt’ was compared against AVC JM18.4, with both models using an internal bit-depth of 10 (or 12 in the case of Traffic_RGB). In addition, both HM9.0.1_RExt and JM18.4 were compared using 8-bit internal accuracy. (Please see Annex A for configuration of JM18.4).

Note that in both of the above sets of comparisons, JM18.4 sometimes failed to produce a matching set of randomaccess encoder and decoder reconstructed frames. These are highlighted in the accompanying spreadsheets.

HM9.0.1_RExt results have also been included for 4:0:0 mode, using the YCbCr-4:4:4 source material, and the results are provided in a spreadsheet with this document. However, no comparisons against other systems have been made.

Finally, as indicated in the software release, issues regarding RDOQ in combination with 4:2:2 formats discovered during the development process of the software caused large swings in the performance between development revisions to be observed. In accordance with the proposal made to the CE when the software was released, HM9.0.1_RExt results for 4:2:2 with RDOQ disabled (--RDOQ=0 --RDOQTS=0) are also provided within a spreadsheet accompanying this document.

Figure 4 - Comparison of AHG7-ECF8.0 (10/12-bit) vs. AVC JM18.4 (10/12-bit).

Figure 5 - Comparison of AHG7-ECF8.0 (8-bit) vs. AVC JM18.4 (8-bit).

## MPEG4 SStP

During the previous BoG meeting (JCTVC-K0365) on Range Extensions an objective was set to benchmark HEVC against the current widely used HDCAM-SR Professional Format.  Considering this request, benchmark results for the following two HDCAM-SR operating points have been provided:

i.e.

HDCAM-SR	YC 4:2:2	1080p   30P   440 Mbit/sec   SQ Mode

HDCAM-SR	RGB 4:4:4	1080p   30P   880 Mbit/sec   HQ Mode

The HDCAM-SR figures provided are for a CBR (Constant Bit Rate) system. HDCAM-SR uses MPEG4 Part 2 Simple Studio Profile for Compression (from now on abbreviated to SStP).

In addition to the CBR results at the HDCAM-SR operating points, we have provided VBR (Variable Bit Rate) results for SStP using the highest and lowest DCT_Precision along with the full range of Quantisation operating points.

The table below summarises the relative BD-rates of the three VBR codecs (i.e., HEVC, AVC and SStP) around the SNR given for the CBR operating points (within a region of +/- 1dB). It can be seen that at these levels, HEVC is only offering around 5% improvement over AVC. Note that no results are available for ‘Traffic’ sequence, since the format is not available. An accompanying interactive Excel Spreadsheet can be used to explore the response for the three codecs at these and other operating points.

Table 1 - Comparison of codecs at professional operating points. DucksAndLegs has previously been encoded with HDCAM-SR, although the original format is unclear.

## Bit distribution between colour components

During the previous BoG meeting on Range Extensions, a query was asked regarding the distribution of bits between the colour components. By adjusting the decoder to account for every bit (and fractional bit) inserted into the bit-stream (a modification that only increases decoder time by ~4%), statistics of every test condition point have been extracted, and have been included in an accompanying spreadsheet, for information purposes.

The table below summarises the spreadsheet, by indicating the average number of luma (or G) bits per chroma (or B+R) bit.

e.g. if Y were to require 1000 bits, and Cb and Cr were to each require 250 bits, the ratio of luma to chroma would be 1000 : (250+250), i.e. 2.0 : 1.0

Obviously, at lower QPs, the chroma (or B+R) data would be expected to occupy a greater percentage of the bits.

i.e. for intra_he10, at QP 17, 1 chroma bit is present for every 5.54, 1.80, 1.17 and 0.41 bits of luma for 4:2:0, 4:2:2, 4:4:4 and 4:4:4 RGB respectively, while at QP37 the ratios are 10.61, 8.43, 3.88 and 0.57.

Of note, for RGB 4:4:4 sequences, the proportion of bits associated with B+R is more than double that of G, which will be due to the encoder search optimising for the G channel in preference over the B+R channels.

Table 2 – Ratio of bits for Y/G to the other two components.
Bits not associated with any component are not included.

# Working Draft Text

The working draft text is the version as delivered at the end of the last committee meeting, JCTVC-K0383. Note, however, that there may be some issues regarding 4:4:4 separate-colour-plane processing (that have been added to the HM WD), where some chroma settings may instead utilise the luma settings.

# Conclusion

This document presents a comparison of the AHG7 Range Extension source code against AVC JM18.4. This Range Extension model is used as the base comparison point for other CE1 (JCTVC-K1121), CE1-related (non-CE1) and AHG7 contributions.

# Annex A: JM18.4 Configurations

The following settings were used for RGB 4:4:4 simulations:

ProfileIDC              = 244# 244=High 4:4:4

YUVFormat               = 3  # YUV format (0=4:0:0, 1=4:2:0, 2=4:2:2, 3=4:4:4)

WeightedPrediction      = 0  # P picture Weighted Prediction (0=off, 1=explicit mode)

WeightedBiprediction    = 0  # B picture Weighted Prediciton (0=off, 1=explicit, 2=implicit)

RGBInput                = 1

The following settings were used for YCbCr 4:4:4 simulations:

ProfileIDC              = 244# 244=High 4:4:4

YUVFormat               = 3  # YUV format (0=4:0:0, 1=4:2:0, 2=4:2:2, 3=4:4:4)

WeightedPrediction      = 0  # P picture Weighted Prediction (0=off, 1=explicit mode)

WeightedBiprediction    = 0  # B picture Weighted Prediciton (0=off, 1=explicit,  2=implicit)

The following settings were used for YCbCr 4:2:2 simulations:

ProfileIDC              = 122# 244=High 4:2:2

YUVFormat               = 2  # YUV format (0=4:0:0, 1=4:2:0, 2=4:2:2, 3=4:4:4)

In addition, the following settings were used for all 3 cases:

SourceBitDepthRescale   = 1  # Rescale bit depth of source for output (0:Disable 1:Enable)

OutputBitDepthLuma      = <8, 10 or 12>  # Output Bit Depth for Luma component (8...14 bits)

OutputBitDepthChroma    = <8, 10 or 12>  # Output Bit Depth for Chroma components (8...14 bits)

SourceBitDepthLuma      = 10 # except for traffic_rgb(=12) and VenueVu_rgb(=8)

SourceBitDepthChroma    = 10 # except for traffic_rgb(=12) and VenueVu_rgb(=8)

SourceWidth             = 1920 # except for traffic(=2560)

SourceHeight            = 1080 # except for traffic(=1600)

FrameRate               = # as specified in test conditions.

FramesToBeEncoded       = # as specified in test conditions.

StartFrame              = 0

QPISlice                = <base QP>

QPBSlice                = <base QP+1>

QPPSlice                = <base QP+1>

IntraPeriod             = <sequence dependent, as specified in HM configurations>

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).