| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0184 |
| --- | --- |

| Title: | TE3: Results of test 4.2.4 on intra prediction based on difference coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wonkap Jang, Jill Boyce, Adeel Abbas Elena Alshina, Chanyul Kim | Email: | {wonkap, jill, adeel}@vidyo.com elena_a.alshina@samsung.com dionism@samsung.com |
| Source: | Vidyo and Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution describes the difference coding algorithm described in section 4.4.2 of the Tool Experiment 3 description document (JCTVC-K1103), and reports the average bitrate reductions of 1.7%, 2.1%, 2.7%, 3.3%, 1.7%, and 2.7% (EL+BL) in RA 2x, RA 1.5x, LD-P 2x, LD-P 1.5x, RA SNR, and LD-P SNR configurations, respectively. In the difference coding algorithm, difference pixels are created by subtracting the upsampled base layer pixels from the enhancement layer pixels, and motion compensation is performed in the difference pixel domain.

# Introduction

TE3 section 4.4.2 studies the performance of difference coding mode in inter prediction. In the inter difference coding mode, motion compensation is not done using reconstructed reference pictures, but using difference reference pictures. A difference reference picture represents the difference of the reconstructed picture and the (upsampled) base layer for the corresponding picture

# Algorithm Description

A new syntax, named “diff_coding_flag” has been added per CU to indicate whether the block is to be coded in “pixel- domain” or “difference-domain”.  In pixel-domain, motion compensation is done in the same way as in HEVC, whereas in the difference-domain, motion compensation is done using difference pixels, which are generated by subtracting the scaled (upsampled) base layer pixels from the enhancement layer pixels. Fig. 1 illustrates the reconstruction process in difference coding mode. The motion vector coding is not modified for the difference mode, but sub-sample positions are interpolated using a bi-linear filter instead of the 8-tap filter for luma and the 4-tap filter for chroma (which are used for standard inter modes).

Figure 1 Illustration of inter difference coding mode

# Simulation Results

Table 1 Performance of RA 2x and 1.5x for TE3 4.4.2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.4% | -3.4% | -3.3% |  |  |  |
| Class B | -1.8% | -2.3% | -2.3% | -2.1% | -3.8% | -3.9% |
| Overall (EL+BL) | -1.7% | -2.6% | -2.6% | -2.1% | -3.8% | -3.9% |
| Overall (EL) | -3.0% | -5.1% | -5.0% | -5.0% | -9.1% | -9.3% |
| Enc Time[%] | 182.1% |  |  | 170.0% |  |  |
| Dec Time[%] | 136.7% |  |  | 138.0% |  |  |
| Enc Mem[%] | 123.1% |  |  | 122.0% |  |  |
| BL Match | 182.1% |  |  | 170.0% |  |  |

Table 2 Performance of LD-P 2x and 1.5x for TE3 4.4.2

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.2% | -3.9% | -3.7% |  |  |  |
| Class B | -2.9% | -2.5% | -2.1% | -3.3% | -4.5% | -4.4% |
| Overall (EL+BL) | -2.7% | -2.9% | -2.5% | -3.3% | -4.5% | -4.4% |
| Overall (EL) | -4.5% | -5.5% | -5.0% | -7.2% | -10.5% | -10.2% |
| Enc Time[%] | 178.6% |  |  | 165.4% |  |  |
| Dec Time[%] | 138.5% |  |  | 139.6% |  |  |
| Enc Mem[%] | 123.7% |  |  | 122.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3 Performance of RA SNR for TE3 4.4.2

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.0% | -3.7% | -3.8% |
| Class B | -2.0% | -3.7% | -4.0% |
| Overall (EL+BL) | -1.7% | -3.7% | -4.0% |
| Overall (EL) | -3.0% | -7.3% | -7.6% |
| Enc Time[%] | 156.7% |  |  |
| Dec Time[%] | 115.2% |  |  |
| Enc Mem[%] | 116.4% |  |  |
| BL Match | Matched |  |  |

Table 4 Performance of LD-P SNR for TE3 4.4.2

|  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.6% | -4.1% | -4.1% |
| Class B | -3.2% | -5.2% | -5.6% |
| Overall (EL+BL) | -2.7% | -4.9% | -5.2% |
| Overall (EL) | -4.5% | -8.3% | -8.7% |
| Enc Time[%] | 154.9% |  |  |
| Dec Time[%] | 117.6% |  |  |
| Enc Mem[%] | 116.5% |  |  |
| BL Match | Matched |  |  |

# Patent rights declaration(s)

Vidyo Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).