| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC- L0187 |
| --- | --- |

| Title: | HEVC Software modifications for field based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zineb AGYO Jean-Marc THIESSE Jérôme VIÉRON ATEME 26 Burospace, Route de Gisy, 91570 Bièvres, FRANCE | Tel: Email: | +33 16 935 8998 z.agyo@ateme.com jm.thiesse@ateme.com j.vieron@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution describes non-normative changes made to the HEVC reference software (HM 9.1) in order to facilitate the handling of interlaced content in case of field coding.

These changes include field separation at the encoder input so that each field can be compressed separately. This way, the interlaced source can be fed to the encoder directly without any preliminary external processing. At the encoder output side, re-interlacing is achieved and frame based PSNR computation is also added in order to facilitate performance evaluation.

Furthermore, this contribution presents a new GOP structure that is more suitable for interlaced content; hence additional modifications have been made in the software so it can support this kind of structure.

These changes are helpful for assessment of HEVC interlace coding.

# Introduction

In order to perform field-based coding on interlaced content using the HEVC reference software, it is currently necessary to separate the fields externally at the entrance of the encoder, and the outputs (PSNR, reconstructed video) are naturally field based. In order to ease the handling of interlaced sequences, pre-processing and post-processing units have been added to separate the fields before encoding, re-interlaced the reconstructed video and compute the PSNR on the interlaced frames.

In the previous JCT-VC Shanghai meeting, contribution JCTVC-K0216 [1] introduced a GOP structure adapted for field coding in order to rigorously assess the HEVC efficiency on legacy contents. Each top and bottom fields were encoded one after the other, and consequently this allows each bottom field to point on its associate top field. However, limitations of this structure have been pointed out since it is not the case for the last top field of the GOP. A new GOP structure that overcomes this issue is presented in the following, and the software has been modified to allow this structure.

Note: The modifications described in this contribution have been presented within the MPEG “Ad-hoc Group on study of interlaced coding in HEVC” and the proposed software has been used for testing and evaluation.

# Contribution

In the in following, we describe the new encoder/decoder parameters that have been added to the reference software HM 9.1.

These are non-normative modifications to enable efficient coding of legacy interlaced content.

## Additional parameters

### Encoder side

Two additional parameters are introduced:

| Option | Description |
| --- | --- |
| - - FieldCoding | Enable field based coding, if disabled, frame based coding is used. |
| - - TopFieldFirst | Defines field parity order; if set to 1, content is considered as top field first, if set to 0, content is considered as bottom field first (default is bottom field first) |

Note that these parameters can be specified in the command line (e.g. TAppEncoder.exe -c Cfg_file.cfg --FieldCoding --TopFieldFirst) or in the cfg file (see Annex 1).

### Decoder side

Since SEI based field coding signalization is not yet implemented in current HM9.1, we suggest to add the parameter “--FieldCoding” to the command line while decoding (e.g. TAppDecoder.exe -b Bin_file.265 --FieldCoding) in order to decode field-based coded bit-stream. This enables similar modification at the decoder side and allows reconstruction of interlaced sequence. If this parameter is not specified the bit-stream is considered as frame-based coded by the decoder.

## Input/Output handling

In case of field based coding, the encoder takes the interlaced video as an input. The pre-processing unit separates the fields according to the specified field parity (“--TopFieldFirst”) before each field is fed to the encoder as a picture to be compressed.

Likewise, the reconstructed video is re-interlaced back before writing the output reconstructed file.

In case of interlaced sequence input with incompatible field size, it is essential to add line padding so the HM can accept it. This can be done by setting the parameter CroppingMode to 1. Modifications have been added to the code so this option works for field case as well.

Note that since so far no signalization specifies the content type (progressive or interlaced), the reconstructed video at the decoder side is consequently field-based (not interlaced) in case of field based coding.

## Interlaced PSNR computation

In case of field based coding, this function computes the PSNR on the interlaced frames which is then displayed in the output log file. The interlaced PSNR over all interlaced frames is computed as well.

## New GOP structure

In the GOP structure described in contribution JCTVC-K0216 [1] (cf figure 1), the last top field of the GOP was coded separately from its related bottom, and we start the next GOP by an orphan bottom field which is not suitable. To overcome this issue, an improved GOP structure is proposed (figure 2) that is more suitable for interlaced content.

The first two fields are compressed separately (I and B), and the same GOP as the one described in JCTVC-K0216 [1] starts at field 2. Thus, GOP always ends by a bottom field, and the correlation between top and bottom fields are fully exploited. References used for each picture are given in more details in Annex 1.

.

Figure 1: GOP structure proposed in JCTVC-K0216.

Figure 2: Proposed improved GOP structure.

# Results

In order to validate the proposed GOP structure, tests have been run on several sequences using the proposed GOP of 16 fields defined in figure 2 and compared to the classical non-improved GOP of 8 fields.

## Experimental settings

Test sequences have been chosen to be consistent with broadcast applications. Three HD sequences (1920x1088/30i) specified within the MPEG interlaced Ad Hoc Group have been considered:

# "Basketball HD" (450 frames)

# "Tennis HD" (300 frames)

# "MacysParade_Cheerleaders1 HD" (299 frames)

And one SD sequence (720x480/30i) which is a “classical” test sequence has been considered as well:

# "Mobile and Calendar SD" (first 200 frames)

Table 1: Coding configuration

|  | Non-optimized classical GOP | Proposed GOP figure 2 |
| --- | --- | --- |
| Intra Period | 96 | 96 |
| GOP Size | 8 fields | 16 fields |
| QP | {27, 32, 37, 42} | {27, 32, 37, 42} |

## Results

The following table reports the BD-rate performance obtained while comparing the new GOP structure to the classical GOP of 8 fields:

Table 2: BD-rate performance comparing the proposed GOP structure to the classical GOP structure of 8 fields

|  | BD-rate |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Basketball HD | -0,3% | -17,3% | -7,0% |
| Tennis HD | -17,3% | -18,8% | -20,0% |
| MacysParade_Cheerleaders1 HD | 4,1% | 15,9% | 12,6% |
| Mobile and Calendar SD | -21,3% | -21,9% | -22,4% |
| Overall | -8,7% | -10,6% | -9,2% |

As we can see from the table above, the proposed GOP structure provides performance gain comparing to the classical GOP structure which is not suitable for field coding; especially for interlaced sequences where frame coding is more efficient than field coding like “Mobile and calendar” where we observe a 21% gain.

Therefore, it is recommended to use this GOP structure to improve field coding efficiency.

## Additional results

It was observed after testing this proposed GOP structure that it had some incontinency concerning the references used for the first B field after an I field. Indeed, with the last configuration, the first B field after an Intra used only this Intra as reference, while it could have more references from the previous B fields as it is the case for progressive coding with classical GOP structures.

This problem has been fixed in a new version of the software. This version is more efficient and offers not only better BD-rate results but also better visual results, where the transition at each Intra is more fluid.

The following tables show the results of BD-rate performance comparing the new version of the proposed GOP to the old GOP structure of 16 fields described in [1] then to the classical GOP structure of 8 fields.

Table 3: BD-rate performance comparing the new version of the proposed GOP structure to the classical GOP structure of 8 fields

|  | BD-rate |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Basketball HD | -4,3% | -22,6% | -14,3% |
| Tennis HD | -25,6% | -33,0% | -33,2% |
| MacysParade_Cheerleaders1 HD | 1,8% | 11,0% | 8,9% |
| Mobile and Calendar SD | -30,4% | -38,5% | -42,3% |
| Overall | -14,7% | -20,8% | -20,2% |

Table 4: BD-rate performance comparing the new version of the proposed GOP structure to the old GOP structure of 16 fields defined in [1]

|  | BD-rate |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Basketball HD | -2,6% | -3,6% | -4,7% |
| Tennis HD | -8,5% | -14,0% | -13,0% |
| MacysParade_Cheerleaders1 HD | -0,4% | -0,4% | -0,5% |
| Mobile and Calendar SD | -25,7% | -29,3% | -32,8% |
| Overall | -9,3% | -11,8% | -12,8% |

As we can see, the gain after this optimization is quite higher. The average gain over the classical GOP increased from 8% to 14%. Moreover, it is obviously more efficient than the GOP structure of [1] according to table 4. Hence, it is recommended to use this optimized GOP structure for better field coding efficiency.

# Conclusion

It is asserted following the results shown in this contribution that the proposed GOP structure is more suitable for interlaced content and hence more efficient. A gain of performance is indeed observed without adding any intrusive changes. We recommend to adopt those modification of HM software in order to allow efficient interlace material encoding.

Acknowledgments: Authors would like to thank Orange Labs or their kind help and their useful comments regarding the modified software described in this contribution.

# References

[1]	J. Viéron, JM. Thiesse, P. Larbier and, A. Depreux, “HEVC interlaced coding assessment and Chroma issue consideration”, JCTVC- K0216, 11th Meeting: Shanghai, CN, 10–19 Oct. 2012

# Patent rights declaration(s)

ATEME does not have any current or pending patent rights relating to the technology described in this contribution.

# Annex 1

Proposed GOP structure of 16 fields:

#======== File I/O =====================

FieldCoding				: 1		# Enable field based coding (0: Frame based coding, 1: Field based coding)

TopFieldFirst			: 0		# Field parity order (1: Top field first, 0: Bottom field first)

#======== Coding Structure =============

IntraPeriod                   : 96           # Period of I-Frame (-1 = only first)

DecodingRefreshType           : 1            # Random Accesss 0:none, 1:CDR, 2:IDR

GOPSize                       : 16           # GOP Size (number of B slice = GOPSize-1)

#     Type POC QPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 temporal_id #ref_pics_active #ref_pics reference pictures     predict deltaRPS #ref_idcs reference idcs

Frame1:  B    1   1        0.442    0	0 	 0            3        1       3         -1 -3 -4               0

Frame2:  B    16  1        0.442    0 	0 	 0            4        1       4         -15 -16 -18 -19        1      -15       4         1 1 1 1

Frame3:  B    8   2        0.3536   0	0 	 0            2        1       3         -7 -8 8                1       8        5         1 1 0 0 1

Frame4:  B    9   2        0.3536   0	0 	 0            2        1       4         -1 -8 -9 7             1      -1        4         1 1 1 1

Frame5:  B    4   3        0.3536   0	0 	 0            2        1       5         -3 -4 4 5 12           1       5        5         1 1 1 1 1

Frame6:  B    5   3        0.3536   0	0 	 0            2        1       5         -1 -4 3 4 11           1      -1        6         1 0 1 1 1 1

Frame7:  B    2   3        0.3536   0	0 	 0            2        1       6         -1 2 3 6 7 14          1       3        6         1 1 1 1 1 1

Frame8:  B    3   4        0.68     0 	0 	 0            2        0       7         -1 -2 1 2 5 6 13       1      -1        7         1 1 1 1 1 1 1

Frame9:  B    6   3        0.3536   0	0 	 0            2        1       6         -1 -2 -5 2 3 10        1      -3        8         0 1 1 1 1 1 1 0

Frame10: B    7   4        0.68     0	0 	 0            2        0       6         -1 -2 -3 1 2 9         1      -1        7         1 1 0 1 1 1 1

Frame11: B    12  3        0.3536   0	0 	 0            2        1       4         -3 -4 -7 4             1      -5        7         0 1 0 1 1 1 0

Frame12: B    13  3        0.3536   0	0 	 0            2        1       4         -1 -4 -5 3             1      -1        5         1 1 0 1 1

Frame13: B    10  3        0.3536   0 	0 	 0            2        1       5         -1 -2 2 3 6            1       3        5         1 1 1 1 1

Frame14: B    11  4        0.68     0	0 	 0            2        0       6         -1 -2 -3 1 2 5         1      -1        6         1 1 1 1 1 1

Frame15: B    14  3        0.3536   0	0 	 0            2        1       4         -1 -2 -5 2             1      -3        7         0 1 0 1 1 1 0

Frame16: B    15  4        0.68     0 	0 	 0            2        0       5         -1 -2 -3 -6 1          1      -1        5         1 1 1 1 1

ListCombination               : 1           # Use combined list for uni-prediction in B-slices