| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0188r2 |
| --- | --- |

| Title: | Unification of scalable and multi-view extensions with HLS only changes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Miska M. Hannuksela Jani Lainema Dmytro Rusanovskyy Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 50 486 0857 kemal.ugur@nokia.com miska.hannuksela@nokia.com jani.lainema@nokia.com dmytro.rusanovskyy@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In this contribution, a high-level syntax only scalable extension standardization track for HEVC is proposed, in addition to a higher performing track that includes low level changes. It is futher proposed to unify the high level syntax only tracks for scalable and multi-view coding in a single extension of HEVC. The goal is to support both scalable and multiview use-cases with no low level changes to HEVC in a single extension, so that HEVC hardware encoders/decoders could be re-used with firmware updates.

Revision 2 of the contribution includes the following changes:

- It attempts to reflect the decisions of the joint BoG report on extension high-level syntax, as documented in JCTVC-L0441v1.

- The capability of using long-term reference pictures across layers was removed from the proposed straw-man text to be discussed separately as part of contribution JCTVC-L0170.

- The proposed specification text changes were divided into two annexes: Annex F contains the proposed common syntax, semantics, and decoding processes for all types of scalable extensions and scalability mechanisms (particularly reference index based and IntraBL-based). Annex G contains the proposed common syntax, semantics, and decoding processes for HLS-only scalable extensions. This division enables IntraBL-based scalable extension to be specified e.g. as Annex H and refer to the common syntax specified in Annex F.

In revision 1, the straw-man specification text was updated so that changes to HEVC v1 are no longer proposed.

# Introduction

In JCT-3V, the 3D extensions of HEVC are currently being developed under the following tracks [JCT3V-B1006]:

- MV-HEVC: In this extension, multiple views can be coded with HEVC by extending the high-level syntax appropriately, and by rearrangement of decoded picture buffers to store the reference pictures as needed, without any changes to the core of the coding layer below the level of coded tree blocks (CTB).

- 3D-HEVC: In this design the inter-component dependencies between texture and depth and are exploited and texture and depth data are jointly coded.

In JCT-VC, the work on scalable extension of HEVC was started at the last meeting and two main approaches are currently being tested in tool experiments:

- Reference index based: In this approach, the upsampled base layer is made available for EL pictures by placing it in the enhancement layer DPB.

- Block based: In this approach, the base layer samples and syntax can be used to predict the enhancement layer and it is indicated at block level (either CU or PU level).

It is argued that the reference index based approach of SHVC could be supported with small changes to the current MV-HEVC draft text. In addition, it could be further harmonized with the MV-HEVC extension. This way, using a single extension and with no low level changes, both scalability and multiview use-cases can be supported. The benefits of this approach are as follows:

- Reduce market confusion by bringing a single high-level syntax extension instead of multiple extensions.

- Support scalability with high-level syntax only changes.

- Support mixed resolution multi-view coding (as mentioned in MPEG requirements document h, section 2.12.3), which is currently not possible in MV-HEVC.

- Support resolution enhancement of multi-view content (as mentioned in MPEG requirements document N12956, section 2.12.3), which is currently not possible in MV-HEVC and SHVC.

# Proposal

We are proposing:

- Similar to MV-HEVC, have a high level syntax only profile in SHVC extension.

- Unify the HLS-only SHVC with MV-HEVC in a single extension.

We also propose that the standardization track including block level changes for SHVC should continue with a target of achieving a high performing scalable operation point.

The text for this joint extension could be based on the draft MV-HEVC working draft text. The major change is the addition of resampling process, so that if the resolution of a reference view/layer is different, the corresponding pictures are upsampled.

We believe much of the work would be done within JCT-VC and JCT-3V in the upcoming meetings, if this unified extension is seen feasible. To initiate this process, we provide a straw-man specification text in the attached document.

The proposed specification text changes are divided into two annexes: Annex F contains the proposed common syntax, semantics, and decoding processes for all types of scalable extensions and scalability mechanisms (particularly reference index based and IntraBL-based). Annex G contains the proposed syntax, semantics, and decoding processes for HLS-only scalable extensions.

The main features of the proposed Annex F (common syntax, semantics, and decoding processes for all scalable extensions) can be summarized as follows:

- Video parameter set design is taken from JCTVC-L0181.

- Activation for picture and sequence parameter sets for individual layers was added similarly to the SVC and MVC.

- Enhancement layer RAP picture behavior as agreed in response to JCTVC-L0039 (see the BoG notes JCTVC-L0441).

- Non-reference pictures at the highest decoded temporal sub-layer are marked as “unused for reference” immediately after their decoding to enable reduction of the DPB usage.

- The DPB size and other characteristics (e.g. sps_max_num_reorder_pics) is indicated for each layer separately in the active (layer) sequence parameter set for that layer. The DPB operates separately for each layer except for the bumping process of the output order DPB which operates across layers. The bumping process outputs consecutively, in ascending nuh_layer_id order, the pictures having the same smallest POC value among all the pictures (marked as “needed for output”) in the DPB.

- The view dependency change SEI message of MV-HEVC was changed to equivalent layer dependency SEI message, which applies to all types of scalability.

The main features of the proposed Annex G (syntax, semantics, and decoding processes for HLS-only scalable extensions) can be summarized as follows:

- The (short-term) inter-layer reference pictures are deduced from the cross-layer dependencies indicated in the VPS and appended at the end of both initial reference picture lists. This is believed to be functionally identical to how reference picture lists are initialized in MV-HEVC.

- The upsampling process was left unspecified in the straw-man text, as no upsampling filter has been agreed by JCT-VC yet (at the time of the initial submission of this contribution).

- Short-term inter-layer reference pictures are temporarily marked as “used for long-term reference” for the decoding of the current picture and marked back as “used for short-term reference” after the decoding of the current picture, as envisioned in several earlier contributions such as JCTVC-J0071.

When compared to MV-HEVC (JCT3V-B1004), the proposed specification text (when applied to view scalability) is believed to include the same functionality but is claimed to be better up-to-date compared to the latest HEVC specification and additionally includes e.g. the activation process for picture and sequence parameter sets.

- Experimental results

TE2 in SHVC is currently testing the reference index based approach with high-level syntax only changes. The results provided by Inter-digital show that high-level syntax only modifications for SHVC bring useful gains over simulcast (configuration 2 of TE2-3.2.1)

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).