| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0193 |
| --- | --- |

| Title: | Base Layer residue upsampling and skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrick Lopez, Philippe Bordes, Pierre Andrivon, Philippe Salmon 1, av. de Belle-Fontaine – CS17616 35576 Cesson-Sévigné Cedex - France | Tel: Email: | +33 2 99 27 32 64 patrick.lopez@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution presents an algorithm for the scalable extension of High Efﬁciency Video Coding standard (SHVC) where the skip mode of the enhancement layer coding can use the up-sampled residue of the Base Layer to predict the residue of the enhancement layer.

It is reported the average gains of the proposed algorithm with the SMUC 0.1.1 anchor are 1.6% in Luma for Random Access (RA) 2x scalability, 1.8% for RA 1.5x scalability, 0.6% in Luma for Low Delay (LD) 2x scalability, 0.9% for LD 1.5x scalability.

Compared to the SVC_MVP mode, it is reported the proposed algorithm gains in Chroma are 0.1% for Random Access (RA) 2x scalability, 0.2% for RA 1.5x scalability, 0.1% for Low Delay (LD) 2x scalability, 0.1% for LD 1.5x scalability.

# Proposed technique for considering the base-layer residue

The reference software SMUC 0.1.1 provided for SHVC has been released on early November 2012. When this software is used in IntraBL framework, a hook (SVC_MVP) is provided which modifies the merging candidate list building [1].

The co-located  base-layer MV is inserted as the first position in the merging candidate list. The base-layer MV is obtained at a location co-located to the central position of the current block. In the case of spatial scalability, the base-layer MV is scaled according to the spatial resolution ratio between the two layers.

The current proposition considers both the base-layer MV and the base-layer residue in the reconstruction process and defines an alternative Skip mode.

In the case this co-located base-layer (upscaled) MV is selected as MV predictor, the co-located base-layer (upscaled) residue may be added to the motion-compensated PU to form the reconstruction.

In this mode, the reconstruction is computed as  :

Reco = MC(RefEL, Upscale(MVBL)) + Upscale(ResidueBL)

This mode is an alternative skip mode since no residual information needs to be transmitted. However, an information must be transmitted in the bitstream to indicate to the decoder whether the base-layer residue is used when this co-located base-layer MV is selected.

The overlapping of PUBL and PUEL in a picture, is represented on Figure 1.

Figure 1 PUs in base-layer and enhanced layer

In this example, the top left enhancement-layer PU “A” is totally included in a single base-layer PU whereas the middle right enhancement-layer PU “B” overlaps two base-layer PUs.

If “A” uses the co-located base-layer MV in skip mode, an information bit signaling whether the co-located  base-layer residue is used will be inserted in the bitstream.

If “B” uses the co-located base-layer MV in skip mode, it is questionable whether the up-sampled residue of the base-layer can improve the prediction. Since “B” overlaps two PUBL, these two PUBL.may have different MV. Consequently, the base-layer residues have not been obtained by the same MV and applying them in the EL with a potentially different MV makes no sense.

Thus, to save bits, it is more efficient to transmit this information only if the current PU is included in one single base-layer PU.

# Modified “Prediction unit Syntax”

idx_mv_baselayer is defined as the index in the merging candidate list corresponding to the motion vector candidate derived from the base layer colocated PU. If such a candidate is not present in the merging candidate list, for instance if the base layer was coded in Intra mode, idx_mv_baselayer is set to -1.

The new flag use_base_residue_flag is conditionnally read from the bitstream if merge_idx corresponds to idx_mv_baselayer and if cu_single_basis[ x0 ][ y0 ] is true. cu_single_basis[ x0 ][ y0 ] indicates that the current PU is included in one single base-layer PU. If use_base_residue_flag is true, the upscaled residue will be added to the prediction.

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if (merge_idx[ x0 ][ y0 ] == idx_mv_baselayer && cu_single_basis[ x0 ][ y0 ]) |  |
| use_base_residue_flag | ae(v) |
| } else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = Pred_BI) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Table 1 : Prediction unit Syntax

# Modified “Decoding process for the residual signal of coding units coded in inter prediction mode”

Depending on rqt_root_cbf, the following applies:

–	If rqt_root_cbf is equal to 0,

–	If use_base_residue_flag  is equal to 0, all samples of the (nCSL)x(nCSL) array resSamplesL and all samples of the two (nCSC)x(nCSC) arrays resSamplesCb and resSamplesCr are set equal to 0.

–	Otherwise (use_base_residue_flag  is equal to 1) , all samples of the (nCSL)x(nCSL) array resSamplesL and all samples of the two (nCSC)x(nCSC) arrays resSamplesCb and resSamplesCr are derived from the colocated base layer residue after upsamping.

# Results

Table 2 reports the results of the proposal vs the anchor SMUC 0.1.1.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -2.0% | -1.9% |  |  |  |
| Class B | -1.8% | -3.0% | -3.3% | -1.8% | -3.0% | -3.3% |
| Overall (EL+BL) | -1.6% | -2.7% | -2.9% | -1.8% | -3.0% | -3.3% |
| Overall (EL) | -4.1% | -6.7% | -7.2% | -5.0% | -7.7% | -8.5% |
| Enc Time[%] | 96.0% |  |  | 94.4% |  |  |
| Dec Time[%] | 129.3% |  |  | 129.9% |  |  |
| Enc Mem[%] | NA |  |  | NA |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.8% | -0.6% |  |  |  |
| Class B | -0.9% | -1.6% | -1.7% | -0.9% | -1.6% | -1.7% |
| Overall (EL+BL) | -0.6% | -1.3% | -1.4% | -0.9% | -1.6% | -1.7% |
| Overall (EL) | -1.5% | -2.8% | -3.1% | -2.0% | -3.4% | -3.8% |
| Enc Time[%] | 96.8% |  |  | 95.5% |  |  |
| Dec Time[%] | 128.1% |  |  | 131.3% |  |  |
| Enc Mem[%] | NA |  |  | NA |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2 : Proposal vs SMUC 0.1.1

Table 3 reports the results of the the SMUC 0.1.1 by setting SVC_MVP vs the SMUC 0.1.1.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -2.0% | -1.9% |  |  |  |
| Class B | -1.8% | -2.9% | -3.2% | -1.8% | -2.9% | -3.2% |
| Overall (EL+BL) | -1.6% | -2.7% | -2.8% | -1.8% | -2.9% | -3.2% |
| Overall (EL) | -4.1% | -6.5% | -6.9% | -5.0% | -7.5% | -8.1% |
| Enc Time[%] | 95.4% |  |  | 94.2% |  |  |
| Dec Time[%] | 101.6% |  |  | 101.1% |  |  |
| Enc Mem[%] | NA |  |  | NA |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.7% | -0.6% |  |  |  |
| Class B | -0.8% | -1.5% | -1.7% | -0.8% | -1.5% | -1.7% |
| Overall (EL+BL) | -0.6% | -1.3% | -1.4% | -0.8% | -1.5% | -1.7% |
| Overall (EL) | -1.4% | -2.7% | -2.9% | -1.9% | -3.3% | -3.6% |
| Enc Time[%] | 96.2% |  |  | 95.4% |  |  |
| Dec Time[%] | 98.6% |  |  | 101.8% |  |  |
| Enc Mem[%] | NA |  |  | NA |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 3 : SMUC 0.1.1 with SVC_MVP set vs SMUC 0.1.1 anchor

# References

- J. Chen, V. Seregin, M. Zhou, B. Jeon, S. Lei, E. Nassor, K. Ugur, A. Segall, A. Tabatabai, H. Yu, J.W. Kang, Y. Ye, W. Zhang, P. Bordes, “A proposal for Scalable HEVC Test Model,” JCTVC-K0348, Shanghai, October 2012.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).