| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0195 |
| --- | --- |

| Title: | Non-TE4: Switchable De-ringing Filter for Inter-layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma Felix C. A. Fernandes 1301 E. Lookout Drive, Richardson, TX 75082, USA | Tel: Email: | +1-972-761-7134 zhan.ma@sta.samsung.com felix.f@sta.samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution presents a PU-level, switchable, de-ringing, inter-layer filter designed to improve the inter-layer prediction efficiency. It is reported that the BD-rate (EL+BL) changes compared to SMuC-0.1.1 are -0.9%/-0.4% for AI HEVC 2x/1.5x, -0.9%/-0.3%/-0.6% for RA HEVC 2x/1.5x/SNR, and -1.2%/-0.8%/-1.0% for LDP HEVC 2x/1.5x/SNR.

# Introduction

For spatial scalability, the reconstructed picture is one of the most important inter-layer predictions. More specifically, the reconstructed low-resolution picture (referred to as the base-layer picture) is up-sampled to form a prediction for the same picture at a higher resolution level (referred to as the enhancement-layer picture). However, the up-sampled base-layer picture often suffers low quality due to the resolution re-sampling error and the quantization error from base-layer encoding. Consequently, we observe that ringing artifacts are commonly present in the up-sampled base-layer pictures around major edge regions. These artifacts degrade the interlayer prediction performance and reduce the overall coding efficiency of the enhancement layer.

# Proposed De-ringing Filter (DRF)

For each pixel in the up-sampled base layer (BL) reconstruction, the De-Ringing Filter (DRF) is applied to remove noise or to enhance an edge. Currently, it uses a 3x3 template for filtering.

Figure 1 Illustration of 3x3 templates for de-ringing filter

Let I(x) be the pixel intensity at position x to be filtered (as shown in Figure 1), and N(x) be the set consisting of x and its 8 immediate neighbors. Now, for any y in N(x), denote the pixel intensity at y as I(y). After applying the de-ringing filter at position x, the filtered intensity now becomes

where w(||y-x||) is a spatial kernel (as shown in Figure 1(b) or (c)) which is neighboring-pixel distance dependent, and g(||Iy - Ix||) is an intensity kernel which is dependent on the neighboring pixel-intensity difference. The spatial-distance metric ||y-x|| is defined as shown below for y in N(x) and x located at the center:

In the current implementation, different w() kernels are used for luma, chroma,2x, 1.5x and SNR scalability as shown in Table 1. Please note that 1.5x and SNR share the same kernels.

Table 1: w for de-ringing filter

|  | Luma | Chroma |
| --- | --- | --- |
| 2x | W_luma_2x = {{w(-4), w(-3), w(-2)}, {{ 3, 4, 3 }, {w(-1), w( 0), w( 1)}, = { 4, 5, 4 }, {w( 2), w( 3), w( 4)}} { 3, 4, 3 }} | W_chroma_2x = {{w(-4), w(-3), w(-2)}, {{1 , 2 , 1 }, {w(-1), w( 0), w( 1)}, = {2 , 4 , 2 }, {w( 2), w( 3), w( 4)}} {1 , 2 , 1 }} |
| 1.5x/SNR | W_luma_15x = {{w(-4), w(-3), w(-2)}, {{ 1, 4, 1 }, {w(-1), w( 0), w( 1)}, = { 4, 12, 4 }, {w( 2), w( 3), w( 4)}} { 1, 4, 1 }} | W_chroma_15x = {{w(-4), w(-3), w(-2)}, {{1 , 2 , 1 }, {w(-1), w( 0), w( 1)}, = {2 , 16 , 2 }, {w( 2), w( 3), w( 4)}} {1 , 2 , 1 }} |

The intensity kernel g()  is controlled by the intensity difference between pixel x and its neighboring pixel y. Please note that g() is different for luma and chroma. Define z = abs(I(y) – I(x)). Then, g(z) is illustrated in Figure 2.

Figure 2 Illustration of intensity kernel

For simplicity, g() is implemented using a look-up table to control the filtering strength, i.e.,

g_luma[65] = { 64, 61, 54, 44, 33, 23, 15, 9, 5, 2, 1, 0, 0, … … , 0 };

g_chroma[33] = { 8, 4, 2, 1, 0, 0, … … 0 };

Since the reconstructed BL pictures are used for intraBL only in current SMuC, DRF is proposed to be coupled with intraBL mode. In this implementation, if the intraBL flag is true, an additional test is conducted using RDOQ to determine whether DRF is required or not. Therefore, the proposed solution is switchable at prediction unit (PU) level.

# Experimental results

The proposed method is implemented in SMuC-0.1.1. Evaluation is done with the test conditions in TE4 document [1]. Table 2 shows the results for each configuration. It is observed that consistent gains over SMuC-0.1.1 are achieved with slight increase of encoding and decoding time, especially for RA and LDP cases

Table 2: Results of Proposed Switchable De-ringing filter (ref. SMuC-0.1.1)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.7% | -2.1% | -2.3% |  |  |  |  |  |  |
| Class B | -0.6% | -0.6% | -0.6% | -0.4% | -0.4% | -0.4% |  |  |  |
| Overall (EL+BL) | -0.9% | -1.0% | -1.1% | -0.4% | -0.4% | -0.4% |  |  |  |
| Overall (EL) | -1.9% | -2.1% | -2.3% | -1.0% | -1.2% | -1.1% |  |  |  |
| Enc Time[%] | 119.3% |  |  | 117.0% |  |  |  |  |  |
| Dec Time[%] | 115.1% |  |  | 111.6% |  |  |  |  |  |
| Enc Mem[%] | 102.9% |  |  | 104.1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -0.5% | -0.8% |  |  |  | -0.6% | -0.2% | -0.1% |
| Class B | -0.6% | -0.1% | 0.0% | -0.3% | 0.2% | 0.2% | -0.5% | 0.3% | 0.4% |
| Overall (EL+BL) | -0.9% | -0.2% | -0.3% | -0.3% | 0.2% | 0.2% | -0.6% | 0.1% | 0.3% |
| Overall (EL) | -1.8% | -0.6% | -0.7% | -0.7% | 0.2% | 0.3% | -1.0% | 0.1% | 0.3% |
| Enc Time[%] | 106.3% |  |  | 104.9% |  |  | 103.9% |  |  |
| Dec Time[%] | 105.7% |  |  | 104.8% |  |  | 106.5% |  |  |
| Enc Mem[%] | 103.3% |  |  | 104.5% |  |  | 112.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -0.8% | -1.0% |  |  |  | -0.8% | -0.5% | -0.2% |
| Class B | -1.0% | -0.6% | -0.4% | -0.8% | -0.1% | 0.0% | -1.1% | -0.1% | 0.2% |
| Overall (EL+BL) | -1.2% | -0.7% | -0.6% | -0.8% | -0.1% | 0.0% | -1.0% | -0.2% | 0.0% |
| Overall (EL) | -2.3% | -1.4% | -1.2% | -2.2% | -0.5% | -0.2% | -1.9% | -0.7% | -0.2% |
| Enc Time[%] | 106.1% |  |  | 104.8% |  |  | 104.0% |  |  |
| Dec Time[%] | 106.4% |  |  | 107.8% |  |  | 104.9% |  |  |
| Enc Mem[%] | 103.3% |  |  | 104.4% |  |  | 112.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

This contribution presents a PU-level, switchable, de-ringing filter design for inter-layer filtering, so as to improve the inter-layer prediction efficiency. It is reported that the BD-rate (EL+BL) changes compared to SMuC-0.1.1 -0.9%/-0.4%, -0.9%/-0.3%/-0.6% and -1.2%/-0.8%/-1.0% for AI HEVC 2x/1.5x, RA HEVC 2x/1.5x/SNR, and LDP HEVC 2x/1.5x/SNR, respectively. It is proposed to investigate this method in TE/CE as a candidate coding tool for SHVC.

# Reference

[1] J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park " Description of Tool Experiment B4: Inter-layer filtering in SHVC ", JCT-VC K1104, JCT-VC 11th Meeting: Shanghai, 2012

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).