| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0198 |
| --- | --- |

| Title: | TE4: Result of Test of Intra Mode Dependent Directional Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Yan Ye, Jie Dong 9710 Scranton R-D, #250 San Diego, CA 92121 USA | Tel: Email: | +1-858-210-4819 {yuwen.he, yan.ye, jie.dong}@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This proposal described TE-B4 Intra Mode Dependent Directional Filter (IMDDF) experimental results based on TE-B4 document [1].  IMDDF was proposed as one of technologies of the HEVC scalable extension solution from InterDigital Communications [2] in the meeting in Oct. 2012 [3]. The goal of this proposal is to provide a comparison results between IMDDF and the upsampling filters used in the TE software SMuC_0.1.1.

# Introduction

Intra mode dependent directional filter (IMDDF) [2] is used for upsampling by considering the directional information from intra coding mode. It is considered in TE-B4 [1]. This proposal provides the comparison results with the default upsampling filter in SMuC_0.1.1 software.

Figure 1. Block diagram of the proposed scalable system

Figure 1 shows the block diagram of the scalable encoding system at the encoder side where IMDDF applies. IMDDF is used to upsample the base-layer reconstructed picture to generate inter-layer reference (ILR) picture, which will be used as reference for enhancement-layer (EL) encoding. There are two ways to use IRL picture in TE software. One is to generate prediction for INTRA_BL mode when REF_IDX_FRAMEWORK is not enabled. Another way is to add ILR in the reference list for motion compensated prediction when REF_IDX_FRAMEWORK is enabled. This proposal only reports the results with REF_IDX_FRAMEWORK disabled.

In HEVC intra coding, the intra mode (Figure 2) indicates some direction information of local textures. That information can be exploited to enhance directional upsampling process in scalable coding. When the base layer blocks are coded using intra prediction, its prediction mode is used to select an appropriate directional filter accordingly. The directional filters are discussed in section 2.

The following sections will describe IMDDF algorithm, upsampling filter selection and signaling.

# Algorithm description

## Intra mode dependent directional filters (IMDDF)

Figure 2 shows all the intra prediction modes supported in HEVC. For intra coded blocks, their prediction modes are highly correlated to the direction of the textures/edges in the block. In the case of scalable coding, when the corresponding base layer blocks are intra coded, the directionality information provided by their prediction modes can be used to improve the inter layer prediction process. IMDDF is specifically designed to take advantage of this “free” information offered by the base layer to improve enhancement layer coding.

Figure 2. Intra prediction modes in HEVC

As show in Figure 3, there are 4 and 9 relative phases for 2x and 1.5x ratios, respectively. IMDDF consists of a set of poly-phase filters that correspond to each prediction mode. IMDDF filter sizes for 2x ratio and for 1.5x ratio are given in Table 1 and , respectively. In implementation, IMDDF filter is only available for 1.5x and 2x luma component.

- 2x ratio 					(b) 1.5x ratio

Figure 3. Upsampling positions for luma samples

Figure 4 shows IMDDF upsampling process for luma component. If the base-layer picture is encoded as intra slice, then all blocks will be upsampled with IMDDF filter according to each block’s intra mode. If the base-layer picture is encoded as inter slice, the picture is firstly upsampled with default filter in SMuC software, then those areas corresponding to intra blocks at base-layer will be updated with IMDDF filter. The chroma upsampling process only uses the default filter.

Figure 4.  IMDDF upsampling process

Table 1 IMDDF filter size for 2x ratio

| Upsampling positions | Filter size (H×V) |
| --- | --- |
| 0 | 7×7 |
| 1 | 8×7 |
| 2 | 7×8 |
| 3 | 8×8 |

Table 2 IMDDF filter size for 1.5x ratio

| Upsampling positions | Filter size (H×V) |
| --- | --- |
| 0 | 7×7 |
| 1 | 8×7 |
| 2 | 8×7 |
| 3 | 7×8 |
| 4 | 8×8 |
| 5 | 8×8 |
| 6 | 7×8 |
| 7 | 8×8 |
| 8 | 8×8 |

## Picture level upsampling filter selection

In each picture coding at enhancement-layer, the upsampling filter will be selected and signaled at slice header to the decoder. Figure 5 shows the filter selection at encoder side. First the base-layer collocated reconstructed picture will be upsampled with default filter and IMDDF filter. If base-layer collocated picture is intra slice, the selection process will compare the SATD value based on each 8x8 block of upsampled picture. If the block distortion with IMDDF is smaller, it will be counted. If the counter is greater than predetermined threshold, then IMDDF filter will be selected. Otherwise default filter will be selected. If base-layer collocated picture is inter slice, the selection is based the distortion of all intra blocks. If the sum of distortion of all intra blocks with IMDDF filter is smaller, then IMDDF will be selected for upsampling. The distortion is measured in SATD.

Figure 5. Upsampling filter selection for intra slice

# Syntax and semantics description

## Syntax

### Sequence parameter set syntax change

The SPS at enhancement layer will signal the availability information of each upsampling filters. The highlighted are changes in EL SPS RBSP payload.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| ... |  |
| if( layer_id > 0 ) { |  |
| num_of_ups_filters_minus1 | ue(v) |
| for( ups_filter_idx=0; ups_filter_idx<= num_of_ups_filters_minus1; ups_filter_idx++) |  |
| ups_filter_enable_flag[ups_filter_idx] | u(1) |
| } |  |
| ... |  |

num_of_ups_filters_minus1 equal to the number of all upsampling methods minus 1.

ups_filter_enable_flag equal to 1 indicates the corresponding upsamling filtering will be used in sequence decoding, otherwise the filter is not used.

### Slice header syntax change

The upsampling method is signaled in EL slice header. If the number of available upsampling methods is greater than 1, then the upsampling method will be signaled explicitly. Otherwise, the upsampling method will be derived from the availability information parsed from EL SPS.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| ... |  |
| if( layer_id > 0 ) { |  |
| if( NumUpsampleMethod >1 ) |  |
| upsample_method_idc | u(1) |
| } |  |
| ... |  |
| } |  |

upsample_method_idc indicates the upsampling method used to upsample BL collocated reconstructed picture. The mapping between upsample_method_idc and the upsampling filters is shown in Table 3. When upsample_method_idc is not present, it is inferred from ups_filter_enable_flag information in SPS.

Table 3. Indication of upsampling method

| Index | Name |
| --- | --- |
| 0 | DEFAULT |
| 1 | IMDDF |

# Compression performance discussion

The compression performance is measured using BD rate as required by the TE group. There are three coding configurations: intra-only, random access, low-delay P. The anchor is generated with SMuC_0.1.1 default upsampling filters (IMDDF_ENABLE is set to 0). The test results are IMDDF enabled (IMDDF_ENABLE is set to 1). The BD rate results are calculated with the template released by TE group. Readers are referred to the accompanying spreadsheet for further details.

## Intra-only configuration

Table 4 shows the average BD rate reduction of the proposed IMDDF upsampling for spatial scalability using intra only configuration. Compared to the HEVC anchors, BD rates of -0.1% and -0.2% are achieved for the luma component for 2x and for 1.5x considering overall bitrate (BL+EL), respectively. BD rates of -0.2% and -0.6% are achieved for the luma component for 2x and for 1.5x considering EL bitrate only.

Table 4. Average BD rate reduction of the proposed system for intra-only spatial scalability

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% |
| Overall (EL+BL) | -0.1% | -0.1% | -0.1% | -0.2% | -0.3% | -0.2% |
| Overall (EL) | -0.2% | -0.2% | -0.1% | -0.6% | -0.7% | -0.7% |
| Enc Time[%] | 96.7% |  |  | 104.0% |  |  |
| Dec Time[%] | 97.4% |  |  | 105.9% |  |  |

## Randomaccess configuation

Table 5 shows the average BD rate reduction of the IMDDF upsampling for spatial and SNR scalability using random access setting. Compared to anchors, no bitrate reductions are achieved because the usage of intra mode at BL is very low and IMDDF can only take effect to those BL intra blocks. For RA HEVC SNR, IMDDF upsampling filter is not applied.

Table 5. Average BD rate reduction of the proposed system for spatial scalability

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94.6% |  |  | 105.2% |  |  | 103.1% |  |  |
| Dec Time[%] | 95.7% |  |  | 106.2% |  |  | 103.8% |  |  |

## Low-delay P configuration

Table 6 shows the average BD rate reduction of the proposed system for spatial and SNR scalability using low-delay P configuration. Compared to anchors, no bitrate reductions are achieved. For LD-P HEVC SNR, IMDDF upsampling filter is not applied.

Table 6. Average BD rate reduction of the proposed system for SNR scalability

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96.9% |  |  | 103.2% |  |  | 98.5% |  |  |
| Dec Time[%] | 96.2% |  |  | 102.4% |  |  | 100.1% |  |  |

## Overall

The IMDDF upsampling can improve coding performance for intra-only configurations compared to default upsampling filter. For random access and low-delay P configurations, it does not take much effect because few intra blocks are used in inter slices at base-layer, and IMDDF only applies to the upsampling process of base-layer intra blocks. For RA SNR and LD-P SNR encoding configurations, IMDDF is not applied. Compared to anchors, the encoding time varies from 94.6% to 105.2% with different configurations, and the decoding time varies from 95.7% to 106.2% on average.

# Memory usages

There are no additional memory spaces allocated for IMDDF upsampling process at encoder and decoder side. IMDDF upsampling reuses those existing allocated memories.

# Patent rights declaration(s)

InterDigital Communications, LLC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J.Chen, A. Segall, E. Alshina, S. Liu, J. D,  J.Park, “Description of Tool Experiment B4: Inter-layer filtering”, JCTVC-K1104, Oct. 2012.

- J. Dong, Y. He, Y. He, G. McClellan, E-S. Ryu, X. Xiu, Y. Ye, “Description of scalable video coding technology proposal by InterDigital Communications”, JCTVC-K0034, Oct. 2012.

- ISO/IEC JTC-1/SC29/WG11 w12957, Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC), July 2012.