| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14-23 Jan., 2013 | Document: JCTVC-L0199 |
| --- | --- |

| Title: | Non-TE1: Fixed Upsampling Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenzhong Chen, Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | +1-408-5261899 { zhenzhong.chen, shan.liu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution provides the new upsampling filters for SHVC. The filter coefficients are 2D separable DCT based upsampling filter coefficients according to the phase shift downsampling method and the sum of coefficients is 128, i.e., 7 bit precision. The 8-tap filter is used for luma, and 4-tap filter is used for chroma. Experimental results with simulcast anchor show luma BD-rate reductions of 22.8% for AI-2x, 32.7% for AI-1.5x, 16.4% for RA-2x, 26.2% for RA-1.5x, 12.3% for LD-P-2x, and 21.9% for LD-P-1.5x. The encoding time increases are 11.3% for AI-2x, 10.1% for AI-1.5x, 8.0% for RA-2x, 9.8% for RA-1.5x, 7.7% for LD-P-2x, and 10.1% for LD-P-1.5x. The decoding time increases are 4.7% for AI-2x, 5.4% for AI-1.5x, 23.9% for RA-2x, 26.5% for RA-1.5x, 28.8% for LD-P-2x, and 35.6% for LD-P-1.5x.

- Introduction

In scalable video coding, the reconstructed BL samples are up-sampled to generate the predictor for co-located EL samples, as shown in Error: Reference source not found. The encoder provides the difference information in the enhancement bit stream and adds the difference to the prediction. In HEVC based scalable extension, when a coding unit (CU) in the EL is coded as inter-layer texture prediction, the corresponding texture in the BL is up-sampled and used as texture predictors for the CU in the EL, as shown in Figure 2. The up-sampling procedure of generating the inter-layer texture prediction samples is based on the adopted up-sampling filter and phase shift.

# Proposed methods

In this contribution we propose to use separable up-sampling method to up-sampling the reconstructed BL signal for inter-layer prediction where DCT based up-sampling filter is used for which the summation of coefficients is 128, i.e., 7 bits precision. The up-sampling filters are separable filters in horizontal and vertical directions. Both directions use the identical 1-D DCT based up-sampling filter. An 8-tap DCT 1D filter is used for luma, and a 4-tap DCT based 1D filter is used for chroma. The coefficients generation method is similar to HEVC DCT interpolation filter [1] and practically these coefficients are rounded to integer. The filter coefficients are shown in Tables 1 and 2. The phase of down-sampled sequences can be shifted according to the down-sampling method. Based on this, position and filter coefficients for up-sampling are determined by the phase of the down-sampled video sequence.

Figure 1. Scalable Video Coding diagram.

Figure 2. Texture in BL up-sampled and used as texture predictors in EL.

Table 1 The 4-Tap up-sampling filter (7 Bits)

| Phase | Filter coefficients |
| --- | --- |
| 0 | { 0, 128, 0, 0}, |
| 1/4 | { -12, 111, 37, -8}, |
| 1/3 | { -12,100, 49, -9}, |
| 3/8 | { -12, 92, 56, -8}, |
| 1/2 | { -8, 72, 72, -8}, |
| 7/12 | { -11, 63, 91,-15}, |
| 2/3 | { -9, 49,100,-12}, |
| 7/8 | { -4, 20,116, -4}, |
| 11/12 | { -3, 11,124, -4} |

Table 2 The 8-Tap up-sampling filter (7 Bits)

| Phase | Filter coefficients |
| --- | --- |
| 0 | { 0, 0, 0, 128, 0, 0, 0, 0}, |
| 1/3 | {-3, 9, -22, 105, 52, -18, 8, -3}, |
| 1/2 | { -2, 9,-23, 80, 80,-23, 9, -2}, |
| 2/3 | {-3, 8, -18, 52, 105, -22, 9, -3}, |

# Simulation results and discussion

The common test conditions defined in Description of Tool Experiment A-1 [2] are used. In Table 1, experimental results for the proposed fixed upsampling filters are provided. Anchor is generated using the simulcast as defined in [2]. As shown in Table 3, Experimental results with simulcast anchor show luma BD-rate reductions of 22.8% for AI-2x, 32.7% for AI-1.5x, 16.4% for RA-2x, 26.2% for RA-1.5x, 12.3% for LD-P-2x, and 21.9% for LD-P-1.5x. The encoding time increases are 11.3% for AI-2x, 10.1% for AI-1.5x, 8.0% for RA-2x, 9.8% for RA-1.5x, 7.7% for LD-P-2x, and 10.1% for LD-P-1.5x. The decoding time increases are 4.7% for AI-2x, 5.4% for AI-1.5x, 23.9% for RA-2x, 26.5% for RA-1.5x, 28.8% for LD-P-2x, and 35.6% for LD-P-1.5x.

Table 3.  BD rate comparison results of the proposed method with simulcast.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.1% | -27.3% | -28.1% |  |  |  |
| Class B | -21.1% | -21.3% | -21.0% | -32.7% | -33.0% | -33.0% |
| Overall (EL+BL) | -22.8% | -23.0% | -23.1% | -32.7% | -33.0% | -33.0% |
| Overall (EL) | -34.8% | -35.3% | -35.5% | -58.3% | -58.3% | -58.4% |
| Enc Time[%] | 113.1% |  |  | 110.1% |  |  |
| Dec Time[%] | 104.7% |  |  | 105.4% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.9% | -9.3% |  |  |  |
| Class B | -15.5% | -9.2% | -7.8% | -26.2% | -20.4% | -18.8% |
| Overall (EL+BL) | -16.4% | -8.9% | -8.3% | -26.2% | -20.4% | -18.8% |
| Overall (EL) | -25.2% | -13.6% | -12.9% | -47.4% | -38.1% | -35.3% |
| Enc Time[%] | 108.0% |  |  | 109.8% |  |  |
| Dec Time[%] | 123.9% |  |  | 126.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.9% | -3.0% |  |  |  |
| Class B | -11.7% | -7.0% | -5.1% | -21.9% | -16.8% | -15.1% |
| Overall (EL+BL) | -12.3% | -5.5% | -4.5% | -21.9% | -16.8% | -15.1% |
| Overall (EL) | -19.6% | -8.6% | -7.2% | -40.7% | -31.9% | -28.8% |
| Enc Time[%] | 107.7% |  |  | 110.1% |  |  |
| Dec Time[%] | 128.8% |  |  | 135.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusions

This contribution proposed new upsampling filters for SHVC. The filter coefficients are 2D separable DCT based upsampling filter coefficients according to the phase shift downsampling method and the sum of coefficients is 128, i.e., 7 bit precision. The 8-tap and 4-tap filters were used for luma and chroma, respectively. Experimental results with the simulcast anchor show luma BD-rate reductions of 22.8% for AI-2x, 32.7% for AI-1.5x, 16.4% for RA-2x, 26.2% for RA-1.5x, 12.3% for LD-P-2x, and 21.9% for LD-P-1.5x. The encoding time increases are 11.3% for AI-2x, 10.1% for AI-1.5x, 8.0% for RA-2x, 9.8% for RA-1.5x, 7.7% for LD-P-2x, and 10.1% for LD-P-1.5x. The decoding time increases are 4.7% for AI-2x, 5.4% for AI-1.5x, 23.9% for RA-2x, 26.5% for RA-1.5x, 28.8% for LD-P-2x, and 35.6% for LD-P-1.5x.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Elena Alshina, Jianle Chen, Alexander Alshin, Nikolay Shlyakhov, and Woo-Jin Han, “CE3: Experimental results of DCTIF by Samsung,” JCT-VC Document, JCTVC-D344, Jan. 2011.

- Andrew Segall, Jianle Chen, Jie Dong, and Elena Alshina, “Description of Tool Experiment A-1: Upsampling filter”, JCT-VC Document, JCTVC-K1201, Oct. 2012.