| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0200 |
| --- | --- |

| Title: | On VPS extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin, V. George, K. Sühring, T. Schierl Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 314 23881 robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The document proposes a flag in the VPS extension syntax to indicate a sparse allocation of layer_id values that allow identification of dedicated scalable identifiers within layer_id when the so-called mapping approach for layer_id is used.

A second proposal additionally omits transmission of the mapping related syntax elements of the VPS extension when dedicated scalable identifiers within layer_id are present.

# Introduction

The Video Parameter Set (VPS) of HEVC [1] provides high level access to a coded bit stream and contains information that facilitates processing of bit streams at intermediate or end devices. The upcoming scalable [2] and multiview [3] extensions of HEVC will benefit from the VPS extension, e.g. for interpretation of the nuh_reserved_zero_6bits or layer_id in future HEVC extensions.

Two general approaches were considered in the design process [4]. Firstly, mapping layer_id values to scalable identifiers, referred to as mapping approach in the following. Secondly, assigning bits (or blocks of bits) of layer_id to specific scalable identifiers, referred to as bit splitting approach in the following. Both approaches require syntax elements in the VPS extension. While the mapping approach allows a more efficient use of the limited amount of bit in the NAL unit header, it requires more complex signalling and processing compared to the simple bit-splitting approach.

The current syntax of the VPS extension as reported in [5] applies the mapping approach and includes two syntax elements that indicate the types of scalability (scalability_map) and the amount of bit necessary to signal the maximum layer per scalability dimension (dimension_id_len_minus1). These two syntax element happen to be similar to the syntax elements required for the bit splitting approach. The mapping approach requires further syntax elements in the VPS extension syntax, namely the actual value of the scalable identifier coded as u(v) and optionally, flag controlled, the respective value of layer_id, if a sparse allocation of layer_id values in a non-consecutive form is desired (e.g. to emulate a bit-splitting like distribution of layer_id values and achieve dedicated scalable identifiers within layer_id).

In many very likely use cases and application scenarios (e.g. two or three spatial or quality layer, two or four views with or without depth, or many combinations thereof) the mapping approach is superfluous. Apart from the simplicity of the bit-splitting approach that requires no indirection or look up, a number of VPS extension syntax elements does not need to be transmitted and intermediate devices (MANEs, MCU) have to store less information per passing video bit stream.

It is considered unlikely that transport systems like RTP or MPEG2-TS will adopt a scalability signalling in the fashion of a mapping approach but will rather resolve the mapping indirection and produce dedicated scalability identifiers for each scalability dimension which will in turn be explicitly signalled such as in the bit splitting approach.

# Proposal 1

The proposal is to add a flag splitting_flag in the VPS extension syntax to indicate that the mapping approach is used for sparse allocation of layer_id values in a non-consecutive form, i.e. producing dedicated scalable identifiers of blocks of bits of layer_id. Changes compared to [5] are marked in yellow.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| splitting_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| … |  |
| } |  |

splitting_flag with a value equal to 1 indicates that the bits of the layer_id field in the NAL unit header can be split according to the values of dimension_id_len_minus1[] and belong to the respective scalability dimensions signalled in scalability_mask. splitting_flag with a value equal to 0 indicates that there is no meaningful split of layer_id bits into dedicated scalability identifiers apart from the layer identifier.

# Proposal 2

This document proposes to add a flag splitting_flag in the VPS extension syntax to switch between the mapping and the bit-splitting approach. Changes compared to [5] are marked in yellow.

Additionally to proposal 1 in this document, splitting_flag controls presence of the mapping related syntax elements (vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ] and dimension_id[ i ][ j ]).

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| splitting_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| if ( !splitting_flag) { |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

splitting_flag with a value equal to 1 indicates that the bits of the layer_id field in the NAL unit header are split according to the values of dimension_id_len_minus1[] and belong to the respective scalability dimensions signalled in scalability_mask. splitting_flag with a value equal to 1 indicates that the syntax elements vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ] and dimension_id[ i ][ j ] are not signalled. splitting_flag with a value equal to 0 indicates that the syntax elements vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ] and dimension_id[ i ][ j ] are signalled and the bits of layer_id in the NAL unit header are not associated to a specific scalability dimension but mapped to scalable identifiers in the VPS extension.

dimension_id_len_minus1[ i ] when splitting_flag is equal to 0 indicates the length in bits of dimension_id[ i ][ j ]. dimension_id_len_minus1[ i ] when splitting_flag is equal to 1 indicates the number of bits of layer_id in the NAL unit header associated with the i-th scalability dimension as indicated by scalability_mask.

# References

- B. Bross, “High Efficiency Video Coding (HEVC) text specification draft 9“, JCTVC-K1003, Shanghai, CN, Oct. 2012

- “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)“, ISO/IEC JTC1/SC29/WG11 N12957, Stockholm, Sweden, July 2012

- G. Tech et al. ,“Working Draft of MV-HEVC based on JCTVC-J1003_d7“, JCT3V-B1004_d0, Shanghai, CN, Oct. 2012

- Y.-K. Wang, "Solutions considered for NAL unit header and video parameter set for HEVC extensions”, JCTVC-J1007, Stockholm, Sweden, July 2012

- J. Boyce, Y.-K. Wang, "NAL unit header and parameter set designs for HEVC extensions", JCTVC-K1007, Shanghai, CN, Oct. 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).