| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0200 |
| --- | --- |

| Title: | On VPS extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin, V. George, K. Sühring, T. Schierl Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 314 23881 robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The document proposes a flag in the VPS extension syntax to indicate a sparse allocation of nuh_layer_id values that allow identification of dedicated scalable identifiers within nuh_layer_id when the so-called mapping approach for nuh_layer_id is used.

A second proposal additionally omits transmission of the mapping related syntax elements of the VPS extension when dedicated scalable identifiers within nuh_layer_id are present.

A first revision of the document reflects the changes proposed in JCTVC-L0181 that were agreed in a joint session of JCT-VC and JCT-3V with respect to the VPS extension. The proposed semantics were updated to clarify the derivation of scalable identifiers.

A second revision of the document reflects the discussion within the BoG and adds a third proposal to impose a conditional restriction on the semantics of the dimension_id[] syntax element in order to allow for dedicated scalability identifiers within the nuh_layer_id syntax element.

# Introduction

The Video Parameter Set (VPS) of HEVC [1] provides high level access to a coded bit stream and contains information that facilitates processing of bit streams at intermediate or end devices. The upcoming scalable [2] and multiview [3] extensions of HEVC will benefit from the VPS extension, e.g. for interpretation of the nuh_reserved_zero_6bits or nuh_layer_id in future HEVC extensions.

Two general approaches were considered in the design process [4]. Firstly, mapping nuh_layer_id values to scalable identifiers, referred to as mapping approach in the following. Secondly, assigning bits (or blocks of bits) of nuh_layer_id to specific scalable identifiers, referred to as bit splitting approach in the following. Both approaches require syntax elements in the VPS extension. While the mapping approach allows a more efficient use of the limited amount of bit in the NAL unit header, it requires more complex signalling and processing compared to the simple bit-splitting approach.

The current syntax of the VPS extension as reported in [5] and the succeeding document [6] applies the mapping approach and includes two syntax elements that indicate the types of scalability (scalability_map_flag) and the amount of bit necessary to signal the maximum layer per scalability dimension (dimension_id_len_minus1). These two syntax element happen to be similar to the syntax elements required for the bit splitting approach. The mapping approach requires further syntax elements in the VPS extension syntax, namely the actual value of the scalable identifier coded as u(v) and optionally, flag controlled, the respective value of nuh_layer_id, if a sparse allocation of nuh_layer_id values in a non-consecutive form is desired (e.g. to emulate a bit-splitting like distribution of nuh_layer_id values and achieve dedicated scalable identifiers within nuh_layer_id).

In many very likely use cases and application scenarios (e.g. two or three spatial or quality layer, two or four views with or without depth, or many combinations thereof) the mapping approach is superfluous. Apart from the simplicity of the bit-splitting approach that requires no indirection or look up, a number of VPS extension syntax elements does not need to be transmitted and intermediate devices (MANEs, MCU) have to store less information per passing video bit stream.

It is considered unlikely that transport systems like RTP or MPEG2-TS will adopt a scalability signalling in the fashion of a mapping approach but will rather resolve the mapping indirection and produce dedicated scalability identifiers for each scalability dimension which will in turn be explicitly signalled such as in the bit splitting approach.

# Proposal 1

The proposal is to add a flag splitting_flag in the VPS extension syntax to indicate that the mapping approach is used for sparse allocation of nuh_layer_id values in a non-consecutive form, i.e. producing dedicated scalable identifiers of blocks of bits of nuh_layer_id. Interpreting the i-th dedicated scalable identifiers yields the same values as the corresponding syntax elements dimension_id[ i ]. Changes compared to [6] are marked in yellow.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| splitting_flag | u(1) |
| for( i = 0, numScalabilityTypes = 0 ; i <16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| numScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( i = 0; i <numScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| … |  |
| } |  |

splitting_flag with a value equal to 1 indicates that the bits of the nuh_layer_id syntax element in the NAL unit header can be split into segments with a length, in bits, according to the values of dimension_id_len_minus1[] and and that the i-th segment has the same value as the scalability identifier dimension_id[ i ] associated with the i-th scalability dimension as signalled with the i-th scalability_mask_flag equal to 1.

For splitting_flag equal to 1, the following applies:

for( i = 0, DimIdx = 0, MaskBitShift= 0; i < 16; i++ ) {

LayerIdSplittingBitmask[ i ] = 0

if (scalability_mask_flag[ i ] == 1) {

LayerIdSplittingBitmask[ i ] =  (2 ^ ( dimension_id_len_minus1[ DimIdx ] + 1 ) - 1 )  >> MaskBitShift

MaskBitShift = MaskBitShift + dimension_id_len_minus1[ DimIdx++ ] + 1

}

}

For each value of nuh_layer_id the following applies:

- DependencyIdNuh[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 1 ]

- QualityIdNuh[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 2 ]

- DepthFlagNuh[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 3 ]

- ViewIdNuh[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 4 ]

A restriction is imposed that for any value of nuh_layer_id that occurs in the bit stream the following applies:

- DependencyIdNuh[ nuh_layer_id ] == DependencyId[ nuh_layer_id ]

- QualityIdNuh[ nuh_layer_id ] == QualityId[ nuh_layer_id ]

- DepthFlagNuh[ nuh_layer_id ] == DepthFlag[ nuh_layer_id ]

- ViewIdNuh[ nuh_layer_id ] == ViewId[ nuh_layer_id ]

dimension_id_len_minus1[ i ] plus 1 when splitting_flag is equal to 0 indicates the length, in bits, of the dimension_id[ i ][ j ] syntax element. dimension_id_len_minus1[ i ] plus 1 when splitting_flag is equal to 1 indicates the length, in bits, of the dimension_id[ i ][ j ] syntax element and the number of bits of the nuh_layer_id syntax element in the NAL unit header associated with the i-th scalability_mask_flag equal to 1 and its associated scalability dimension.

# Proposal 2

This document proposes to add a flag splitting_flag in the VPS extension syntax to switch between the mapping and the bit-splitting approach. Changes compared to [6] are marked in yellow.

Additionally to proposal 1 in this document, splitting_flag controls presence of the mapping related syntax elements (vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ] and dimension_id[ i ][ j ]).

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| splitting_flag | u(1) |
| for( i = 0, numScalabilityTypes = 0 ; i <16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| numScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( i = 0; i <numScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| if ( !splitting_flag) { |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

splitting_flag with a value equal to 1 indicates that the bits of the nuh_layer_id syntax element in the NAL unit header are split into segments with a length, in bits, according to the values of dimension_id_len_minus1[] and that the i-th segment serves as scalability identifier for the i-th scalability dimension as signalled with the i-th scalability_mask_flag equal to 1. splitting_flag with a value equal to 1 indicates that the syntax elements vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ] and dimension_id[ i ][ j ] are not signalled. splitting_flag with a value equal to 0 indicates that the syntax elements vps_nuh_layer_id_present_flag, layer_id_in_nuh[ i ] and dimension_id[ i ][ j ] are signalled.

For splitting_flag equal to 0, the following applies:

for( i = 0; i <= vps_max_layers_minus1; i++ )

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  !=  0 )  &&  scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

The following applies:

- For each layer indexed by i = 0..vps_max_layers_minus1 + 1

- DependencyId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

- QualityId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 1 ]

- DepthFlag[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][2 ]

- ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 3 ]

For splitting_flag equal to 1, the following applies:

for( i = 0, DimIdx = 0, MaskBitShift= 0; i < 16; i++ ) {

LayerIdSplittingBitmask[ i ] = 0

if (scalability_mask_flag[ i ] == 1) {

LayerIdSplittingBitmask[ i ] =  (2 ^ ( dimension_id_len_minus1[ DimIdx ] + 1 ) - 1 )  >> MaskBitShift

MaskBitShift = MaskBitShift + dimension_id_len_minus1[ DimIdx++ ] + 1

}

}

For each value of nuh_layer_id the following applies:

- DependencyId[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 1 ]

- QualityId[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 2 ]

- DepthFlag[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 3 ]

- ViewId[ nuh_layer_id ] = nuh_layer_id && LayerIdSplittingBitmask[ 4 ]

dimension_id_len_minus1[ i ] plus 1 when splitting_flag is equal to 0 indicates the length, in bits, of the dimension_id[ i ][ j ] syntax element. dimension_id_len_minus1[ i ] plus 1 when splitting_flag is equal to 1 indicates the number of bits of the nuh_layer_id syntax element in the NAL unit header that serve as scalability identifier for the i-th scalability dimension as signalled with the i-th scalability_mask_flag equal to 1.

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

The following applies:

for( i = 0; i <= vps_max_layers_minus1; i++ )

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  !=  0 )  &&  scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

The following applies:

- For each layer indexed by i = 0..vps_max_layers_minus1 + 1

- DependencyId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

- QualityId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 1 ]

- DepthFlag[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][2 ]

- ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 3 ]

# Proposal 3

The proposal incorporates comments received during the BoG discussion. Improved semantics for splitting_flag are proposed as well as a restriction for the semantics of the dimension_id[][] syntax element. Syntax and semantic changes with respect to [6] are highlighted in yellow.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| splitting_flag | u(1) |
| for( i = 0, numScalabilityTypes = 0 ; i <16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| numScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( i = 0; i <numScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| … |  |
| } |  |

splitting_flag equal to 1 indicates that a restriction on the dimension_id[ ] syntax element is imposed and the bits of the nuh_layer_id syntax element in the NAL unit header are split into n segments with a length, in bits, according to the values of dimension_id_len_minus1[ ] and that the n segments are associated with the n scalability dimensions signalled in scalability_mask_flag[ ]. splitting_flag equal to 0 indicates that the no restriction is imposed.

dimension_id_len_minus1[ j ] plus1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

The variable dimBitOffset[ j ] is derived as follows.

dimBitOffset[j]=dimIdx=0jdimension_id_len_minus1[ dimIdx ]	(X-1)

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. When not present, the value of dimension_id[ i ][ j ] is inferred to be equal to 0. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits. When splitting_flag is equal to 1, it is a requirement of bitstream conformance that dimension_id[ i ][ j ] shall be equal to layer_id_in_nuh[ i ] && ( (1 << (dimension_id_len_minus1[ j ] + 1) − 1) >> dimBitOffset[ j ] ).

The following applies:

for( i = 0; i <= vps_max_layers_minus1; i++ )

for( smIdx= 0, j =0; smIdx< 16; smIdx ++ )

if( ( i  !=  0 )  &&  scalability_mask_flag[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

else

ScalabilityId[ i ][ smIdx ] = 0

The following applies:

- For each layer indexed by i = 0..vps_max_layers_minus1 + 1

- DependencyId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 0 ]

- QualityId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 1 ]

- DepthFlag[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][2 ]

- ViewId[ layer_id_in_nuh[ i ] ] = ScalabilityId[ i ][ 3 ]

When splitting_flag is equal to 1, a restriction is imposed that for every value of dimension_id[ i ][ j ] the following condition shall be true.

dimension_id[ i ][ j ] = = layer_id_in_nuh[ i ] && ( (2 ^ ( dimension_id_len_minus1[ j ] + 1 ) - 1 )  >> bitshift[ j ]) 	(X-1)

- References

- B. Bross et al., “High Efficiency Video Coding (HEVC) text specification draft 9“, JCTVC-K1003, Shanghai, CN, Oct. 2012

- “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)“, ISO/IEC JTC1/SC29/WG11 N12957, Stockholm, Sweden, July 2012

- G. Tech et al.,“Working Draft of MV-HEVC based on JCTVC-J1003_d7“, JCT3V-B1004_d0, Shanghai, CN, Oct. 2012

- Y.-K. Wang, "Solutions considered for NAL unit header and video parameter set for HEVC extensions”, JCTVC-J1007, Stockholm, Sweden, July 2012

- J. Boyce, Y.-K. Wang, "NAL unit header and parameter set designs for HEVC extensions", JCTVC-K1007, Shanghai, CN, Oct. 2012.

- J. Boyce, Y.-K. Wang, S. Deshpande, "Proposed VPS extension semantics and editorial clean-ups to syntax", JCTVC-L0181, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).