| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0202 |
| --- | --- |

| Title: | Sign data hiding flag for chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The PPS sign_data_hiding_flag specifies whether sign data hiding is enabled. Sign data hiding provides coding efficiency improvements for luma, while losses are observed for chroma. Therefore, it is proposed to have two flags to enable sign data hiding: one for luma and one for chroma components. This gives encoders more flexibility to select a better complexity/efficiency trade-off.

# Technical description

## Sign Data Hiding PPS Flag

Sign data hiding (SDH) requires more additional complexity at the encoder than at the decoder side, since an encoder has to perform the search of the coefficients to modify. An encoder can disable SDH if its usage doesn’t meet the complexity requirements. To that end, sign_data_hiding_flag is signaled in the picture parameter set (table 7.3.2.3 “Picture parameter set RBSP syntax” of the HEVC specification draft).

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| … | … |
| sign_data_hiding_ flag | u(1) |
| … | … |

The semantics of this flag are the following:

sign_data_hiding_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled.

## Proposal

Since the complexity/performance trade-off of SDH for luma and chroma are highly asymmetrical (see results in section 2), it is proposed to use two flags to enable SDH instead of one:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| … | … |
| sign_data_hiding_luma_flag | u(1) |
| sign_data_hiding_chroma_flag | u(1) |
| … | … |

The semantics being:

sign_data_hiding_luma_flag equal to 0 specifies that sign bit hiding is disabled for the luma component. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled for the luma component.

sign_data_hiding_chroma_flag equal to 0 specifies that sign bit hiding is disabled for the chroma component. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled for the chroma component.

# Simulation results

The propsoed signaling is implemented in HM9.0 and simulations carried under the common test conditions. The performance is compared to HM9.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate saving. Three cases are tested: SDH off (luma and chroma), SDH luma off, and SDH chroma off. Table 1 summarizes these results for the main profile. Losses are observed when SDH is applied to the chroma components.

| BD-rate | HM9.0 SDH off | SDH luma off | SDH chroma off |
| --- | --- | --- | --- |
| AI-Main | 0.85 / 1.43 / 1.44 / 0.99% | 0.67 / 2.15 / 2.15 / 1.04% | 0.19 / −0.72 / −0.69 / −0.03% |
| RA-Main | 0.77 / 0.64 / 0.63 / 0.74% | 0.64 / 1.68 / 1.72 / 0.90% | 0.13 / −1.18 / −1.06 / −0.18% |
| LB-Main | 0.71 / 0.90 / 0.85 / 0.75% | 0.60 / 1.59 / 1.41 / 0.83% | 0.05 / −0.54 / −0.64 / −0.11% |

Table 1. BD-rates for the 3 test cases and 3 configurations: for Y, U, and V components and the average in bold (using weight 1/8 for chroma).

Results for the HE10 configurations are similar to those of the main profile. On the other hand, results for SDH chroma improve when RDOQ is off: weighted BD-rate gains range from 0.05% to 0.2%. Complete results are in the accompanying spreadsheets.

# Conclusions

Given the asymmetric performance observed for luma and chroma, having two flags enabling SDH usage in HEVC would help encoders select better operating points.

# WD text

#### 7.3.2.3 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| … | … |
| sign_data_hiding _flag | u(1) |
| sign_data_hiding_luma_flag | u(1) |
| sign_data_hiding_chroma_flag | u(1) |
| … | … |

#### 7.4.2.3Picture parameter set RBSP semantics

sign_data_hiding_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled.

sign_data_hiding_luma_flag equal to 0 specifies that sign bit hiding is disabled for the luma component. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled for the luma component.

sign_data_hiding_chroma_flag equal to 0 specifies that sign bit hiding is disabled for the chroma component. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled for the chroma component.

#### 7.3.5.11 Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| … | … |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| firstSigScanPos = 16 |  |
| lastSigScanPos = −1 |  |
| … | … |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| … | … |
| if( lastSigScanPos = = −1) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 && !cu_transquant_bypass_flag ) |  |
| … | … |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && !sign_data_hiding_flag ( ( !sign_data_hiding_luma_flag && cIdx==0 ) ¦ ¦ ( !sign_data_hiding_chroma_flag && cIdx > 0 ) ¦ ¦ !signHidden ¦ ¦ n != firstSigScanPos ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag( ( sign_data_hiding_luma_flag && cIdx == 0) ¦ ¦ ( sign_data_hiding_chroma_flag && cIdx > 0 ) ) && signHidden ) { |  |
| sumAbsLevel += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstSigScanPos && ( ( sumAbsLevel % 2 ) = = 1) ) |  |
| TransCoeffLevel[x0][y0][cIdx][xC][yC] = − TransCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).