| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0204 |
| --- | --- |

| Title: | AHG11: DST transform for Intra-BL mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Tzu-Der Chuang Do-Kyoung Kwon Fabrice Le Leannec Patrice Onno | Email: | vseregin@qti.qualcomm.com peter.chuang@mediatek.com d-kwon@ti.com Fabrice.LeLeannec@crf.canon.fr Patrice.Onno@crf.canon.fr |
| Source: | AHG11 |  |  |

_____________________________

# Abstract

This contribution proposes to use forward and inverse DST transform for luma 4x4 transform units coded with Intra-BL mode. Experiments results show 0.4% in AI, 0.2% in RA and 0.1% in LDP luma BD-rate gains for EL+BL, when compare to the SMuC0.1.1 software.

# Introduction

In the SMuC0.1.1 software for HEVC scalable extension, there is a misalignment in forward and inverse transforms for luma 4x4 transform units (TU) in Intra-BL mode. Currently, DCT is used as a forward transform and DST is used as inverse transform.

# Solution

There can be two solutions to the problem with misaligned forward and inverse transforms in Intra-BL mode:

- Use DCT as forward and inverse transforms

- Use DST as forward and inverse transforms

Luma BD-rate for both solutions are summarised in the next table.

|  | AI |  | RA |  |  | LD-P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2x | 1.5x | 2x | 1.5x | SNR | 2x | 1.5x | SNR |
| 1. DCT | -0.3 | -0.3 | -0.2 | -0.2 | -0.2 | -0.0 | -0.0 | -0.1 |
| 2. DST | -0.4 | -0.4 | -0.2 | -0.2 | -0.2 | -0.1 | -0.0 | -0.1 |

# Conclusion

In HEVC, there is no 4x4 DCT transform for luma component coded with intra mode and DST 4x4 is used for all intra directions. It is desirable to keep HEVC design unchanged, so Intra-BL mode should not introduce additional transform core for luma 4x4 TU. Also, according to the results, DST solution is slightly better. Based on these observations, it is recommended to use DST4x4 as forward and inverse transforms for Intra-BL mode.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).