| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0205 |
| --- | --- |

| Title: | TE5: Using base layer MV in Merge and AMVP modes (test 5.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results of the test 5.2.2 in TE5. In the tested methods, base layer motion vector obtained from the center of the co-located block is added as a candidate to the enhancement layer Merge and AMVP candidate list. There are two approaches, in the first one the base layer motion vector candidate is added during candidate list construction and the second one where base layer motion vector candidate is inserted into the completed candidate list. Additionally, results for base layer motion vector derivation with applied motion filed compression and without it are provided.

# Introduction

In HEVC scalable extension, additional motion vector (MV) candidate obtained from the base layer (BL) can be used to generate candidate list for Merge and AMVP modes.

# Solution

In this contribution, BL MV is derived from the center location of the co-located block; this center position is the same as that used for TMVP derivation in HEVC. The maximum number of candidates is kept unchanged; it is five for Merge mode and two for AMVP mode. Additionally, when the spatial resolutions of BL and EL pictures are different, the BL MV is scaled according to the spatial resolution ratio between the two layers.

In HEVC, motion vectors are compressed after being coded and such compressed motion vectors can be later used as TMVP for coding other frames. For better coding efficiency, the uncompressed BL MVs can be used in enhancement layer coding. In other words, motion vector compression for a base layer is performed after it is used for inter-layer prediction.  In this contribution, both methods, using compressed BL MVs and uncompressed BL MVs, are investigated.

In this contribution two solutions are presented:

- Adding BL MV candidate to Merge and AMVP candidate lists during list construction [1].

For Merge mode, available BL MV candidate is added as a first candidate into the list, and pruning is applied between spatial candidates and BL MV. Bottom left spatial candidate is added conditionally to guarantee the spot for TMVP in the candidate list.

In AMVP mode, available BL MV is added as a last candidate in the list, if the existing AMVP list has less than two candidates.

- Inserting available BL MV into the already completed Merge and AMVP candidate lists as a first MV candidate.

Above mentioned two solutions were implemented on top of SMuC 0.1.1, and SMuC 0.1.1 was used as an anchor. The experiments results under test condition specified in TE5 [2] for luma BD-rate for EL+BL rate of the solutions using uncompressed BL motion field are summarised in the next tables.

Table 1. Base layer MV is derived from uncompressed base layer motion field.

| Test |  | RA |  |  | LD-P |  |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x | 1.5x | SNR | 2x | 1.5x | SNR |  |
| Solution 1 | Merge | -2.28 | -2.36 | -2.49 | -1.32 | -1.28 | -2.03 | -1.96 |
|  | AMVP | -1.49 | -1.48 | -1.54 | -1.01 | -0.83 | -1.05 | -1.23 |
|  | Merge+AMVP | -2.42 | -2.49 | -2.61 | -1.44 | -1.35 | -2.11 | -2.07 |
| Solution 2 | Merge | -1.99 | -2.16 | -2.22 | -1.04 | -1.06 | -1.76 | -1.71 |
|  | AMVP | -1.47 | -1.47 | -1.53 | -1.00 | -0.82 | -1.06 | -1.23 |
|  | Merge+AMVP | -2.11 | -2.28 | -2.32 | -1.16 | -1.14 | -1.81 | -1.80 |

Additionally, the results of the methods where compressed motion field of the base layer is used for BL MV derivation are summarised in the next table.

Table 2. Base layer MV is derived from compressed base layer motion field.

| Test |  | RA |  |  | LD-P |  |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x | 1.5x | SNR | 2x | 1.5x | SNR |  |
| Solution 1 | Merge | -1.16 | -1.83 | -2.01 | -0.37 | -0.83 | -1.43 | -1.27 |
|  | AMVP | -0.83 | -1.19 | -1.32 | -0.47 | -0.63 | -0.81 | -0.88 |
|  | Merge+AMVP | -1.17 | -1.93 | -2.11 | -0.39 | -0.87 | -1.46 | -1.32 |
| Solution 2 | Merge | -0.94 | -1.63 | -1.74 | -0.13 | -0.60 | -1.13 | -1.03 |
|  | AMVP | -0.80 | -1.18 | -1.31 | -0.46 | -0.63 | -0.81 | -0.87 |
|  | Merge+AMVP |  |  |  |  |  |  |  |

# Conclusion

In this contribution, two approaches of using base layer motion vector as a candidate in Merge and AMVP candidate lists are investigated. In the first approach, the candidate list generation is modified, however in the second approach candidate list generation is not changed and base layer MV candidate is inserted into completed candidate list. Also, it was noticed that AMVP list changes on top of Merge list modification provide only about 0.1 additional performance improvement for both solutions.

# References

[1] J. Chen, et al. “Description of scalable video coding technology proposal by Qualcomm (configuration 1)”, JCTVC-K0035, Shanghai, China, Oct. 2012.

[2] V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, and H. Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, China, Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).