| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0206 |
| --- | --- |

| Title: | TE3: Combined inter mode (test 4.6.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results of the combined inter mode (method 4.6.2.2) tested in TEB3. The prediction in this mode is defined as an average of two predictions which are done in Intra-BL mode and enhancement layer inter prediction mode. Experiments results show about 1.2% for luma and 3.9% for both chroma components performance improvement for EL+BL rate on average among RA 2x, RA 1.5, RA SNR, LDP 2x, LDP 1.5x and LDP SNR test configurations with about 113% encoder complexity on average.

# Introduction

Under the framework of multi-loop decoding, the reconstructed base layer pixel data can be available for enhancement layer coding. And this additional information can be used to improve the enhancement layer prediction. In this contribution, the reconstructed base layer pixels or Intra-BL prediction is weighted with the inter prediction to form a new prediction mode called combined mode.

# Solution

The combined prediction mode [1] is applied to inter predicted CUs. The prediction in this mode is defined as an average of two predictions which are done in intra-BL mode and enhancement layer inter prediction mode. Let Pb denote intra-BL prediction and Pe denote enhancement layer inter prediction. The final prediction P for the CU is defined as

This method is applied only to the luma component. The enhancement layer inter prediction is used for chroma components when the combined prediction mode is enabled. The combined_mode_flag is used to indicate the usage of the mode at CU level.

Combined inter prediction mode was implemented on top of SMuC 0.1.1, and SMuC 0.1.1 was used as an anchor. The experiments results under test condition [2] are summarized in the next table.

# References

[1] J. Chen, et al. “Description of scalable video coding technology proposal by Qualcomm (configuration 1)”, JCTVC-K0036, Shanghai, China, Oct. 2012.

[2] X. Li, E. Francois, P. Lai, D. Kwon, and A. Saxena, “Description of Tool Experiment B3: Combined Prediction in SHVC”, JCTVC-K1103, Shanghai, China, Oct. 2012.

# Specification sample

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] [Ed. (GJS): Syntax element name convention violation – subset of name transform_skip_flag.] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| combined_mode_flag |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( PredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| combined_mode_flag |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).