| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0207 |
| --- | --- |

| Title: | TE5: Cross-verification of the test 5.2.5 on base layer MV candidate |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-L0032 on inter-layer motion vector prediction. It is confirmed that implementation is consistent with the proposal description and the experiment results are matched with the proponent data.

# Introduction

The method [1] verified in this contribution is the test 5.2.5 of TE5 [2] on inter-layer motion prediction. In the studied method base layer (BL) motion vector (MV) derived from the co-located base layer PU is added as a first motion vector candidate into the Merge candidate list. The implementation details are the following:

- Base layer MV is derived from the center of the co-located block

- Base layer MV is added as a first candidate into the candidate list

- Spatial MV scaling is performed by reusing the TMVP scaling function

- Check on the spatial candidate is performed to guarantee TMVP in the candidate list

- Uncompressed base layer motion field is used for BL MV derivation (*)

- Redundancy check is performed between spatial candidates and BL MV candidate (*)

The results without last two modifications (*) were additionally provided for this method and also verified in this contribution.

# Experimental results

The software based on SMuC0.1.1 was distributed among TE5 participant by the proponent. It was compiled and executed on Linux cluster. The simulation results based on the common test conditions described in TE5 and released by AHG10 [3] were compared against anchor software SMuC0.1.1, and summarised in the following table. The more detailed results can be found in the accompanied excel tables.

| Test |  | RA |  |  | LD-P |  |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x | 1.5x | SNR | 2x | 1.5x | SNR |  |
| Uncompressed base layer motion field | With check | -2.32 | -2.36 | -2.49 | -1.35 | -1.28 | -2.03 | -1.97 |
|  | Without check | -2.02 | -2.14 | -2.20 | -1.05 | -1.04 | -1.74 | -1.56 |
| Compressed base layer motion field | With check | -1.18 | -1.83 | -2.01 | -0.39 | -0.83 | -1.43 | -1.28 |
|  | Without check | -0.92 | -1.61 | -1.71 | -0.12 | -0.57 | -1.13 | -1.01 |

# References

[1] D. Bugdayci, and K. Ugur, “TE5: Results of 5.2.5 on inter-layer motion vector prediction”, JCTVC-L0032, Geneva, CH, Jan. 2013.

[2] V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, and H. Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, China, Oct. 2012.

[3] X. Li, E. Francois, P. Lai, D. Kwon, and A. Saxena, “Description of Tool Experiment B3: Combined Prediction in SHVC”, JCTVC-K1103, Shanghai, China, Oct. 2012.