| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0208 |
| --- | --- |

| Title: | AHG9 / HEVC v1: Updated text of the SOP description SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This contribution reportedly proposes:

- A definition of a structure of pictures (SOP).

- To update the specification text related to structure of pictures (SOP) description SEI message to reflect the latest HEVC specification text.

- To enable description of multi-picture SOPs starting from an IDR picture.

- To specify value ranges for the syntax elements for the SOP description SEI message.

- Editorial changes to the specification text related to the SOP description SEI message.

# Introduction

The structure of pictures (SOP) description SEI message was adopted in the HEVC standard from contribution JCTVC-H0423. In the latest HEVC specification text, the text related to the SOP description SEI message has certain claimed shortcomings, which this contributions attempts to solve. More specifically, this contribution proposes:

- During the text integration, the definition of an SOP was not copied from JCTVC-H0423 to the specification text. This contribution proposes the definition of a SOP. It is asserted that the definition is the same as in JCTVC-H0423 but is updated to the latest HEVC terminology.

- In order to reflect the JCT-VC decisions on the removal of nal_ref_flag and the use of nal_unit_type to indicate different types of pictures, sop_desc_nal_ref_flag[ i ] is proposed to be replaced with sop_desc_vcl_nalu_type[ i ], which contains a copy of the nal_unit_type value of the i-th picture in the described SOP.

- It is proposed that sop_desc_st_rps_idx[ i ] which is the short-term reference picture set index for the i-th picture in the described SOP is present for non-IDR pictures only. This change enables the description of multi-picture SOPs starting from an IDR picture.

- The allowed value ranges for all the syntax elements are proposed.

In addition, several editorial suggestions are proposed.

# Proposed specification text

The proposed additions are indicated with red font color. The text proposed to be removed is indicated with strikethrough and red font color.

3.x	structure of pictures (SOP): One of more coded pictures consecutive in decoding order, among which the first coded picture in decoding order has TemporalId equal to 0 and is not a sub-layer non-reference picture and among which no coded picture except potentially the first coded picture in decoding order is a RAP picture.

# 4	Abbreviations

…

SOP	Structure of pictures

D.1.18	SOP description SEI message syntax

| sop_description( payloadSize ) { | Descriptor |
| --- | --- |
| sop_seq_parameter_set_id | ue(v) |
| num_pics_in_sop_minus1 | ue(v) |
| for( i = 0; i <= num_pics_in_sop_minus1; i++ ) { |  |
| sop_desc_nal_ref_flag[ i ] | u(1) |
| sop_desc_vcl_nalu_type[ i ] | u(6) |
| sop_desc_temporal_id[ i ] | u(3) |
| if( sop_desc_vcl_nalu_type[ i ] != IDR_W_DLP && sop_desc_vcl_nalu_type[ i ] != IDR_N_LP ) |  |
| sop_desc_st_rps_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| sop_desc_poc_delta[ i ] | se(v) |
| } |  |
| } |  |

- D.2.18	SOP description SEI message semantics

The SOP description SEI message indicates constraints that apply in the described SOP that starts with the current access unit. [Ed.(GJS): Define SOP – what is it?]

The SOP description SEI message shall not be present in any access unit with TemporalId greater than 0.

When an SEI NAL unit that contains a SOP description SEI message and has nuh_reserved_zero_6bits equal to 0 is present, the SEI NAL unit shall precede, in decoding order, the first VCL NAL unit in the access unit.

sop_seq_parameter_set_id specifies the sps_seq_parameter_set_id value for the sequence parameter set. The value of sop_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

num_pics_in_sop_minus1 + 1 specifies the number of pictures in the described SOP. num_pics_in_sop_minus1 shall be in the range of 0 to 1023, inclusive.

sop_desc_nal_ref_flag[ i ] equal to 0 indicates the i-th picture in decoding order within the SOP has a nal_unit_type equal to TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12, or RSV_VCL_N14. [Ed. TK. This was changed since nal_ref_flag has been removed. Perhaps sop_desc_nal_ref_flag[ i ] can be removed if no longer useful.]

sop_desc_vcl_nalu_type[ i ] specifies the nal_unit_type value of the i-th picture in decoding order within the SOP.

sop_desc_temporal_id[ i ] specifies the TemporalId value of the i-th picture in decoding order within the described SOP. The value of 7 for sop_desc_temporal_id[ i ] is reserved for future use by ITU-T | ISO/IEC.

sop_desc_st_rps_idx[ i ] specifies the short-term reference picture set included in the sequence parameter set identified by sop_seq_parameter_set_id and used by the i-th picture in decoding order within the described SOP. sop_desc_st_rps_idx[ i ] shall be in the range of 0 to 63, inclusive.

sop_desc_poc_delta[ i ] specifies the difference between the picture order count values of the i-th picture in decoding order within the described SOP and the (i−1)-th picture in decoding order within the described SOP. sop_desc_poc_delta[ i ] shall be in the range of ( -MaxPicOrderCntLsb ) / 2 + 1 to MaxPicOrderCntLsb / 2 – 1, inclusive, when MaxPicOrderCntLsb is derived from currSeqParamSet, which is specified below.

The bitstream may not contain all the pictures described in a SOP description SEI message.

NOTE – For example, the bitstream may have been subject to TemporalId based sub-bitstream extraction, but pictures that have TemporalId values no longer existing in the extracted bitstream may still be present in the SOP description SEI message.

The following constraints specify that the pictures that are present in the bitstream must match to the information given in the SOP description SEI message. [Ed. (GJS): This paragraph seems normative. Improve phrasing. Does it contradict other statements within the annex?]

The variable picOrderCntExp[ i ] is derived as follows.

picOrderCntExp[ 0 ] = PicOrderCntVal of the current picture
for( i = 1; i <= num_pics_in_sop_minus1; i++ )
	picOrderCntExp[ i ] = picOrderCntExp[ i − 1 ] + sop_desc_poc_delta[ i ]

The variable nalRefFlag[ j ] is specified as follows for the j-th picture in decoding order starting from j equal to 0 for the first picture of the SOP containing the picture associated with the SEI message.

–	If the j-th picture had nal_unit_type equal to TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RSV_VCL_N10, RSV_VCL_N12, or RSV_VCL_N14, nalRefFlag[ j ] is set to 0.

–	Otherwise, nalRefFlag[ j ] is set to 1.

Let sdVclNaluType[ j ], sdTtId[ j ], sdSstRpsIdx[ j ], and sdPpicOrderCntVal[ j ] be the values of nal_unit_type, TemporalId, short_term_ref_pic_set_idxst_rps_idx, and PicOrderCntVal that are in effect for the j-th picture in decoding order starting from j equal to 0 for the first picture of the described SOP containing the picture associated with the SEI message. Let currSeqParamSet be the previous sequence parameter set RBSP in decoding order with sps_seq_parameter_set_id equal to sop_seq_parameter_set_id. long_term_ref_pics_present_flag shall be equal to 0 in currSeqParamSet. [Ed. (GJS): This seems to introduce some confusion with respect to uses of the same variable names in other places in the text. I suggest using subscripts rather than array symbols here.]

It is a requirement of bitstream conformance that when the SOP description SEI message, the following constraints shall apply for each picture i from picture 0 to picture num_pics_in_sop_minus1 when picOrderCntExp[ i ] is equal to sdPpicOrderCntVal[ j ], where j is greater than 0 and sdPpicOrderCntVal[ j − 1 ] is less than or equal to picOrderCntExp[ num_pics_in_sop_minus1 ]:

–	nalRefFlag[ j ] shall be equal to sop_desc_nal_ref_flag[ i ].

–	sdVclNaluType[ j ] shall be equal to sop_desc_vcl_nalu_type[ i ].

–	sdTtId[ j ] shall be equal to sop_desc_temporal_id[ i ].

–	short_term_ref_pic_set_sps_flag shall be equal to 1 for each slice segment header of picture j.

–	sdSsstRpsIdx[ j ] shall be equal to sop_desc_st_rps_idx[ i ], when present.

–	currSeqParamSet shall be the active sequence parameter set RBSP for picture j.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).