| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14 –23 January, 2013 | Document: JCTVC-L0209 WG11 Number: m27546 |
| --- | --- |

| Title: | On Implicit Signaling of Scalability Dimension Identifiers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In current video parameter set extension design dimension Id values for each scalability dimension of each layer are explicitly listed and transmitted. This document additionally proposes a method for implicit signaling of scalability dimension identifiers in video parameter set extension.

# Introduction

HEVC Draft 9 [1] includes the syntax and semantics for signalling video parameter set (VPS).

NAL unit header and parameter set designs for HEVC extensions - JCTVC-K1007 [2] provides a video parameter set design for HEVC Extension.

In current video parameter set extension design [2] dimension Id values (dimension_id[i][j]) for each scalability dimension of each layer are explicitly listed and transmitted.

It is asserted that in many typical situations the dimension Id values for each scalability dimension for each layer can be derived based on signaling a parameter about each scalability dimension. This document proposes such a method for implicit signaling of scalability dimension identifiers in video parameter set extension. The proposed approach also supports current explicit signaling of dimension_id[i][j] values if required by any particular applications.

It is asserted that the main advantages of the proposed approach are that:

- It allows a simple signalling mechanism which does not need to explicitly list all the scalability dimension identifiers for each layer_id.

- It requires fewer bits to signal when the dimension IDs are assigned according to a typical manner providing full scalability on various scalability dimensions.

- It supports explicit signalling of scalability dimension identifiers for each layer_id if required by some application.

# Proposal

We propose following syntax and semantics. The proposed changes are highlighted below compared to HEVC Draft 9 [1] and NAL unit header and parameter set designs for HEVC extensions - JCTVC-K1007 [2]. Two variant syntax (Variant A, Variant B) are proposed.

.

## Variant A

## Syntax and Semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| vps_implicit_dimension_ids_flag | u(1) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| if(vps_implicit_dimension_ids_flag) |  |
| max_dimension_id_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if(!vps_implicit_dimension_ids_flag) { |  |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency information signaling |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

vps_implicit_dimension_ids_flag with a value of 1 specifies that the explicit listing of dimension_id[i][j] values for various scalability dimensions is not signaled and the dimension_id[i][j] values are inferred. vps_implicit_dimension_ids_flag with a value of 0 specifies that the explicit listing of dimension_id[i][j] values for various scalability dimensions is signaled.

max_dimension_id_minus1[i] specifies the maximum value of the dimension_id for the scalability dimension i. The scalability dimensions are listed in the scalability_mask table.

## Decoding Process:

When dimension_id values are implicitly signaled they are inferred as DimId[i][j]  as described by the code below during the decoding process.

int Id[]={0,0,0,…,0};

for(i=0;i<=vps_max_layers_minus1;i++)

{

for(j=0;j<NumScalabilityTypes;j++)

{

DimId[i][j]= Id[j];

}

Id[NumScalabilityTypes-1]= Id[NumScalabilityTypes-1]+1;

for(k= NumScalabilityTypes-1;k>=0;k--)

{

if(Id[k]==(max_dimension_id_minus1[k]+1))

{

Id[k]=0;

Id[k-1]= Id[k-1]+1;

}

}

}

## Variant B

Variant B is similar to variant A except the syntax elements vps_nuh_layer_id_present_flag and layer_id_in_nuh[ i ] are also not signaled when implicitly signaling scalability dimension identifiers.

## Syntax:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| vps_implicit_dimension_ids_flag | u(1) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| if(vps_implicit_dimension_ids_flag) |  |
| max_dimension_id_minus1[ i ] | u(3) |
| } |  |
| if(!vps_implicit_dimension_ids_flag) { |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency information signaling |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

## Semantics and Decoding Process:

Semantics and decoding process for Variant B is same as that for variant A except that additionally the value of NUH layer id is inferred to be equal to the entry index i of the loop for(i=0;i<=vps_max_layers_minus1;i++).

# Conclusion

This document proposes syntax, semantics, and decoding process for implicit signaling of scalability dimension identifiers in video parameter set extension. Two syntax variants are proposed. We propose to adopt one of the proposed approaches to VPS extension design.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bros, W-J. Han, J-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 8,” JCTVC-K1003, Shanghai, December 2012.

[2] J. Boyce, Y.-K. Wang, “NAL Unit Header and Parameter Set Design for HEVC Extensions,” JCTVC-K1007, Shanghai, October 2012.