| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0211 |
| --- | --- |

| Title: | NonTE3/4: Combination of TE-B4 4.2.1 Adaptive Up-sampling Filter and TE3 4.6.2.1 Generalized Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Jianle Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 | Tel: Email: | +1-858-658-3923 lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Adaptive up-sampling filter and generalized residual prediction were studied in TE-B4 4.2.1 and TE-3 4.6.2.1, respectively. In this proposal, the combination of the two techniques is investigated. It is reported that 3.2% and 4.91% luma BD-rate reduction (EL+BL) is obtained on average for RA, and LD-P cases, respectively. It is asserted that the coding gain of two methods is additive.

# Introduction

Adaptive up-sampling filter (AUF) [1] and generalized residual prediction (GRP) [2] were investigated in SHVC TE4 and TE3, respectively. In this proposal, the combination of the two techniques is experimentally studied.

# Test results

The table below summarizes the performance of the combination of AUF and GRP with three weighting modes (0, 0.5, 1) and fast GRP mode selection method [2]. Full results are provided in the attached Excel sheets. On average, 3.2% and 4.91% luma BD-rate reduction (EL+BL) is obtained on average for RA, and LD-P cases, respectively.

Table 1 Test results of AUF + GRP (0, 0.5, 1) with fast GRP mode selection

As reference, the individual performance of AUF and GRP is listed below. It can be observed that, the gains by AUF and GRP are additive.

Table 2 Test results of AUF only [1]

Table 3 Test results of GRP(0, 0.5, 1) with fast GRP mode selection [2]

# Conclusions

In this proposal, the combination of adaptive up-sampling filter in TE4 and generalized residual prediction in TE3 is studied. The gain by the two method is additive. On average, 3.2% and 4.91% luma BD-rate reduction (EL+BL) is obtained on average for RA, and LD-P cases, respectively

# References

- J. Chen, X. Li, M. Karczewicz, “TE-B4: Results of test 4.2.1 on adaptive up-sampling filter”, doc. JCTVC-L309, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, 14–23 Jan. 2013

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, doc. JCTVC-L0078, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, 14–23 Jan. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).