| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0213 |
| --- | --- |

| Title: | Differential coding for RefIdx based scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou Jani Lainema Kemal Ugur Miska Hannuksela |  |  |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a way to enable differential coding in RefIdx based scalable coding. The approach is based on placing a differential reference picture to the enhancement layer DPB together with an upsampled base layer picture. In order to be able to represent the sample values of the differential reference pictures as positive integers an offset is added to the sample differences. When reconstructing predicted sample values the HEVC version 1 weighted prediction process is utilized to add the differential reference picture to the base layer prediction and to compensate for the offset in differential reference picture. The method is reported to improve the objective compression performance of the SHVC 0.1.1 RefIdx based operation by -1.8 %, -3.5 % and -2.8% for RA-2x, RA-1.5x and RA-SNR scalability, respectively (“EL only” results).

# Introduction

Differential coding has been reported to improve coding efficiency of scalable video codecs (e.g. in JCTVC-F290). This contribution proposes a way to enable a form of differential coding in RefIdx based scalability. In order to allow a differential reference pictures to be stored as unsigned integer, an offset is added to the differential sample values before storing those to the reference picture buffer. Motion compensation for the differential reference picture takes place as weighted prediction and is always utilizing bi-prediction with base layer reference picture as the second predictor. In this way, the differential sample values can be added to the base layer samples (instead of averaging as the non-weighted bi-prediction would do) and the offset can be subtracted from the result.

The implementation is fully conforming to the spirit of RefIdx based scalability and does not introduce any new coding tools. Reference picture list is constructed in a way an unmodified HEVC version 1 weighted prediction implementation can be used for all motion compensation purposes. Figure below illustrates the proposed design.

Figure 1: Illustration of the proposed method. Unmodified HEVC version 1 decoder takes care of the decoding. Additional sample level operations are limited to construction of differential reference pictures.

# Algorithm description

## Reference picture construction

In order to be able to perform differential coding, two differential reference pictures Di (i=POC) are generated and placed to the enhancement layer decoded picture buffer. One differential reference picture is generated for the first picture in the reference picture list 0 and one for the first picture in the reference picture list 1. Sample values of the differential reference pictures Di are based on the enhancement layer reference picture Ri and the corresponding upsampled base layer reference picture Bi as follows:

Di(x, y) = Clip ( Ri(x, y) – Bi(x, y) + 128 )

## Reference picture list construction

Reference picture lists are constructed considering that the reconstruction of differentially coded blocks takes place utilizing weighted bi-prediction. The two differential reference pictures are placed to the end of list 0 with a weight of two and an extra entry for the upsampled base layer picture also with a weight of two is placed to the end of reference picture list 1. Di frames are generated based on the first reference picture in reference picture list 0 and list 1 (in the case those are the same, only one is generated), and corresponding upsampled base layer reference pictures. All the new picture references are also associated with an offset of -128 in order to allow the weighted prediction process to compensate for the offset added to the differential reference picture samples. In order to limit the increase in the size of the reference picture list 0, the last two enhancement layer reference pictures are removed from the list when adding the differential reference pictures. However, at least one enhancement layer reference picture is always kept in the list. The following two examples illustrate the reference picture list construction when coding a picture with POC 4 and POC 16 in the case of GOP size 8:

| Ref pic list 0: | R0 | B4 | D0w2 | D8w2 |
| --- | --- | --- | --- | --- |
| Ref pic list 1: | R8 | R0 | B4 | B4w2 |

(a)

| Ref pic list 0: | R8 | R6 | B16 | D8w2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ref pic list 1: | R8 | R6 | R4 | R0 | B16 | B16w2 |

(b)

Figure 2: Example of reference picture list construction for picture with (a) POC 4 and (b) POC 16 in the case of GOP size 8. Green slots refer to the base layer references added by introduction of RefIdx based scalability. Blue slots refer to the references added by the proposed method.

## Weight parameters

For all enhancement layer pictures (Ri) and the base layer references (Bi), weight and offset values are set to 1.0 and 0, respectively. So we disable the weights by setting “bPresentFlag” to zero in the reference software for those reference frames. For new base layer reference (Biw=2) and differential reference frame, weight and offset are set to 2 and -120, respectively. So we enable weights by setting “bPresentFlag” to 1, and the values of “iWeight” and “iOffset” are set to 2 and -120, respectively. The value of “uiLog2WeightDenom” is set to 0 for all reference frames.

## Motion estimation

The normal SHVC 0.1.1 motion estimation is executed for the enhancement layer pictures (Ri) and the traditional base layer references (Bi). Motion vector of the candidate PUs with reference to Biw2 are set to (0, 0), and motion vectors for differential reference frames (Di) are set to the motion vectors found when evaluating motion for corresponding enhancement layer reference picture (Ri).

The new inter prediction case (the combination of base layer reference frame and differential reference frame) is checked after bi-prediction motion search in “predInterSearch()” function. If one of these modes provides the lowest R-D cost, that is selected as the best prediction mode.

## Motion compensation

Motion compensation follows strictly the HEVC version 1 weighted prediction process. Due to the weight and offset selection described above the differential frame is effectively added to the upsampled base layer samples.

# Objective results

The table below summarizes the objective coding performance of the proposed approach compared to SHVC 0.1.1 software with RefIdx framework turned on. The timing information for SNR scalability is not accurate as the results were obtained from a computing cluster with different kinds of nodes. Accurate timing for SNR scalability is to be provided as a revision to this contribution.

Table 1: Results with REF_IDX_ME_AROUND_ZEROMV on

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.2% | -2.4% | -2.5% |  |  |  | -1.1% | -2.8% | -3.1% |
| Class B | -0.6% | -2.4% | -2.7% | -1.0% | -2.8% | -3.0% | -1.2% | -3.0% | -3.2% |
| Overall (EL+BL) | -0.8% | -2.4% | -2.6% | -1.0% | -2.8% | -3.0% | -1.2% | -2.9% | -3.2% |
| Overall (EL) | -1.8% | -4.4% | -4.8% | -3.5% | -7.1% | -7.9% | -2.8% | -5.6% | -6.1% |
| Enc Time[%] | 124.1% |  |  | 124.1% |  |  | N/A |  |  |
| Dec Time[%] | 112.9% |  |  | 112.2% |  |  | N/A |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

The anchor for the simulation results is the SHVC 0.1.1 refIdx framework with motion estimation around zero. It means that the following macros have been activated in the reference software:

#define REF_IDX_FRAMEWORK           1 ///< inter-layer reference framework

#define REF_IDX_ME_AROUND_ZEROMV   1 ///< added ME around zero MV for inter-layer reference picture

The table below summarizes the objective coding performance of the proposed approach compared to SHVC 0.1.1 software with RefIdx framework turned on and motion estimation around zero motion vector is switched off. The timing information is not accurate as the results were obtained from a computing cluster with different kinds of nodes. That is the macros mentioned above have been set to:

#define REF_IDX_FRAMEWORK           1 ///< inter-layer reference framework

#define REF_IDX_ME_AROUND_ZEROMV   0 ///< added ME around zero MV for inter-layer reference picture

Table 2: Results with REF_IDX_ME_AROUND_ZEROMV off

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.2% | -2.4% | -2.6% |  |  |  | -1.1% | -2.7% | -3.1% |
| Class B | -0.6% | -2.4% | -2.7% | -1.0% | -2.8% | -3.0% | -1.2% | -3.0% | -3.2% |
| Overall (EL+BL) | -0.8% | -2.4% | -2.7% | -1.0% | -2.8% | -3.0% | -1.2% | -2.9% | -3.1% |
| Overall (EL) | -1.8% | -4.4% | -4.9% | -3.5% | -7.1% | -7.9% | -2.7% | -5.6% | -6.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  | N/A |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).