| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0220r1 |
| --- | --- |

| Title: | Non-TE5: Inter-layer Intra MPM derivation for EL in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

In this proposal for the intra coded enhancement layer CU, the MPM derivation is modified to use base layer intra prediction modes. Experimental results show that comparing to SMuC0.1.1, BD-rate numbers for combined BL+EL are -0.31% for Y, -0.12% for U, -0.10% for V in AI 2x and -0.13% for Y, 0.11% for U, 0.09% for V in AI 1.5x.

# Introduction

In SMuC0.1.1, Enhancement Layer (EL) reuses HEVC intra mode coding scheme. Due to the similarities between the content of EL and up-scaled Base Layer (BL), there is a large correlation between intra prediction modes of EL and BL. Several proposals in using such intra prediction mode correlation are studied in TE5 Section 5.1.[1].

With the macro SVC_BL_CAND_INTRA in SMuC0.1.1 enabled, co-located BL intra prediction mode is used for MPM derivation if co-located BL CU is intra coded. Otherwise, the MPM derivation stays the same as HEVC. The MPM derivation is as follows.

- If co-located BL intra prediction mode is different from left and above neighbours in EL, co-located BL intra prediction mode is set to be the first MPM followed by intra modes of left and above neighbours.

- Otherwise, co-located BL intra prediction mode is equal to either left or above neighbour of EL. HEVC MPM derivation is applied.

Since the new IntraBL mode is treated as non-intra coded in SMuC, when either left or above neighbour is IntraBL mode, it is tagged as “unavailable” and treated as DC mode in MPM derivation. There could be cases however, that co-located BL CUs for left and above neighbours, can be used for intra prediction mode derivation. Accordingly, this contribution proposes to use intra mode from co-located BL neighbours for MPM derivation.

# Proposed solution

In proposed solution, SVC_BL_CAND_INTRA is enabled. The change to MPM derivation about neighbouring modes is as follows.

- If the left or above neighbour is IntraBL mode and the co-located BL CU of left or above neighbour is intra coded, the intra mode of the co-located BL CU of left or above neighbour is used as left or above mode for MPM derivation.

- Otherwise, left or above mode is set to DC.

The proposed solution is demonstrated and compared to SMuC0.1.1 in Figure 1.

Figure 1 Proposed derivation of MPMs in EL

# Simulation results

The proposed solution is implemented in SMuC0.1.1 and tested using All Intra configurations suggested in TE5 [1]. The simulation results are summarized in Table 1. Running time is not available because simulations are run in a cluster. Detailed results are available in accompanying excel files.

Table 1 Summary of simulation results

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.23% | -0.04% | 0.06% |  |  |  |
| Class B | -0.34% | -0.16% | -0.17% | -0.13% | 0.11% | 0.09% |
| Overall (EL+BL) | -0.31% | -0.12% | -0.10% | -0.13% | 0.11% | 0.09% |
| Overall (EL) | -0.55% | -0.16% | -0.12% | -0.32% | 0.33% | 0.29% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| Enc Mem[%] |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Since the intra prediction mode of BL is used for EL, there is parsing dependency on BL. To avoid the dependency, MDCS is disabled for EL. The results with MDCS off are summarized in Table 2. Detailed results are available in accompanying excel files.

Table 2 Summary of simulation results with MDCS OFF

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.17% | 0.09% | 0.16% |  |  |  |
| Class B | -0.18% | 0.05% | 0.13% | -0.12% | 0.13% | 0.14% |
| Overall (EL+BL) | -0.18% | 0.06% | 0.14% | -0.12% | 0.13% | 0.14% |
| Overall (EL) | -0.33% | 0.13% | 0.26% | -0.30% | 0.37% | 0.40% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
| Enc Mem[%] |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

In this contribution, MPM derivation using BL intra prediction mode is proposed and BD-rate saving is reported by simulation. It is recommended to further investigate the proposed solution in TE or AHG.

# Reference

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim and H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, October 2012.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).