| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0221r2 |
| --- | --- |

| Title: | Non-TE5: Memory reduction for MV data of EL in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com |
|  | Kazushi Sato, Shuo Lu Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com Shuo.lu@jp.sony.com |
| Source: | Sony Electronics Inc. Sony Corp. |  |  |

_____________________________

# Abstract

This contribution proposes to reduce memory for MV data of EL by removing temporal MV in MV predictor and merge candidate list in EL of SHVC. Potential performance drop can be compensated by using inter-layer MV prediction algorithms. The proposed solution is tested in combination with inter-layer MV prediction proposed in JCTVC-K0037. Simulation results indicate  that compared to SMuC0.1.1 anchor, BD-rate numbers for BL+EL combination  in Luma merge mode are -1.67% for RA 2x, -1.99% for RA 1.5x, -2.12% for RA SNR and -0.58% for LP 2x, -0.67% for LP 1.5x, -1.60% for LP SNR. For AMVP, BD-rate numbers for BL+EL combination in Luma merge mode are -0.84% for RA 2x, -0.97% for RA 1.5x, -1.09% for RA SNR and -0.51% for LP 2x, -0.49% for LP 1.5x, -0.79% for LP SNR. For both merge and AMVP, BD-rate numbers for BL+EL combination in Luma merge mode are -1.92% for RA 2x, -2.20% for RA 1.5x, -2.32% for RA SNR and -0.86% for LP 2x, -0.91% for LP 1.5x, -1.78% for LP SNR.

# Introduction

In SHVC, Base layer (BL) motion vector (MV) can be used to predict MV of enhancement layer (EL). Several inter-layer MV prediction algorithms are proposed in the 11th JCTVC meeting and tested in TE5 Section 5.2[1]. The inter-layer MV prediction algorithm from JCTVC-K0037 [2] and SMuC0.1.1 example hook are demonstrated in Figure 1.  In JCTVC-K0037, the MV compression process is performed after encoding and decoding of EL. The proposed advantage of this approach is that EL can access a more accurate MV data from BL. The improved BD-rate is confirmed by the results of TE5.

Figure 1 Inter-layer MV prediction algorithms of ETRI[2] and SMuC’s example

# Proposed solution

## Idea

As shown in TE5 report, the inter-layer MV prediction can improve the coding performance. Since EL applies the same MV prediction algorithm as HEVC, temporal MV of EL is one of the candidates for merge list and MV predictor list. Although temporal MV is compressed to save storage, temporal MV size is still large given that EL has large resolution.  To reduce the storage of MV data for EL and achieve a better trade-off between memory usage and coding efficiency, it is proposed to remove temporal MV from merge candidate list and MV predictor list for EL and add instead the BL MV to the list. The proposed solution is demonstrated in Figure 2.

Since a BL MV is added but temporal MV is removed, the total length of merge list and AMVP list stay the same as HEVC. So no additional pruning is needed for the proposed solution.

Figure 2 Proposed inter-layer MV prediction algorithm

## WD changes

### WD change A

The temporal MV can be removed totally from EL by using the existing SPS level flag. The corresponding WD text changes are marked as highlighted.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| …. |  |
| If (layer_id == 0) // base layer only |  |
| sps_temporal_mvp_enable_flag | u(1) |
| strong_intra_smoothing_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| …. |  |

sps_temporal_mvp_enable_flag equal to 1 specifies that slice_temporal_mvp_enable_flag is present in the slice headers of pictures with IdrPicFlag equal to 0 in the coded video sequence. sps_temporal_mvp_enable_flag equal to 0 specifies that slice_temporal_mvp_enable_flag is not present in slice headers and that temporal motion vector predictors is not used in the coded video sequence. When sps_temporal_mvp_enable_flag is not present, sps_temporal_mvp_enable_flag is set to 0.

### WD change B

A new slice level syntax can be added to signal the usage of BL MV for inter-layer MV prediction. The corresponding WD text changes are marked as highlighted.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| .... |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| ..... |  |
| If(layer_id>0) |  |
| bl_mv_enable_flag | u(1) |
| if( sps_temporal_mvp_enable_flag && !bl_mv_enable_flag) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| } |  |

bl_mv_enable_flag equal to 1 specifies that MV from co-located BL can be used to merge mode candidate list and MV prediction list.  bl_mv_enable_flag equal to 0 specifies that no inter layer MV prediction is applied.

# Simulation results

The proposed solution is implemented in the software of TE5 5.2.3[1]. The implementation disables the temporal MV for EL in both merge list and AMVP list in software directly. Note that the implementation does not include WD change A and B. So simulation results do not exactly reflect the WD change A or B. It’s believed that the WD changes will not affect the PSNR but the bit-rate slightly.  Simulations are conducted using configurations suggested in TE5 [1]. Running time is not available because simulations are run in a cluster.

The simulation results for merge only are summarized in Table 1. Detailed results are available in accompanying excel files.

Table 1 Summary of results for merge only

|  | RA HEVC 2x |  |  |  |  |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  |  | Y |  | U |  | V | Y | U | V |
| Class A | -2.20% |  | -5.19% |  | -4.94% |  |  |  |  |  |  |  | -2.54% | -6.47% | -6.73% |
| Class B | -1.46% |  | -3.48% |  | -3.58% |  |  | -1.99% |  | -3.72% |  | -4.03% | -1.95% | -4.02% | -4.26% |
| Overall (EL+BL) | -1.67% |  | -3.97% |  | -3.97% |  |  | -1.99% |  | -3.72% |  | -4.03% | -2.12% | -4.72% | -4.97% |
| Overall (EL) | -3.36% |  | -7.36% |  | -7.42% |  |  | -5.46% |  | -9.32% |  | -10.04% | -4.40% | -9.11% | -9.58% |
| Enc Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Enc Mem[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| BL Match | Matched |  |  |  |  |  |  | Matched |  |  |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  |  |  |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |
|  | Y |  | U |  | V |  |  | Y |  | U |  | V | Y | U | V |
| Class A | -1.13% |  | -2.79% |  | -2.55% |  |  |  |  |  |  |  | -2.29% | -4.06% | -4.00% |
| Class B | -0.36% |  | -1.86% |  | -1.90% |  |  | -0.67% |  | -1.89% |  | -2.05% | -1.33% | -2.33% | -2.51% |
| Overall (EL+BL) | -0.58% |  | -2.12% |  | -2.08% |  |  | -0.67% |  | -1.89% |  | -2.05% | -1.60% | -2.82% | -2.93% |
| Overall (EL) | -1.22% |  | -3.77% |  | -3.67% |  |  | -1.69% |  | -4.07% |  | -4.35% | -2.93% | -4.95% | -5.12% |
| Enc Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Enc Mem[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| BL Match | Matched |  |  |  |  |  |  | Matched |  |  |  |  | Matched |  |  |

The simulation results for AMVP only are summarized in Table 2. Detailed results are available in accompanying excel files.

Table 2 Summary of results for AMVP only

|  | RA HEVC 2x |  |  |  |  |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  |  | Y |  | U |  | V | Y | U | V |
| Class A | -1.20% |  | -2.33% |  | -2.27% |  |  |  |  |  |  |  | -1.32% | -2.76% | -2.98% |
| Class B | -0.69% |  | -1.62% |  | -1.79% |  |  | -0.97% |  | -1.53% |  | -1.82% | -0.99% | -1.97% | -2.21% |
| Overall (EL+BL) | -0.84% |  | -1.82% |  | -1.93% |  |  | -0.97% |  | -1.53% |  | -1.82% | -1.09% | -2.20% | -2.43% |
| Overall (EL) | -1.61% |  | -3.23% |  | -3.47% |  |  | -2.57% |  | -3.53% |  | -4.26% | -2.17% | -3.94% | -4.41% |
| Enc Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Enc Mem[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| BL Match | Matched |  |  |  |  |  |  | Matched |  |  |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  |  |  |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |
|  | Y |  | U |  | V |  |  | Y |  | U |  | V | Y | U | V |
| Class A | -1.05% |  | -1.81% |  | -1.60% |  |  |  |  |  |  |  | -1.23% | -2.09% | -1.97% |
| Class B | -0.30% |  | -1.18% |  | -1.30% |  |  | -0.49% |  | -1.07% |  | -1.22% | -0.61% | -1.25% | -1.27% |
| Overall (EL+BL) | -0.51% |  | -1.36% |  | -1.38% |  |  | -0.49% |  | -1.07% |  | -1.22% | -0.79% | -1.49% | -1.47% |
| Overall (EL) | -0.94% |  | -2.27% |  | -2.30% |  |  | -1.17% |  | -2.18% |  | -2.39% | -1.36% | -2.42% | -2.40% |
| Enc Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Enc Mem[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| BL Match | Matched |  |  |  |  |  |  | Not matched |  |  |  |  | Not matched |  |  |

The simulation results for merge and AMVP only summarized in Table 3. Detailed results are available in accompanying excel files.

Table 3 Summary of results for merge and AMVP

|  | RA HEVC 2x |  |  |  |  |  |  | RA HEVC 1.5x |  |  |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  |  | Y |  | U |  | V | Y | U | V |
| Class A | -2.47% |  | -5.52% |  | -5.28% |  |  |  |  |  |  |  | -2.76% | -6.67% | -6.93% |
| Class B | -1.70% |  | -3.76% |  | -3.89% |  |  | -2.20% |  | -3.89% |  | -4.23% | -2.15% | -4.24% | -4.54% |
| Overall (EL+BL) | -1.92% |  | -4.26% |  | -4.29% |  |  | -2.20% |  | -3.89% |  | -4.23% | -2.32% | -4.93% | -5.23% |
| Overall (EL) | -3.79% |  | -7.82% |  | -7.91% |  |  | -5.93% |  | -9.65% |  | -10.44% | -4.76% | -9.41% | -9.95% |
| Enc Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Enc Mem[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| BL Match | Matched |  |  |  |  |  |  | Matched |  |  |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  |  |  |  |  | LD-P HEVC 1.5x |  |  |  |  | LD-P HEVC SNR |  |  |
|  | Y |  | U |  | V |  |  | Y |  | U |  | V | Y | U | V |
| Class A | -1.48% |  | -3.18% |  | -3.04% |  |  |  |  |  |  |  | -2.48% | -4.27% | -4.26% |
| Class B | -0.62% |  | -2.21% |  | -2.29% |  |  | -0.91% |  | -2.09% |  | -2.29% | -1.50% | -2.58% | -2.72% |
| Overall (EL+BL) | -0.86% |  | -2.48% |  | -2.51% |  |  | -0.91% |  | -2.09% |  | -2.29% | -1.78% | -3.06% | -3.16% |
| Overall (EL) | -1.66% |  | -4.29% |  | -4.29% |  |  | -2.18% |  | -4.44% |  | -4.79% | -3.19% | -5.27% | -5.44% |
| Enc Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| Enc Mem[%] | #NUM! |  |  |  |  |  |  | #NUM! |  |  |  |  | #NUM! |  |  |
| BL Match | Matched |  |  |  |  |  |  | Matched |  |  |  |  | Matched |  |  |

# Conclusion

In this contribution, solution for inter-layer MV prediction with reduced memory is proposed. Simulation results show coding efficiency improvement without additional temporal MV storage in EL. It is recommended to investigate the proposed solution under CE or AHG.

# Reference

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim and H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, October 2012.

- J. W. Kang, H. Lee, J. Lee, J. S. Choi, J. W. Kim, J. Nam, H. Choi, D. Sim, “Description of scalable video coding technology proposal by ETRI and Kwangwoon Univ.”, JCTVC-K1105, Shanghai, October 2012.

# Patent rights declaration(s)

Sony Electronics Inc. and Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).