| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0222 |
| --- | --- |

| Title: | TE3 : Results of Test 4.2.2 on Intra prediction based on differential picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Jianle Chen Xiang Li Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | krapaka@qti.qualcomm.com cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports SHVC TE3 results of Test 4.2.2 on Difference domain Intra Prediction (DIP). DIP is a predictive coding tool which uses reconstructed pixels from spatial neighbors of current enhancement layer and co-located base layer to predict the current enhancement layer block. It is reported that an average of (EL Only) 2%, (EL+BL) 0.9%, luma BD-rate reduction is obtained for AI Spatial scalability cases.

# Technical description

This test studies the Difference domain Intra Prediction (DIP) proposed in [1][2] to improve the coding efficiency of SHVC. In the DIP, as shown in Figure 1, the difference between the pixels of current neighbors and those of collocated BL neighbors are used to generate a difference prediction based on the intra prediction mode. The generated difference prediction signal is added to the collocated BL block signal to form the final prediction.

The intra prediction method for difference signal remains unchanged with respect to HEVC except for the planar mode. After intra prediction is performed to get the difference prediction signal, the bottom-right portion of the difference prediction for the planar mode is set to zero (that is each pixel(x, y) satisfying the condition (x + y) >= N-1, where N is the width of the current block is set to zero). Typically farther the current pixel from its reference pixels, higher is the prediction error. To reduce this error, in this method the farther pixels do not predict from reference pixels but only from collocated Base layer pixels.

Due to the high frequency nature of the difference signals, HEVC mode dependent intra smoothing (MDIS) process is disabled in difference domain intra prediction mode. As a result of no MDIS filtering, the latency and complexity are reduced with gain in BD-Rate.

The difference domain Prediction mode is indicated by a flag intra_resi_pred_flag at CU level.

Figure 1 Intra residual prediction

# Test Results

In this section, the difference domain intra prediction is experimentally verified under SHVC common test conditions defined by AHG10. Thanks to Samsung and UGENT for crosschecking the tests.

Table 1:  Experimental results of Difference Domain Intra Prediction over TE3 Anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.7% | -0.2% | -0.4% |  |  |  |
| Class B | -1.2% | -0.3% | -0.5% | -0.8% | 0.2% | 0.0% |
| Overall (EL+BL) | -1.1% | -0.3% | -0.5% | -0.8% | 0.2% | 0.0% |
| Overall (EL) | -1.8% | -0.5% | -0.8% | -2.1% | 0.6% | 0.1% |
| Enc Time[%] | 160.4% |  |  | 151.3% |  |  |
| Dec Time[%] | 108.9% |  |  | 106.7% |  |  |
| Enc Mem[%] | 103.3% |  |  | 102.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusions

In this proposal, the performance of Difference domain Intra Prediction for SHVC is investigated. By utilizing base layer picture information in predicting enhancement layer picture for Intra Coded blocks, significant improvement in coding efficiency is achieved. It is suggested to adopt Difference domain Prediction into SHVC and reference software.

# References

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 1) JCTVC-K0035”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 2) JCTVC-K0036”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).