| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0223 |
| --- | --- |

| Title: | Frame packing with 2D compatible output |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Rickard Sjöberg | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution discusses and presents proposed changes for frame packing arrangement. It is asserted in the document that the current scheme for frame packing arrangement has drawbacks regarding error robustness as well as drawbacks regarding detection of whether a sequence is frame packed or not. It is stated in the contribution that it would be an advantage to define how one view should be extracted and output from a frame packed sequence in order to ensure compliance and interoperability.

What is proposed in the contribution is:

- add frame_packing_seq_flag in SPS to indicate whether the sequence is a frame packed sequence or not

- FPA SEI shall be present if frame_packing_seq_flag is equal to 1, FPA SEI shall not be present if frame_packing_seq_flag is equal to 0

- replace frame_packing_arrangement_cancel_flag with frame_packing_arrangement_reserved_zero_1bit

- add sps_frame_packing_arrangement_type in SPS

- if sps_frame_packing_arrangement_type = 5 send frame_packing_temporal_cut_level_idc that indicates which temporal layers belong to which view

- if and only if sps_frame_packing_arrangement_type = 3, 4 or 7 send default_display_window

- perform cropping based on parameters from the SPS that is active when a picture is decoded instead of parameters from the SPS that is active when the picture is output

- change cropping process to depend on external means selecting if conformance cropping window (stereo) should be used or if default display window (2D) should be used

# Introduction

Frame packing arrangement SEI is used to indicate to a decoder that the pictures that are output from the decoder contains samples of multiple distinct spatially (or temporally) packed constituent frames.

A typical use case for the frame packing arrangement SEI message is to transmit a stereo sequence where the two frames representing the left view and the right view hereafter called frame compatible stereo service.

A constituent frame is herein referred to as a “view”. A view does not have to be the left or the right view of a stereo sequence but could represent something else not defined in the HEVC specification.

An important feature of the frame compatible stereo service is that when the decoder is used with a stereo display, two views are displayed, but when the decoder is used with a 2D display only one view should be displayed. Currently it is not defined in HEVC what a decoder

## Frame packing signaling

A frame packed sequence generally does not change frame packing arrangement type within the sequence and does not change from frame packed to regular video (i.e. stereo to 2D) within a coded video sequence. Such changes are performed at random access points.

When the frame packing SEI was added to AVC/H.264 it was not possible to put frame packing indication information in the SPS since the standard was already released and SPS did not have an extension mechanism.

Having a frame packing sequence flag in the SPS would improve robustness and consistency and overcome problems that exist in AVC/H.264. For example, the DVB 3DTV specification [1] states that “In order to maximize the robustness of the decoding process in the IRD, it is recommended that the frame compatible plano-stereoscopic 3DTV service applies the frame packing arrangement SEI message also during carriage of the HDTV format, at least for a period of two seconds before and after a format switch between the HDTV video format and the frame compatible plano-stereoscopic 3DTV video format.”

SEI messages can be lost (or removed) without affecting the decoding process. A service that depends on the presence or absence of a specific SEI message is very vulnerable to packet losses. If the frame packing indication information is contained in the SPS it is much easier to ensure that the frame packing information is received for example through using different SPS IDs for 2D sequences and stereo sequences.

## Stereo and 2D compatibility

If a decoder that decodes a stereo sequence is connected to a stereo display the decoder should of course output both views. If a decoder that decodes a stereo sequence is connected to a 2D display the desired behavior is that the decoder outputs only one view. However, when the frame packing SEI was added to AVC/H.264 it was not possible to change the output behavior to support these two modes of operation since there were already a large number of legacy clients present that did not support these two modes of operation (and that were not aware of frame packing arrangements). For HEVC there are no legacy clients. Currently there is no definition of how to extract one view from a frame packed sequence which may lead to inconsistency across the market since there are no means of validating that the output is correct i.e. there is no conformance test for single view output.

In AVC/H.264 the following four combinations of decoders and displays could occur:

Frame packing aware decoder and stereo display. Works fine (stereo video displayed).

Frame packing aware decoder and 2D display. Works fine (2D video displayed) only if there is an interface in which the decoder can indicate that only parts of the output pictures are to be displayed or if the encoder performs additional cropping and delivers non-conforming output.

Frame packing unaware decoder and stereo display. Works fine (stereo video displayed) only if the display is able to deduce that the pictures that are output from the decoder are representing stereo video.

Frame packing unaware decoder and 2D display. Does not work (both views will be displayed side by side).

For HEVC there are no frame packing unaware decoders since the FPA SEI message exists in the first version of the standard. Currently there are only two cases:

Frame packing aware decoder and stereo display. Works fine (stereo video displayed).

Frame packing aware decoder and 2D display. Works fine (2D video displayed) only if there is an interface in which the decoder can indicate that only parts of the output pictures are to be displayed or if the encoder performs additional cropping and delivers non-conforming output.

What we would like to achieve is the following two cases:

Frame packing aware decoder and stereo display. Works fine (stereo video displayed).

Frame packing aware decoder and 2D display. Works fine (2D video output from the decoder and displayed).

The only thing that is required is that the decoder is aware of whether it is used with a stereo display or a 2D display. Such awareness can be achieved with addition of a variable that is set by external means.

The current syntax in HEVC with the default display window does not specify how the information provided with the default display window should be used. Different systems may use it in different ways. For example, it is not clear what a decoder should do with default display window if frame packing arrangement type is not 3, 4 or 7 or if there are no FPA SEI messages. For frame packing arrangement type 5 (temporal interleaving) it is not possible to use default display window as a guidance to retrieve one view. Currently there is no information in the SPS that indicates that one view can be extracted (through discarding every second picture) when frame packing arrangement type 5 is used.

To avoid confusion in the market and to achieve the goal of creating standardized, interoperable means of operation across different systems and different applications it is suggested to add selectable stereo and 2D output in the HM decoder and in the HEVC specification.

# Example use case with frame packing arrangement type 5

The frame packing arrangement SEI message contains the syntax elements frame0_self_contained_flag and frame1_self_contained_flag that indicates that no inter prediction operations within the decoding process for the samples of one view refer to samples of the other view. However, it is not possible to discard the information that belongs to one view since there is no guarantee that the resulting bitstream is a conforming bitstream.

However, it is possible to reuse the temporal layering for frame packing arrangement type 5 separating the views by using different temporal Ids. The bitstream extraction process defined for temporal layers ensures that the resulting bitstream will be a conforming bitstream.

A simple example is shown in figure 1.

Figure 1. Simple example of using temporal id to separate the views.

In order to support trick play operations it is beneficial to let each view consist of several temporal layers but with a clear cut of level that separates the views. Figure 2 shows an example where the two lowest layers belong to the left view and the other layers belong to the right view. We propose to add a syntax element to SPS frame_packing_temporal_cut_level_idc to represent this cut off level and that the syntax element is signalled when frame packing arrangement type is equal to 5 as described in the next section of this document.

Figure 2. Example where two temporal layers are used for the left view.

When using the frame_packing_temporal_cut_level it is possible to extract only those temporal layers that represent one view and provide the bitstream to a system that only supports half the frame rate as illustrated in Figure 3. It is also possible for a decoder that is connected to a 2D display to reduce the resources used for decoding since only every second picture has to be decoded.

Figure 3. A sub-bitstream containing one view is created through the sub-bitstream extraction process and usage of frame_packing_temporal_cut_level.

# Proposal

To address the robustness problems described in the background section in regards to signalling of frame packing we propose the following:

- add frame_packing_seq_flag in SPS to indicate whether the sequence is a frame packed sequence or not

- FPA SEI shall be present if frame_packing_seq_flag is equal to 1, FPA SEI shall not be present if frame_packing_seq_flag is equal to 0

- replace frame_packing_arrangement_cancel_flag with frame_packing_arrangement_reserved_zero_1bit

To address the problems with ambiguity regarding usage of the default display window and to provide functionality for extracting a sub-bitstream when frame packing arrangement type 5 is used we propose the following:

- add sps_frame_packing_arrangement_type in SPS

- if sps_frame_packing_arrangement_type = 5 send frame_packing_temporal_cut_level_idc that indicates which temporal layers belong to which view

- if and only if sps_frame_packing_arrangement_type = 3, 4 or 7 send default_display_window

In order to make it well defined what a decoder should output depending on whether it is used in a system with two views (stereo) or one view (2D) we propose the following:

- perform cropping based on parameters from the SPS that is active when a picture is decoded instead of parameters from the SPS that is active when the picture is output

- change cropping process to depend on external means selecting if conformance cropping window (stereo) should be used or if default display window (2D) should be used

# Syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| frame_packing_seq_flag | u(1) |
| if ( frame_packing_seq_flag ) { |  |
| sps_frame_packing_arrangement_type | ue(v) |
| if ( sps_frame_packing_arrangement_type = = 5 ) |  |
| frame_packing_temporal_cut_level_idc | u(3) |
| if ( sps_frame_packing_arrangement_type = = 3 ¦ ¦ sps_frame_packing_arrangement_type = = 4 ¦ ¦ sps_frame_packing_arrangement_type = = 7 ) { |  |
| default_display_window_flag | u(1) |
| if ( default_display_window_flag ) { |  |
| def_disp_win_left_offset | ue(v) |
| def_disp_win_right_offset | ue(v) |
| def_disp_win_top_offset | ue(v) |
| def_disp_win_bottom_offset | ue(v) |
| } |  |
| } |  |
| … |  |

The proposed changes to semantics, cropping and frame packing arrangement SEI are attached in JCTVC-K1003_v13_L0233.doc.

# References

[1] 		DVB A154 Frame Compatible Plano-Stereoscopic 3DTV, February 2011,
		(www.dvb.org/technology/standards/a154_DVB-3DTV_Spec.pdf)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).