| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0224 |
| --- | --- |

| Title: | TE5 : Results of Test 5.1.2 on Inter-Layer Intra Mode Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | krapaka@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the results of TE-5.1.2 on inter-layer intra mode prediction. In the tested method, the intra mode of the co-located unit at base layer is included as a most probable intra mode candidate for the enhancement layer. Further, in order to avoid parsing dependency on base layer, Mode Dependent Coefficient Scanning (MDCS) is disabled and diagonal scanning pattern is always used for all modes. It is reported that an average of 0.2% luma BD-rate reduction (EL+ BL) is obtained for AI Spatial scalability cases where MDCS is enabled. When MDCS is disabled an average of 0.1% luma BD-rate reduction (EL+ BL) is obtained for AI Spatial scalability cases.

# Technical description

In this proposal, the intra prediction mode of the co-located BL block is used to refine the final Most Probable Intra Mode (MPM) list. The intra prediction mode of the co-located BL block, iColBaseDir, is added to the final MPM list, candModeList[ x ] x=0..2, as shown below:

if iColBaseDir is not one of the candModeList[ x ] x=0..2 and iColBaseDir is not equal to Intra_DC then,

candModeList[0] = iColBaseDir
                                candModeList[1] = candModeList[0]
                                candModeList[2] = candModeList[1]

The total number of candidates in the MPM list is limited to three as that in HEVC.  Also, The MPM list generation at the enhancement layer remain the same as that in HEVC thereby keeping the modifications to minimal.

# Test Results

The proposed method is experimentally verified under SHVC common test conditions defined by AHG10 and the results are summarized in the following tables. Thanks to MediaTek for crosschecking the tests.

Table 1:  Experimental results of inter layer intra mode prediction (MDCS on)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% |  |  |  |
| Class B | -0.3% | -0.2% | -0.2% | -0.1% | 0.1% | 0.1% |
| Overall (EL+BL) | -0.3% | -0.1% | -0.1% | -0.1% | 0.1% | 0.1% |
| Overall (EL) | -0.4% | -0.2% | -0.1% | -0.2% | 0.2% | 0.2% |
| Enc Time[%] | 101.2% |  |  | 100.8% |  |  |
| Dec Time[%] | 101.7% |  |  | 100.7% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2:  Experimental results of inter layer intra mode prediction (MDCS off)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.2% |  |  |  |
| Class B | -0.1% | 0.0% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall (EL+BL) | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |
| Overall (EL) | -0.2% | 0.1% | 0.2% | -0.2% | 0.2% | 0.3% |
| Enc Time[%] | 100.3% |  |  | 100.3% |  |  |
| Dec Time[%] | 100.5% |  |  | 98.8% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# References

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 1) JCTVC-K0035”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 2) JCTVC-K0036”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).