| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0225 WG11 Number: m27562 |
| --- | --- |

| Title: | Video Sequence Characteristics Signaling in VPS and VPS Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sony Electronics Inc., Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes new syntax elements in vps_extension for the support of signaling mixed video types in the coded video sequences.

In version r1 of this document two of the new syntax elements are also proposed to be included in the video parameter set for HEVC version1.

# Mixed Sequences Signaling support in VPS Extension

## Motivation for Mixed Sequences Signaling

VPS [2] contains metadata to describe the overall characteristics of coded video, enables standards extension compatibility in terms of systems layer signaling so that a legacy decoder can ignore additional information about the extension bitstream structure. The current VPS syntax structure and its extension contain information for multi-layered video sequences, where a layer could be a base layer (for HEVC version-1 and its extensions), or could be a scalable enhancement layer (spatial or quality) for SHVC, or a view layer (texture view or depth view) for multi-view coding of HEVC extension (MHVC). All such layers could have their sub-layers also (temporal). Each video layer could be dependent upon its neighboring lower layers or it could be independent with no inter-layer predictions, etc.

The motivation behind "mixed sequences signaling support" is to develop new layer-specific properties in the VPS extension syntax structure, as suggested in "JCTVC-K_Notes_dD" in [4]. The layered syntax arrangement for independent layers in VPS can be suitably exploited to support a variety of mixed contents (source-types, coding types, etc.) in the high level syntax structure for HEVC extensions.

Some possible application examples which contain mixed sequences in broadcasting or network transmission environment for a VPS with 2-layers are:

- Frame structured video in layer-1 and Field structured video in layer-2: Each video layer has its respective SPS, etc. Legacy mono-view (2-D) decoders can use either of these 2 layers for its base-layer application.

- Frame-structured video (2D, mono-view) in layer-1 and Frame Packing Arrangement (FPA) video (3D, stereo-view) in layer-2: Legacy mono-view (2-D) decoder uses the layer-1 video, while an advanced stereo-view (3D) Frame Packing Arrangement (FPA) video decoder can use layer-2 for 3D stereo-view video application.

It is highly anticipated that such combination of mixed video types may be present in the video bitstreams to support two such decoders (legacy and advanced one). As an example, an ATSC work-plan on Hybrid Delivery Real-time Broadcast and Broadband / Mobile channels is considering a shared 2D and 3D video by using hybrid variations of legacy 2D mono-view / left-view video and 3D (stereo)  right-view video.

Many such combinations of coded video contents with varied source-types and coding types may co-exist in the current and future applications where either legacy or advanced decoders will sort out the respective bitstream for decoding.

The inherent layer-based descriptions of VPS can be extended to support such applications, at least for the Systems level solutions (Ad-insertions, Splicing, etc.).

Thus support is needed in VPS extension to signal information about mixed source characteristics.

## Mixed Sequences Signaling Proposal for HEVC Extension

In the following we describe syntax and semantics for signaling mixed sequences information in VPS extension.

### Syntax

Four new syntax elements are introduced in VPS extension – mixed_video_present_flag, source_scan_type_info_idc , source_2d_3d_info_idc, source_mixed_codec_flag. These new syntaxes signal what types of source video combination present in a particular VPS Layer.

The updated syntax structure of vps_extension based on JCTVC-K1007 [1], is shown below. The new syntax elements are highlighted.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| mixed_video_present_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| if( mixed_video_present_flag ) { |  |
| source_scan_type_info_idc[i] | u(2) |
| source_2d_3d_info_idc[i] | u(2) |
| source_mixed_codec_flag[i] | u(1) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

### Semantics

mixed_video_present_flag syntax parameter equals 1indicates presence of mixed video information syntax elements - source_scan_type_info_idc, source_2d_3d_info_idc, source_mixed_codec_flag, in VPS layers. If mixed_video_present_flag equals 0, no such mixed video syntax elements are present in the layers.

source_scan_type_info_idc[i] equal to 0 indicates that the layer i has pictures whose scan type should be interpreted as interlaced for all pictures. source_scan_type_info_idc[i] equal to 1 indicates that the layer i has pictures whose scan type should be interpreted as progressive for all pictures. source_scan_type_info_idc[i] equal to 2 indicates that the layer i has pictures whose scan type is unknown. source_scan_type_info_idc[i] equal to 3 indicates that the layer i has pictures which are a mixture of scan type progressive and scan type interlaced.

Tables below shows the proposed indicator information:

| source_scan_type_info_idc[i] | Video Layer Characteristics |
| --- | --- |
| 0 | All pictures in layer i have interlaced scan type |
| 1 | All pictures in layer i have progressive scan type |
| 2 | Pictures in layer i have unknown scan type |
| 3 | Pictures in layer i have a mixture of interlaced and progressive scan type |

source_2d_3d_info_idc[i] equal to 1 indicates that all pictures for layer i are packed in a frame compatible 3D format. source_2d_3d_info_idc[i] equal to 0 indicates that all pictures for layer i are 2D frames i.e. none of them are packed in a frame compatible 3D format. source_2d_3d_info_idc[i] equal to 2 indicates that the layer i has pictures whose frame packing arrangement is unknown. source_2d_3d_info_idc[i] equal to 3 indicates that the layer i has pictures which are a mixture of frame compatible 3D and 2D frames.

Tables below shows the proposed indicator information:

| source_2d_3d_info_idc[i] | Video Layer Characteristics |
| --- | --- |
| 0 | All pictures in layer i are 2D frames |
| 1 | All pictures in layer i are packed in frame compatible 3D format |
| 2 | Pictures in layer i have unknown frame packing arrangement |
| 3 | Pictures in layer i have a mixture of frame compatible 3D and 2D frames |

source_mixed_codec_flag[i] syntax parameter equals 1 indicates presence of AVC encoded video in a VPS layer i. If 0, the default HEVC encoded video is present in a VPS layer i. This syntax parameter is identical to avc_base_codec_flag of base layer.

Note: In the current VPS, "avc_base_codec_flag" is restricted to 1-bit. This is why the "source_mixed_codec_flag[i]" is set to 1 bit. However, for future applications, this could be extended to 2-bits perhaps.

| source_mixed_codec_flag / avc_base_codec_flag | Mixed video coding-type |
| --- | --- |
| 0 | HEVC |
| 1 | AVC |

In addition to VPS extension the syntax elements source_scan_type_info_idc, source_2d_3d_info_idc and source_mixed_codec_flag could also be included in SPS of HEVC extension.

# Video Sequence Characteristics Signaling Proposal for HEVC Version1

In version r1 of this document we propose to include the source_scan_type_info_idc and source_2d_3d_info_idc syntax elements also for the HEVC version1, in the video parameter set.

The proposed syntax is shown below.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_max_num_layers_minus1 //reserved_zero_6bits in the base spec | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| next_essential_info_byte_offset //vps_reserved_zero_16bits in the base spec | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| source_scan_type_info_idc | u(2) |
| source_2d_3d_info_idc | u(2) |
| vps_num_hrd_parameters | ue(v) |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| … |  |
| bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantics of the syntax elements source_scan_type_info_idc and source_2d_3d_info_idc syntax elements are same as those of the syntax elements source_scan_type_info_idc[i] and source_2d_3d_info_idc[i] syntax elements except they apply to the base layer.

In addition to VPS the syntax elements source_scan_type_info_idc and source_2d_3d_info_idc could also be included in SPS for HEVC version1.

# Conclusion

In this contribution, the vps_extension() syntax structures is updated with new syntaxes for the HEVC Extensions in scalable video coding, multi-view coding and 3D video coding areas to support mixed video sequences in various layers of the VPS.

This contribution extends the layer-based properties of VPS to address some possible emerging applications for better communication and system integration purposes in both System layers and also the overall decoding path.

In version r1 we also propose to include the source_scan_type_info_idc and source_2d_3d_info_idc syntax elements for the HEVC version1, in the video parameter set.

We propose adoption of the new syntax elements in VPS and VPS extension.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Boyce and Y-K. Wang, “NAL unit header and parameter set designs for HEVC extensions”, JCTVC-K1007, Shanghai, China, October 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 9,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K1003_v13, Shanghai, China, October 2012.

- M. Haque, K. Sato and A. Tabatabai, "On support of mixed video sequences in High Level Syntaxes", JCTVC-K0232, Shanghai, China, October 2012.

- G. Sullivan and J. Ohm, " Meeting report of the 11th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Shanghai, CN, 10–19 Oct 2012", JCTVC-K_Notes_dD.