| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0226 WG11 Number: m27563 |
| --- | --- |

| Title: | VPS and vps_extension updates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution presents some updates on VPS and syntax cleanups for vps_extension syntax structures.

- HEVC VPS Syntax Structure

The VPS syntax structure in the current DIS specification [2] for HEVC Version-1 is shown below. It is listed in this contribution as a reference to show some changes needed for new HEVC extensions beyond Version-1 for SHVC, MHVC and 3DV.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bit_rate_pic_rate_info( 0, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| vps_num_op_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_op_sets_minus1; i++ ) |  |
| operation_point_set( i ) |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The yellow-highlighted area above is updated in next section to support various HEVC extensions.

## VPS Syntax structure modification

The VPS syntax structure is modified as shown below in yellow-highlighted areas.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_max_num_layers_minus1 //vps_reserved_zero_6bits in version-1 | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| next_essential_info_byte_offset //vps_reserved_0xffff_16bits in version-1 | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bit_rate_pic_rate_info( 0, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| vps_num_op_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_op_sets_minus1; i++ ) |  |
| operation_point_set( i ) |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag // bit_equal_to_one in JCTVC-K1007_v1 | u(1) |
| if( vps_extension_flag ) { |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| vps_extension( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

This update is made according to the BoG meeting report [1] from the last meeting. It also includes some additional cleanups while adding the vps_extension() syntax structure.

- The vps_reserved_zero_6bits in version-1 is changed as vps_max_num_layers_minus1 in [1]. However in vps_extsnion() syntax strucre in [1] this parameter is used as  vps_max_layers_minus1. This is also consistent with a similar parameter vps_max_sub_layers_minus1 in versuion-1. So we should use vps_max_layers_minus1 in VPS.

- For HEVC extensions, "vps_extension()" syntax structure as defined in [1] is inserted in VPS under the condition of vps_extension_flag (yellow high-lighted areas near the end of VPS).

- The vps_reserved_0xffff_16bits in version-1 is replaced by next_essential_info_byte_offset, as mentioned  in [1]. This new syntax parameter can be used to locate the essential information available in "vps_extension()" syntax structure.

- The syntax parameter next_essential_info_byte_offset also includes the byte-alignment related syntaxes near top of the "vps_extension()" syntax structure in [1].  For the sake of cleanliness, these byte-alignment syntaxes are taken out and placed inside the VPS under the condition of vps_extsnion_flag, just before the "vps_syntax()" structure.

- It also be noted that the "rbsp_trailing_bits" also contain a number of byte alignment bits for the VPS_rbsp() and such bit-counts can be determined as follows:

nByteAlignmentBits = ( (vps_extension_length + 7) / 8) * 8 - vps_extension_length,

where  vps_extesnion_length is the total number of bits in vps_extension() and these can be computed easily as vps_extension() syntax structure so far contains Fixed-length unsigned integer bits.

## VPS Extension Syntax structure and its cleanup

The syntax structure of vps_extension() from the BoG report [1] in the last meeting, is shown below.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| num_dimensions_minus1[ i ] | u(4) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) { |  |
| dimension_type[ i ][ j ] | u(4) |
| dimension_id[ i ][ j ] | u(v8) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

While reviewing the above syntax structure and semantics description, a number of issues are discussed below as cleanups or for editing purposes.

For the scalability_mask  synatx parameter, the following parameter table is listed, while the corresponding entries show the bit-location for the register-type data of  "scalability_mask". But it is not described well.

| scalability_mask | Scalability dimension |
| --- | --- |
| 0 | none (base HEVC) |
| 1 | spatial |
| 2 | quality |
| 3 | depth |
| 4 | multiview |
| 5 | unspecified |
| 6-15 | Reserved |

In place of the above table for scalability_mask, the following register-type description may be better for a clear understanding:

| 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| unspe-cified | multi-view | depth | quality | spatial | none |

Bit-positions

Scalability dimension

| 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| reserved |  |  |  |  |  |  |  |  |  |

Also the highlighted parts of the syntax structure of vps_extension() shown below, indicate some necessary cleanups.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) |  |
| dimension_id_len_minus1[ i ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 10; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j < num_dimensions_minus1NumScalabilityTypes; j++ ) { |  |
| v = dimension_id_len_minus1[j] + 1 |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1 ; i++ ) |  |
| profile_tier_level(1, vps_max_sub_layers_minus1 ) |  |
| for( i = 10; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

- The byte-alignments related syntaxes are removed, as indicated in the previous section for VPS update.

- The undefined parameter "num_dimensions_minus1" is replaced by "NumScalabilityTypes".

- There are some questions about why all the "for" loops start from "i=1" in vps_extension(). For the "profile_tier_level(.,.)",  it is present for once in the VPS for the Base Layer, and so for" loops start from "i=1"  is valid here. But for the other two cases, this may not be true, as "Base Layer = Layer=0" may not be excluded for "dimension_id" or “layer dependency” case. So the "for" loop should start from "i=0". As an example, for "Layer specific info" with the "for( i = 1; i <= vps_max_layers_minus1; i++ ) {  ...... }" loop, if we have a coded video sequence containing Stereo-view with depth-view (2+1=3 Layers), there are 3 Layers. So vps_max_layers_minus1 = 3-1 = 2. If we follow the current "for" loop,

i=1 => Layer-1 = depth-view;

i=2 (vps_max_layers_minus1) => Layer-2 = View-1

Where is  i=3 => Layer-3 = View-2?

As a correction, this loop should be "for( i = 0; i <= vps_max_layers_minus1; i++ ) {  ...... }" for two cases, except for the "profile_tie_level(.,.)", as shown in the syntax structure above for vps_extension().

- The semantics description for the syntax parameter “dimension_id[i][j]" is also not available with respect to “Scalability dimension” and “dimension_id_len_minus1[j]”. An example table is shown below to show the relationship among these parameters.

| Scalability dimension | dimension_id | dimension_id_len (example) |
| --- | --- | --- |
| none (base HEVC) | - | - |
| spatial | dependency_id | 4 |
| quality | quality_id | 4 |
| depth | depth_flag | 1 |
| multiview | view_idx | 4 |
| unspecified | - | - |
| Reserved | - | - |

# Conclusion

In this contribution, the VPS and vps_extension() syntax structures are updated with some cleanups in the later one for the HEVC Extensions in scalable video coding, multi-view coding and 3D video coding areas.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Boyce and Y-K. Wang, “NAL unit header and parameter set designs for HEVC extensions”, JCTVC-K1007, Shanghai, China, October 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 9,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K1003_v13, Shanghai, China, October 2012.

- M. Haque, K. Sato and A. Tabatabai, "On support of mixed video sequences in High Level Syntaxes", JCTVC-K0232, Shanghai, China, October 2012.